This is some short test code that I put together to drive the

Soundblaster's DAC in both DMA and direct mode. There are two programs:

DACDMA.EXE                      Outputs a raw 8 bit sample file to the DAC in

DMA mode.

DACDIR.EXE                      Outputs a raw 8 bit sample file to the DAC in

direct mode.

Unfortunately DACDIR has seemed to have stopped working for some reason.

If someone can tell me what has gone wrong, please send me some mail.

The usage of bot programs is quite simple. DACDMA can take up to two

arguments. The first is the name of the sample file. The second is an

optional sample rate to play the sample back. The sample rate will

default to 11KHz. DACDIR only takes one arguement, the name of the raw

sample to play.

The IO address and IRQ for the Soundblaster is set in the file SBP.ASM.

It is fairly important that these are set correctly. I have it set up

for port 0x220 and IRQ 5 (I sort of found a conflict with IRQ 7 under

OS/2 2.0 8-(). Note also, that the samples are only played in mono on

the SB Pro. I haven't yet worked out how to use it in stereo.

I have written in support for the BLASTER environment variable. There is

a function that will read and parse this variable to determine what the

settings for the Soundblaster are. If the variable isn't set, the

function will return a 1 if the BLASTER variable is not set, and 0 if it

is ok. If it returns a 1, the arguments are set to the default

Sounblaster parameters.

As a bonus, I have included a program that will read the Soundblaster

Pro's mixer chip and report what the current volume settings are. I have

a little more documentation for it, but I haven't got around to writing

it up properly yet. It is just some random scribblings.

I have now added a short program that makes use of the CT-VOICE.DRV

driver to play .VOC files. It isn't very well commented but it should

give you an idea of how to use the driver.

Any comments or suggestions (or especially more programming information)

can be directed to cejjb@marlin.jcu.edu.au.

Jeff Bird         12 Feb 92