README for ANKLUNG

ANKLUNG is an interactive digital composing machine. ANKLUNG.ZIP

contains the following files:

ANKLUNG.EXE	VBRUN200.DLL	ANKLUNG.WRI	THE_MUSE.WRI

THE_MUSE.TIF	README.WRI	DANCE.ANK	PRAYER.ANK

STRAUSS.ANK	TV_SCENE.ANK	EXAMPLES.WRI

This package represents work intermittently in progress. ANKLUNG is

based on a description and functional diagram in Hal Chamberlin's

book: The Musical Applications of Microprocessors.  THE_MUSE.TIF is a

redrawing of Chamberlin's diagram and THE_MUSE.WRI is a transcript of

the relevant passage.

ANKLUNG.EXE is the executable produced by Visual Basic 2. It needs

VBRUN200.DLL to run. ANKLUNG.WRI is a rudimentary description of how

to use the application. However, using ANKLUNG is a fairly intuitive

process with immediate feedback.

Files with the *.ANK extension are saved configurations I include as

examples. They reflect my particular musical interests which happen

not to be related to popular dance. ANKLUNG is capable of helping to

generate ideas in virtually every musical genre. EXAMPLES.WRI contains

brief descriptions of the *.ANK files and, more importantly, details

of the instrumentation they need to work properly.

Paul@kriv.demon.co.uk --- Paul Kriwaczek --- 24.3.1994

------------------------------------------------------------------------

ANKLUNG

programmed by Paul Kriwaczek

This version of ANKLUNG has five "Players", all driven by the same

Main Program Clock, allowing for up to five-note polyphony.

Each Player can generate a tune: a monophonic pattern of notes,

repeated over a shorter or longer period, and output through any

installed Windows MIDI driver.

Getting Started

The simplest way to start is to think of each Player as divided into

four regions: top left, top right, lower left, lower right.

Stage 1: Set one or more of the righthand scrollbars in the top right

region. This will generate a simple pattern.

Stage 2: Set one or more of the lefthand scrollbars in the top left

region. Now set one or more of the righthand scrollbars in the lower

right region. This will generate a more complex pattern.

Stage 3: Keeping at least one of the lefthand scrollbars in the top

left region, move one or more into the lower left region. This will

create the most complex patterns of all.

Stage 4: Press the Beat button and uncheck some of the

checkboxes. This will make the same pattern of notes more irregular in

time. Select a Tempo, an octave and a mode.

THE MAIN CONTROLS

Each Player consists of a Counter and a Shift Register, to which are

connected a Note Selector and a Parity Generator. (See Chamberlin's

original description in THE_MUSE.WRI. And THE_MUSE.TIF for visual

reference).

The Counter has 9 outputs including always OFF and always ON; each is

labelled with the number of the divider connected to that output. Thus

C2 divides the Main Program Clock by 2, C4 divides it by four, C8 by

8, C16 by 16, C3 by 3, C6 by 6.

The Shift Register has sixteen outputs labelled S1 to S16.

The Parity Generator is controlled by the four scrollbars labelled

VARIATION. When set to the Counter outputs, the sum of all four will

be either even or odd. If the sum is even, the Parity generator will

feed an input into the Shift Register. Setting one or more of the

VARIATION scrollbars to the Shift Register outputs can make the

resulting pattern which shifts through the register arbitrarily

complex and difficult to predict. A visual display mode in the

interface makes it possible to observe the changes.

The Note Selector is labelled PATTERN. The note is generated by

connecting the four PATTERN scrollbars to the Counter and Shift

Register outputs. The scrollbars are the input to a four-bit register,

able to convert them into different 16 notes.

THE BUTTONS

TEMPO

Adjusts the Main Program Clock. A beat is ANKLUNG'S unit of

time. There are no subdivisions of a beat. Thus fast short notes

require a high Tempo.

Volume

Allows for selecting the MIDI Velocity of the player's notes.

Mute

Mutes the Player.

Slip

Selects a number of beats by which to slide the output of the Player

backwards or forwards relative to the other Players. When slipped, the

contents of all counters and shift registers are first reset to zero.

Reset

Resets the Player to its initial condition.

PAUSE

Toggles the entire ANKLUNG to stop and start. When paused, Counters

and Shift Registers retain their contents, ready for restarting.

Channel

Selects the MIDI Channel through which to output the Player's

notes. The Player may also be set to send its output to another

Player, offsetting the second Player's note by the amount of the first

Player's note.

Rhythm

Allows for selected clock ticks in a repetitive pattern to be missed

by the Player. Uncheck the checkboxes representing the beats to be

missed. The pattern may be made of groups of 3, of 4, of 5, of 7, or

of 11. The marks above the checkboxes are to help with counting the

groups.

Pitch

Adjusts the output of the Note Selector up or down by an selectable

number of semitones.

Octave

The Player begins with its base note C = 440Hz. This is the Alto

voice. Soprano is up one octave, Tenor down two and Bass octave is

down by three.

Mode

The Note Generator outputs a series of numbers. These are converted

to whichever scale is set in Mode. Major (Ionian), Dorian (with two

flats), Phrygian (four flats), Lydian (one sharp), Mixolydian (one

flat), Aeolian (three flats), Locrian (five flats) and Melodic Minor

are available.

Voice

Selects a synthesizer voice for the Player. A MIDI Change Program

message is sent at the time of changing Voice and also whenever a

configuration is loaded from file.

THE OPTIONS MENU

Collect in MIDI File

When the Start button is pushed, the collecting of notes into the

file begins. Thus a particular moment in the sequence may be selected

as the beginning. When the Stop button is pushed, the user is asked to

give a name to the resulting MIDI file. The ANKLUNG may be paused

while collecting a MIDI file, allowing the configuration to be

changed. When the machine is restarted, MIDI collection continues

after a seamless join.

Visual Display

Presents the user with a visual display of the state of the Counter

and of the Shift Register. This is one of the most helpful

features. The Visual Display can select which Player's setting to

show.

Print Configuration

Produces a printout of all the settings of each player on the

currently selected Windows default printer.

OTHER MENUS

The File and Windows menus contain the usual facilities.