Juno-106 Patch Librarian for Windows

------------------------------------

This is a Windows 3.1 patch editor and librarian for the Roland

Juno-106 synthesiser.

System requirements: Windows 3.1, an installed and working MIDI

interface, a Juno-106 hooked up for MIDI input and output.

To Install the Juno-106 Librarian

1.  Copy the files from the distribution archive into any directory on

your PC.

2.  If you don't already have it, get a copy of VBRUN300.DLL and put it

in your Windows system directory.  It's available for Internet ftp from

ftp.cica.indiana.edu:/pub/pc/win3/util/vbrun300.exe.

2.  Run JUNOLIBR.EXE using Program Manager or File Manager.

3.  Select MIDI... from the Options menu and choose the correct input

and output devices and MIDI channel for the Juno

4.  Make sure the Function switch on the back of the Juno (next to the

MIDI in socket) is set to position III

5.  Select bank A, patch 11 on the Juno using the front panel switches

on the synth

6.  If everything is working the description for patch A11 on the screen

should change to "Undescribed patch", and the sliders and switches on

the screen display should change to match the synth. If this doesn't

happen, see the Troubleshooting section in the help file JUNOLIBR.HLP.

If you have any problems please contact davec@wsti.demon.co.uk.

Thanks to everyone who sent email with comments and bug reports, I

appreciate it!

Change history

--------------

Version 2.0:

Export to Cakewalk 2 was broken in the released version,

now fixed.

Added Export to Cakewalk 3.

Fixed an "index out of range" error that could occur when you pressed a

high-numbered patch button after loading a partially-empty patch file.