# JAWS for Windows 95 & NT Version 3.00 Setup Guide

## About this Setup Guide

This guide provides information about installing JFW (JAWS for Windows) version 3.00, ensuring basic settings, registering your software, and moving Authorization Keys. JAWS (Job Access With Speech) is a registered trademark of Henter-Joyce, Inc. Information about using JAWS and Windows is contained in our manuals, training tapes, quick reference cards, and the on-line help system.

It’s important to note that before you open the envelope containing the JAWS software diskettes, you should read the Henter-Joyce, Inc. (Henter-Joyce) Software License Agreement. It is enclosed in the JAWS software package and in the JAWS Basics manual. When you open the envelope containing the JAWS software diskettes, you automatically accept all terms and conditions of the license agreement.

As stated in the Henter-Joyce Software License Agreement, you are licensed to use JAWS under certain conditions and prohibited from giving copies of JAWS software to others to use on their computers. Our license agreement allows you to use your JAWS program on more than one computer as long as you are the only user of the software. For example, you could set up one copy of JAWS on a desktop PC at work and a second on your traveling laptop or on a PC at home. Purchases of Site Licenses are permitted to set up additional copies of JAWS and to use them simultaneously.

## Documentation Conventions

This document was written in Word 97 and contains hyperlinks to help you maneuver from topic to topic with “Go to” statements. Any time you encounter a line of text beginning with the words “Go to” you can press ENTER anywhere between the first or last character of the line. Your cursor automatically goes to the title of that section. This hyperlinked document can be used in any Internet browser, Word 95, or Word 97. A text version of this document is also available without the hyperlinks, with the same file name and a .TXT extension.

In order to make reading this guide as simple as possible we've included some conventions for you to be aware of while following instructions.

- JAWS and JFW are used synonymously to reference the product JAWS For Windows.

- When instructing you to type text into a command line or input field, brackets like this will surround the actual text: [a:\setup]. The setup command is within the set of brackets.

- A section heading title and number references the sections of the document. For example, Section 1 Installing JAWS for Windows.

Go to the Table of Contents to choose another topic or continue reading.

## Table of Contents

Select any of the following options to go to that specific heading within this guide. You may also find other "go to" references within those topics that open other related documents.

Go to About this Setup Guide

Go to Documentation Conventions

Go to Section 1 Installing JAWS for Windows

Go to Section 2 Authorization Key

Go to Section 3 Starting JAWS

Go to Section 4 Loading JAWS

Go to Section 5 Program Changes

Go to Section 6 Making Backup Copies of JFW Diskettes

Go to Section 7 Troubleshooting

Go to Section 8 Registering JFW

Go to Section 9 Technical Support

## Section 1 Installing JAWS for Windows

This section covers both standard and custom installations for JFW v3.00. When selecting the standard installation you will be able to select one synthesizer and one Braille device. The program is installed in the default directory of C:\JFW30. For more control over your installation process you can choose the custom installation. You will be able to select up to three synthesizers. You will also be able to install the program on a specified drive and directory and select a specific language version, if appropriate

NOTE: When installing a new version of JFW, it will overwrite/replace existing JFW files if you install the new version into the old version's directory. To make sure that the older version is still available after installing the new version, use the Windows Explorer to rename the old JFW folder before installing the new version. For example, you can rename or copy your current JFW folder (directory) to JFW-OLD.

Section 1.1 Setup Instructions

The instructions for running the setup program are below. When the disks are being copied, you will be notified to change diskettes by a message on the screen that may be accompanied by a beep or tone from the computer. This audible tone is dependent on your particular computer configuration and system setup.

- Insert Disk 1 in Drive A.

- Choose Run from the Start menu.

The Run dialog box appears.

- Type [A:\Setup] and press ENTER.

- Press ENTER at the welcoming message.

- A series of messages appear that may be strictly for information like the welcoming message or may require you to make a choice, like choosing a synthesizer or Braille display. Before installing these files, you will be given a chance to confirm your choices. Follow the instructions on the screen, pressing ENTER each time you insert the next diskette.

- After the setup program completes, press ENTER to exit and then be sure to reboot your computer for the setup choices to take affect.

Go to the Table of Contents to choose another topic or continue reading.

## Section 2 Authorization Key Information

An Authorization Key is simply a file with computer information originally located on your Authorization diskette. When JFW starts, this key should either be present on your hard drive in the JFW directory or on the Authorization disk in the diskette drive. The Authorization Key is crucial to JAWS working properly. If an Authorization Key is not present, JAWS will stop talking shortly after it is loaded. Usually, the Authorization Key automatically loads onto your computer during the installation process from the Authorization diskette.

Purchasers of a Single User License of the JFW software receive three Authorization Keys. Those who purchase an evaluation package receive one Authorization Key. Refer to the Authorization Key Guide for more information.

Go to the Table of Contents to choose another topic or continue reading.

## Section 3 Starting JAWS

Begin by removing any diskette that may remain in the diskette drive and reboot your PC. If you have not installed your speech synthesizer, do so now.

NOTE: If your speech synthesizer uses a software device driver, the software must be installed before you can use JAWS. Follow the installation instructions included with your synthesizer to ensure proper hardware and software installation.

## Section 4 Loading JAWS

JAWS can be loaded by your computer system automatically or you can load it manually from the Run dialog or the Start menu.

## Section 4.1 Automatic Loading

## When installing JAWS, the JAWS icon is added to your Start menu under Programs. There are two ways to automatically load JAWS. First, you can select the Run JAWS First option on the JAWS File menu to have JAWS begin speaking before you sign on to Windows. Windows NT users may find this option unsatisfactory and may get better results using the option described next. Second, you can copy the icon to your Startup menu and JAWS will start speaking after you sign on to Windows.

NOTE: If you decide you no longer want JAWS to start automatically, you can delete the JAWS icon from the Startup menu and unselect the Run JAWS First option.

## Section 4.2 Manual Loading

## When installing JAWS, the JAWS icon is added to your Start menu under Programs. There are two ways to manually load JAWS. First, all you need to do is open the Start menu, open the Programs submenu, then select JFW30 to open the JFW submenu. Choose the JAWS icon and the program launches.

The second way to load JAWS is from the Run dialog. Open the Start menu, select Run, and then type [C:\JFW30\JFW.EXE] and press ENTER.

Go to the Table of Contents to choose another topic or continue reading.

## Section 5 Program Changes

Henter-Joyce continues to develop enhancements for JFW, and as a result, there may be new information that does not appear in our printed documentation. You can find the most current information in the CHANGES.TXT file, located JFW30 subdirectory on your hard disk. It contains the most important notes on these enhancements. Another place to find information on program changes is the What's New option on the JAWS Help menu. It describes all of the program enhancements in an organized manner in one location.

Go to the Table of Contents to choose another topic or continue reading.

## Section 6 Making Backup Copies of JFW

Use the DOS XCOPY command to make backup copies of your JFW diskettes. IMPORTANT: You cannot make copies of your Authorization diskette.

Assuming you are using the A drive, the DOS command would be: XCOPY A: A: /S.

NOTE: Substitute the appropriate drive letter for drives other than A.

For best results, use newly formatted empty diskettes to make copies.

Go to the Table of Contents to choose another topic or continue reading.

## Section 7 Troubleshooting

If, after starting JAWS, you immediately hear an error message that tells you an Authorization Key is not present, follow the procedure for moving the Authorization Key from the diskette to the hard drive in the Authorization Key Guide.

If Windows does not load after rebooting your PC, refer to your Microsoft Windows setup manual or contact Microsoft Technical Support for assistance.

If Windows loads but JAWS does not speak, follow these suggestions.

- If you have a new speech synthesizer, you should verify the tightness of the synthesizer connections including external speaker or headphone connections, and be sure the volume level is set to a mid-range loudness setting. If your synthesizer installation instructions require the installation of a synthesizer driver, then be sure it has been installed properly. If you installed an external synthesizer, be sure you connected to the port you indicated during the setup process and that its power supply is connected and the synthesizer is turned on.

- If your speech synthesizer had been working with a DOS screen reader and does not work with JAWS, begin your troubleshooting by looking at your Windows Taskbar. Press CONTROL+ESC to open the Start menu, press ESC, and then TAB to activate the Taskbar. You should find JAWS by using the arrow key to move through the list of running applications. If JAWS is not in the list, then it has not been loaded. Follow the steps to load JAWS in the Starting JAWS procedure.

- If you load JAWS and it appears in the Windows Taskbar, and it does not speak, you may have to run the JAWS Installation again. If you did not choose the correct synthesizer or port when doing the first installation, JAWS is unable to communicate with the synthesizer.

- Call Henter-Joyce Technical Support if JAWS still does not speak after trying these suggestions.

FYI: We provide documentation on some speech synthesizers, though the documentation that came with your synthesizer should be your primary source of information. You may find some documentation for your synthesizer in the \JFW30\TECNOTES directory.

If JAWS does not read the text on the screen, or if you receive a General Protection Fault (GPF) "blue screen" after attempting to load JAWS, or other similar symptoms occur, refer to Series 1 Video Cards and JAWS of the Compatibility Series. You can find this document at the Henter-Joyce web site, www.hj.com.

Refer to the Troubleshooting JAWS guide when it is available.

Go to the Table of Contents to choose another topic or continue reading.

## Section 8 Registering JFW

Once you’ve completed the installation and proper setup of JAWS, you are ready to register JFW. When registering your copy of the software, you confirm your eligibility to receive technical support from Henter-Joyce. Please register today!

After loading JAWS, place the JAWS Registration diskette in your floppy disk drive. Open the Start menu and select the Programs option to open the JAWS submenu. Choose the Register option and follow the registration process by filling in the registration form.

Your answers to the Registration questions are copied to the JAWS Registration diskette. Please send the diskette to Henter-Joyce, Inc. in the self-addressed postage-paid diskette mailer included in the JFW software package. The diskette mailer is made of cardboard and usually has the name Henter-Joyce, Inc. imprinted in Braille.

Go to the Table of Contents to choose another topic or continue reading.

## Section 9 Technical Support

The Henter-Joyce training tapes, manuals, quick reference cards, and on-line help address the needs of most computer users with a range of experience. If you are a new computer user, then you should plan on using the training tapes to learn about JAWS and Windows.

You can find even more assistance outside of the training tapes. Henter-Joyce provides information and help in a variety of formats and places.

- A Quick Reference Help system is available to assist you with learning JAWS.

- The Program Changes help system provides information about program changes.

- Information about using the help system is provided on the training tapes and in the Help tips section in each of the help systems. The information provided makes using help quick and easy. To start the JAWS on-line help system, press INSERT+J to activate the JAWS application window and press ALT+H to display the Help menu. From there you can choose any of the sub menu options such as JFW Help Topics, Quick Reference, What's New, Customizing JFW, or Technical Support.

- The manuals are available in various formats. The JAWS Basic manual is provided in print and is also available in Braille. All of the manuals are in the JAWS on-line help system and provided in text files. The text files for North American English are in the \JFW30\MANUALS\ENU directory. Text files for other languages are in other user language subdirectories of \JFW30\MANUALS directory.

- A large amount of technical information for those who want to set up JAWS to respond to the greatest challenges of Windows is available in the appropriate user language subdirectory in \JFW30\TECNOTES, and in the on-line help system. Refer to the Customizing JAWS manual for more information.

- More information about authorization issues can be found in the Authorization Key Guide.

- You can also refer to the Troubleshooting JAWS guide when it is available.

- Our Technical Support staff provides assistance for the setup of JAWS and other Henter-Joyce, Inc. products. We cannot provide free support for those products we do not produce or sell. On-going Technical Support is only available for those users who purchase and register JFW. When calling Technical Support, please have your JFW serial number available and be ready to take notes. To display your JAWS serial number, choose the About option from the Help menu. You should also find the serial number in print and Braille format on the cover of your user manual and on the Authorization diskette. You can also e-mail your technical support questions to JAWS@hj.com.

- Henter-Joyce has an Internet home page and FTP site for additional sources of technical support and information. You can use your computer and modem to download software programs, technical bulletins, and JAWS program updates from these sources.

- For those who are interested, you can subscribe to the JFW List Server. Henter-Joyce does not maintain this List Server. See the Henter-Joyce home page at www.hj.com for more details.

This concludes the JFW Setup Guide.

Go to the Henter-Joyce, Inc. address and phone number section.

Go to the top of this guide.

Go to the Table of Contents to choose another topic or continue reading.

Henter-Joyce, Inc.

11800 31st Court North, St. Petersburg, FL  33716

Phone: (800) 336-5658 or (813) 803-8000

Fax: (813) 803-8001

E-Mail Address: jaws@hj.com

FTP Site for Downloading Files: ftp://ftp.hj.com