DECtalk 5.x

Product Description

Fonix DECtalk provides the ideal TTS solution for embedded applications. Fonix DECtalk small footprint and extensive OS and CPU support allows developers to add speech output to many devices with small CPU capacity or minimal memory.

Version 5.1 includes three new languages: Mandarin Chinese, Japanese and Korean.  It also incorporates improvements in German, French, Italian, two Spanish languages (European and Latin American), US English and overall prosody.

Fonix Engineering Product Requirements Document

Current Revision Date: December 13, 2005

Version History

| Revision Chart |  |  |  |
| --- | --- | --- | --- |
| Version | Author(s) | Description | Date Completed |
| 1.0 | R. Brian Moncur | Original |  |

# Product or Project Introduction

Technology continues to advance at an ever increasing pace. Everyday, new companies surface that specialize in applying these new technologies to solve ‘real world’ problems and assist others in integrating these advances into new products and applications.

## Features

### Version 5.1

Languages

- Asian Languages – Chinese, Japanese, Korean

- European Languages – French, German, Castilian Spanish, Italian, UK English

- America’s Languages – US English, Latin American Spanish

Improvements

- All improvements made in DECtalk 4.6.4

- New nasal architecture

- Intonation will be separated by language

- Bug Fixes from 4.64 and 5.0

Maintenance

- Nand file support will be a part of the product branch

Customers need to be able to configure their memory usage – what flexibility can we give them in designing with NOR and NAND flash?

- Separate vocal tract parameters so that SLPs can tune and model sounds and duration

Demos

- The Speak demo will be the primary application for development, testing and customer trials.  The Speak demo will support both the Standard API and the Simple API.

- Demos will be available for other Platforms as resources become available.

### Version 5.2

Regression Test Tools for

- Normalized Text

- Phoneme output

- Audio output

Update PHEdit

- Output in text so that comparisons can be made with previous versions.

Cleanup code.

- Remove #ifdefs where possible.

- Document code with consistent headers and comments.

- Separate additional controls so that SLPs can tune prosody

- Remove dead code

Change DecTalk to support loadable voices.

- Voice changes are made by loading a voice parameter file.

- Create a Voice Control Panel to “Build your own voice”

Build a UI tool to allow others to edit text normalization rules

Improved Parsing

- Breath Groups

- Grammatical parsing for prosody

Audio

- Add 16, 22 & 44 KHZ output options to HC DECtalk, Switch selectable on low Compute

Improvements

- Female voice improvements

## Technology Requirements

### Supported Platforms

| Hardware | Win32 | WinCE/PPC/PE/SM | Linux 2.2/2.4/2.65 | Solaris 8+ | MAC OS 10+ (OS X) | Palm OS 5.x** | QNX 6.3 | Symbian-Nokia Series | No OS** Native |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analog Devices |  |  |  |  |  |  |  |  |  |  |
| Blackfin 533/535 |  |  |  |  |  |  |  |  | X |  |
| Apple |  |  |  |  |  |  |  |  |  |  |
| MAC |  |  |  |  | X |  |  |  |  |  |
| ARM |  |  |  |  |  |  |  |  |  |  |
| ARM 7 |  | X |  |  |  |  |  |  | * |  |
| ARM 9 |  | X | X |  |  | * |  | X | * |  |
| Epson |  |  |  |  |  |  |  |  |  |  |
| S1C33 Family, GNU33 |  |  |  |  |  |  |  |  | X |  |
| Freescale (Motorola) |  |  |  |  |  |  |  |  |  |  |
| i.MXL |  | * | * |  |  | * |  | * | * |  |
| PowerPC 5100/5200 |  | * | * |  |  |  | * |  |  |  |
| Intel |  |  |  |  |  |  |  |  |  |  |
| SA-1110 |  | X | X |  |  | * |  | X | * |  |
| XScale |  | X | X |  |  | * |  | X | * |  |
| X86 | X | * | X | X |  |  | * |  |  |  |
| MIPS |  |  |  |  |  |  |  |  |  |  |
| R4xxx |  | * |  |  |  |  |  |  |  |  |
| NeoMagic |  |  |  |  |  |  |  |  |  |  |
| MiMagic 3 |  | X |  |  |  |  |  |  | * |  |
| MiMagic 5 |  | X | * |  |  | * |  | X | * |  |
| Renesas (Hitachi) |  |  |  |  |  |  |  |  |  |  |
| SH3 | * | * |  |  |  |  |  |  | X |  |
| SH4 |  | * | * |  |  |  | * |  |  |  |
| Samsung |  |  |  |  |  |  |  |  |  |  |
| S3C ARM Family |  | X | X |  |  | * |  | X | * |  |
| Texas Instruments |  |  |  |  |  |  |  |  |  |  |
| OMAP 710 / 720 |  | X |  |  |  |  |  |  | * |  |
| OMAP 5910 |  | X | X |  |  | * |  | X | * |  |
| Sun |  |  |  |  |  |  |  |  |  |  |
| Sparc |  |  |  | X |  |  |  |  |  |  |

* - Built by request from sales only

## Required Performance

### Speed

Low Compute – Process speech with less than 1 second delay for a 10 word sentence running on an SH3 at 20 MHz

High Compute – Process speech with less than 1 second delay for a 10 word sentence running on XXXX

### Memory

All languages:

RAM – 	Less than 128 KB (program memory space)

ROM – 	Less than 1 MB for executables for all languages (NOR ROM space)

Less than 8 MB for data for all languages (NAND ROM space)

## Testing Requirements

Fonix Dectalk 5.1 will be tested using a variety of methods that will be included in the overall test plan.  These methods will include but are not limited to the following.

Test applications:
Previous versions of DECtalk were verified by using a test application “TtsTest.exe” to verify that the program functions in a default expectation.  Another test application “DTErrCheck.exe” has been created and will be used to check the error states of DECtalk.  This application passes in invalid parameters to the DECtalk SDK and monitors the error returns.

Samples and Demos:
In addition to the Test applications the SDK will be tested by using the samples and demos to insure their functionality as well.  A section of the test plan will be developed to ensure test coverage for each sample and demo.

Baseline and Listening:
Testing of each language and voice for TTS generation errors will be done by comparing output of a set list of sentences against a baseline (the previous version of DECtalk 4.6.4) and listening to any differences that are found.  It will then be determined if the product has an improvement (if so the baseline will be reset) or a defect that needs fixing.  This will enable us to process larger amounts of TTS and only dedicate time to areas that are changing.  Areas where there is no previous baseline available (new languages) sentences can be used without a baseline and will report an error until one is provided.  This will allow the SPL's to track progress of new languages as well as previously shipped languages.

Installation:
Final application testing will occur using the install application.  A section will be included in the test plan for verifying the install to ensure the application is ready to be releases to the public.

## Documentation Requirements

## Quality Requirements

Fonix DECtalk 5.x uses both letter-to-sound rules and a dictionary to determine the pronunciation of all input text. All common words for a specific language are pronounced according to letter-to-sound rules. Fonix DECtalk 5.x letter-to-sound rules are based on a language specific pronunciation standard. Words that are exceptions to these rules are maintained in a separate language specific dictionary for more accurate pronunciation.

Fonix assures that pronunciations for most common words of each language will be correct. This does not include foreign words or specialty words such as medical terms, terms related to specific occupations, etc. Specific words can be added to the dictionary if they are common words and as space allows.

| Language | Pronunciation Standard |
| --- | --- |
| Castilian Spanish | Castilian Dialect Standard is TBD |
| French | Parisian French based on the “Oxford Hachette French Dictionary” |
| German | High Northern German based on the “Duden Dictionary” |
| Italian | Florence Dialect, which is considered the broadcast standard |
| US English | American Western English (US Broadcasting standard) |
| Japanese | Tokyo Dialect. Standard may be the NHK Japan Broadcast accent dictionary |
| Chinese | Mandarin – Standard dialect. Standard is TBD. |
| Korean | Seoul South Korean younger dialect used mostly by those under 60 years of age. Standard is TBD. |

## Release Schedule

| Date | Customer | Release Version & Type | US English | French | German | Italian | UK English | Spanish | LA Spanish | Chinese | Japanese | Korean |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12/19/05 | Epson Scotland | 5.1 Pre-Alpha |  |  |  |  |  |  |  |  | A1 |  |
| Monday 12/30/05 | Epson Scotland | 5.1 Pre-alpha |  |  |  |  |  | A |  |  |  |  |
| 1/16/05 | Epson Scotland | 5.1 Pre-alpha Update |  |  |  |  |  |  |  |  | A2 |  |
| 3/31/06 | Epson Scotland | 5.1 Alpha | A | A | A | A |  | B | A | A | A | A |
| 3/31/06 | Casio | 5.1 Alpha | A | A | A | A |  | B | A | A | A | A |
| 3/31/06 | Besta | 5.1 Alpha | A | A | A | A |  | B | A | A | A | A |
| 3/31/06 | General | 5.1 Alpha | A | A | A | A |  | B | A | A | A | A |
| 4/30/06 | Epson Scotland | 5.1 Beta | B | B | B | B | A | B | B | B | B | B |
| 4/30/06 | Casio | 5.1 Beta | B | B | B | B | A | B | B | B | B | B |
| 4/30/06 | General | 5.1 Beta | B | B | B | B | A | B | B | B | B | B |
| 4/30/06 | Besta | 5.1 Beta (T900 Port) | B | B | B | B | A | B | B | B | B | B |
| 5/31/06 | Epson Scotland | 5.1 Release | R | R | R | R | B | R | R | R | R | R |
| 5/31/06 | Casio | 5.1 Release | R | R | R | R | B | R | R | R | R | R |
| 5/31/06 | General | 5.1 Release | R | R | R | R | B | R | R | R | R | R |
| 6/15/06 | Besta | 5.1 Release (T900 Port) | R | R | R | R | B | R | R | R | R | R |

A – Alpha, B – Beta, R – Release Candidate, X – Final Release

- Japanese will be as-is using the Onsei.dic.

- Add more intonation processing.

* European languages are 4.64 Quality”

## Release Requirements

The following are minimum requirements for the release type:

Alpha

- Minimal input will be ASCII text.  Conversion for Asian language wide characters may or may not be available.

- Some intonation processing will be available; however, it may be very minimal

- Text normalization for Asian languages is NOT to be expected.

- Word position indexing for Asian languages is NOT to be expected.

- Demos will only be available for the Win32 platform.

- Number of allowable minor, major and critical bugs … ?

Beta

- Conversion for Asian languages will be included.

- Most of the intonation processing should be included.

- Text normalization for all languages will be included.

- Word position indexing for Asian languages will not be included.

- Demos will only be available for the Win32 platform.

- Number of allowable minor, major and critical bugs … ?

Release