# Spanish Vowel Rules:

“y” is pronounced as the vowel /i/ when final in a word (as in “ley”) and in the conjunction “y” = (and).

Muy = stress on the u. This is composed of two weak vowels.

If a weak vowel is followed by a weak vowel, the first becomes a glide (ruido, viuda, juicio, ciudad, ruin).

If a weak vowel is followed by a weak vowel, and the first has stress and bears the accent mark, then both vowels retain their syllabic values (día, continúa, fíen, flúido, vacío, guía).

# Y, when before a vowel, is pronounced as English y in “you”.

In Spanish, the vowels a, e, o are the "strong vowels". In DT, the combination of two consecutive strong vowels is too long. Let's add the rule below. This will be trial and error until we get the length, but let's start with a decrease of about 1/2 (see below).

Strong vowel rule: When the vowels "a", "e", or "o" are unstressed, and follow one another in any combination (aa, ae, ao, ee, ea, eo, oo, oa, oe), , even if the consecutive vowels are both the same vowel (example aa), shorten the duration of each of the vowels by about ½ (on 10/23/06 sent bug to increase duration slightly because it seemed to short esp. on two like consecutive vowels). If any of the consecutive vowels are stressed, this rule should not be used.

For Example: baja altura, que en, que existen, que esperara

Gu rule:  The letters “gu” have different pronunciations depending on the following:

- The “u” in the “gu” combination is silent when followed by  the vowels “e” or “i”.  Example words: aguerrido, aguila.

- The “u” in the “gu” combination is pronounced as the vowel “u” when the “gu” combination is followed by a consonant.  Example words: aguja

- The “u” in the “gu” combination is pronounced as a “wx” (glide) when the “gu” is followed by the vowels “a” or “o”. Example words: Antiguo, agua.

- The umlaut “ü” in the “gü” combination is pronounced as a “wx” when in the combinations of güi or güe. Example words: vergüenza, cigüeña, pingüino, agüero

In the “gu” combination, the g can be either the stop /g/ or the approximate /gh/ depending on the context of the g.

- It should be /g/ if it is preceded by a pause and followed by a, o, or u.  It should be /g/ if it is in the middle of the word and preceded by a nasal.

- It should be /gh/ if in the middle of a word and followed by a, o, or u and not preceded by a nasal.

# Notes on Spanish Trilled rr

(compiled by Randall Pierce M.S.CCC-SLP)

Information based on research and/or information from Spanish Linguist (Erik Willis)

Typically trills in Spanish will front with both the passive and active articulators

(dental-alveolar and tongue tip) in the presence of a high front vowel.

Natural trills have a period of vocalic element between the taps that is roughly equivalent to the duration of the taps.  Also, the taps cause a break in the voicing bar.

The trilled R in spont. conversation typically has 2 closures with an incomplete (approximant) third.  Duration mean is about 75 ms.

Within the trill, the tap duration should be about 15 ms long with intervening vocalic element in the 20 ms range.

Allophonic variations of trilled rr:

-A. /r/ in word initial position is typically trilled

-B. /r/ in word final position may be trilled or faded out as discussed earlier. However, if a vowel follows the word final /r/, it must be a tap [ɾ].

In sentences 1 and 2 below:  the only trill would be on the word reserva. The other {r}s are phonological taps.

1. Llegaré tarde pero mantenga mi reserva por favor.

2. Perdón por el retraso.

El crecimiento sobre el terreno se mantuvo a un mínimo.

arriba

cerrar

sierra Leona

## Tapped r in Spanish

- Spanish taps are typically in the 15-25 ms range.  They often have formant structure.

- Typically described as voiced.

- The epenthetic vowel (vocalic element) should be present in Spanish blend sequences between the consonant and tap. This would only be in the context of when a consonant precedes a tap. For example, blend sequences such as tr, br, gr, cr, etc. would have this vocalic element. This vocalic element should have a duration of 20-25 ms.

- There should be no epenthetic vowel when a tap is followed by a consonant (example: rq, rc, rs, rd, rn, rt, rg, rm, etc).

- coda rhotics could be the implementation of the current tap specification.  Natural speech also seems to show a voiced vocalic element.

## VOT for Spanish Voiceless Stops /p,t,k/

- The VOT specifications mentioned below are based on a duration that starts at the burst onset until the beginning of voicing. Right now as small as DT can go with the resolution is 6.4 ms.

- The stops /p,t,k/ in phrase and word initial position have a short lag VOT, roughly 5-15 ms.

- Some basic guidelines from past research: VOT p=12, t=17, k=23

## Pre-voicing for Spanish Voiced Stops /b,d,g/

The stops /b,d,g/ have “pre-voicing” of 30-50 ms. (in another report he said 45 – 70 ms). If this pre-voicing is not present it could result in a word like “bomba” sounding like “pompa”.

Voice Bar

There needs to be a strong voicing bar when the voiced stop follows a pause and a reduced voice bar duration and voicing following a voiced consonant.  Yet to be determined are preceding voicessless segments  {tb}  the only permissible combinations in Spanish are /tb/ as in football and I am not convinced the /t/ is actually produced in normal speech.

## Closure info for stops

In Spanish; a syllable final stop is typically elided (omitted).  Therefore, when two stops occur together the duration should be limited to reflect a single stop (example of "pt" in abruptamente). The duration of Spanish voiceless stops (p, t, k) should be about 50-60 ms in length. Additionally, with the voiceless stops, voicing continues into the “closure” for at least 30% of the gesture with a gradual fade to voicelessness.

## Intonation

Nuclear Stress:  In Spanish it is the stress on the last stressed syllable of the phrase (or isolated words). The peak is realized about 2/3 the way through the tonic syllable, and then starts to fall off. Within this tonic syllable there is an increase in duration and amplitude.

PreNuclear Stress: Stress on syllables before the final syllable of the phrase. The pitch starts to rise at the onset of the tonic syllable and goes beyond the tonic syllable. For example in the word lámina, the pitch rise would start at the beginning of the á and continue rising beyond the /m/,  into the /i/ and then decline. There is an increase in duration, but not as much as in the nuclear syllable.

Question Intonation:

- 2.5 semitones is an acceptable pitch rise for humans regarding “question final” tonal rise (based on height of the nuclear pa and the plateau of the rise); however, for a computer it is not sufficient because we lack additional cues.  Therefore, for DT, our linguist suggests a final tonal rise of double (5 Semitones).  For DT we are on a Hz basis right now instead of semitones.

- Questions with penultimate stress should have a LOW TONE throughout the final stressed syllable with a tonal rise beginning at the offset of the tonic syllable.

- Questions with oxytonic or final syllable stress should begin the tonal rise midway through the final syllable.

- Ask Erik about questions with antepenultimate stress. ¿Dónde mira la nómada el número de lámina?

- Navarro-Tomás (1968) and Canellada & Madsen (1987) claim that both utterance-initial point and first peak are higher in Spanish interrogatives

- Sosa (1992, 1998) claims that only the first peak (and other prenuclear peaks) is more prominent.

Single Words: Treat the same as nuclear stress, because single words are technically end of phrase.

Utterances with two-three lexical words:

- The prenuclear pitch accents should extend beyond the tonic syllable.

- The nuclear pitch accents should have a peak and tonal fall within the tonic syllable—as they should according to the literature.

- The tonal rise of a prenuclear pitch accent (tonal rising gesture on a non-final lexical word) will typically begin to rise at the onset of the tonic syllable.  The following quote comes from an in press paper of Erik’s “From the utterance onset there is typically a slight tonal fall to the Low tone near the onset of the tonic syllable.  Following the Low tone, there is a tonal rise that typically extended beyond the boundaries of the tonic syllable, followed by a tonal fall to a point near the onset of the final tonic syllable.  After the Low tone associated with the final stressed syllable, there was either a small tonal rise or plateau to a point past the midpoint of the tonic syllable, which then began a tonal fall through the end of the utterance.”  The F0 track described corresponds to a target sentence like Miraba la luna.

- In general, each successive peak should be lower than the previous (in the case of Pablo, perhaps 7-10 Hz).  You may wish to write some sort of rule that calculates all of the tonic syllables in the utterance and create some algorithm for peak height decay or declination.  The same holds true for the scaling of the Low tones.

- Studies have found that the initial peak initially reaches a ceiling for the High tonal values of the utterance, while the final tonal value of the utterance is likewise invariably the lowest of the utterance (and constitutes the floor threshold).  These High and Low values are typically met in the case of utterances with two-three lexical words.  So the challenge is to hit the target High and Low, and vary the intervening peaks with respect to the High and Low.

- For example,

  - Two stresses= 1 at 130 Hz, 2=120 Hz.

  - Three stresses=1 at 130 Hz, 2=123 Hz, 3= 116 Hz.

  - Four stressed=1 at 130, 2=124 Hz, 3= 119 Hz., 4=114

Contrastive Final Pitch Behaviors form Five Categories:

    - Rise

    - Fall

    - Suspension

    - Semi rise

    - Semi fall

Rounded Vowels:

Spanish has considerably more lip rounding for the /u,o/ vowels than English.  A lack of this rounding will make the segment sound non-native (Barrutia and Schwegler 1994).  Erik bases this comment on the fact that f3 is basically the same for the front and back vowels in the sentences, and as he understands it, lip-rounding has the effect of lowering f3.

Mid-vowel allophones of /o > c/ and /e > E/

Currently DT doesn’t have the mid vowel allophones /c/ and /E/. The linguist feels that adding two new mid-vowel allophones of /o/ and /e/ would not contribute to a better sound in a significant. He feels that adding these phonemes would more likely than not create a hindrance to rule interaction.

Nasal Assimilation:

Spanish has an assimilation rule that applies to nasals preceding consonants. Nasalization definitely applies across morpheme and word boundaries. Nasal assimilation occurs regardless of word boundary and absolutely word internally. This assimilation rule requires the coda nasal to assimilate or "take on" the place specification of the following consonant. There is also a word-final neutralization rule in effect in all dialects of Spanish. This word-final neutralization rule results in word-final alveolar nasals preceding a pause or a vowel (in the cases of word-final nasals preceding consonants, the nasal assimilation rule mentioned above applies).

In Puerto Rican Spanish word-final nasals before pauses and vowels are velarized and in Mexican Spanish they are neutralized to an alveolar place of articulation.

Coda nasals assimilate to the place of articulation of the following consonant. The two extremes are the most important for DT [m, ŋ ].

/V+nasal+C/        [ m ]  ___bilabial  	 {un beso}    [um.be.so]

[ɱ ]   ___labio-dental	 {un flaco}    [uɱ.fla.ko]

[n̪  ]   ___dental		{un diente}  [un̪ .di̯en̪. te]

[ n ]   ___alveolar	{un loco}    [ un. lo.ko]

[ɲ ]    ___palatal	 {un chico}   [uɲ. tʃ i.ko]

[ŋ ]    ___velar		 {un gato}     [uŋ.ga.to]

Stop following nasals:  Any stop following a nasal should be a stop (not an approximate), however, the closure (stop gap) is minimal and should be cut by about ½ (15-20 ms).  Example words: envío, combinar

## Phonological variable /s/

The voiceless sibilant fricative /s/ has several realizations in Spanish. The fricative /s/ can undergo deletion, aspiration, voicing, or can be retained as /s/. In Spanish, there is a general voicing assimilation rule where the fricative /s/ is voiced before a voiced consonant (example: “desde” the s becomes voiced). The processes of deletion and aspiration, mentioned above, do not occur in the dialect of Mexican Spanish discussed here. These processes do, however, occur in many dialects of Spanish, including Puerto Rican Spanish (http://www.lingref.com/cpp/hls/8/paper1255.pdf)

Approximants:

Postvocalic /b, d, g/ are realized as approximants in most varieties of Spanish.

Hiatus Rules:

In Hiatus, the strong vowels a, e, o in combinations are to be treated as separate syllables. According to the linguist (Erik), we tried reducing the vowels slightly, 20% when in hiatus and unstressed. The linguist reported that he didn’t think there is a preference for length based on segment ordering. Hiatus samples below:

>traerá

>autoestima

>sahara oeste

>asamblea

Palatal Approximate:

The linguist reported that the palatal approximate (IPA j) should be about 130-150 ms. In Spanish, some regions pronounce the Spanish letter y as a palatal voiced affricate ] in the initial position and following nasals.

The palatal approximate /j/ should be more like a glide instead of a strong /i/. There should be a reduction in the waveform amplitude (a significant pinch) and a break or reduction in the formant structure.

Approximate Rules gh:

Rule 1: g should be the stop /g/ in the following cases:

-after a pause (gato)

-after a nasal (un gato, mango)

Rule 2: g should become the approximate gh in the following cases:

-Everywhere else not mentioned in rule 1. (el gato, la gata, lago, algo,

agnóstico)

Rule 3: The word initial g turns into the approximate gh across word boundaries when not followed by /i/ or /e/ or not preceded by n. (extremadamente gordo)

Semi-Vowels yx and wx:

yx should be about 50 ms in length. When in an “iu” combination, the yx should be longer (somewhere between yx and y duration). Maybe about 100?  The duration of “u” in the iu combination, should be somewhere between the u and the wx.

In a word that follows a pause, and has a silent "h" in the initial position, followed by an "i" or “u” plus a vowel (example: hierba, hueso), the "i" should be pronounced like the consonant "y" as in yo. For “u”, it should be pronounced as “w” (as is wake). If these semi-vowels are not following a pause, they should be pronounced “yx”  or “wx” (i.e. across word boundaries).

Spanish:

Rules for letter "i" (when always pronounced as phoneme "i"):

1. Word initial, before a consonant (both when preceding a pause and when no pause):  invitar

2. Word final, following a consonant (both when followed by a pause, and when not followed by a pause):

casi.

3. Word-medially, between two consonants: colibrí, invitar, víctimas, martin, tráfico

4. When preceded by the letter u (u is pheneme wx in this case): cuida

5. When stressed in all contexts: país, río

When letter "i" is pronounced as phoneme "yx":

1. When not stressed and preceded or followed by all vowels except u, it is phoneme "yx": bien, reino,

ciudad, cambiáis.

2. When in a word that does not follow a pause, and has a silent "h" in the initial position, followed by letter

"i" plus a vowel (example: hierba) the "i" should be pronounced as phoneme "yx".

Rule for "i" when in "iu" or "iu" combinations:

1. When unstressed "i" precedes u it becomes yx and u stays phoneme u: viuda

2. When "i" follows "u" it is phoneme "i": cuidado

When letter "i" is pronounced as letter "y":

1. When in a word that follows a pause, and has a silent "h" in the initial position, followed by letter "i" plus

a vowel (example: hierba) the "i" should be pronounced as phoneme "y".

When letter "y" is pronounced as phoneme "yx":

1. In final position of words: soy, ley, muy, guay.

U and WX Rules:

Rules for letter "u" (when always pronounced as phoneme "u"):

1. Word initial, before a consonant (both when preceding a pause and when no pause):  uno, unido

2. Word final, following a consonant (both when followed by a pause, and when not followed by a pause):

(unstressed final u not common).

3. Word-medially, between two consonants: cuba, abductor

4. When preceded by the letter i (i is pheneme yx in this case): ciudad

5. When stressed in all contexts: cacatúa, charrúa, transeúnte, dúo

When letter "u" is pronounced as phoneme "wx":

1. When not stressed and preceded or followed by all vowels except i, it is phoneme "wx": cuanto, bueno,

cuota.

When letter "u" is pronounced as phoneme "wx":

1.  When in a word that does not follow a pause, and has a silent "h" in the initial position, followed by letter "u" plus a vowel (example: hueso) the "u" should be pronounced as phoneme "wx".

When letter "u" is pronounced as phoneme "w":

1.  When in a word that follows a pause, and has a silent "h" in the initial position, followed by letter "u" plus a vowel (example: hueso) the "u" should be pronounced as phoneme "w".

Rule for "u" when in "iu" or "iu" combinations:

1. When unstressed "u" precedes i it becomes wx and i stays phoneme u: cuidado

2. When letter "u" follows letter "i" it is phoneme "u": viuda

/s/ Duration Rules:

In DT, when /s/ is followed by another /s/ (tres semanas) it will sound too long if not shortened. When two /s/ are back to back shorten each /s/ about 30%.  The Current rules in DT regarding /s/ duration are:

1. When two /s/'s are back to back shorten each /s/ about 30% (tres semanas).

2. When /s/ is at end of a multi-syllable word shorten the duration 8%.

3. When /s/ is at the beginning of an open syllable shorten 8%.

4. /s/ in front of plosive shorten 53%

5. /s/ is also shortened when between two vowels.

umlaut ü:

Here are the rules for the umlaut ü: The letter u, when following g or q and preceding e or i, is silent, unless it has a dieresis (ü). If the u has a dieresis (¨) it becomes a wx. The dieresis indicates that the two adjacent vowels both need to be pronounced (üe or üi). The dieresis is only found on the U, and it can only precede an E or I.

Dh rules:

The intervocalic -d- and the word-final -d (as in ciudad) are different allophones of the phoneme /d/. The intervocalic allophone is universally pronounced as a voiced interdental fricative (similar to the th- in English these, those, etc.). On the other hand, the word-final -d shows some variation in the Spanish-speaking world. In the standard language of Spain (based on a Castilian model), it is usually articulated as a voiceless interdental fricative (similar to the th- in thin). In many other varieties, it is completely elided (as if thr word were written ciudá).  The other option is as an unreleased dental stop [d]. Spanish seems to have a phonotactic constraint against word-final stops. To clarify, the allophonic variant of final d is only in phrase final position.  Otherwise it will turn to an approximant if followed by a vowel

B and V variance:

The recommendation of our linguist is to keep both these graphemes as belonging to the same phon /b/.  If a Native Spanish person is recording and reading from a list of words, they may vary the production of the v sound like an English v. According to our linguist, they may be consciously producing something that doesn’t exist in normal speech, for example, releasing a /t/ in word final position.

Y sound:

There should be significant pinch in the waveform and reduction in the formant structure.