BrailleNote Apex

KeySoft 9.0.1

Release Notes

What’s new?

Keysoft 9.0.1 build 753

Version 9.01 is a maintenance release for BrailleNote Apex. This new version includes English and French installations depending on license language.

Main fixes included in this version are:

- Web browser: better support of pop-up dialogs such as the security dialogs found when downloading from NLS web Braille site.

- Improved Wi-Fi connexion.  KeySoft also announces if a page is not found or if the connexion is not established using Wi-Fi.

- LAN and WLAN MAC addresses are available in the information menu.

- USB to IEEE-1284 parallel adapter are supported in this version. The Product #: 16898 available from Cables to GO. http://www.cablestogo.com/ is recommended.

- Spanish Eloquence is now supported as in Keysoft 8.0. Therefore, it is now possible to read Spanish books from Bookshare.

- Dictionaries have updated user guides New versions of the dictionaries are available. We updated the user guides and the document is available directly in the Oxford folder.

- Format SD cards is available in support information mode.

Other issues fixed:

- Improved pronunciation of some words using Eloquence.

- When copying a folder from an external drive to the flash disk through the file manager, KeySoft only copies the current folder instead of all the card content.

- When navigating by level in Daisy books, reading can now be stopped.

- Corrections of QT commands are included.

- Restoration issues have been fixed.

- Zeroing the stopwatch is now working when the stopwatch is running

- Words are now inserted and replaced correctly in thesaurus.

- Backup is done in the appropriated default folder.

- Sub-directories can be deleted using Backspace with Dots 1 + 4

- Under wireless options, types of networks is now set to all available.

- Incremental backup is working correctly.

- Non live streams lost if Enter with X is used to pause playback

- In the Web browser, Space with M now provides all reading modes

- In the Planner, reading using the Back key behaves normaly,

- In the Address book, Grade 2 is working to lookup an address

- When starting or resuming Keysoft, the voice is not crackling anymore

- Backspace with Enter clears the braille display when time is displayed

- Planner - Problem using the visual display has been fixedEmbossing using the serial port of the Index with the USB/serial adapter is now working.

KeySoft 9.0.0

The new Apex notetaker is the continuation of the BrailleNote family’s tradition of excellence. Thinner (0,78 in.) and lighter (1,8 lb) than any other notetaker in the industry, the BrailleNote Apex is designed for maximum comfort during extended reading and writing tasks:

- Comfortable full-size Braille keyboard makes all your notetaking quick and easy.

- Signature thumb keys make extensive reading a real pleasure with effortless navigation by line, sentence or paragraph.

- Newly designed scroll wheel allows for effortless navigation in menus, documents and e-mail lists.

Apex’s main characteristics are:

Hardware

- 32-cell refreshable Braille display

- Computer-style keyboard (QT model)

- 8-dot Braille keyboard (BT model)

- Scroll Wheel (BT model)

- Speech output with choice of TTS (Eloquence is default output)

- Stereo speakers

- Internal Omni Directional Microphone

- Real-time Clock with 30-day battery backup

- External speaker / headphone and microphone jacks

- 8GB of non-volatile user storage

- 256 MB SDRAM

- Weight: 812 g (1.8 lbs)

- Dimensions: 19 mm (0.78 in) H x 244 mm (9.61 in) W x 143 mm (5.63 in) D

- Processor: Freescale iMX32

Connectivity

- One SD card socket supporting SDHC cards

- Built-in Ethernet port (10/100Mbps)

- Built-in Wi-Fi 802.11 b/g protocol

- Built-In Bluetooth 2.1

- One high-speed USB 2.0 client port for Active Sync and Braille Terminal Mode

- 3 high speed USB 2.0 host ports

- Optional USB voice modem

- Optional GPS receiver via Bluetooth

Apex available USB device, driver list

- USB memory sticks and mass storage like External Drives

- USB CDROM

- USB QWERTY Keyboard

- USB to Serial cable adapter

- USB Printers (most popular HP Printers)

- Radicom USB Modem

Batteries

- Rechargeable and user-replaceable lithium-ion Battery

- No specific calibration of batteries is required. However, a full charge cycle is needed for the percentage to be accurate.

- When starting a recharge cycle, batteries at 90% and over will not be recharged to prevent premature wear.

Operating system

- Microsoft Windows CE 6 operating system

- Completely new BSP developed under Windows CE 6

- 3 resets are available: Hard Reset, Soft Reset, Clear Settings

  - Hard Reset: Unconditional reboot  (press & hold reset 5 seconds)

  - Soft Reset: Software reboots and  will try to save information before closing (press Reset button)

- Clear Settings: Reinitializes the registries (press Reset with dots 4-5-6 (BT) or J-K-L (QT)

- BrailleTerminal Mode is now available through USB Client. A PC driver is required. The driver called ‘ApexPCDrivers’ is available through the companion CD. You must install the driver corresponding to your type of computer processor, 32 bits or 64 bits. Please refer to your computer properties.

- Real-Time Clock set through an online Secure Service.

- Support of USB printers and embossers equivalent to KeySoft 8.0 on the mPower.

- Serial connectivity is available through USB\serial adapter provided in the box.

## End User Software (KeySoft 9.0)

- KeySoft 9.0 for Apex is equivalent in terms of end user functionality to the KeySoft 8.0 for mPower.

- Bluetooth connections can now be established on Apex or PC. Similarly, Bluetooth configuration can also be done on Apex or PC. Please consult user guide for information on connecting using Bluetooth.

- To use Apex in Braille terminal mode via Bluetooth, initiate the connection from the PC, not from the BrailleNote. Use pin 0000.

- Two Bluetooth devices can be connected simultaneously.

- Main Bluetooth services supported include:

  - Serial Port Profile (SPP)

    - More than one serial connection at same time.

    - PC Serial Connection

    - GPS Receiver

    - Braille display

    - BT Serial Printers, etc

  - Dial-up Networking (DUN) Profile (Depends on SPP)

    - PC Network connexion

    - Cell phone tethering

    - Allows Apex to access the Internet using your mobile phone as a wireless dial-up modem

  - File Transfer Profile (FTP) (Depends on GEOP)

    - Mainly with PC when available

    - Other Apex

  - Hands-Free Profile (HFP) (Depends on SPP)

    - headset with a button to open and close conversation like cell phone hand-free headsets.

  - Headset Profile (HSP) (Depends on SPP)

    - Dummy Bluetooth headsets with volume control on microphone and speakers.

  - Human Interface Device (HID) Profile

    - Standards BT Keyboards

  - Personal Area Networking (PAN) Profile

    - Allows Apex to be part of an ad-hoc network and the same mechanism can be used to access a remote network through a network access point

  - Audio Gateway Services

    - Used by Headset and Hands-Free profiles.

- KeySoft 9.0 is built on a brand new Framework.  This software architecture will allow HumanWare R&D to develop new KeySoft version and new applications.

- Installation of Software upgrade and product keys has totally changed and it is much easier for the end user.  They can be downloaded directly to the Apex or installed from an SD Card or USB Stick.

- The default Microphone is the Apex internal microphone.  But if an external microphone is plugged, it becomes the main input.  The user cannot choose the input of their choice.

- The newly designed scroll wheel allows for effortless navigation in menus, documents and e-mail lists.  It allows you to intuitively perform a set of actions without using Braille commands. The Scroll wheel can perform different actions depending on the current application. The Scroll Wheel is a feature only found on the BrailleNote BT.

To activate or deactivate the Scroll wheel: Press SPACE with O, then W. This is an On/Off toggle command; in other words it will alternate between on and off each time you press SPACE with O, then W.

- VGA Output is open from the KeySoft Option Menu, Visual Display answered “V” for VGA and “F” for off.  VGA is turned off automatically when Apex is switch to Off, Soft Reset, Hard Reset or goes to Suspend mode on Battery.

# Software Update

Installation procedure using removable memory:
To upgrade BrailleNote Apex to a more recent version, you will need to acquire the installation upgrade file. The Installation upgrade file has an .hwi extension.

You will also need a memory media such as an SD card or a USB thumb drive (it will be referred to as the Installation SD card later in the text) to perform the installation.

Use the same procedure to install an optional application such as the Oxford Dictionary.

To prepare the installation SD card, follow the instructions below:

- Insert the SD card in the computer's SD card reader. You may also use an external SD card reader connected to the computer's USB port.

- Download the installation upgrade file or the optional application program to the PC.

- Make sure to save this file in a location you will be able to locate.

- Copy this file into the root directory of your SD card.  It does not matter if there are other files and folders present.

Once the installation SD card is ready, please follow this procedure:

- Connect your BrailleNote Apex device to a power outlet. Neglecting to do so may cause device memory corruption.

- Make sure the BrailleNote Apex is powered on.

- Verify that there is no SD card in the BrailleNote's SD card slot and that the USB drives are not being used.

- Insert the installation SD card. The BrailleNote Apex will confirm that an SD card was inserted. You will be asked to perform a soft reset.

- When the Apex has rebooted, it will ask you if you want to install this new software version or the optional application. Press Y to proceed with installation or N to abort.

- Beeps and progress messages will be heard throughout the installation process. This will only take a few short moments.

- When installation is complete, the BrailleNote will restart. The new installation has been completed when the startup chime is heard and you are in the main menu.  If you installed an optional application, the user guide will open. Note that the installation file has been deleted from the SD card.

Installation procedure using the Apex internal memory:
Alternatively, you can download the file directly on the Apex Flash disk. If using the Apex to download the file, make sure to do this through an Ethernet connection, not a Wi-Fi connection. When downloaded, the file should be saved in the Flash Disk root folder. Please consult user guide for information on downloading files.

In case of problems during installation: If an installation failure message is heard, do not reset the device. Remove the installation SD card and insert it again. If the problem persists, contact technical support.

# Restoring mPower data onto the Apex

It is possible to backup the data found on an mPower unit and restore it onto the Apex. This is an effective method to transfer the contents of your flash disk, including your emails, texts, contact list, planner and LAN settings etc. To obtain details on creating a backup copy of your mPower data, or how to restore the information to your Apex, please consult the Backup or Restore section of the Utilities chapter in the User guide.

# Known Limitations, Workarounds and Tips

- FM Radio can’t be recorded in version 9.0.1.

- Only one USB external Modem is supported (Radicom V92MBU-E for Windows CE 6.0). Users must order the USB modem from Humanware.

- Radicom USB Modem may not work properly for some countries like UK, France and Australia.

- When using Apex with Jaws, make sure to select a serial port below 10 with Bluetooth or the USB client driver.

- When Daylight saving time is set to yes, every reset moves the time ahead by one hour.

- Copying item from Address list into word processor document may produce errors.  Use instructions described in User guide, section 10.6 Inserting an Address into a document to do so correctly.

- WPA/WPA2 mixed router configuration is not supported.

- In Windows CE 6.0, Ethernet/Wireless download is limited to 60 Mb. A fix is expected from Microsoft in a near future.

- Download from NLS bard web site for audio books is not supported.