BrailleNote Apex family

KeySoft 9.2.2

Release Notes

Users upgrading to version 9.2.2 from a version earlier than 9.2 must obtain a product key. Please see below for the procedure to obtain this Product key.  Users upgrading from KeySoft 9.2.x to 9.2.2 can do so without using a product key.

***KeySoft 9.2.2***

- Fixed an issue where a small amount of units were freezing during general usage.

- Fixed issue where the keyboard would not immediately respond upon boot up.

- Other bug fixes and general stability enhancements

***KeySoft 9.2.1***

- Implemented a disk checking sequence during boot up to verify the integrity of the flash disk.  If problems are found, they are rectified before KeySoft loads.  Note that while problems are being fixed, there may be a slight delay in the boot up process.

- General stability enhancements

# What's new in KeySoft 9.2?

## KeySoft PDF Converter:

It is now possible to convert a PDF document to a text file. The text file opens directly in KeyWord, thus allowing it to be read and edited at your convenience.  This capability allows for any PDF document with imbedded text to be converted to a text file, allowing professional documents to be received as attachments by email on your apex, and opened and read without the use of a computer or other external converter. To initiate the conversion process, a "Convert PDF file," option has been added to the file manager menu.

### Steps to convert a PDF file are as follows:

- Enter the File Manager from the Main Menu

- Select Convert PDF file or press O.

- Select the drive and folder where the PDF file is located.

- Press Enter on the PDF file

- KeySoft prompts with the message “Converting PDF to text” and 3 second beeps while the conversion is in progress.

- You are now presented with the message to "Review the options?" Simply press enter if you want to immediately enter the converted text file.  Press Y for Yes if you want to immediately save this file as a Braille document.

- The PDF file has now been converted to text and is now shown in KeyWord.

## PDF Files In KeyBook:

If you are planning on reading the contents of a PDF document but do not intend on editing the file, KeySoft's book reader allows for a simple and efficient method to accomplish this task.

### Steps to open a PDF file via the book reader:

- Enter the book reader from KeySoft's main menu.

- Select the drive and folder where the PDF file is located.

- Press enter on the file and it will convert and open the file in the book reader.

## Full QWERTY input and control when using WindowEyes:

With this new version, the Apex QT keyboard can be used to either represent the computer's keyboard, to input text and perform windows commands, or by pressing function plus B the ASDF and JKL; keys can emulate a Perkins style braille keyboard for commands and data entry. Simply select the, "BrailleNote with keyboard input," option when choosing a Braille device in Window-Eyes.

# Enhancements and Issues Fixed in KeySoft 9.2

## Capability to copy a sender to the address list:

When the command “Copy Sender to Address List” is performed on a received email, the information is now correctly preserved in the address list.

## Braille input in terminal mode on Apex QT units:

Certain screen readers do not allow for full keyboard usage when a Braille terminal is used.  Because of this the command function plus B has been created to emulate Braille input using the letters ASDF and JKL; on the QWERTY keyboard.  When emulating Braille input using this command, screen readers recognize the QT keyboard as a Braille keyboard and all functions available to displays with Braille input keys are available to the QT Apex.

### Steps to use Braille input with QWERTY keyboard are as follows:

- From the main menu enter the Braille Terminal application.

- Select the communication port, (U for USB or B for Bluetooth.)

- Press function with B to activate Braille input.

- Now ASDF and JKL; emulate the keys on a standard Perkins style keyboard and can be used to enter information into devices that allow for Braille control.

## Connection configuration modifications:

The first time you need to connect to the internet for any reason you will be prompted to verify which connection should be used.  This ensures that the appropriate connection is used, along with validating proxy server settings if necessary.  Upon verifying the connection the first time, Keysoft remembers the connection and will not prompt for this again until the unit is reset.

## Improved Internet file download:

Downloading of internet files have been improved to allow larger file downloads such as KeySoft updates or audio files. In addition, when the help command is used during a file download, the user is offered significantly more information: including filename, transfer rate (in KB/sec), percentage of completion (when total file size is available) along with the progress of the file download.

## File Manager modifications:

In the file manager menu, the item called "Directory," has been replaced by an item called "Browse Files."  This item is used to browse the drives, folders and files available to the BrailleNote Apex.  Simply press enter on the "Browse Files" item or press the letter B.

# Updating to KeySoft 9.2:

## How to obtain the Product key?

The product key  can be downloaded by logging in to license.humanware.com with your serial number and email address.

## Upgrade Procedure:

To upgrade BrailleNote Apex to a major version, you will need to:

- Install the Product key you received by email or acquire from the Humanware license site at http://license.humanware.com/  your serial number and email address is required. Please note that it is important to install the key prior to the KeySoft version.

- Acquire the .hwi installation upgrade file. Installation and upgrade procedures remain the same as version 9.1.

Users running a version earlier than 9.1 may upgrade directly to 9.2 after the 9.2 product key has been obtained.  There is no need to first install KeySoft 9.1.

Product Key installation procedure

Prior to installing KeySoft, you will need to install the Authorization key.  The Authorization Key file has an .hwl extension. You will also need a memory media such as an SD card or a USB thumb drive (it will be referred to as the Installation SD card later in the text) to perform the installation.

To prepare the installation SD card, follow the instructions below:

1. Insert the SD card in the computer's SD card reader. You may also use an external SD card reader connected to the computer's USB port.
2. Download the product key file to the PC.
3. Make sure to save this file in a location you will be able to locate.
4. Copy this file into the root directory of your SD card. It does not matter if there are other files and folders present.

Note: If you use KeyWeb from your Apex to download the license, save the file into the root directory (none folder,) of your Flash Disk.

Once the installation SD card is ready, please follow this procedure to install your produc key:

1. Make sure the BrailleNote Apex is powered on.
2. Verify that there is no SD card in the BrailleNote's SD card slot and that the USB drives are not being used.
3. Insert the installation SD card. The BrailleNote Apex will confirm that an SD card was inserted and will install all licenses available on the card.
4. You are now ready to install the Keysoft 9.2 update.

Note: If you use KeyWeb to download the license, simply perform a soft reset when the download is completed to install the license on your unit.

If the product key license has been successfully installed, you will hear the following message: "1 valid license installed". If this message is not heard, do not proceed with the software upgrade.

KeySoft Upgrade Procedure

Installation procedure using removable memory:

To upgrade BrailleNote Apex to a more recent version, you will need to acquire the installation upgrade file. The Installation upgrade file has a .hwi extension. You will also need your installation SD card previously used.

To prepare the installation SD card, follow the instructions below:

- Insert the SD card in the computer's SD card reader. You may also use an external SD card reader connected to the computer's USB port.

- Download the installation upgrade file to the PC.

- Make sure to copy this file in a location you will be able to locate.

- Copy this file into the root directory of your SD card.  It does not matter if there are other files and folders present.

Once the installation SD card is ready, please follow this procedure:

- Connect your BrailleNote Apex device to a power outlet. Neglecting to do so may cause device memory corruption.

- Make sure the BrailleNote Apex is powered on.

- Verify that there is no SD card in the BrailleNote's SD card slot and that the USB drives are not being used.

- Insert the installation SD card. The BrailleNote Apex will confirm that an SD card was inserted. You will be asked to perform a soft reset. When the Apex has rebooted, it will ask you if you want to install this new software version. Press Y to proceed with installation or N to abort.

- Beeps and progress messages will be heard throughout the installation process. This will only take a few short moments.

- When installation is complete, the BrailleNote will restart. The new installation has been completed when the startup chime is heard and you are in the main menu.  Note that the installation file has been deleted from the SD card.

- You are now ready to use KeySoft 9.2.

Installation procedure using the Apex internal memory:

Alternatively, you can download the installation upgrade file directly on the Apex Flash disk. When downloaded, the file should be saved in the Flash Disk root (none) folder. Please consult the user guide for information on downloading files.

In case of problems during installation:

If an installation failure message is heard, do not reset the device. Remove the installation SD card and insert it again. If the problem persists, contact technical support at support@humanware.com.

Please contact your regional Humanware office in the USA, Canada, UK, and Australia for more information.

USA: 1 800 722-3393

UK/Europe: +44 1933 415 800

Canada: 1 888 723-7273

Australia/Asia: +61 2 9686 2600

To keep up to date with the BrailleNote family of products, sign up for the Newswire at: www.humanware.com/braillenote or www.humanware.com/braillenote_support .