F.A.Q.

KeySoft version 8.0

July 2009

Table of Contents

1.	KeyChat	1

2.	Audible.com Books and Audible Manager	2

3.	New Bookshare Library Access	5

4.	WPA-PSK capability	6

5.	Sendero GPS : Try before you buy	7

6.	Printer support	9

# KeyChat

Q. What is instant messaging?

A. Instant messaging is a form of real time text-based communication between two or more participants using a computer network and associated chat software.  This means that even when people reside in different countries, short typed messages maybe instantly exchanged, read and replied to.

Messaging services like those available in KeyChat can be used locally over a company intranet or more widely via the internet with the same easy interface you are used to when using the KeySoft suite of applications.

Q. What requirements are needed to use KeyChat and with whom can I communicate using instant messaging?

A.  To use KeyChat, you must have an account with a service that provides XMPP messaging.  The most popular service and simplest way to obtain this membership is by creating a Gmail account.  To do this, visit www.gmail.com.

Once the Gmail account is set up, your KeyChat username will be your Gmail e-mail address, and your KeyChat password will be your Gmail password.  The Host server that KeyChat needs to connect to is the same host server that the Google Talk application on a pc connects to.  This server is currently talk.google.com, using port 5222, but is subject to change at Google's discretion.  For more information on Google's server connections we invite you to visit the Google Talk setup help section at: http://www.google.com/support/talk/?hl=en.

Once Keychat has been setup and you are able to log in, you can chat with members of any XMPP messaging service.  Three of the major organizations that provide this service are Google, IChat, and Jabber.

Q.  What is XMPP/ Jabber instant messaging?

A. The Extensible Messaging and Presence Protocol, (XMPP) is the specific method that Google Talk, KeyChat and many other XMPP messaging programs use to transmit and receive messages between their users.

Jabber servers are used to offer XMPP instant messaging services.

Q.  My friends use MSN or Yahoo! How can I chat with them?

A.  The current XMPP protocol is not compatible with alternative chat programs such as AIM, Yahoo, or MSN.  The instant messaging community hopes that a common messaging protocol will be adopted to allow chatting from multiple services and using different contact lists. As XMPP is the only open standard, the community is heading towards the establishment of XMPP as the common protocol.  AOL and Google are currently working together to allow both Google Talk and AIM users to communicate with each other in the future.  Apple already supports this protocol.

Until this collaboration occurs, there are multifunctional programs that can be installed on a PC that will allow any user of any legacy instant messaging service to communicate with members of the Google Talk community.  A free and accessible downloadable program for windows XP or Vista is called Miranda.  The link to download along with setup instructions from Google is found here.

http://www.google.com/support/talk/bin/answer.py?answer=24885

A separate web-based site which allows AOL, Yahoo and MSN users to contact Google talk users, along with users of other IM communities is Meebo.  This site can be accessed by a PC’s web browser by going to:

http://www.meebo.com/

# Audible.com Books and Audible Manager

Q. Can the BrailleNote play recorded books from Audible.com?

A. Yes. However, you must first be a member of the Audible service and use the Audible Manager program to authorize your BrailleNote to play the protected Audible books.

Q. Can I try out the Audible service without subscribing?

A. Yes. HumanWare has a partnership with Audible that allows HumanWare customers to try the Audible service free for 14 days including the download of a free Audible book.  To learn more visit:http://www.audible.com/humanware

You can cancel at any time during the trial period and still keep your downloaded book.

Q. What is Audible Manager?

A. Audible Manager is a Windows software program that you download and install from Audible.com when you become an Audible member. Audible Manager is then used to activate your BrailleNote to enable it to play Audible books. Audible Manager will also keep a list of your downloaded Audible books and allow you to play them on your computer and transfer them to your BrailleNote. Audible Manager runs on Windows 2000, XP, and Vista. It does not run on Windows 98 or ME.

Q. Can I listen to a sample Audible book before subscribing?

A. Yes. Visit: http://www.audible.com/humanware to sign up for a free 14-day trial of Audible including a free book download and the Audible Manager software. You may keep the book if you decide not to subscribe to Audible.

There are also excerpts of Audible content on our Web site at www.humanware.com/braillenote_support . Sample books do not need authorization through the Audible Manager. You will need to create a folder called “Audible” in your selected memory. We recommend you use a card reader and use Windows Explorer to copy the .aa Audible book files to the Audible folder on your storage card.

Q. How do I activate my BrailleNote with Audible Manager?

A. First, be sure you have version 5.5.0.4 or higher of Audible Manager, and Microsoft ActiveSync (Windows XP or earlier), or windows Mobile Device Center, Windows Vista.)  If you are an existing Audible customer, you can download the latest version of Audible Manager from

http://download.audible.com/AM50/ActiveSetupN.exe

When Install Shield asks you to choose a player device, you may choose ‘Humanware BrailleNote’. Alternatively, you can also choose "No mobile Device" during the installation because you can also add the Humanware BrailleNote device later from Audible Manager.

Once it is installed, be sure to remain connected to the Internet and run Audible Manager. With Audible Manager running you are now ready to activate your BrailleNote's storage card to play Audible books. Follow these steps:

- Insert the storage card into the BrailleNote. Power off your BrailleNote and connect it to your computer using the provided USB cable. Then power the BrailleNote back on. You should hear a rising chiming sound as the BrailleNote is recognized by the PC.  Important: When using Audible Manager, you must connect the BrailleNote. You cannot activate your storage card in a card reader.

- In Audible Manager, press Alt+D to open the Device pull down menu.

- If you did not specify the ‘Humanware BrailleNote’ during Audible Manager installation, then choose the Add New Device option.

- You will then be asked to connect to the Internet so Audible Manager can update necessary components. Select the YES button to proceed.

- Audible Manager will then present a list of possible devices. Arrow down the list to Humanware BrailleNote and select it. Then tab to the OK button and activate it

- Audible Manager will download the plug-in for the BrailleNote and then ask to close Audible Manager to install the plug-in on your computer. Select the OK button.

- Audible Manager will then close and reopen.

- Once open, press the F6 key to select the mobile device that Audible Manager should scan for content, and arrow to the Humanware Braille Notes option.  If “Humanware BrailleNote” is not an option, press the f5 key to refresh the list of devices.

- Right-click and select properties from the drop down menu, or alternatively press Control Shift D to open the device properties.

- Tab to the Storage selection combo box and arrow to the storage card you would like to activate, where you plan on storing your future Audible Books.

- Tab to the OK button.

- Again open the Device menu and choose the option to activate your device.

- Audible Manager will prompt you to enter your Audible Username and password. Then press the OK button to activate your BrailleNote's storage card.

- Note that Audible refers to the BrailleNote device as Humanwre BrailleNote. Please do not select ‘Humanware Players’ as this refers to the Victor Reader Stream and the ClassMate.

- After you press OK, the storage card should become activated. You can verify this as there will be a folder named Audible on your storage card and a file named AudibleActivation.sys in that folder. Note that this is a system file and will not appear unless your Windows folder options are set to show hidden system files. However, you do not need to change your folder options as this will not affect use of Audible. If the Audible folder is created then likely the Activation file is there.

Q. Can I use more than one storage card for Audible books on the BrailleNote?

A. Yes. If you need to use a second storage card for Audible, simply repeat the procedure listed above or create an Audible folder on the second card and copy the AudibleActivation.sys file to it.

Q. After activating my BrailleNote, how do I transfer Audible books to it?

A. You may use Audible Manager with the BrailleNote connected to transfer your Audible books to the BrailleNote. An easy way to do this is to arrow down to the book you want in the Audible Manager library list. Then press the applications key on your keyboard and from the resulting context menu, select the "Add to Humanware BrailleNote". Audible Manager will place the Audible book .aa file in the Audible folder of the storage card.

Instead of transferring books with Audible Manager, you may also place your storage card into a card reader and use Windows Explorer to copy the .aa Audible book files from the Audible download folder to the Audible folder. The default Audible download folder is:

C:\Documents and Settings\All Users\Documents\Audible\Downloads

Or, if you updated an older version of Audible Manager it may be:

C:\Program Files\Audible\Programs\Downloads

You cannot create separate subfolders in Audible for each book.

Q. Which Audible formats are supported on the BrailleNote?

A.  Audible format 4 books should be downloaded for use on the BrailleNote.

Q. After resetting my BrailleNote I am unable to play Audible content.

A. If a 4 5 6 reset (J K L on QT models) was done for diagnostic or troubleshooting purposes you may need to reactivate using the Audible Manager software.

Q. Is there a limit to the number of devices or storage cards I can activate for use with the Audible service?

A. Audible allows customers to activate up to 8 different devices per account, allowing someone to potentially activate a BrailleNote, a Victor Reader Stream, and an Apple iPod all for use with content purchased with the same Audible user account.  Note that the BrailleNote may only be activated for one Audible user account at a time.

# New Bookshare Library Access

Q. What's new in Bookshare site?

A. The bookshare.org web site has been updated and redesigned, offering a number of benefits and changes, and also a new protected format for downloaded books, newspapers and magazines.

Q. Downloading books with .bks2 extension

Books downloaded with bks2 format are unpackaged and recognized like Bookshare books. They will be seen in the folder along with any previously downloaded books.

Q. Downloading books with .zip extension

If books were downloaded with the default .zip extension, it will be necessary to change the BookReader filter to all. Alternatively, use the KeySoft File Manager to unpack the books as they will not appear when browsing via the Book Reader application.  The downloaded books may later be renamed to have a .bks2 file extension, or you may wish to change your preferred default file type within your bookshare.org account preferences to be .bks2 instead of .zip.

Q. Security process on the new Bookshare site

Tip to avoid security dialog boxes: When entering the Bookshare site for the first time, after typing your user name and password, you can check the “Remember Me” check box. By doing that, you’ll only have to go through the security dialog boxes once. Next time you open the Bookshare page, you will be automatically logged on to your account without having to pass through security.

Q. Where do I stock downloaded books?

We recommend downloading and unpacking books on memory cards instead of using the internal memory.

Q. The system is slow when loading page and navigating.

It is a known issue that browsing the new bookshare.org web site and downloading books can be slower than times experienced using the Legacy Bookshare site. This will be particularly noticeable on the Classic models.  Because the BrailleNote mPower and BrailleNote PK are more recent, they are not affected by this performance constraint.

# WPA-PSK capability

Q. What is WPA encryption?

A. WPA, Wi-Fi Protected Access, is a security protocol created to secure wireless computer networks. This protocol was created to reinforce the security level offered by its previous Wired Equivalent Privacy (WEP).

Q. What type of WPA encryption is supported in KeySoft 8.0?

A. Keysoft 8.0 supports WPA PSK (Pre Shared Key).

Q. Is WPA available on my BrailleNote?

A. WPA PSK encryption is only available for use on the mPower platforms when using a wireless G card.

Q. What type of Wi-Fi card is recommended for the Mpower?

A. The recommended card for the mPower is the Ambicom 54G.

Q. How can I enable WPA PSK capability on my BrailleNote MPower?

Now, two options are available when creating a connection configuration entitled: “Use WPAPSK?” If one is using either the Ambicom WL54G or the US Robotics Wireless MAXg USB Adapter USR5421, and the wireless signal is encrypted using WPA personal, set this option to yes and press enter.

The next option prompts the user for the WPA Preshared Key.  This is a passphrase that is between 8 and 64 characters and must match the passphrase that was originally entered in the wireless router.

Once this information has been entered, exit the connection configuration list with a space with E, or escape key on a QT and type a Y to save changes.  Connect to the internet as you would normally and The MPower will successfully connect to a wireless G signal that is encrypted using WPA.

# Sendero GPS for BrailleNote: Try before you buy

KeySoft version 8.0 includes the Sendero GPS version 6.0.  During the 90-day free trial period, users can browse their preferred map or navigate outdoors. Users are required to download the desired maps from the Sendero site. For real time navigation, you will need a GPS receiver. After this trial period, users can continue to enjoy their system at a special price.

The new BrailleNote mPower and PK units will ship with the GPS trial and KeySoft version 8.0 as well.

How can I activate the BrailleNote GPS software?

mPower platforms

Version 8.0 will contain BrailleNote GPS V 6.0 installed by default. From the Main menu, go to the "Navigation" option. You will be prompted to press T to run the trial version or register your copy of Sendero GPS (R).  After pressing T, information will be provided on how to get the Sendero application up and running.

To virtually browse a geographic area or create routes using the BrailleNote GPS application, you will need to install an electronic map on your SD or CF card. To download your maps and POIs, go to www.gosendero.com. In the serial number box, enter "demo2009" and use the password “sendero”.  Once logged in, select the country and maps you wish to download.  Unzip them on your PC and copy those unzipped files to a storage media card, in a subdirectory called maps.  Exit this program, insert the storage media card into your mPower or PK and start Sendero GPS again by pressing N.

To use Sendero’s BrailleNote GPS for outdoors navigation, connect a GPS receiver in addition to loading the electronic map of your area.  Please refer to the www.gosendero.com site for a list of recommended GPS receivers.

PK platforms

PK users will find the navigation option in the Main menu, however, they will need to copy the GPS_6_0.exe file on the CF Card. Users will find the GPS_6_0.exe file on our BrailleNote download page at:  www.humanware.com/braillenote_support

Installation procedure:

1. Install KeySoft 8.0

2. Copy GPS_6_0.exe to the root of /, or /windows, or /flash disk, or /Compact Flash.

3. Then type N for Navigation option from the main menu.

Is this version compatible with earlier BrailleNote GPS versions?

The first time you run the GPS after installing KeySoft 8, the GPS program will automatically detect if a version prior to 6.0 exists on the system. You will hear the message: “An older version of GPS has been detected.  Please reset while holding down J K L (QT models) or dots 4 5 6 (BT models).  After resetting, an additional reset will be required. Afterwards, upon resetting a second time, you will be able to use your GPS version without issue.

If you have already upgraded to version 6.0, the system can continue to use it without any problems.

Users with previous GPS versions who want to upgrade to Sendero version 6.0, should purchase the upgrade at the standard price.

How is the trial period calculated?

The 90-day trial period starts when you run the GPS software for the first time. The system will announce the trial expiration date.

The software license will expire once the clock passes the cutoff date announced at time of registration or after using the software for 90 days, regardless of the current date of the system.

Users can find out when the trial version will end by going to Help/Information Center/About GPS.

What maps can I use?

During the 90-day trial period, users are allowed to download map areas of their choice.

What memory card size is required?

The memory card size needed depends on the region covered by the maps.  For maps covering the entire United States, we recommend a 16 GB card or greater.

How can I learn more about BrailleNote GPS?

In order to facilitate the trial experience, we are including the BrailleNote GPS tutorial in the BrailleNote Companion CD. The document is also available through the HumanWare or Sendero Web site.

Users are invited to take part in the Sendero chat sessions and planned Webinars. For information, please refer to the Sendero Web page.

How can I purchase a BrailleNote GPS license?

After this trial period, users can continue to enjoy their system at a preferred price. To order a BrailleNote GPS license after your trial period has expired, please contact your HumanWare representative.

Can I transfer the GPS system from one unit to another?

These will be handled on a case by case basis for a fee. Units will need to be first authorized to run KeySoft Verion 8.

# Printer support

Q: What printers are supported by the generic HP option?

- Current retail HP Printers and Legacy Devices 12/2008

- Deskjet 350C

- Deskjet 400, 400L

- Deskjet 450, 460 Series

- Deskjet 500 Series, 520, 540

- Deskjet 6XX series

- Deskjet 810C, 812C, 815C, 816C, 830, 832, 840, 841, 842, 843

- Deskjet 825, 845

- Deskjet 850, 855, 870, 880, 882, 890, 895

- Deskjet 9XX Series

- Deskjet 11XX Series, 12XX Series

- Deskjet 3810, 3816, 3818, 3819, 3820 Series, 3822, 3870

- Deskjet 51XX, 54XX, 55XX, 56XX, 57XX, 58XX, 59XX

- Deskjet 61XX, 65XX, 66XX, 68XX, 69XX

- Deskjet 5600 & 5100 & 5800

- Deskjet 6540 & 6520

- Deskjet 5740

- Deskjet 6840

- Deskjet 3320 & 3420 & 3325

- Deskjet 3600 & 3500

- Deskjet 3740 & 3840

- Deskjet 9300

- Deskjet 96XX

- Deskjet 98XX

- Deskjet D25XX

- Deskjet F42XX

- Officejet 3XX, 5XX, 6XX, 7XX

- Officejet 11XX

- Officejet 51XX, 61XX, 62XX, 63XX, 4100, 4105, 4200, 5500, 9100

- Officejet 71XX, 72XX, 73XX, 74XX

- Officejet 9x11

- Officejet d Series

- Officejet G Series

- Officejet J Series

- Officejet K Series

- Officejet Lx

- Officejet R Series

- Officejet t Series

- Officejet v Series

- Officejet Pro K Series

- Officejet Pro L Series

- Officejet H470

- Officejet J35XX, J36XX, J55XX, J57XX, J64XX,

- Officejet K71XX

- PSC 500, 7XX, 9XX

- PSC 15XX, 16XX

- PSC 21XX, 22XX, 23XX, 24XX, 25XX

- PhotoSmart 1XX, 2XX, 3XX, 4XX

- PhotoSmart 11XX, 12XX, 13XX

- PhotoSmart 25XX, 26XX, 27XX

- PhotoSmart 31XX, 32XX, 33XX

- PhotoSmart 71XX, 72XX, 73XX, 74XX, 75XX, 76XX, 77XX, 78XX, 79XX

- PhotoSmart 80XX, 81XX, 82XX, 84XX, 87XX

- PhotoSmart A3XX, A4XX, A5XX, A6XX, A7XX, A8XX

- PhotoSmart C31XX, C41XX, C42XX, C43XX, C44XX, C51XX, C52XX, C61XX, C62XX, C71XX, C72XX, C81XX

- PhotoSmart D50XX, D51XX, D53XX, D54XX, D61XX, D71XX, D72XX, D73XX, D74XX

- PhotoSmart Pro B83XX

- PhotoSmart P1000, P1100

- All Monochrome Laserjet Printers and MFPs (except LJ 1000, 1005, 1018, 1020, P10XX, M1005)

- All Color LaserJet Printers and MFPs (except LJ 1600, 2600)

- Apollo P2100 & P2150

- Apollo P2200 & P2250

- E-Printer e20

- Business InkJet 1000, 1100 Series, 1200 Series, 22XX Series, 2300 Series, 2600 Series, 2800 Series, 3000 Series

- Color Inkjet cp1700 Series

- HP 2000 Series, 2500 Series

- Apollo 2200, 2500, 26XX, P2XXX

[End]