Sendero’s I-Blue Getting Started Guide

Copyright 2005-2011

By Sendero Group

See the I-Blue User’s Guide for details about receiver specifications.

I-Blue Package

a. The first layer of the I-Blue box contains the receiver in its case with the battery already inserted.

b. Under the receiver, you will find a car power adapter and the AC charger and USB cable which can be used either with the AC adapter or plugged into a computer for charging. Make sure the device you plug into is turned on if you want the receiver to charge.

c. On the bottom of the box, you will find a print manual, warranty card and CD with computer connectivity software not relevant for connection to the BrailleNote, Braille Sense, or mobile phones.

I-Blue Description

Situate the receiver with the rubber pad on the table and the three ridges near the bottom edge closest to you. Those ridges are the status lights, orange, blue and green/red.

The Power switch is on the left edge toward the bottom, it has 3 positions. The first position or switch all the way down is Off, the second position is Navigate, the third is Navigate plus data logging mode.  You will hear a single beep when the switch is moved to the second or third position.  Both the second and third positions turn the receiver on and track equally.  If the user switches from Position 2 Navigation to Position 3 Data logging while connected to Sendero GPS, the connection to the hardware will be lost. One can reconnect easily with the reconnect command.

On the right side you will find the DC jack (mini USB type) rectangular slot for the power/interface cable. The connector slides in only one way. You may be able to feel that the edge of the plug, which is supposed to face up, is slightly narrower than the lower edge. The sleeve of the connector is also smoother on the top than the bottom. This is a friction fit connection.

Installing or removing the Battery:

Place the receiver on a flat surface so that the USB port is on the left and the switch is on the right side.  Feel along the back of the receiver near the bottom and you will find a lever.  It looks like something you may find on a remote control.  Hold the receiver with one hand, with your other hand lightly push the lever away from you and lift the battery cover.

The battery cover will lift away from the receiver.  You will not slide it.

Once the compartment is open, the battery contacts are in the right corner nearest you. Remove the battery from the plastic bag.  You will notice a seam in the center that runs the entire length of the battery.  This seam faces down when placed in the receiver.  If you feel both ends of the battery you will notice that one end is smooth across the edge and on the other end the corners look like they are cut off.  Place the battery into the receiver with the seam facing down and the  cut off edge towards the bottom of the receiver.  Slide the battery down the length of the receiver.  Push on the top edge of the battery until you feel the battery snap into place.

How to Remove the battery:

With the battery cover off and the receivers’ switch on the right side as described above, feel along the top edge of the receiver.  Your fingertip should come across a long thin edge running from left to right, there should be enough room for you to press your finger against the battery and lift it out.

Replacing the battery cover:

Feel the battery cover. Along the edge you will notice two small plastic tabs protruding out.  Place the battery cover on the receiver so the tabs are pointing away from you.  Slide the battery cover in the same direction until the tabs slide home.  Lower the other end of the battery cover until it snaps closed.

Important Points about the I-Blue Receiver

The receiver uses the highest sensitivity MTK GPS chip set and is WAAS/EGNOS enabled.  WAAS can increase accuracy to 10 feet. The Sendero GPS software indicates when the WAAS satellite is engaged.

Because of higher satellite sensitivity, the I-Blue may pick up signals even inside buildings although the heading information may not be reliable.

The receiver may work fine inside a pocket or purse, however, a clear view of the sky is always recommended for best results.

The battery life should be over 30 hours and it takes about 3-4 hours to fully charge. Coincidentally, the battery is the same type as the battery used in many Nokia phones such as the 6620 and 6682. The I-Blue battery is 3.7 volts and 1000 MAH.

The GPS receiver can alert you of status changes by playing a series of beeps.  Here is a short description to get you started.

- Switch the receiver to log mode: one beep.

- Switch the receiver to Nav mode: one beep.

- Pressing the POI button: three beeps.  This records a POI on the receiver, it has no effect on Sendero GPS user POIs.

- Acquire GPS position: two beeps.  You will hear two beeps when the GPS receiver acquires a position even if the Sendero GPS software is not loaded.

The Sendero I-Blue case offers two methods for attachment. It has a slide-on clip as well as Velcro wings which can be looped around the shoulder strap for extra security.

The strap and case:

The receiver is best situated on top of the shoulder. Clip it to the sliding shoulder strap if your device has one. Sendero offers a long shoulder strap that converts into  a short handle strap. Once you have the receiver case firmly clipped into the shoulder pad, wrap the Velcro wings around the strap. When you are using the receiver, situate the unit on top of your shoulder so it faces up.

In order to successfully use SenseNav in a live environment, the Sense notetaker must be paired with a GPS receiver.

[Please note: All Sense notetakers except the original Braille Sense have built-in Bluetooth capabilities, so a compatible Bluetooth adapter such as the Princeton(PTM-UBT3S) USB dongle will need to be inserted into this specific notetaker's USB port. The following steps assume this adapter or a similar one is already being used.]

Steps for pairing the receiver with the Sense notetaker

1. Turn on both the GPS receiver and the Sense notetaker.

2. Press BACKSPACE-DOTS-3-4-5-6(BT) or F9(QT) until Bluetooth is enabled. For the Braille Sense Original, insert the USB Bluetooth adapter  so that it displays, "Bluetooth card inserted." If Bluetooth is already enabled, this hot key will turn it off.

3. Press F1(BT) or WINDOWS(QT) followed by L to open the Bluetooth Manager. The Sense notetaker will immediately begin scanning for available Bluetooth devices in range. After several seconds the Sense notetaker will display a list of available Bluetooth devices. [Please note: If multiple GPS receivers are found, there will not be an accurate way of determining which is the correct receiver.]

4. Press UP or DOWN arrow to select the GPS receiver (IBTa+) then press ENTER to begin scanning for its services. When the service list is displayed, press ENTER on ActiveSync to establish a connection.

5. Once the ActiveSync connection has been established, close the Bluetooth manager with SPACE-Z(BT) or ALT-F4(QT).

At this point the Sense notetaker and the GPS receiver know about each other and will work together when they are in range. [Please note: If the Sense notetaker is ever reset back to its factory defaults from a revive or a firmware upgrade, the pairing information will be lost. This means that the above steps will need to be performed prior to using SenseNav in a live environment.]

++ Launching and configuring SenseNav for the first time

The first time SenseNav is launched, a few items will need to be configured. These items include the location of the maps, the GPS receiver connection type, and the POI author name.

-- Installing the memory card and launching for the first time

1. Power on both the GPS receiver and the Sense notetaker. [Please note: for best performance, we recommend that the GPS receiver be powered on a couple minutes prior to launching SenseNav.]

2. Press F1(BT) or WINDOWS(QT) to focus the main program menu. The Sense notetaker will display, "File manager (f)."

3. Insert the supplied memory card into the appropriate slot and wait for the confirmation sound to play.

4. Press BACKSPACE-DOTS-3-4-5-6(BT) or F9(QT) until Bluetooth is enabled. For the Braille Sense Original, insert the USB Bluetooth adapter  so that it displays, "Bluetooth card inserted." If Bluetooth is already enabled, this hot key will turn it off.

5. Press V or arrow down to " Sense Navigation" then press ENTER to launch it.

When SenseNav launches for the first time, the map data location and the GPS receiver type will not be set.

-- Map and POI data configuration

1. Press ENTER-O(BT) or ALT-O(QT). SenseNav will then ask you to choose a map folder.

2. You will be placed on one of the memory storage devices such as the Flashdisk or SD card. If your maps are located in a place other than the default location, press BACKSPACE until the correct storage device is displayed. Press ENTER on that device and then navigate to the correct folder using a combination of the ENTER key and the arrow keys. It is important that you not press ENTER once you reach the folder containing your map files.

3. Press SPACE to select the map folder, then press ENTER and SenseNav will load the maps contained in the selected map folder.

[Please note: The authorization key named SenseGPS.key must either be located in the root of the memory card, or inside the map folder itself. If it is not, SenseNav will not FUNCTION. By default, the authorization key is already present in the root of the memory card, so this should not be an issue as long as the key has not been removed or renamed.]

Once SenseNav has been authorized and the maps have loaded successfully, it will only operate in virtual mode. In order to operate in live mode, the GPS receiver connection type will need to be set. By default, SenseNav is set to no GPS receiver.

-- Setting the GPS receiver type

1. From within SenseNav press BACKSPACE-G(BT) or CONTROL-G(QT). This will open the GPS receiver selection dialog box.

2. The GPS receiver is set to none by default. Press SPACE or BACKSPACE until the correct type of GPS receiver connection is selected. I.E. Bluetooth (ActiveSync) or Internal GPS.

3. Press ENTER and SenseNav will look for the GPS receiver and connect to it.

For Technical Support on the Braille Sense and GPS, contact the dealer from whom you purchased your product. Sendero Group direct customers call 1-888-757-6810, extension 111.