Sendero’s I-Blue GPS receiver Getting Started Guide

Copyright 2005-2011

By Sendero Group

See the I-Blue User’s Guide for details about receiver specifications.

I-Blue Package

a. The first layer of the Senedero GPS box contains the receiver in its case with the battery already inserted.

b. Under the receiver, you will find a car power adapter and the AC charger and USB cable which can be used either with the AC adapter or plugged into a computer for charging. Make sure the device you plug into is turned on if you want the receiver to charge.

c. On the bottom of the box, you will find a print manual, warranty card and CD with computer connectivity software not relevant for connection to the BrailleNote, Braille Sense, or mobile phones.

I-Blue Description

Situate the receiver with the rubber pad on the table and the three ridges near the bottom edge closest to you. Those ridges are the status lights, orange, blue and green/red.

The Power switch is on the left edge toward the bottom, it has 3 positions. The first position or switch all the way down is Off, the second position is Navigate, the third is Navigate plus data logging mode.  You will hear a single beep when the switch is moved to the second or third position.  Both the second and third positions turn the receiver on and track equally.  If the user switches from Position 2 Navigation to Position 3 Data logging while connected to Sendero GPS, the connection to the hardware will be lost. One can reconnect easily with the reconnect command.

On the right side you will find the DC jack (mini USB type) rectangular slot for the power/interface cable. The connector plugs in only one way. You may be able to feel that the edge of the plug, which is supposed to face up, is slightly narrower than the lower edge. The sleeve of the connector is also smoother on the top than the bottom. This is a friction fit connection.

Installing or removing the Battery:

Place the receiver on a flat surface so that the USB port is on the left and the switch is on the right side.  Feel along the back of the receiver near the bottom and you will find a lever.  It looks like something you may find on a remote control.  Hold the receiver with one hand, with your other hand lightly push the lever away from you and lift the battery cover.

The battery cover will lift away from the receiver.  You will not slide it.

Once the compartment is open, the battery contacts are in the right corner nearest you. Remove the battery from the plastic bag.  You will notice a seam in the center that runs the entire length of the battery.  This seam faces down when placed in the receiver.  If you feel both ends of the battery you will notice that one end is smooth across the edge and on the other end the corners look like they are cut off.  Place the battery into the receiver with the seam facing down and the  cut off edge towards the bottom of the receiver.  Slide the battery down the length of the receiver.  Push on the top edge of the battery until you feel the battery snap into place.

How to Remove the battery:

With the battery cover off and the receivers’ switch on the right side as described above, feel along the top edge of the receiver.  Your fingertip should come across a long thin edge running from left to right, there should be enough room for you to press your finger against the battery and lift it out.

Replacing the battery cover:

Feel the battery cover. Along the edge you will notice two small plastic tabs protruding out.  Place the battery cover on the receiver so the tabs are pointing away from you.  Slide the battery cover in the same direction until the tabs slide home.  Lower the other end of the battery cover until it snaps closed.

Important Points about the I-Blue Receiver

The receiver uses the highest sensitivity MTK GPS chip set and is WAAS/EGNOS enabled.  WAAS can increase accuracy to 10 feet. The Sendero GPS software indicates when the WAAS satellite is engaged.

Because of higher satellite sensitivity, the I-Blue may pick up signals even inside buildings although the heading information may not be reliable.

The receiver may work fine inside a pocket or purse, however, a clear view of the sky is always recommended for best results.

The battery life should be over 30 hours and it takes about 3-4 hours to fully charge. The I-Blue battery is 3.7 volts and 1000 MAH.

The GPS receiver can alert you of status changes by playing a series of beeps.  Here is a short description to get you started.

- Switch the receiver to log mode: one beep.

- Switch the receiver to Nav mode: one beep.

- Pressing the POI button: three beeps.  This records a POI on the receiver, it has no effect on Sendero GPS user POIs.

- Acquire GPS position: two beeps.  You will hear two beeps when the GPS receiver acquires a position even if the Sendero GPS software is not loaded.

The Sendero I-Blue case offers two methods for attachment. It has a slide-on clip as well as Velcro wings which can be looped around the shoulder strap for extra security.

The strap and case:

The receiver is best situated on top of the shoulder. Clip it to the sliding shoulder strap if your device has one. Sendero offers a long shoulder strap that converts into  a short handle strap. Once you have the receiver case firmly clipped into the shoulder pad, wrap the Velcro wings around the strap. When you are using the receiver, situate the unit on top of your shoulder so it faces up.

Configuring the BrailleNote Apex, PK or mPower for a Bluetooth GPS receiver with KS7 or higher

Summary of connection sequence:

* Options plus C then B gets you to the Bluetooth Menu. (BrailleNote Options followed by C for Connectivity and B for Bluetooth)

* Select the receiver and Activate Active sync on the PK and mPower, activate Serial for the Apex.  Note: Do not activate the printer service.

Installing the Sendero GPS software:

See the GPS ReadMe or BrailleNote manual for details on installing the Sendero GPS software. If you purchased your BrailleNote and GPS together, the software is probably already installed and the receiver activated.

I-Blue Connection Details:

- Charge the I-Blue for 4 hours and then turn it on with the Power switch on the left edge. Up is On. See the receiver layout description below.

- From the BrailleNote Main Menu, Press SPACE with O on the Braille keyboard or FUNCTION with O on the QWERTY keyboard for Options, C for the Connectivity Menu, and B for Bluetooth, Y for Yes (Bluetooth On), and Y for Search for Bluetooth devices. Beeps will indicate searching.  Press Enter on the IBT A+ selection. Space through the list of services and press Enter on ActiveSync for the PK or mPower and Serial for the Apex. Select Yes for Pairing. Make sure all other ActiveSync connections are closed before trying to use the GPS Bluetooth.

- Enter 0000 for the authentication code when asked and press enter. Use a number sign before the zeros for Grade 1 or 2 Braille. If it doesn't pair, repeat this step.

- Space through the list until you see ActiveSync and press Enter and A to Activate. Make sure you see “ActiveSync Active.” If you don’t see the word, “Active”, repeat the steps to activate ActiveSync. If it still won't activate, turn the receiver off and on and turn the BrailleNote Bluetooth off and on.

- Go to the BrailleNote Main menu, wait 10 seconds and then select the GPS Navigation application. There is a pause followed by an announcement of the number of maps loaded and the receiver status. If you hear Bluetooth not detected, go back to the main menu, wait a bit and try again. Otherwise, return to the Connectivity Menu and check to see that ActiveSync is indeed Active.

- From the Main Menu, press N for Navigation. Once Sendero GPS loads, press O for GPS Options, G for GPS receiver and B for Bluetooth. If you received a new BrailleNote and GPS from Sendero, the receiver should already be paired and ready to go and these latter steps are not necessary.

Pairing with the Bluetooth Receiver

It is best to be familiar with Bluetooth pairing as the GPS receiver may become unpaired for several reasons like resetting or perhaps when pairing with other Bluetooth devices. It is not hard to pair once you become familiar with the drill.

** If you open the GPS application without first turning on the I-Blue, you will receive an error message indicating you should turn it on and/or reactivate the Bluetooth connection in the BrailleNote Options menu. Unless you have reset the BrailleNote, you should return to the Main Menu, turn on the I-Blue and load GPS again. If you turn the BrailleNote off while in the midst of the GPS application, you will have to turn it back on, go to the Main Menu and launch the GPS application. If all the Connectivity settings are correct, try waiting 10 or 15 seconds at the Main Menu before loading the GPS application.

The I-Blue may stay connected with the BrailleNote 30 or 40 feet away. If the receiver and BrailleNote connection is lost, it will usually be necessary to go to the Main Menu and reload the GPS again after the units are within range of each other.  Note: you can press BACKSPACE with R on the BT and CONTROL with R on the QT to try and reconnect to the receiver.

If you connect the BrailleNote to another Bluetooth device like a cell phone, you may first have to deactivate the I-Blue or reset the BrailleNote. It would then be necessary when you want to run the I-Blue GPS again to reactivate the PK or mPower for the ActiveSync GPS connection.

When you reset, the Bluetooth connection must be reestablished as follows:

From the Main Menu, press Space O for Options, C for Connectivity Menu and B for Bluetooth and Y for Yes to Turn Bluetooth on if it isn’t already. Say No to Searching for devices and space through the list until you see “IBT A+” GPS and press Enter.

- Space through the list and press Enter on ActiveSync.

- You will see the message, “remove pairing with IBT A+ or activate ActiveSync.  Press A to activate the receiver. Make sure it says ActiveSync Active.

- Return to the Main Menu, wait 10 seconds and open the GPS application.

Note: If you have to do a hard reset, or your BrailleNotes battery goes flat, it will be necessary to search for devices and pair with the receiver.

For Technical Support on the BrailleNote and GPS, contact the dealer from whom you purchased your product. Sendero Group direct customers call 1-888-757-6810, extension 111. HumanWare customers call 1-800-722-3393.