Sendero GPS, Version 3.5 ReadMe, May 15, 2006

Copyright 2002 2006, Sendero Group, LLC

This Read Me contains last minute information for GPS Version 3.5 for the Keysoft 6 and 7 BrailleNotes, mPower and PK as well as important information you should read before using the products.  Documents listing changes between all versions are in a file posted on the Sendero Download site as well as on the media you receive with your product. See the respective User manuals for BrailleNote or GPS questions and instructions. Listen to the Audio Tutorial on the Compact Flash card or CD for a play-by-play of basic GPS functions. Optional GPS training courseware is available for purchase in Braille or print as is a Braille GPS manual. A document called GPS Accuracy is in the Documents folder. See the Getting Started document in the Receiver folder for specifics on the GPS receiver you are using.

Keysoft GPS compatibility list:

Keysoft 5 or below runs GPS 1.0 to GPS 3.0

Keysoft 6 or greater runs GPS 3.1 to GPS 3.5 with a few exceptions.
Keysoft 7 with GPS 3.1 or 3.3 will work but is not recommended. There may be issues with Eloquence on the mPower.

Keysoft 7 PK requires a special GPS 3.3 version.

Keysoft 7 and GPS 3.5 work beautifully on all models,, Classic, mPower and PK.

Software Installation:

If you purchased the GPS at the same time as your VoiceNote or BrailleNote, the software should already be installed. If you purchased the GPS separately, or if you are installing an upgrade, see your BrailleNote or VoiceNote user manual for instructions on installing software. Make sure you have Keysoft 6.1 or later installed. The latest version of Keysoft is highly recommended. See the BrailleNote manual for a complete explanation how to install applications on the BrailleNote or we have a brief description at the end of this ReadMe.

Key 3.5 things to know:

* Map and POI data are new in 3.5. Previous commercial data files will not work with 3.5. Your User.poi file will work with any version. Put all map and POI files in the same folder. If you put the old files in the folder, you will hear an error message, “unable to load xxx maps or POIs.”

* Map and POI data are in the same folder.

* There are no spaces in state names. When you enter an address and pick the state, scroll through the list or type the first couple letters of the state name. States like NorthCarolina are all one word.

* Maps can be downloaded with a high speed connection from MySendero.com. Otherwise, the Map files are available on DVDs.

* Under KS7, GPS is now under the Navigation Menu. The hot key from the Main Menu is N. In Keysoft 6 and below, the hot key was G. You can use NG or NN from the Main Menu to go to GPS and start the program.

* When inserting cards in the Compact Flash or Storage slots, wait 5 seconds or so after they are recognized before loading the GPS application. If you hear the message that no maps were loaded, go back to the main menu and start the GPS application again.

* If you get a Bluetooth not detected message, first make sure the receiver is turned on amd activated in the Bluetooth Connectivity menu. See the Getting Started document in the Receivers folder. It may be necessary to  simply go back to the Main Menu, wait 10 seconds and open the GPS application again.

* It is highly recommended that you remove memory cards when your BrailleNote is turned off or before doing a reset. This is particularly important with 8 GB compact flash cards. They may be damaged if they are not removed.

* If you are operating in a route and change to a new destination, strange things may happen to the original route. It is recommended that you calculate a new route if you establish a new destination.

* We recommend that you do not reverse a vehicle route because freeways and one-way streets may not be respected. If you wish to reverse a vehicule route, do so and then immediately recalculate the route.

* It is recommended that you rerecord manual waypoints in 3.5. New functionality was added to manual route recording in version 3.3 to include description and turn fields. Although older routes may work fine, if you are experiencing problems, rerecord those routes in 3.5.

* When the GPS program loads, it opens every POI file in the active folder. By having an image of the POI file in memory, searches and Look-Around functions are grately sped up. Although you can load up to 75 POI files from one folder on the mPower or PK, be aware that this is using up a lot of memory. We recommend that you limit the number of POI files to those you actively need. The maximum on the Classic is 12 maps and 30 POI files. There are over 120 POI files for the U.S. If you wish to calculate a route across the U.S., at most load the Core POI files so you have less than 75.

* When data from the GPS program are appended to the clipboard and pasted into a BrailleNote document, be aware computer Braille is being used. You would want to open this in a text document and not a Grade II Braille document.

Other Important Items:

The number of lanes indicated by pressing Chord C is less evident in version 3.5 maps than in the previous maps.

The GPS Receiver Option defaults to Serial GPS for Classic BrailleNotes with Keysoft 6 or 7. This works for the Delorme Earthmate and most other brands. If you have any of the Magellan models, especially the 310 or Sportrak, select the Magellan receiver option. The PK and mPower BrailleNote GPS defaults to a Bluetooth receiver.

The commercial points of interest for your state are located on the 1 GB Compact Flash card supplied with your GPS package in the Maps folder and important documents are in their respective folders. The POI file names begin with the numerals 35. The map files are also in the Maps folder beginning with the state name. If maps or POIs do not appear when you load the program, go to the GPS Options menu to point to the relevant Map/POI folder.

If you purchased a high capacity compact flash card, the maps may be organized regionally with the respective POI files in each folder.

Under Keysoft 7 with GPS version 3.5, there is a slightly different behavior after one presses a command. You will not hear the title of the command announced. You will hear the information generated by that command. This “key-echo” functionality was available under Keysoft 6 and will likely return in a future release. For now, there is no confirmation of the command.

In order to automatically hear the street names of some types of roads in the UK map data, it is necessary to set the GPS Options Street Names item to “Long.” The default is “short” street names. This may also be necessary in other countries as well. Note the Chord C command gives both the short and long street names no matter what you have the Street Name set to.

In 3.5, there is a new feature to announce what side of the street the final address is on. If you set the destination based upon a POI in a country outside the U.S. or Canada, this information may not be announced because the POI address syntax is different. For the POI to trigger the side of the street information, the POI name must be followed by a comma, the street number and then the street name.

Known Issues

* For the ten states that have more than one section, like California with 4 sections or Florida with two, you must load the first state section which contains the .CTY file, 35California_core.cty

If for some reason you wish to only have Northern California loaded, you would need to put the 3 files for California4 plus the .CTY file in the maps folder plus any POI files you need like California4_35.bpi.

There are two POI files for most states and provinces. One has the word Core in it and the other has the word Bonus. The file called bonus has points which have not yet been categorized. This means that POIs like restaurants may be in the Bonus file and not categorized in the core file. Using the “All” category option will give access to the restaurants in this situation. Remember that these maps and POIs will not work with versions previous to 3.5.

In the unlikely case that you are listening to music while using the GPS and wish to lower the volume on the media player, Space with dots 1-3 does not work. You can either leave the GPS program or go into the GPS Options menu. We of course advise against any distraction while using the GPS program as a pedestrian.

If you are using a serial GPS receiver with the mPower, it will be necessary to turn the receiver off before a reset on the mPower can be performed successfully.

The BrailleNote “go to page” command does not work from within the GPS Menu User Guide. It will work if you open the GPS manual from within the BrailleNote Book Reader application.

If you are experiencing slow response to commands, switch the Look-Around mode off while performing these other functions. It will remain in the status you choose until you do a hard reset, at which time it will revert to its default status. Look-Around scans for nearby POIs and intersections every 12 seconds and this takes some of the BrailleNote’s processing power. Besides turning Look-Around off, you can also make adjustments to look for POIs or Intersections only. See the manual for details. This is normally not an issue on the Keysoft 6 or higher systems.

In the case of a shopping mall, there could be dozens of POIs, which have different names but the same lat/lon based on a single geocoded address for that shopping center. The Look-Around mode will trigger on the first of these multiple points at the same lat/lon but it won’t automatically announce the rest of the points at that lat/lon. If you wish to hear all the POI names, you must press F or Chord F to generate a list.

The indexing of some towns in the map data may require a round about way of finding an address match. It is best to assume a town is in the database and to use the suggestions in the manual to find a means of pulling that town up if you don’t find it using a normal Address Look Up. In London for example, some addresses must be matched by putting in the council name rather than London as the city name. Medium to large cities may also be in the Points of Interest database and you can place your virtual position in the vicinity of a city by doing a POI search in this way.

There is no distance limit placed on route calculations, however, there is a limit of 2500 route waypoints placed on the BrailleNote Classic and 10000 on the PK and mPower.

The limit on POI searches is 2500 on the Classic and 5000 on the PK and mPower. The User.poi file is the last to be searched. Therefore, if you happen to have a search return more than the limit of points, the User points will not be displayed, even if they are close by.

Collective User POI files are available as free downloads from the Sendero site. These files have a .BPI extension just like the commercial POI files. These files with user points must have the word “user” in the file name. Otherwise, when you select the User category for a POI search, these points will not be found.

Should the system ever stop responding or if you need to abort a long route calculation, press reset. Avoid excessive searches like all points within 1000 miles.

If you ask for a POI search and immediately afterwards the automatic “look-around” function triggers a nearby POI, the current point announced by your POI search may be replaced by the Look-Around POI.

When searching for a city or street, which contains an accented character, it is necessary to enter that character exactly in order to find the match. You can enter the first part of a city name and find matches containing that string at the beginning of the city name.

When using Grade II, the “\” character will be spelled out as Backslash when a Map folder or POI folder name is displayed. Also, if you are entering a city name during an address match, be aware of single letter contractions. For example, if you only put the letter “c” it will be interpreted as the contracted word, “can.” Use a letter sign or additional letters to avoid this confusion.

Some of the duplicate commands on the Braille style keyboard do not work in Grade 1 or Computer Braille modes.

With Computer Braille the dot 4 Find command does not work nor do the dot 3 or dot 2 for recording a waypoint turningpoint.

With Grade 1 Braille the keyboard lock 123456 command, dot 5 Heading and Dot 2 Current Waypoint commands do not function.

If you turn on to a U-shaped street in Explore mode and press the Forward command, you will probably hear that you have looped back to the intersection where you started. If you continue along the street that spans the mouth of the U, it is likely that the next intersection ahead will be the other end of the U. There is no way to travel on the U street itself unless it has intersecting segments.

There are occasional streets, which split off in multiple directions with the same name. This may happen at a Y in a road or at a roundabout or intersection with an unusual island. The result might be the announcement of a street being, ahead, behind and to the left or right, all at the same time.

When doing an address match, it is best, but not always necessary, to enter the street name without a prefix like Northwest or a suffix like Avenue. You will receive a list of options once the matches are found. In Portland Oregon, it is necessary to enter the prefix before numbered streets, for example, SE 12th. If you just put in 12th, no match will be made. Add the two-letter abbreviation before 12th, and it works. This is the exception to the rule however.

Map Data

See the GPS manual for complete instructions on adding and subtracting maps and POIs from your Compact Flash or other storage cards. Make sure all files for each state are in the folder and not in a sub folder for that state. States with multiple segments will have one extra file with the entire state city list. Do not transfer maps from the Storage card to the Compact Flash card on the BrailleNote as this can take a very long time and there is the potential for a crash. Maps can also be stored on compatible hard drives or on an SD card.

There is a Multiple City folder containing listings for the ten states with multiple sections. If you wish to have for example only 1 of the four California maps loaded and need to find out which map your California city is in, do a search on that city in the California Multiple City document. The relevant map, like CA1 will be listed after the city name.

Note: You must have the California.cty file included in the map folder for one or more of the California map segments to work properly. This means however that any city in the state can be matched even if it is not in the individual map segment loaded. To avoid confusion, we recommend against loading individual map segments.

When entering the first few letters of a state name in the Address State field like AL for Alabama, you will need a letter-sign before the letters AL on the BT keyboard to distinguish it from the contraction for “also.”

Points of Interest:

To learn about the available POI categories, press Chord F on your BrailleNote and enter the search distances. The categories will then be presented and you can arrow through them. They are not in alphabetical order. The more popular categories appear at the top of the list. Once you know what the category options are, it is best to select a category by pressing its first letter. A POI list is also available within the Look Around mode, Chord M.

Note: Washington DC is within the Maryland points of interest file.

Should you wish to change the way a category name is pronounced, you can use the BrailleNote’s pronunciation dictionary. For example, rename Gas to be pronounced as Fuel or Petrol.

By default, the GPS program first looks for map and POI files in the Maps folder on the PC card and second on the Compact Flash card. If you wish to use other folders, you must set the desired map folder under the GPS Options menu.

See the Map section in the User’s manual for details about copying and using maps. We recommend transferring the maps directly from a Compact Flash card on the PC to the BrailleNote rather than using Active Sync or by transferring from the BrailleNote’s PC card.

The entire U.S. maps and POIs total just over 6 GB. This means you can easily put them on two 4GB cards or on a single 8 GB card. As stated elsewhere, you should make it a practice to load only the maps you regularly use in a folder. However, for purposes of driving or flying across country, you could divide the U.S. in to a few folders depending upon the card size available to you. If you purchase the 8 GB option from Sendero, the maps are divided into regional folders for you.

Installing Applications on the BrailleNote

1. Copy your serialized  GPS.exe file on to the compact flash card. If the file was zipped up when you received it, make sure and unzip it first. If the extension had to be changed from Zip to another extension, make sure and rename it back to a Zip extension before trying to unzip it.

2. Press and release the Reset with the 3 keys held down, dots 123 on the Braille keyboard or the SDF keys held together on the QT keyboard.  You will hear a beep after releasing the Reset. Wait for the subsequent chime before you release the 3 keys. The PK uses C3 with C4 as the equivalent of the Reset button.

Note, if you don’t hold the 3 keys down long enough, normally around ten seconds, you will get a message that some files are in use and you cannot install over them. Do the reset again and hold those 3 keys longer.

3. From the Main Menu, press U for Utilities, A for Applications and then select the Compact Flash card and select the GPS application. When it says, Command Prompt, just press Enter and continue pressing Enter through the other prompts. If you receive a message saying that the existing files are newer than the version you are installing, ignore this and press Enter to overwrite the former files with the new ones. At the end you will be prompted to press Reset with no keys held down.

Thank you for purchasing Sendero GPS and for reading this document. Happy travels.

For technical support, contact the dealer from whom you purchased your GPS. Direct US Sendero customers call, 1-888-757-6810.

Please read other documents for a complete understanding of the BrailleNote GPS software including the manual, the audio tutorial and the optional GPS CourseWare.