setup guide   	GPS 10

wireless GPS receiver

Unpacking the GPS 10

1. Remove the contents of the GPS 10 Package and compare the contents with the list of items on the 		carton. If any items are missing, contact your Garmin Dealer.

2. 	Identify the GPS 10 wireless GPS receiver, Velcro mounting patch and 12 Volt DC Cigarette Lighter 		Charging Adapter. If you have purchased the GPS 10 Deluxe, the package also contains installation 		CDs for Garmin Que™and nRoute™ GPS navigation software and MapSource City Select® detailed 		mapping data.

Locating the GPS 10 in a vehicle

1. Determine the desired mounting location for the GPS 10 receiver.

• Place the GPS 10 in a location with a clear view of the sky in order to receive GPS satellite signals 		   and within the limits of the Charging Cable when inserted in the vehicle cigarette lighter.

• The GPS 10 which is equipped with Bluetooth® wireless technology, must be within approximately 		  30 feet (10.5 meters) of the Bluetooth wireless technology equipped PC or PDA in order to be in		  communication.

2. Securely mount the GPS 10 in or on the vehicle: If placing on the outside of a vehicle, the GPS 10 		must be placed on a smooth horizontal surface that is compatible with the built-in magnet in the 		GPS 10. If placing inside a vehicle, use the Velcro® patch provided with the unit. Remove the adhesive 		backing from one side of the Velcro patch and position over bottom of the GPS 10. Then remove the   	    adhesive backing from the other side of the patch and carefully press the unit onto the desired 		mounting  location.  After installation, separate the two Velcro strips to remove the GPS 10 from the 		vehicle.

Note: It is recommended that the GPS 10 be removed from the exterior of the vehicle when left unat-		tended for a prolonged period of time.

Powering Up the GPS 10 Receiver

1. The GPS 10 receiver contains an internal battery which must be charged before use. Charging requires 			approximately 3 hours and the battery will provide operating power for approximately 10 hours. While 			you are charging the GPS 10, you can operate the receiver using auxiliary power from the charging 			adapter.

2.	With the GPS 10 mounted in place in the vehicle, connect the cigarette lighter adapter end of the 			charging cable into the vehicle’s cigarette lighter. The LED on the front of the GPS 10 flashes yellow-green, 		indicating charging is in progress. When fully charged the LED displays as steady yellow-green. When 			recharging is required, the LED displays as flashing orange.

Note: If power to the vehicle cigarette lighter is turned off whenever the ignition switch is turned off, 			it may be more efficient to charge the GPS 10 while driving and use the auxiliary power provided by the 		charging adapter to operate the GPS 10. This is also the best method to use if you desire to setup 			your GPS 10 and a PC or PDA for the first time without waiting for the GPS 10 battery to charge.

3.	Power up the GPS 10 by pressing the POWER button on the top of the unit. A flashing blue LED displays 		to indicate that the unit is operational.

To communicate with a PC or Pocket PC or other Bluetooth® wireless technology enabled mobile device:

Before you can use the GPS 10 for navigation it is necessary to establish a connection between the GPS 10 receiver and a Bluetooth wireless technology enabled device (PC, Pocket PC, or other interface device).  A PC or Pocket PC must have factory installed Bluetooth wireless technology or a Bluetooth wireless technology adapter. Interface devices must be Bluetooth wireless technology enabled and be compatible with NMEA 0183 Version 3.0 Approved Sentences.

(Refer to the product packaging for minimum PC and Pocket PC requirements and to the GPS 10 Technical Specification posted on the Garmin web site, www.garmin.com, for specific interfacing information for other devices.)

1.	Turn on the GPS 10 receiver. The blue status LED will flash rapidly (once per second) to indicate that 		the receiver is operating.

2.	Turn on your mobile device (PC Laptop or Pocket PC) and perform a Bluetooth device discovery.

3.	Pair and bond with the GPS 10 by entering the passkey number: 1234.  (After the GPS 10 and mobile 		device have connected, the blue status LED slows the flash to once every other second)

4.	Determine which communication port (COM Port) number your mobile device is using for outbound 		serial communication. Refer to the documents provided with your Bluetooth hardware/software for 		this information.

5.	Access the GPS Navigation application on the mobile device and set the COM Port for the device by 		following instructions in the Garmin nRoute or Garmin Que Setup Guide. At start-up, Garmin nRoute 		and Garmin Que applications allow you to view GPS position fix status information.

Anytime that the link between your mobile device and the GPS 10 has been broken for fifteen min-		utes, the GPS 10 will turn Off to conserve power. If you are using switched external power (12 V DC 		from a switched power source such as a cigarette lighter controlled by the ignition switch) to operate 		the GPS 10, the GPS 10 automatically powers On when the switch is activated. If external power 		is not switched, use Power Button to turn the GPS 10 back On after it has automatically turned Off.

© Copyright 2005 Garmin Ltd, or its subsidiaries

All rights reserved. Except as expressly provided herein, no part of this manual may be reproduced, copied, transmitted, disseminated, downloaded or stored in any storage medium, for any purpose without the express prior written consent of Garmin. Garmin hereby grants permission to download a single copy of this manual onto a hard drive or other electronic storage medium to be viewed and to print one copy of this manual or of any revision hereto, provided that such electronic or printed copy of this manual must contain the complete text of this copyright notice and provided further that any unauthorized commercial distribution of this manual or any revision hereto is strictly prohibited.

Information in this document is subject to change without notice. Garmin reserves the right to change or improve its products and to make changes in the content without obligation to notify any person or organization of such changes or improvements. Visit the Garmin web site (www.garmin.com) for current updates and supplemental information concerning the use and operation of this and other Garmin products.

Garmin®, TracBack® and AutoLocate® are registered trademarks of Garmin Ltd. or its subsidiaries and may not be used without the express permission of Garmin.  Bluetooth is a certification mark of Bluetooth SIG, Inc.

Safety Warnings and Cautions

WARNING: For use in vehicles, it is the sole responsibility of the owner/operator of a vehicle with the GPS 10 installed to place and secure the device so that it will not interfere with the vehicle operating controls and safety devices, obstruct the driver’s view of driving conditions, or cause damage or personal injury in the event of an accident.

Do not mount the unit over airbag panels or in the field of airbag deployment. Airbags expand with a rapid force that can propel objects in their path towards the vehicle driver or passengers causing possible injury. Refer to airbag safety precautions contained in the vehicle owner’s manual.

Do not mount the unit in a place where the driver or passengers are likely to impact it in a collision. The mounting hardware provided by Garmin is not warranted against collision damage or the consequences thereof.

WARNING: When used in vehicles, it is the sole responsibility of the driver of the vehicle to operate the vehicle in a safe manner, maintain full surveillance of all driving conditions at all times, and not become distracted by the unit to the exclusion of safe driving practices. It is unsafe to operate the controls of the unit  or other connected devices while you are driving. Failure by the driver of a vehicle equipped with a GPS 10 installed to pay full attention to operation of the vehicle and road conditions while the vehicle is in motion could result in an accident with property damage and personal injury.

CAUTION: Use the GPS 10 at your own risk.  To reduce the risk of unsafe operation, carefully review and understand all aspects of this Owner’s Manual — and thoroughly practice operation using the Demo Mode prior to actual use. When in actual use, carefully compare indications from the GPS 10 to all available navigation sources, including the information from other NAVAIDs, visual sightings, charts, etc.  For safety, always resolve any discrepancies before continuing navigation.

CAUTION: It is the user’s responsibility to use this product prudently.  This product is intended to be used only as a navigational aid and must not be used for any purpose requiring precise measurement of direction, distance, location or topography. This product should not be used as an aid to determine ground proximity for aircraft navigation.

CAUTION: The electronic chart is an aid to navigation and is designed to facilitate the use of authorized government charts, not replace them. Only official government charts and notices to mariners contain all information needed for safe navigation – and, as always, the user is responsible for their prudent use.

MAP DATA INFORMATION:  One of the goals of Garmin is to provide customers with the most complete and accurate cartography that is available to us at a reasonable cost. We use a combination of governmental and private data sources, which we identify as required in product literature and copyright messages displayed to the consumer. Virtually all data sources contain inaccurate or incomplete data to some degree.  This is particularly true outside the United States, where complete and accurate digital data is either not available or prohibitively expensive.

WARNING: The Global Positioning System (GPS) is operated by the United States government, which is solely responsible for its accuracy and maintenance. The system is subject to changes which could affect the accuracy and performance of all GPS equipment. Although the Garmin GPS 10 is a precision electronic NAVigation AID (NAVAID), any NAVAID can be misused or misinterpreted and, therefore, become unsafe.

WARNING: This product, its packaging, and its components contain chemicals known to the State of California to cause cancer, birth defects, or reproductive harm. This Notice is being provided in accordance with California’s Proposition 65. If you have any questions or would like additional information, please refer to our web site at http://www.garmin.com/prop65.

Specifications

GPS Performance

Receiver:		WAAS enabled, 12 parallel channel

Acquisition Time:*	Reacquisition: Less than 2 seconds

Warm: Approximately 15 seconds

Cold: Approximately 45 seconds

AutoLocate™:  Approximately 5 minutes

SkySearch:  Approximately 5 minutes

Update Rate:	1 to 900 seconds between updates:  programmable in 1 second increments

GPS Accuracy

Position: 		< 15 meters (49 feet), 95% typical **

Velocity:		0.1 knot RMS steady state steady state

DGPS (WAAS) Accuracy

Position:		3 meters (10 feet), 95% typical**

Velocity:		0.1 knot RMS steady state steady state

Dynamics:			999 knots, 6g’s

Temperature

Operating:		-10o to +60oC - internal power,  -30o to +60oC - external power

Charging:		+5o to +45oC

Storage:		-40o to +90oC

Interface Protocol:		NMEA 0183, sentences GGA/GSA/GSV/GLL/RMC/VTG

Physical

Size:		.75” H x 1.77”W x 3.46” D (19 x 45 x 88 mm)

Weight:		2.8 ounces (80 g)

Case:		Waterproof to IEC 60529 ipx7 standards

Power Supply:	Rechargeable Lithium Ion

12 hours (typical use)

12 V DC Adapter/Charging Cable

Auto Power On when connected to a switched external power source

Wireless Interface:		Connects to Bluetooth wireless technology (Class 2) enabled Pocket PC or PC

* Warm = all data known, Cold =time and almanac known, Autolocate = almanac known, position and time unknown, SkySearch = no data known.

** Subject to accuracy degradation to 100m 2DRMS under the U.S. Department of Defense imposed Selective Availability Program.

FCC Compliance Information

The GPS 10 complies with Part 15 of the FCC interference limits for Class B digital devices FOR HOME OR OFFICE USE. These limits are designed to provide more reasonable protection against harmful interference in a residential installation, and are more stringent than “outdoor” requirements.

Operation of this device is subject to the following conditions: (1) This device may not cause harmful interference, and (2) this device must accept any interference received, including interference that may cause undesired operation.

This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation.  If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one of the following measures:

•  Reorient or relocate the receiving antenna.

•  Increase the separation between the equipment and the receiver.

•  Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

•  Consult the dealer or an experienced radio/TV technician for help.

Software License Agreement

BY USING THE GPS 10, YOU AGREE TO BE BOUND BY THE TERMS AND CONDITIONS OF THE FOLLOWING SOFTWARE LICENSE AGREEMENT. PLEASE READ THIS AGREEMENT CAREFULLY.

Garmin grants you a limited license to use the software embedded in this device (the “Software”) in binary executable form in the normal operation of the product. Title, ownership rights and intellectual property rights in and to the Software remain in Garmin.

You acknowledge that the Software is the property of Garmin and is protected under the United States of America copyright laws and international copyright treaties. You further acknowledge that the structure, organization and code of the Software are valuable trade secrets of Garmin and that the Software in source code form remains a valuable trade secret of Garmin. You agree not to decompile, disassemble, modify, reverse assemble, reverse engineer or reduce to human readable form the Software or any part thereof or create any derivative works based on the Software. You agree not to export or re-export the Software to any country in violation of the export control laws of the United States of America.

Limited Warranty

This Garmin product is warranted to be free from defects in materials or workmanship for one year from the date of purchase.  Within this period, Garmin will at its sole option, repair or replace any components that fail in normal use.  Such repairs or replacement will be made at no charge to the customer for parts or labor, provided that the customer shall be responsible for any transportation cost.  This warranty does not cover failures due to abuse, misuse, accident or unauthorized alteration or repairs.

THE WARRANTIES AND REMEDIES CONTAINED HEREIN ARE EXCLUSIVE AND IN LIEU OF ALL OTHER WARRANTIES EXPRESS OR IMPLIED OR STATUTORY, INCLUDING ANY LIABILITY ARISING UNDER ANY WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE, STATUTORY OR OTHERWISE.  THIS WARRANTY GIVES YOU SPECIFIC LEGAL RIGHTS, WHICH MAY VARY FROM STATE TO STATE.

IN NO EVENT SHALL GARMIN BE LIABLE FOR ANY INCIDENTAL, SPECIAL, INDIRECT OR CONSEQUENTIAL DAMAGES, WHETHER RESULTING FROM THE USE, MISUSE, OR INABILITY TO USE THIS PRODUCT OR FROM DEFECTS IN THE PRODUCT.  Some states do not allow the exclusion of incidental or consequential damages, so the above limitations may not apply to you.

Garmin retains the exclusive right to repair or replace the unit or software or offer a full refund of the purchase price at its sole discretion.  SUCH REMEDY SHALL BE YOUR SOLE AND EXCLUSIVE REMEDY FOR ANY BREACH OF WARRANTY.

To obtain warranty service, call your local Garmin authorized dealer.  Or call Garmin Customer Service at one of the numbers listed below for shipping instructions and an RMA tracking number.  The unit should be securely packed with the tracking number clearly written on the outside of the package.  The unit should be sent, freight charges prepaid, to any Garmin warranty service station.  A copy of the original sales receipt is required as the proof of purchase for warranty repairs.

Online auction confirmations are not accepted for warranty verification.  To obtain warranty service, an original or copy of the sales receipt from the original retailer is required.  Garmin will not replace missing components from any package purchased through an online auction.