GPS 10

TECHNICAL

SPECIFICATIONS



Garmin International, Inc.

1200 E. 151st Street

Olathe, KS  66062 USA

190-00516-00, Revision B

February 2005

© Copyright 2005 Garmin Ltd. or its subsidiaries

Garmin International, Inc.
1200 East 151st Street, Olathe, Kansas 66062, U.S.A.
Tel. 913/397.8200 or 800/800.1020
Fax 913/397.8282

Garmin (Europe) Ltd.
Unit 5, The Quadrangle, Abbey Park Industrial Estate, Romsey, SO51 9DL, U.K.
Tel. 44/0870.8501241
Fax 44/0870.8501251

Garmin Corporation 
No. 68, Jangshu 2nd Road, Shijr, Taipei County, Taiwan
Tel. 886/2.2642.9199
Fax 886/2.2642.9099

All rights reserved. Except as expressly provided herein, no part of this manual may be reproduced, copied, transmitted, disseminated, downloaded, or stored in any storage medium, for any purpose without the express prior written consent of Garmin. Garmin hereby grants permission to download a single copy of this manual onto a hard drive or other electronic storage medium to be viewed and to print one copy of this manual or of any revision hereto, provided that such electronic or printed copy of this manual must contain the complete text of this copyright notice and provided further that any unauthorized commercial distribution of this manual or any revision hereto is strictly prohibited.

Information in this document is subject to change without notice. Garmin reserves the right to change or improve its products and to make changes in the content without obligation to notify any person or organization of such changes or improvements. Visit the Garmin Web site (www.garmin.com) for current updates and supplemental information concerning the use and operation of this and other Garmin products.

Garmin and AutoLocate are registered trademarks of Garmin Ltd. or its subsidiaries and may not be used without express permission of Garmin.

Web site address: www.garmin.com

RECORD OF REVISIONS

| Revision | Revision Date | Description | ECO # |
| --- | --- | --- | --- |
| A | 12/10/04 | Initial Release | -- |
| B | 02/01/05 | Revised and Redrawn | 29554 |

TABLE OF CONTENTS

1	Introduction	1

1.1	Cautions	1

1.2	Limited Warranty	2

1.3	Overview	3

1.4	Features	3

1.4.1	GPS 10 Wireless Receiver	4

1.5	Technical Specifications	5

1.5.1	Physical Characteristics	5

1.5.1.1	Size	5

1.5.1.2	Weight	5

1.5.1.3	Color	5

1.5.1.4	Case Material	5

1.5.2	Electrical Characteristics	5

1.5.2.1	Input Voltage	5

1.5.2.2	Input Current	5

1.5.2.3	Battery	5

1.5.2.4	Battery Charger	5

1.5.2.5	GPS Receiver Sensitivity	5

1.5.3	Environmental Characteristics	5

1.5.3.1	Operating Temperature Range	5

1.5.3.2	Battery Charging Temperature Range	5

1.5.3.3	Storage Temperature	5

1.5.4	GPS Performance	6

1.5.4.1	Receiver	6

1.5.4.2	Acquisition Times	6

1.5.4.3	Update Rate	6

1.5.4.4	Accuracy	6

1.5.5	Interfaces	6

1.5.5.1	GPS 10 Electrical Characteristics	6

1.5.5.2	Garmin Interface	6

1.5.5.3	GPS 10 Protocol	6

2	Mechanical Characteristics & Mounting	7

3	GPS 10 Software Interface	8

3.1	Received NMEA 0183 Sentences	8

3.1.1	Almanac Information (ALM)	8

3.1.2	Sensor Initialization Information (PGRMI)	9

3.1.3	Sensor Configuration Information (PGRMC)	9

3.1.4	Additional Sensor Configuration Information (PGRMC1)	10

3.1.5	Output Sentence Enable/Disable (PGRMO)	10

3.2	Transmitted NMEA 0183 Sentences	11

3.2.1	Sentence Transmission Rate	11

3.2.2	Transmitted Time	12

3.2.3	Global Positioning System Almanac Data (ALM)	12

3.2.4	Global Positioning System Fix Data (GGA)	12

3.2.5	GPS DOP and Active Satellites (GSA)	13

3.2.6	GPS Satellites in View (GSV)	13

3.2.7	Recommended Minimum Specific GPS/TRANSIT Data (RMC)	13

3.2.8	Track Made Good and Ground Speed (VTG)	13

3.2.9	Geographic Position (GLL)	14

3.2.10	Estimated Error Information (PGRME)	14

3.2.11	GPS Fix Data Sentence (PGRMF)	14

3.2.12	Sensor Status Information (PGRMT)	14

3.2.13	3D velocity Information (PGRMV)	15

3.2.14	DGPS Beacon Information (PGRMB)	15

3.3	Baud Rate Selection	15

Appendix A: Earth Datums	16

Appendix B: Binary Phase Output Format	19

Appendix C: GPS 10 and Windows Serial Mouse Issue	23

Appendix D: Sensor Configuration Software	25

Downloading the Sensor Configuration Software	25

Selecting a Model	25

Connecting to the Sensor	25

File Menu	26

Comm Menu	26

Config Menu	26

View Menu	27

Help Menu	27

Appendix E: Host System Considerations………………...…………………………..28

PIN Code…………………………………………………………..…………………………………………..28

LIST OF TABLES AND FIGURES

Figure 1. GPS 10 Bottom Case Dimensions	7

Figure 2. GPS 10 Operational Characteristics	7

Table 1: NMEA 0183 Output Sentence Order and Size	11

Table 2: Characters per Second for Available Baud Rates	11

# Introduction

## Cautions

CAUTION

The GPS system is operated by the government of the United States, which is solely responsible for its accuracy and maintenance. Although the GPS 10 is a precision electronic NAVigation AID (NAVAID), any NAVAID can be misused or misinterpreted, and therefore become unsafe. Use these products at your own risk. To reduce the risk, carefully review and understand all aspects of these Technical Specifications before using the GPS 10. When in actual use, carefully compare indications from the GPS to all available navigation sources including the information from other NAVAIDs, visual sightings, charts, etc. For safety, always resolve any discrepancies before continuing navigation.

FCC Compliance

The GPS 10 complies with Part 15 of the FCC interference limits for Class B digital devices FOR HOME OR OFFICE USE. These limits are designed to provide reasonable protection against harmful interference in a residential installation, and are more stringent than “outdoor” requirements.

Operation of this device is subject to the following conditions: (1) This device may not cause harmful interference, and (2) this device must accept any interference received, including interference that may cause undesired operation.

This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

The GPS 10 does not contain any user-serviceable parts. Unauthorized repairs or modifications could result in permanent damage to the equipment, and void your warranty and your authority to operate this device under Part 15 regulations.

## Limited Warranty

This Garmin product is warranted to be free from defects in materials or workmanship for one year from the date of purchase. Within this period, Garmin will at its sole option repair or replace any components that fail in normal use. Such repairs or replacement will be made at no charge to the customer for parts or labor, provided that the customer shall be responsible for any transportation cost. This warranty does not cover failures due to abuse, misuse, accident, or unauthorized alteration or repairs.

THE WARRANTIES AND REMEDIES CONTAINED HEREIN ARE EXCLUSIVE AND IN LIEU OF ALL OTHER WARRANTIES EXPRESS OR IMPLIED OR STATUTORY, INCLUDING ANY LIABILITY ARISING UNDER ANY WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE, STATUTORY OR OTHERWISE. THIS WARRANTY GIVES YOU SPECIFIC LEGAL RIGHTS, WHICH MAY VARY FROM STATE TO STATE.

IN NO EVENT SHALL GARMIN BE LIABLE FOR ANY INCIDENTAL, SPECIAL, INDIRECT OR CONSEQUENTIAL DAMAGES, WHETHER RESULTING FROM THE USE, MISUSE, OR INABILITY TO USE THIS PRODUCT OR FROM DEFECTS IN THE PRODUCT. Some states do not allow the exclusion of incidental or consequential damages, so the above limitations may not apply to you.

Garmin retains the exclusive right to repair or replace the unit or software or offer a full refund of the purchase price at its sole discretion. SUCH REMEDY SHALL BE YOUR SOLE AND EXCLUSIVE REMEDY FOR ANY BREACH OF WARRANTY.

To obtain warranty service, contact your local Garmin authorized dealer or call Garmin Product Support at one of the numbers listed below for shipping instructions and an RMA tracking number. The unit should be securely packed with the tracking number clearly written on the outside of the package. The unit should then be sent, freight charges prepaid, to any Garmin warranty service station. A copy of the original sales receipt is required as the proof of purchase for warranty repairs.

Garmin International, Inc.
1200 E 151st Street, Olathe, Kansas 66062 U.S.A.
Tel. 913/397.8200 or 800/800.1020
Fax. 913/397.8282

Garmin (Europe) Ltd.
Unit 5, The Quadrangle, Abbey Park Industrial Estate, Romsey, SO51 9DL U.K.
Tel. 44/0870.8501241 
Fax 44/0870.8501251

Online Auction Purchases: Products sold through online auctions are not eligible for rebates or other special offers from Garmin. Online auction confirmations are not accepted for warranty verification. To obtain warranty service, an original or copy of the sales receipt from the original retailer is required. Garmin will not replace missing components from any package purchased through an online auction.

International Purchases: A separate warranty is provided by international distributors for units purchased outside the United States. This warranty is provided by the local in-country distributor and this distributor provides local service for your unit. Distributor warranties are only valid in the area of intended distribution. Units purchased in the United States or Canada must be returned to the Garmin service center in the United Kingdom, the United States, Canada, or Taiwan for service.

## Overview

The GPS 10 product is a GPS receiver with WAAS capability, a rechargeable lithium-ion battery, integral magnetic base and Bluetooth wireless technology capability. Based on the proven technology found in other Garmin 12-channel GPS receivers, the GPS 10 tracks up to 12 satellites at a time while providing fast time-to-first-fix, one-second navigation updates, and low power consumption. This generation of GPS sensors adds the capability of FAA Wide Area Augmentation System (WAAS) differential GPS. The GPS 10’s far-reaching capability meets the sensitivity requirements of land navigation as well as the dynamics requirements of high-performance aircraft.

The GPS 10 design uses the latest technology and high-level circuit integration to achieve superior performance while minimizing space and power requirements. All critical components of the system including the GPS RF/IF receiver hardware and the GPS digital baseband are designed and manufactured by Garmin to ensure the quality and capability of the GPS. The hardware capability combined with software intelligence makes the GPS 10 easy to integrate and use for laptop/tablet PC, PDA, and smart phone applications by incorporating a Class 2 Bluetooth module within the unit.

The GPS 10 product is designed to withstand rugged operating conditions and is waterproof to IEC 60529 IPX7, immersion in 1 meter of water for 30 minutes. This complete GPS receiver requires no additional components to be supplied by an OEM or system integrator. An internal rechargeable lithium-ion battery and 12 VDC charging adapter are provided with the receiver. The only requirement is a clear view of the GPS satellites. Internal FLASH memory allows the GPS to retain critical data such as satellite orbital parameters, last-known position, date and time. End user interfaces such as keyboards and displays are the responsibility of the application designer.

## Features

- Integrated Bluetooth (Class 2) wireless technology module for communication with a mobile device.

- 12-channel GPS receiver tracks and uses up to 12 satellites for fast, accurate positioning and low power consumption.

- Differential DGPS capability using real-time WAAS corrections yielding position errors of less than 3 meters.

- Compact, rugged design ideal for applications with minimal space. Receiver position information can be displayed directly on a chartplotter or laptop/tablet PC, PDA, or smart phone with Bluetooth wireless technology capability.

- User initialization is not required. Once installed, unit automatically produces navigation data after position fix is acquired.

- User-configurable navigation mode (2-dimensional or 3-dimensional fix).

- Configurable for binary format carrier phase data output.

- FLASH-based program and non-volatile memory. New software revisions available through Web site download. Non-volatile memory does not require battery backup.

- On-board rechargeable main power battery to maintain the real-time clock

- The real-time clock can run for up to 2 months after the main battery enters a low battery mode.

- Configurable parameters include expected position, current time and date, preferred position fix type (2D, 3D, or automatic), and velocity filter time constant (none, automatic, or your choice between 2 and 255 seconds).

- Magnetic mount for attachment to ferrous surfaces. Velcro adhesive backed patch for non-magnetic surfaces.

- Includes 12 VDC Cigarette Lighter Charging Adapter.

- Waterproof design allows continuous exposure to the prevailing weather conditions at most locations.

- Auto Power On Mode: When in the Off mode and connected to a switched 12 V DC external power source, the GPS 10 automatically turns On when external power is turned on. (example: a cigarette lighter controlled by the vehicle ignition switch.)

### GPS 10 Wireless Receiver

The GPS 10 uses Bluetooth wireless technology to interface with a mobile device (Computer/PDA) that is Bluetooth wireless technology enabled.

Class 2 Bluetooth device with power control capability based on the Bluetooth System Specification, Version 1.1. The supported profile is Serial Profile (SSP).

Accessories provided with the GPS 10:

- Velcro Mounting Patch (Not shown)

- 12 VDC Cigarette Lighter Charging Adapter (010-10412-00)

12 VDC Charging Adapter

Optional Accessories:

AC Charging Adapter

Suction Cup Windshield Mount (010-10616-00)

- 110-115 V AC Charging Adapter (010-10413-00)

Suction Cup Mount

Power Plug Specifications for charging the GPS 10 from user provided power source: 5 VDC (Range 4.8 to 5.5 VDC @ 600 mA)

Power Plug Dimensions

## Technical Specifications

Specifications are subject to change without notice.

### Physical Characteristics

#### Size

45 mm wide, 88 mm long and 19 mm in height.

#### Weight

80.0 grams

#### Color

Black w/Gray Accents

#### Case Material

Polycarbonate thermoplastic case that is waterproof to IEC 60529 IPX7 level (immersion in 1 meter of water for 30 minutes)

### Electrical Characteristics

#### Input Voltage

4.8–6.5 V

#### Input Current

500mA @ 5.0 V

#### Battery

- Capacity:  	900 mAHr

- Charging rate: 	Approx. 3 Hrs

- Operating time: 	Approx. 12 Hours with WAAS enabled.

#### Battery Charger

The included Cigarette Lighter Adapter (320-00187-00) can be used to charge the GPS 10’s internal battery or an AC Wall Charger (362-00028-00) can be purchased separately to charge the battery.

- Input  		10 to 30 V AC

- Output 		4.9 to 5.25 V DC @ 1.0 A

#### GPS Receiver Sensitivity

-165 dBW minimum

### Environmental Characteristics

#### Operating Temperature Range

- -30°C to + 60°C  	Powered from external supply

- -10°C to +60°C	Powered by internal batteries

#### Battery Charging Temperature Range

5°C to 45°C	(Outside this temperature range, battery charging is not possible)

#### Storage Temperature

- -40°C to +90°C

### GPS Performance

#### Receiver

WAAS Enabled™; 12 parallel channel GPS receiver continuously tracks and uses up to 12 satellites to compute and update your position.

#### Acquisition Times

- Reacquisition:	Less than 2 seconds

- Warm:		Approx. 15 seconds (all data known)

- Cold:	Approx. 45 seconds (initial position, time, and almanac known; ephemeris unknown)

- AutoLocate®:	5 minutes (almanac known; initial position and time unknown)

- SkySearch:	5 minutes (no data known)

#### Update Rate

1 record per second

#### Accuracy

- GPS Standard Positioning Service (SPS)
Position:		< 15 meters, 95% typical 
Velocity:	0.1 knot RMS steady state

- WAAS
Position:		< 3 meters, 95% typical
Velocity:	0.1 knot RMS steady state

- Dynamics:	999 knots velocity (only limited at altitude greater than 60,000 feet), 6g dynamics

### Interfaces

#### GPS 10 Electrical Characteristics

- Communicate with Host Platform via Bluetooth Serial Profile

- Typical Range: up to 32 feet (10 meters)

#### Garmin Interface

Refer to the Garmin Device Interface Specification for specific information about the Garmin Protocol. The document is located on the Garmin Web site at Appendix B: Binary Phase Output Formatr additional information concerning access to raw pseudo-range and carrier phase data from the GPS 10.

#### GPS 10 Protocol

- NMEA 0183 Version 2.0 or NMEA 0183 Version 2.30 (Version 2.0 is factory default, programmable by data field 7 of the PGRMC1 sentence described in Section 3.1.4 Additional Sensor Configuration Information (PGRMC1).

- Available NMEA 0183 output sentences include GPALM, GPGGA, GPGSA, GPGSV, GPRMC, GPVTG, GPGLL, PGRME, PGRMF, PGRMT, PGRMV, and PGRMB (Garmin proprietary sentences). See Section 3.2 Transmitted NMEA 0183 Sentences for format descriptions.

- Configuration and initialization is accomplished with NMEA 0183 input sentences. Allows initialization of information such as expected position, date, time, earth datum, and differential mode. See Section 3.1 Received NMEA 0183 Sentences for format descriptions.

- Configurable for binary data output including GPS carrier phase data.

# Mechanical Characteristics & Mounting

The unit contains an integrated magnetic mount and a Velcro patch for mounting on non-ferrous surfaces. The following drawings show example geometry for mounting hardware in case you wish to design your own custom mount. Figure 1 shows the mounting footprint. Figure 2 shows operational characteristics of the GPS 10.

Figure 1. GPS 10 Bottom Case Dimensions

Figure 2. GPS 10 Operational Characteristics

# GPS 10 Software Interface

The interface protocol design of the GPS 10 product is based on the National Marine Electronics Association’s NMEA 0183 ASCII interface specification. This standard is fully defined in NMEA 0183, Version 2.30. Copies may be obtained from NMEA, http://www.nmea.org/.

In addition to the standard NMEA 0183 sentences, the GPS 10 may also be configured to transmit information over their serial interface using NMEA 0183 compliant Garmin proprietary sentences. These proprietary sentences begin with the characters, “$PGRM”, instead of the characters “$G” that are typical of the standard NMEA 0183 sentences. The characters “$P” indicate that the sentence is a proprietary implementation and the characters and “GRM” indicate that it is Garmin’s proprietary sentence. The letter (or letters) that follow the characters “$PGRM” uniquely identifies that particular Garmin proprietary sentence.

It is also possible to configure the GPS 10 to transmit binary phase data information over their serial interface. See Appendix B: Binary Phase Output Format for details.

The following sections describe the NMEA 0183 data format of each sentence transmitted and received by the GPS 10 product.

## Received NMEA 0183 Sentences

The following paragraphs define the sentences that can be received on the GPS sensor’s port. Null fields in the configuration sentence indicate no change in the particular configuration parameter. All sentences received by the GPS sensor must be terminated with <CR><LF>, the ASCII characters for carriage return (0D hexadecimal) and line feed (0A hexadecimal). The checksum *hh is used for parity checking data and is not required, but is recommended for use in environments containing high electromagnetic noise. It is generally not required in normal PC environments. When used, the parity bytes (hh) are the ASCII representation of the exclusive-or (XOR) sum of all the characters between the “$” and “*” characters, non-inclusive. The hex representation must be a capital letter, such as 3D instead of 3d. Sentences may be truncated by <CR><LF> after any data field and valid fields up to that point will be acted on by the sensor.

### Almanac Information (ALM)

The $GPALM sentence can be used to initialize the GPS sensor’s stored almanac information in the unlikely event of non-volatile memory loss or after storing longer than six months without tracking GPS satellites.

$GPALM,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>,<10>,<11>,<12>,<13>,<14>,<15>*hh<CR><LF>

| <1> | Total number of ALM sentences to be transmitted by the GPS sensor during almanac download. This field can be null or any number when sending almanac to the GPS sensor. |
| --- | --- |
| <2> | Number of current ALM sentence. This field can be null or any number when sending almanac to the GPS sensor. |
| <3> | Satellite PRN number, 01 to 32 |
| <4> | GPS week number |
| <5> | SV health, bits 17-24 of each almanac page |
| <6> | Eccentricity |
| <7> | Almanac reference time |
| <8> | Inclination angle |
| <9> | Rate of right ascension |
| <10> | Root of semi major axis |
| <11> | Omega, argument of perigee |
| <12> | Longitude of ascension node |
| <13> | Mean anomaly |
| <14> | af0 clock parameter |
| <15> | af1 clock parameter |

### Sensor Initialization Information (PGRMI)

The $PGRMI sentence provides information used to initialize the GPS sensor’s set position and time used for satellite acquisition. Receipt of this sentence by the GPS sensor causes the software to restart the satellite acquisition process. If there are no errors in the sentence, it will be echoed upon receipt. If an error is detected, the echoed PGRMI sentence will contain the current default values. Current PGRMI defaults (with the exception of the Receiver Command, which is a command rather than a mode) can also be obtained by sending $PGRMIE to the GPS sensor.

$PGRMI,<1>,<2>,<3>,<4>,<5>,<6>,<7>*hh<CR><LF>

| <1> | Latitude, ddmm.mmm format (leading zeros must be transmitted) |
| --- | --- |
| <2> | Latitude hemisphere, N or S |
| <3> | Longitude, dddmm.mmm format (leading zeros must be transmitted) |
| <4> | Longitude hemisphere, E or W |
| <5> | Current UTC date, ddmmyy format |
| <6> | Current UTC time, hhmmss format |
| <7> | Receiver Command, A = Auto Locate, R = Unit Reset |

### Sensor Configuration Information (PGRMC)

The $PGRMC sentence provides information used to configure the GPS sensor’s operation. Configuration parameters are stored in non-volatile memory and retained between power cycles. The GPS sensor will echo this sentence upon its receipt if no errors are detected. If an error is detected, the echoed PGRMC sentence will contain the current default values. Current default values can also be obtained by sending $PGRMCE to the GPS sensor.

$PGRMC,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>,<10>,<11>,<12>,<13>,<14>*hh<CR><LF>

| <1> | Fix mode, A = automatic, 2 = 2D exclusively (host system must supply altitude), 3 = 3D exclusively |
| --- | --- |
| <2> | Altitude above/below mean sea level, -1500.0 to 10000.0 meters |
| <3> | Earth datum index. If the user datum index (96) is specified, fields <4> through <8> must contain valid values. Otherwise, fields <4> through <8> must be null. Refer to Appendix A: Earth Datums for a list of earth datums and the corresponding earth datum index. |
| <4> | User earth datum semi-major axis, 6360000.000 to 6380000.000 meters (.001 meters resolution) |
| <5> | User earth datum inverse flattening factor, 285.0 to 310.0 (10-9 resolution) |
| <6> | User earth datum delta x earth centered coordinate, -5000.0 to 5000.0 meters (1 meter resolution) |
| <7> | User earth datum delta y earth centered coordinate, -5000.0 to 5000.0 meters (1 meter resolution) |
| <8> | User earth datum delta z earth centered coordinate, -5000.0 to 5000.0 meters (1 meter resolution) |
| <9> | Differential mode, A = automatic (output DGPS data when available, non-DGPS otherwise), D = differential exclusively (output only differential fixes) |
| <10> | NMEA 0183 Baud rate, 1 = 1200, 2 = 2400, 3 = 4800, 4 = 9600, 5 = 19200, 6 = 300, 7 = 600 |
| <11> | Velocity filter, 0 = No filter, 1 = Automatic filter, 2-255 = Filter time constant (e.g., 10 = 10 second filter) |
| <12> | Not used |
| <13> | Not used |
| <14> | Dead reckoning valid time 1-30 (sec) |

All configuration changes take effect after receipt of a valid value except baud rate. Baud rate change takes effect on the next power cycle or an external reset even.

### Additional Sensor Configuration Information (PGRMC1)

The $PGRMC1 sentence provides additional information used to configure the GPS sensor operation. Configuration parameters are stored in non-volatile memory and retained between power cycles. The GPS sensor will echo this sentence upon its receipt if no errors are detected. If an error is detected, the echoed PGRMC1 sentence will contain the current default values. Current default values can also be obtained by sending $PGRMC1E to the GPS sensor.

$PGRMC1,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>,<10>,<11>,<12>*hh<CR><LF>

| <1> | NMEA 0183 output time 1-900 (sec) |
| --- | --- |
| <2> | Binary Phase Output Data, 1 = Off, 2 = On. |
| <3> | Automatic Position Averaging when Stopped, 1 = Off, 2 = On |
| <4> | No Effect (DGPS beacon frequency – 0.0, 283.5 – 325.0 kHz in 0.5 kHz steps) |
| <5> | No Effect (DGPS beacon bit rate – 0, 25, 50, 100, or 200 bps) |
| <6> | No Effect (DGPS beacon scanning, 1 = Off, 2 = On) |
| <7> | NMEA 0183 version 2.30 mode indicator, 1 = Off, 2 = On |
| <8> | DGPS mode, W = WAAS Only, N = None (DGPS disabled) |
| <9> | Power Save Mode for GPS, P = Power Save mode, N = Normal |
| <10> | Adaptive Transmission Enabled, 1 = Off, 2 = On |
| <11> | Auto Power Off, 1 = Off, 2 = On |
| <12> | Power On with External Charger, 1 = Off, 2 = On |

Configuration changes take effect immediately, with the exception of Binary Phase Output Data, which takes effect on the next power cycle or a reset event. A reset can be commanded by sending the sentence “$PGRMI,,,,,,,R” (refer to Section 3.1.2 Sensor Initialization Information (PGRMI)). If the GPS sensor is in the Binary data mode, it is necessary to send the following eight-byte data stream to temporarily change the data format to NMEA 0183. Then follow by sending a PGRMC1 sentence that turns off the Binary Phase Output Data format:

10 0A 02 26 00 CE 10 03 (Hexadecimal)

### Output Sentence Enable/Disable (PGRMO)

The $PGRMO sentence provides the ability to enable and disable specific output sentences. The following sentences are enabled at the factory: GPGGA, GPGSA, GPGSV, GPRMC, and PGRMT.

$PGRMO,<1>,<2>*hh<CR><LF>

| <1> | Target sentence description (e.g., PGRMT, GPGSV, etc.) |
| --- | --- |
| <2> | Target sentence mode, where: 0 = disable specified sentence 1 = enable specified sentence 2 = disable all output sentences 3 = enable all output sentences (except GPALM) 4 = restore factory default output sentences |

The following notes apply to the PGRMO input sentence:

- If the target sentence mode is ‘2’ (disable all), ‘3’ (enable all), or ‘4’ (restore defaults), the target sentence description is not checked for validity. In this case, an empty field is allowed (e.g., $PGRMO,,3), or the mode field may contain from 1 to 5 characters.

- If the target sentence mode is ‘0’ (disable) or ‘1’ (enable), the target sentence description field must be an identifier for one of the sentences that can be output by the GPS sensor.

- If either the target sentence mode field or the target sentence description field is not valid, the PGRMO sentence will have no effect.

- $PGRMO,GPALM,1 will cause the GPS sensor to transmit all stored almanac information. All other NMEA 0183 sentence transmission will be suspended temporarily.

- $PGRMO,,G will cause the COM port to change to Garmin Data Transfer format for the duration of the power cycle. The Garmin mode is required for GPS 10 series product software updates.

.

## Transmitted NMEA 0183 Sentences

The subsequent paragraphs define the sentences that can be transmitted by the GPS 10.

### Sentence Transmission Rate

Sentences are transmitted with respect to the user selected baud rate.

The GPS sensor will transmit each sentence (except where noted in particular transmitted sentence descriptions) at a periodic rate based on the user selected baud rate and user selected output sentences. The GPS sensor will transmit the selected sentences contiguously. The length of the transmission can be determined by the following equation and Tables 2 and 3:

total characters to be transmitted

length of transmission     =	           ---------------------------------------------

characters transmitted per second

| Sentence | Output by Default? | Maximum Characters |
| --- | --- | --- |
| GPRMC |  | 74 |
| GPGGA |  | 82 |
| GPGSA |  | 66 |
| GPGSV |  | 70 |
| PGRME |  | 35 |
| GPGLL |  | 44 |
| GPVTG |  | 42 |
| PGRMV |  | 32 |
| PGRMF |  | 82 |
| PGRMB |  | 40 |
| PGRMT | Once per minute | 50 |

Table 1: NMEA 0183 Output Sentence Order and Size

| Baud | Characters per Second |
| --- | --- |
| 300 | 30 |
| 600 | 60 |
| 1200 | 120 |
| 2400 | 240 |
| 4800 | 480 |
| 9600 | 960 |
| 19200 | 1920 |
| 38400 | 3840 |

Table 2: Characters per Second for Available Baud Rates

The maximum number of fields allowed in a single sentence is 82 characters including delimiters. Values in the table include the sentence start delimiter character “$” and the termination delimiter <CR><LF>. The factory set defaults will result in a once per second transmission at the NMEA 0183 specification transmission rate of 4800 baud.

Regardless of the selected baud rate, the information transmitted by the GPS sensor is referenced to the one-pulse-per-second output pulse immediately preceding the GPRMC sentence, or whichever sentence is output first in the burst (see Table 2 above).

### Transmitted Time

The GPS sensor outputs UTC (Coordinated Universal Time) date and time of day in the transmitted sentences. Before the initial position fix, the on-board clock provides the date and time of day. After the initial position fix, the date and time of day are calculated using GPS satellite information and are synchronized with the one-pulse-per-second output.

The GPS sensor uses information obtained from the GPS satellites to add or delete UTC leap seconds and correct the transmitted date and time of day. The transmitted date and time of day for leap second correction follow the guidelines in National Institute of Standards and Technology Special Publication 432 (Revised 1990). This document is for sale by the Superintendent of Documents, U.S. Government Printing Office, Washington, D.C., 20402, U.S.A.

When a positive leap second is required, one second is inserted at the beginning of the first hour (0h 0m 0s) of the day that the positive leap is occurring. The minute containing the leap second is 61 seconds long. The GPS sensor would have transmitted this information for the leap second added December 31, 1998 as follows:

$GPRMC,235959,A,3851.3651,N,09447.9382,W,000.0,221.9,071103,003.3,E*69

$GPRMC,000000,A,3851.3651,N,09447.9382,W,000.0,221.9,081103,003.3,E*67

$GPRMC,000000,A,3851.3651,N,09447.9382,W,000.0,221.9,081103,003.3,E*67

$GPRMC,000001,A,3851.3651,N,09447.9382,W,000.0,221.9,081103,003.3,E*66

If a negative leap second should be required, one second will be deleted at the end of some UTC month. The minute containing the leap second will be only 59 seconds long. In this case, the GPS sensor will not transmit the time of day 0h 0m 0s (the “zero” second) for the day from which the leap second is removed.

$GPRMC,235959,A,3851.3650,N,09447.9373,W,000.0,000.0,111103,003.3,E*69

$GPRMC,000001,A,3851.3650,N,09447.9373,W,000.0,000.0,121103,003.3,E*6A

$GPRMC,000002,A,3851.3650,N,09447.9373,W,000.0,000.0,121103,003.3,E*69

### Global Positioning System Almanac Data (ALM)

Almanac sentences are not normally transmitted. Almanac transmission can be initiated by sending the GPS sensor a $PGRMO,GPALM,1 command. Upon receipt of this command, the GPS sensor will transmit available almanac information on GPALM sentences. During the transmission of almanac sentences, other NMEA 0183 data output will be suspended temporarily.

$GPALM,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>,<10>,<11>,<12>,<13>,<14>,<15>*hh<CR><LF>

<field information> can be found in Section 3.1.1 Almanac Information (ALM).

### Global Positioning System Fix Data (GGA)

$GPGGA,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>,M,<10>,M,<11>,<12>*hh<CR><LF>

| <1> | UTC time of position fix, hhmmss format |
| --- | --- |
| <2> | Latitude, ddmm.mmmm format (leading zeros will be transmitted) |
| <3> | Latitude hemisphere, N or S |
| <4> | Longitude, dddmm.mmmm format (leading zeros will be transmitted) |
| <5> | Longitude hemisphere, E or W |
| <6> | GPS quality indication, 0 = fix not available, 1 = Non-differential GPS fix available, 2 = Differential GPS (DGPS) fix available, 6 = Estimated |
| <7> | Number of satellites in use, 00 to 12 (leading zeros will be transmitted) |
| <8> | Horizontal dilution of precision, 0.5 to 99.9 |
| <9> | Antenna height above/below mean sea level, -9999.9 to 99999.9 meters |
| <10> | Geoidal height, -999.9 to 9999.9 meters |
| <11> | Differential GPS (RTCM SC-104) data age, number of seconds since last valid RTCM transmission (null if not an RTCM DGPS fix) |
| <12> | Differential Reference Station ID, 0000 to 1023 (leading zeros will be transmitted, null if not an RTCM DGPS fix) |

### GPS DOP and Active Satellites (GSA)

$GPGSA,<1>,<2>,<3>,<3>,<3>,<3>,<3>,<3>,<3>,<3>,<3>,<3>,<3>,<3>,<4>,<5>,<6>*hh<CR><LF>

| <1> | Mode, M = manual, A = automatic |
| --- | --- |
| <2> | Fix type, 1 = not available, 2 = 2D, 3 = 3D |
| <3> | PRN number, 01 to 32, of satellite used in solution, up to 12 transmitted (leading zeros will be transmitted) |
| <4> | Position dilution of precision, 0.5 to 99.9 |
| <5> | Horizontal dilution of precision, 0.5 to 99.9 |
| <6> | Vertical dilution of precision, 0.5 to 99.9 |

### GPS Satellites in View (GSV)

$GPGSV,<1>,<2>,<3>,<4>,<5>,<6>,<7>,...<4>,<5>,<6>,<7>*hh<CR><LF>

| <1> | Total number of GSV sentences to be transmitted |
| --- | --- |
| <2> | Number of current GSV sentence |
| <3> | Total number of satellites in view, 00 to 12 (leading zeros will be transmitted) |
| <4> | Satellite PRN number, 01 to 32 (leading zeros will be transmitted) |
| <5> | Satellite elevation, 00 to 90 degrees (leading zeros will be transmitted) |
| <6> | Satellite azimuth, 000 to 359 degrees, true (leading zeros will be transmitted) |
| <7> | Signal to noise ratio (C/No) 00 to 99 dB, null when not tracking (leading zeros will be transmitted) |

NOTE:  Items <4>,<5>,<6>, and <7> repeat for each satellite in view to a maximum of four (4) satellites per sentence. Additional satellites in view information must be sent in subsequent bursts of NMEA 0183 data. These fields will be null if unused.

### Recommended Minimum Specific GPS/TRANSIT Data (RMC)

$GPRMC,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>,<10>,<11>,<12>*hh<CR><LF>

| <1> | UTC time of position fix, hhmmss format |
| --- | --- |
| <2> | Status, A = Valid position, V = NAV receiver warning |
| <3> | Latitude, ddmm.mmmm format (leading zeros will be transmitted) |
| <4> | Latitude hemisphere, N or S |
| <5> | Longitude, dddmm.mmmm format (leading zeros will be transmitted) |
| <6> | Longitude hemisphere, E or W |
| <7> | Speed over ground, 000.0 to 999.9 knots (leading zeros will be transmitted) |
| <8> | Course over ground, 000.0 to 359.9 degrees, true (leading zeros will be transmitted) |
| <9> | UTC date of position fix, ddmmyy format |
| <10> | Magnetic variation, 000.0 to 100.0 degrees (leading zeros will be transmitted) |
| <11> | Magnetic variation direction, E or W (westerly variation adds to true course) |
| <12> | Mode indicator (only output if NMEA 0183 version 2.30 active), A = Autonomous, D = Differential, E = Estimated, N = Data not valid |

### Track Made Good and Ground Speed (VTG)

$GPVTG,<1>,T,<2>,M,<3>,N,<4>,K,<5>*hh<CR><LF>

| <1> | True course over ground, 000 to 359 degrees (leading zeros will be transmitted) |
| --- | --- |
| <2> | Magnetic course over ground, 000 to 359 degrees (leading zeros will be transmitted) |
| <3> | Speed over ground, 000.0 to 999.9 knots (leading zeros will be transmitted) |
| <4> | Speed over ground, 0000.0 to 1051.8 kilometers per hour (leading zeros will be transmitted) |
| <5> | Mode indicator (only output if NMEA 0183 version 2.30 active), A = Autonomous, D = Differential, E = Estimated, N = Data not valid |

### Geographic Position (GLL)

$GPGLL,<1>,<2>,<3>,<4>,<5>,<6>,<7>*hh<CR><LF>

| <1> | Latitude, ddmm.mmmm format (leading zeros will be transmitted) |
| --- | --- |
| <2> | Latitude hemisphere, N or S |
| <3> | Longitude, dddmm.mmmm format (leading zeros will be transmitted) |
| <4> | Longitude hemisphere, E or W |
| <5> | UTC time of position fix, hhmmss format |
| <6> | Status, A = Valid position, V = NAV receiver warning |
| <7> | Mode indicator (only output if NMEA 0183 version 2.30 active), A = Autonomous, D = Differential, E = Estimated, N = Data not valid |

### Estimated Error Information (PGRME)

$PGRME,<1>,M,<2>,M,<3>,M*hh<CR><LF>

| <1> | Estimated horizontal position error (HPE), 0.0 to 999.9 meters |
| --- | --- |
| <2> | Estimated vertical position error (VPE), 0.0 to 999.9 meters |
| <3> | Estimated position error (EPE), 0.0 to 999.9 meters |

### GPS Fix Data Sentence (PGRMF)

$PGRMF,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>,<10>,<11>,<12>,<13>,<14>,<15>*hh<CR><LF>

| <1> | GPS week number (0 - 1023) |
| --- | --- |
| <2> | GPS seconds (0 - 604799) |
| <3> | UTC date of position fix, ddmmyy format |
| <4> | UTC time of position fix, hhmmss format |
| <5> | GPS leap second count |
| <6> | Latitude, ddmm.mmmm format (leading zeros will be transmitted) |
| <7> | Latitude hemisphere, N or S |
| <8> | Longitude, dddmm.mmmm format (leading zeros will be transmitted) |
| <9> | Longitude hemisphere, E or W |
| <10> | Mode, M = manual, A = automatic |
| <11> | Fix type, 0 = no fix, 1 = 2D fix, 2 = 3D fix |
| <12> | Speed over ground, 0 to 1051 kilometers/hour |
| <13> | Course over ground, 0 to 359 degrees, true |
| <14> | Position dilution of precision, 0 to 9 (rounded to nearest integer value) |
| <15> | Time dilution of precision, 0 to 9 (rounded to nearest integer value) |

### Sensor Status Information (PGRMT)

The Garmin Proprietary sentence $PGRMT gives information concerning the status of the GPS sensor. This sentence is transmitted once per minute regardless of the selected baud rate.

$PGRMT,<1>,<2>,<3>,<4>,<5>,<6>,<7>,<8>,<9>*hh<CR><LF>

| <1> | Product, model and software version (variable length field, e.g., “GPS 10 SW VER 2.01 BT VER 1.27 764”) |
| --- | --- |
| <2> | ROM checksum test, P = pass, F = fail |
| <3> | Receiver failure discrete, P = pass, F = fail |
| <4> | Stored data lost, R = retained, L = lost |
| <5> | Real time clock lost, R = retained, L = lost |
| <6> | Oscillator drift discrete, P = pass, F = excessive drift detected |
| <7> | Data collection discrete, C = collecting, null if not collecting |
| <8> | GPS sensor temperature in degrees C |
| <9> | GPS sensor configuration data, R = retained, L = lost |

### 3D velocity Information (PGRMV)

$PGRMV,<1>,<2>,<3>*hh<CR><LF>

| <1> | True east velocity, 514.4 to 514.4 meters/second |
| --- | --- |
| <2> | True north velocity, 514.4 to 514.4 meters/second |
| <3> | Up velocity, 999.9 to 9999.9 meters/second |

### DGPS Beacon Information (PGRMB)

NOTE: PGRMB is not supported at this time.

$PGRMB,<1>,<2>,<3>,<4>,<5>,K,<6>,<7>*hh<CR><LF>

| <1> | Beacon tune frequency, 0.0, 283.5 – 325.0 kHz in 0.5 kHz steps |
| --- | --- |
| <2> | Beacon bit rate, 0, 25, 50, 100, or 200 bps |
| <3> | Beacon SNR, 0 to 31 |
| <4> | Beacon data quality, 0 to 100 |
| <5> | Distance to beacon reference station in kilometers |
| <6> | Beacon receiver communication status (0 = Check Wiring, 1 = No Signal, 2 = Tuning, 3 = Receiving, 4= Scanning) |
| <7> | DGPS fix source (R = RTCM, W = WAAS, N = Non-DGPS Fix) |
| <8> | DGPS mode, A = Automatic, W = WAAS Only, R = RTCM Only, N = None (DGPS disabled) |

## Baud Rate Selection

Baud rate selection can be performed by sending the appropriate configuration sentence to the GPS sensor as described in the $PGRMC Section 3.1.2 Sensor Initialization Information (PGRMI), field <10>.

# Appendix A: Earth Datums

The following is a list of the Garmin GPS 10 Earth datum indices and the corresponding earth datum name (including the area of application):

0	ADINDAN - Ethiopia, Mali, Senegal, Sudan

1	AFGOOYE - Somalia

2	AIN EL ABD 1970 - Bahrain Island, Saudi Arabia

3	ANNA 1 ASTRO 1965 - Cocos Island

4	ARC 1950 - Botswana, Lesotho, Malawi, Swaziland, Zaire, Zambia, 	Zimbabwe

5	ARC 1960 - Kenya, Tanzania

6	ASCENSION ISLAND 1958 - Ascension Island

7	ASTRO BEACON “E” - Iwo Jima Island

8	AUSTRALIAN GEODETIC 1966 - Australia, Tasmania Island

9	AUSTRALIAN GEODETIC 1984 - Australia, Tasmania Island

10	ASTRO DOS 71/4 - St. Helena Island

11	ASTRONOMIC STATION 1952 - Marcus Island

12	ASTRO B4 SOROL ATOLL - Tern Island

13	BELLEVUE (IGN) - Efate and Erromango Islands

14	BERMUDA 1957 - Bermuda Islands

15	BOGOTA OBSERVATORY - Colombia

16	CAMPO INCHAUSPE - Argentina

17	CANTON ASTRO 1966 - Phoenix Islands

10	CAPE CANAVERAL - Florida, Bahama Islands

19	CAPE - South Africa

20	CARTHAGE - Tunisia

21	CHATHAM 1971 - Chatham Island (New Zealand)

22	CHUA ASTRO - Paraguay

23	CORREGO ALEGRE - Brazil

24	DJAKARTA (BATAVIA) - Sumatra Island (Indonesia)

25	DOS 1968 - Gizo Island (New Georgia Islands)

26	EASTER ISLAND 1967 - Easter Island

27	EUROPEAN 1950 - Austria, Belgium, Denmark, Finland, France, Germany, Gibraltar, Greece, Italy, Luxembourg, Netherlands, Norway, Portugal, Spain, Sweden, Switzerland

28	EUROPEAN 1979 - Austria, Finland, Netherlands, Norway, Spain, Sweden, Switzerland

29	FINLAND HAYFORD 1910 - Finland

30	GANDAJIKA BASE - Republic of Maldives

31	GEODETIC DATUM 1949 - New Zealand

32	ORDNANCE SURVEY OF GREAT BRITAIN 1936 - England, Isle of Man, Scotland, Shetland Islands, Wales

33	GUAM 1963 - Guam Island

34	GUX 1 ASTRO - Guadalcanal Island

35	HJORSEY 1955 - Iceland

36	HONG KONG 1963 - Hong Kong

37	INDIAN - Bangladesh, India, Nepal

38	INDIAN - Thailand, Vietnam

39	IRELAND 1965 - Ireland

40	ISTS O73 ASTRO 1969 - Diego Garcia

41	JOHNSTON ISLAND 1961 - Johnston Island

42	KANDAWALA - Sri Lanka

43	KERGUELEN ISLAND - Kerguelen Island

44	KERTAU 1948 - West Malaysia, Singapore

45	L.C. 5 ASTRO - Cayman Brac Island

46	LIBERIA 1964 - Liberia

47	LUZON - Mindanao Island

48	LUZON - Phillippines (excluding Mindanao Island)

49	MAHE 1971 - Mahe Island

50	MARCO ASTRO - Salvage Islands

51	MASSAWA - Eritrea (Ethiopia)

52	MERCHICH - Morocco

53	MIDWAY ASTRO 1961 - Midway Island

54	MINNA - Nigeria

55	NORTH AMERICAN 1927 - Alaska

56	NORTH AMERICAN 1927 - Bahamas (excluding San Salvador Island)

57	NORTH AMERICAN 1927 - Central America (Belize, Costa Rica, El Salvador, Guatemala, Honduras, Nicaragua)

58	NORTH AMERICAN 1927 - Canal Zone

59	NORTH AMERICAN 1927 - Canada (including Newfoundland Island)

60	NORTH AMERICAN 1927 - Caribbean (Barbados, Caicos Islands, Cuba, Dominican Republic, Grand Cayman, Jamaica, Leeward Islands, Turks Islands)

61	NORTH AMERICAN 1927 - Mean Value (CONUS)

62	NORTH AMERICAN 1927 - Cuba

63	NORTH AMERICAN 1927 - Greenland (Hayes Peninsula)

64	NORTH AMERICAN 1927 - Mexico

65	NORTH AMERICAN 1927 - San Salvador Island

66	NORTH AMERICAN 1983 - Alaska, Canada, Central America, CONUS, Mexico

67	NAPARIMA, BWI - Trinidad and Tobago

68	NAHRWAN - Masirah Island (Oman)

69	NAHRWAN - Saudi Arabia

70	NAHRWAN - United Arab Emirates

71	OBSERVATORIO 1966 - Corvo and Flores Islands (Azores)

72	OLD EGYPTIAN - Egypt

73	OLD HAWAIIAN - Mean Value

74	OMAN - Oman

75	PICO DE LAS NIEVES - Canary Islands

76	PITCAIRN ASTRO 1967 - Pitcairn Island

77	PUERTO RICO - Puerto Rico, Virgin Islands

78	QATAR NATIONAL - Qatar

79	QORNOQ - South Greenland

80	REUNION - Mascarene Island

81	ROME 1940 - Sardinia Island

82	RT 90 - Sweden

83	PROVISIONAL SOUTH AMERICAN 1956 - Bolivia, Chile, Colombia, Ecuador, Guyana, Peru, Venezuela

84	SOUTH AMERICAN 1969 - Argentina, Bolivia, Brazil, Chile, Colombia, Ecuador, Guyana, Paraguay, Peru, Venezuela, Trinidad and Tobago

85	SOUTH ASIA - Singapore

86	PROVISIONAL SOUTH CHILEAN 1963 - South Chile

87	SANTO (DOS) - Espirito Santo Island

88	SAO BRAZ - Sao Miguel, Santa Maria Islands (Azores)

89	SAPPER HILL 1943 - East Falkland Island

90	SCHWARZECK - Namibia

91	SOUTHEAST BASE - Porto Santo and Madeira Islands

92	SOUTHWEST BASE - Faial, Graciosa, Pico, Sao Jorge, and Terceira Islands (Azores)

93	TIMBALAI 1948 - Brunei and East Malaysia (Sarawak and Sabah)

94	TOKYO - Japan, Korea, Okinawa

95	TRISTAN ASTRO 1968 - Tristan da Cunha

96	User defined earth datum

97	VITI LEVU 1916 - Viti Levu Island (Fiji Islands)

98	WAKE-ENIWETOK 1960 - Marshall Islands

99	WORLD GEODETIC SYSTEM 1972

100	WORLD GEODETIC SYSTEM 1984

101	ZANDERIJ - Surinam

102	CH-1903 - Switzerland

103	Hu - Tzu - Shan

104	Indonesia 74

105	Austria

106	Potsdam

107	Taiwan - modified Hu-Tzu-Shan

108	GDA - Geocentric Datum of Australia

- Dutch

# Appendix B: Binary Phase Output Format

In binary phase output mode, GPS 10 series products will transmit two types of packets once per second.  One record contains primarily post-process information such as position and velocity information.  The second record contains receiver measurement information.  For the GPS 10, the records are sent at a default baud rate of 9600 baud, 8 data bits, and no parity.

To turn these records on, use the $PGRMC1 NMEA sentence as described in Section 3.1.4.

Refer to the Garmin Device Interface Specification for details on how to form and parse Garmin packets. At the time of this printing, these specs are available from the technical suppport section of our Web site at http://www.garmin.com/support/commProtocol.html.

For the GPS 10, the two records must be enabled independently by commands to the unit. The ID of each command should be 10 decimal to signify that the record is a command.  The data portion of the packet should be one of the following:

| Function | Command (base 10) |
| --- | --- |
| Position Record On | 49 |
| Position Record Off | 50 |
| Receiver Measurement Record On | 110 |
| Receiver Measurement Record Off | 111 |

Note that the satellite data information is also enabled when the position record is enabled.

Records sent over RS232  begin with a delimiter byte (10 hex).  The second byte identifies the record type (33 hex for a position record, 34 hex for a receiver measurement and 72 hex for a satellite data record).  The third byte indicates the size of the data.  The fourth byte is the first byte of data.  The data is then followed by a checksum byte, a delimiter byte (10 hex), and an end-of-transmission character (03 hex).  Additionally, any DLEs (0x10) that appear between the delimeters are escaped with a second DLE.  There is sample code at the end of this section that will strip off the DLEs and ETXs.

RS232 Packet:

- 0x10 			(DLE is first byte)

- 0x##			(Record ID – single byte)

- 0x##			(Number of data bytes – single byte)

- data bytes		(See descriptions below)

- 0x##			(2’s complement of the arithmetic sum of the bytes between the delimiters)

- 0x10			(DLE)

- 0x03			(ETX is last byte)

USB packets contain only the record ID, size, and data payload. Refer to the Garmin Device Interface Specification for details on how to form and parse Garmin USB packets. The document is located on the Garmin Web site at http://www.garmin.com/support/ commProtocol.html.

USB Packet:

- 0x##			(Record ID – single byte)

- 0x##			(Number of data bytes – single byte)

- data bytes		(See descriptions below)

The data bytes of each packet contain the record specified by the record ID.  A description of each record follows.

Satellite Data Record

The satellite data has a record ID of 0x72 with 84 (0x54) data bytes. The data bytes contain the data for the 12 channels as described below. For each satellite, the following data is available:

typedef	struct

{

uint8	svid;	//space vehicle identification (1-32 and 33-64 for WAAS)

uint16 	snr;		//signal-to-noise ratio

uint8	elev;	//satellite elevation in degrees

uint16	azmth;	//satellite azimuth in degrees

uint8	status;	//status bit-field

} cpo_sat_data;

The status bit field represents a set of booleans described below:

| Bit | Meaning when bit is one (1) |
| --- | --- |
| 0 | The unit has ephemeris data for the specified satellite. |
| 1 | The unit has a differential correction for the specified satellite. |
| 2 | The unit is using this satellite in the solution. |

This pattern is repeated for all 12 channels for a total of 12 X 7 bytes = 84 (0x54) bytes :

typedef	struct

{

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

cpo_sat_data

} cpo_all_sat_data

The RS-232 Packet for the Satellite Record looks like:

- 0x10 			(DLE is first byte)

- 0x72			(Record ID – single byte)

- 0x54			(Number of data bytes – single byte)

- cpo_all_sat_data

- 0x##			(2’s complement of the arithmetic sum of the bytes between the delimiters)

- 0x10			(DLE)

- 0x03			(ETX is last byte)

The USB Packet for the Satellite Record looks like:

- 0x72			(Record ID – single byte)

- 0x54			(Number of data bytes – single byte)

- cpo_all_sat_data

Position Record

The Position Record has a record identifier of

typedef struct

{

float	alt;

float	epe;

float	eph;

float	epv;

int	fix;

double	gps_tow;

double	lat;

double	lon;

float	lon_vel;

float	lat_vel;

float	alt_vel;

float	msl_hght;

int	leap_sec;

long	grmn_days;

} cpo_pvt_data;

alt 	Ellipsoid altitude (meters)

epe 	Est pos error (meters)

eph 	Pos err, horizontal (meters)

epv 	Pos err, vertical (meters)

fix 	0 = no fix; 1 = no fix; 2 = 2D; 3 = 3D; 4 = 2D differential; 5 = 3D differential; 6 + = undefined

gps_tow 	GPS time of week (sec)

lat	Latitude (radians)

lon	Longitude (radians)

lon_vel	Longitude velocity (meters/second)

lat_vel	Latitude velocity (meters/second)

alt_vel	Altitude velocity (meters/second)

msl_hght	Mean sea level height (meters)

leap_sec	UTC leap seconds

grmn_days	Garmin days (days since December 31, 1989)

Receiver Measurement Record

typedef struct

{

unsigned long	cycles;

double	pr;

unsigned int	phase;

char	slp_dtct;

unsigned char	snr_dbhz;

char	svid;

char	valid;

} cpo_rcv_sv_data;

typedef struct

{

double	rcvr_tow;

int	rcvr_wn;

cpo_rcv_sv_data	sv[ 12 ];

} cpo_rcv_data;

rcvr_tow 	Receiver time of week (seconds)

rcvr_wn	Receiver week number

cycles	Number of  accumulated cycles

pr	Pseudorange (meters)

phase	To convert to (0 -359.999) multiply by 360.0 and divide by 2048.0

slp_dtct	0 = no cycle slip detected; non-zero = cycle slip detected

snr_dbhz	Signal strength - db Hz

svid	Satellite number (0–31, 19–138 for WAAS)  Note: add 1 to offset to current svid numbers.

valid	0 = information not valid; non-zero = information valid

DLE and ETX bytes:

Sample C code to receive the two records should filter DLE and ETX bytes as described below:

typedef enum

{

DAT,

DLE,

ETX

} rx_state_type;

/* Declare and initialize static variables */

static char	in_que[ 256 ];

static int	in_que_ptr = 0;

static rx_state_type	rx_state = DAT;

.

.

.

void add_to_que( char data )

{

#define DLE_BYTE 0x10

#define ETX_BYTE 0x03

if ( rx_state == DAT )

{

if ( data == DLE_BYTE )

{

rx_state = DLE;

}

else

{

in_que[ in_que_ptr++ ] = data;

}

}

else if ( rx_state == DLE )

{

if ( data == ETX_BYTE )

{

rx_state = ETX;

}

else

{

rx_state = DAT;

in_que[ in_que_ptr++ ] = data;

}

}

else if ( rx_state == ETX )

{

if ( data == DLE_BYTE )

{

rx_state = DLE;

}

}

if ( in_que_ptr > 255 )

{

in_que_ptr = 0;

}

}

# Appendix C: GPS 10 and Windows Serial Mouse Issue

Problem: It is possible for Windows to incorrectly interpret the NMEA 0183 output of the GPS 10 as the output of a Microsoft Serial BallPoint Mouse. When that happens, Windows loads drivers for the Serial BallPoint Mouse. This causes the cursor to move erratically about the display, clicking, right clicking, double clicking, dragging and dropping displayed items as it goes. On Windows 2000 and Windows XP, you may not experience the problem if you wait until after the computer is booted before connecting the GPS 10 to the serial communications port. This problem is not specific to the GPS 10. Any NMEA 0183 device connected to a Windows computer’s serial port is likely to cause this problem. Below are several possible solutions to this problem.

Note: If your GPS 10 came with Garmin nRoute™ Navigation Software, you will not have this problem because the unit is programmed to produce data according to the binary format that is described in Appendix B: Binary Phase Output Format.

If you wish to use a different navigation software application with your GPS 10, you will need to de-select the “Phase Output Data” option that is discussed in Solution 3 below.

Solution 1: The easiest solution is to disable the Serial BallPoint mouse in the Device Manager. Of course, this solution assumes that you do not need to use a Serial BallPoint mouse with your computer. When the wild mouse problem occurs follow the steps below.

- Unplug the DB9 connector.

- Right-click on the My Computer icon on your desktop and select Properties to open the Device Manager.

- Go to the hardware tab of the resulting pop-up window and click on the Device Manager button.

- Right-click on Serial BallPoint Mouse and choose the option to Disable (NOT uninstall) this device.

Solution 2: Do not plug the DB9 connector into the computer until the computer is turned on and the operating system is completely booted up and running. Every time you use the GPS 10, you will need to start the computer and operating system before making the connection between the GPS 10 PC and the computer. This solution usually works on Windows 2000 and Windows XP computers.

Solution 3: Change the default output of the GPS 10 so that it does not automatically send NMEA 0183 data to the computer (change the default configuration to enable “Phase Output Data”). Use the Garmin Sensor Configuration Software, named SNSRCFG.EXE, provided on the Web site to enable “Phase Output Data.” SNSRCFG.EXE is included with the latest GPS 10 firmware.

Please note that by enabling “Phase Output Data,” the device will no longer appear as a NMEA 0183 device to your computer. As a result, mapping applications that expect to hear NMEA 0183 data will not recognize your GPS 10 until you re-enable the NMEA 0183 output, disabling the “Phase Output Data” in the process.

Refer to Appendix D: Sensor Configuration Software for complete information about downloading the Sensor Configuration Software application.

After the Sensor Configuration Software is installed and running on your computer, follow the steps below to change the default data output of the GPS 10.

- Select the GPS 10 from the list of sensors and click OK. The Sensor Configuration Software opens with the default configuration file for the GPS 10, as shown on the next page.

- Select Config > Switch to NMEA Mode (or press the F10 key).

- Select Comm > Setup to open the Comm Setup Window.

- Select the Serial Port to which the GPS 10 is connected. Select Auto to have the program automatically determine the Baud Rate, or select Manual to manually select the Baud Rate of the GPS 10. Click OK when done.

- Click the Connect icon , or select Comm > Connect to connect to the GPS 10.

- To view the current programming of the GPS 10, select Config > Get Configuration from GPS (or press the F8 key). The current programming of the GPS 10 is displayed in the window, as shown on the previous page.

- Open the Sensor Configuration Window by pressing the F6 key or selecting Config > Sensor Configuration.

- Place a check mark in the box next to “Binary Output Data” to change the GPS 10 to Garmin Mode. The unit will keep this change in the programming until you change it again in the software.

- Note: Selecting Config > Switch to Garmin Mode (or pressing the F11 key) only changes the unit to work in Garmin Mode until power is cycled through the unit again. For a more permanent change, refer to step 8 above.

- Click OK when completed enabling “Binary Output Data.”

- When you are ready to upload the changes into the GPS 10, select Config > Send Configuration to GPS (or press the F9 key). The new configuration is then loaded into the GPS 10.

- You may disconnect and close the software when finished. The software configuration can also be saved for future reference.

Refer to Error: Reference source not foundAppendix D: Sensor Configuration Software for complete information about downloading the Sensor Configuration Software application.

# Appendix D: Sensor Configuration Software

SNSRCFG configures the GPS sensors based on user-selected parameters. Some application features include the ability to download GPS sensor configuration, maintain different configurations in files, and perform GPS sensor configurations quickly with the use of one function key.

This section provides a brief overview of the Sensor Configuration Software. Refer to this section when using the software to configure your Garmin sensor.

### Downloading the Sensor Configuration Software

The Garmin Sensor Configuration Software (SNSRCFG.exe) is available from the Garmin Web site. To download the software, start at http://www.garmin.com/oem, select the GPS 10, and then select Software Updates. The Garmin Sensor Configuration Software (SNSRCFG.exe) is included in the software update download.

### Selecting a Model

After opening the program (snsrcfg.exe), the following screen appears. Select the radio button next to the type of Garmin sensor you are configuring.

### Connecting to the Sensor

After selecting the type of sensor, the following window opens. This is the Main Interface Screen for the program.  To configure your sensor, you must first connect to the sensor.

- Select Config > Switch to NMEA Mode (or press the F10 key).

- Select Comm > Setup to open the Comm Setup Window.

- Select the Serial Port to which the sensor is connected. Select Auto to have the program automatically determine the Baud Rate, or select Manual to manually select the Baud Rate of the GPS 10. Click OK when done.

- Click the Connect icon , or select Comm > Connect.

- To view the current programming of the sensor, select Config > Get Configuration from GPS (or press the F8 key). The current programming of the sensor is displayed in the window shown to the right.

### File Menu

The File Menu allows you to open, save, and print sensor configurations. The items in the File Menu work like most Windows-based programs.

### Comm Menu

The Comm (Communication) Menu allows you to set the port number, baud rate, and then connect and disconnect from the sensor.

Setup: Opens the Comm Setup Window, shown to the left. Select the Serial Port to which the sensor is connected from the drop-down list. Then select Auto (the program determines the baud rate on its own) or Manual (you then enter the baud rate) for the Baud Rate entry.

Connect: Select Connect to connect to the sensor to change or view the configuration.

Disconnect: Select Disconnect to disconnect from the sensor.

### Config Menu

The Config (Configuration) Menu allows you configure the sensor as it is connected.

Sensor Configuration (F6): Opens the Sensor Configuration Window, shown to the right. Many of the fields in this window should be left alone. Please refer to the beginning of this manual for clarification about some of these fields. For the most part, this window is used to enter a new Latitude, Longitude, and Altitude for the sensor. This is especially helpful when you are programming the sensor for use in a particular geographic location.

Resetting the Unit (Reset Unit) performs a reset on the unit, much like cycling the power.

Resetting the non-volatile memory (Reset NonVol) will clear all of the data from the non-volatile memory.

NMEA Sentence Selections (F7): Displays the NMEA Sentence Selections Window. If the sentence is enabled, a check mark appears in the box to the left of the sentence name. Click the box to enable or disable to the sentence.

Get Configuration From GPS (F8): Retrieves the current programming from the sensor. The programming is then displayed in the Main Interface Window.

Send Configuration To GPS (F9): Sends the changes you have made to the programming to the sensor.

Switch to NMEA Mode (F10): Switches the unit to NMEA Mode. The sensor must be in NMEA Mode when connected to this software.

Switch to Garmin Mode (F11): Switches the unit to Garmin Mode.

Update Software (F12): After you have downloaded a new software version for the sensor, you can update the sensor with the new software. Select Update Software and then select the file using the Open dialog box. You must locate both the *.rgn file and the updater.exe file.

Do not attempt to update the GPS 10 software unless Bluetooth reception conditions are optimal. Devices such as microwave ovens and 802.11b/g wireless networking gear can interfere with the Bluetooth link, and may cause the software update to fail.

Note that, failed software update attempts are generally recoverable. If a software attempt fails, identify the source of interference and try again.

### View Menu

The View Menu allows you to view the NMEA sentences transmitted by the sensor. You can also customize how the program looks by showing and hiding the Toolbar and Status Bar.

### Help Menu

The Help Menu displays the software version and copyright information.

APPENDIX E: HOST SYSTEM CONSIDERATIONS: PIN Code.

Some systems may or may not require a PIN Code in order to recognize the GPS 10.

The default PIN code for GPS 10 is “1234”. When you initially make this connection or the connection is terminated unexpectedly.

Just type “1234” and click OK.