Getting Started with the Magellan 310 GPS Receiver

Connections: The serial cable should be connected to the back of the receiver. Plug the female DB-9 connector into the serial port on the left of the back panel of your BrailleNote or VoiceNote per the instructions in the User Manual. The connector is longer on the top edge than the bottom.  Make sure the connector is up the right way. Gently tighten the thumbscrews.

Power: To power up the Magellan 310 GPS receiver, depress the button in the lower right corner.  To turn the unit off, press the Power button. It will take 6 seconds to power off after you let up on the button.

Batteries are already installed in your unit. To change batteries, look on the back of the receiver and find a ring handle like the one on the Magellan end of the serial cable, on a screw in the middle bottom back of the unit. Lift out the lid and find 2 double A batteries.  The right battery positive end should face down toward the spring and the left battery positive end faces up with the flat side on the spring.  You will normally get 18 hours or so of continuous use on a full charge or new set of batteries.

Can I use NiCad Batteries in my GPS 310?

Yes. However, the battery life of your GPS 310 will be diminished with the use of NiCad batteries versus regular batteries.

Can I attach my GPS 310 to external power?

Yes. However, this requires the optional 12-volt power cable available from Magellan Corporation or one of its dealers.

Will I lose all my settings when my batteries die?

No. As long as you leave the batteries inside the GPS 310, memory will be retained for up to one month, even with dead batteries. (With good batteries installed, you can store your GPS 310 for six months without losing any memory.) When you remove batteries, you have 30 minutes to install new batteries before memory is lost.

Note: The bottom left button is a screen brightness toggle. Press it once to turn the light on.  In a quiet environment, you will be able to hear a buzzing.  Press it again and the buzz will stop when the light is switched off. Hearing this buzz is one way to check if the unit is on.  You can also tell from the GPS software. When you turn the receiver off and wait 6 seconds, you will hear, “GPS not detected” on your BrailleNote or VoiceNote.

Acquiring a Position: Place the receiver on the dash or near a window to begin acquiring GPS satellites. When you travel to a new area or if the unit has been off for several days, it will take longer to determine the GPS position. Turn on your VoiceNote or BrailleNote and select the GPS application in the main menu. To find out tracking information, either select GPS Status in the GPS menu or use the hot key from within the program.

HOW TO INITIALIZE THE MAGELLAN 310 GPS RECEIVER

It will not normally be necessary to initialize the receiver. The receiver is shipped from the factory with batteries and is preconfigured. According to the manual, if you leave bvatteries out of the receiver for mor than 30 minutes, it may be necessary to initialize it again.  If the batteries simply run low, there will still be enough power to keep the receiver initialized for several days or weeks.

When you get your GPS receiver, take a moment to familiarize yourself with the layout of the unit.  On the front panel, you'll find 7 small buttons and one large round control, which acts as a multidirectional arrow for menu navigation.

Description of controls:

Column of Buttons on the left side:

Bottom:  Light

Middle:   Mark

Top:  Navigation

Column of buttons on the Right side:

Bottom, Power

Middle, Menu

Top,  GoTo

Center:

Top,  Enter

Round button, Up-Down, Left Right

If your Magellan 310 needs to be initialized, it is recommended that you have sighted assistance. However, if you follow the instructions below, you may be able to get the unit to communicate on your own.

Summary for US users: Power on, Menu key, Down Arrow once, Enter, Down Arrow 3 times, Enter 5 times and finally press the Navigation key.

Detailed initialization instructions

Turn the unit on with the power button in the bottom right corner.  The GPS receiver will bring you up to the satellite status screen.

Press menu, the middle button on the right.

You will be at the Landmarks option menu.

Press the down arrow once by pressing the bottom part of the middle round key.

The Set-Up options will be displayed.

Press enter to Select the Set-Up options. This is the long button above the center round button.

The first option is Coordinates.  Press the Down Arrow button 3 times.  You will then be at the EZ setup selection.  Press Enter 5 times to accept the defaults.

Press the navigation key (top left button) and wait for the receiver to begin tracking. It may take a while the first time you track after initializing.

Should you wish to change the default settings while initializing, here are some details to be aware of:

Item 1 under the EZ Start section is Region/country. The default is USA. When you press Enter, that item is selected. Press Down Arrow once for Europe, twice for Australia and New Zealand. Going up from USA, you get the following:

Africa, Americas, East Asia, West Asia, Atlantic, Australia, Europe, Pacific, USA.

Item 2 is State if USA is selected. The default is AK for Alaska at the beginning of the Alphabet. Press Up Arrow to go from A to W in the state list.

Item 3 is Elevation

Item 4 is Time

Item 5 is Date.

This is the end of the EZ Start items. Other options are:

Item 6 is Demo.

Item 7 is Contrast. Press Right or Left Arrow to adjust.

Item 8 is coordinate system, Lat Lon.

Item 9 is Degrees Minutes resolution.

Item 10 is Map Datum. Default is WGS84.

Item 11 is Units. Default Miles per hour.

Item 12 is time format. Default 12 hour.

Item 13 is another Time setting.

Item 14 is North reference. Default is magnetic.

Again, it is not necessary to initialize unless the receiver will not track.

Be sure to read the BrailleNote or VoiceNote GPS User Manual for more information. For technical support, refer to the full Magellan 310 manual also available in the Documents folder of your Storage card or review the Q&A at the end of the User Guide.

Thales Magellan Corporation technical support can be reached at 1-909 394-5000 for support on the receiver only.

Pulse Data HumanWare technical support at 1-925-680-7100