To connect a Bluetooth GPS receiver or other Bluetooth device to a Windows phone, do the following:

Step 1: First go to the phone’s menu, to Settings, Connectivity, Bluetooth. Select Add new device by pressing select.  Select the GPS receiver.

It is necessary to be patient as the Bluetooth receiver is searched for. It will take several seconds. Press HOME then ENTER to verify you are connecting to the Holux.  When prompted for the pass code, type 0000 and press NEXT.  You will receive verbal confirmation after entering the passcode that a connection has been made.  Press LEFT SOFT to close the confirmation dialog.

Step 2:  At the display name prompt just press next to accept the default display name and continue unless you wish to change it.

Step 3:  Press enter to enable the Serial port checkbox, then press soft key 1 when you are done.

Step 4: In the connectivity menu, select the GpS receiver with right soft.

Arrow to the menu item Com Ports and then select “Add a com port” and press enter.

Press right soft for menu.

Arrow to new outgoing GPS receiver.

Then Arrow right or left to select Com 0 with the center Enter key. You should hear, Holux Com 0 checked.

Then Press Left Soft until you return to the “Today screen.”

Step 5: Start Mobile Geo. Mobile Geo asks if you wish to configure your receiver, answer Yes by pressing Left soft.  Scroll to the left and right until the Bluetooth GPS receiver on com0 is selected and press enter.

If a connection is not established, reboot the phone. This may be particularly necessary after switching from one GPS receiver to another.

Note, if your phone has an internal GPS receiver, it may be on Com 4