01 Intro

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator.

"Welcome to the Sendero GPS audio tutorial.  I'm Kim Casey, GPS training coordinator at Sendero group."

"SENDERO Group specializes in accessible location and information technology.  For more information about Sendero Group visit our website at: www.senderogroup.com or email GPS@senderogroup.com. Sendero customers call call toll-free in the  United States and Canada- 888-757-6810."

"All other customers or international callers dial: 1-530-757-6800."

"This tutorial copyright Sendero Group LLC 2008 should not be copied or distributed without written consent of Sendero Group.  The content of this tutorial may change as new versions of Sendero GPS are released."

"See the readme document for the latest changes and version updates."

"Sendero GPS is a powerful orientation tool.  It uses GPS satellites  so that you can receive announcements about your position and nearby points of interest."

"You can easily find specific addresses, have access to millions of points of interest, and calculate a route between 2 places."

"So say, for example, that you plan to travel to an unfamiliar place.  Before you even go you can use GPS to familiarize yourself with the area."

"Simply input the address of the location that you're trying to go to and you'll be able to look around the area using the virtual explore mode."

"You will then be able to see points of interest and nearby addresses.  Once you are there, you will actually be able to have the GPS location information in real-time as you're traveling around.”

"You will then have access to a wealth of information such as intersections you're crossing, points of interest that you pass by, nearby addresses, direction of heading and also speed of travel."

"If you have created a route, you will have detailed step-by-step directions automatically anounced as you proggress in your route."

"This product is an excellent navigation aid, but it does not replace the need for careful orientation and good judgement.  Never rely solely on GPS or any one device for navigation."

"There are numerous factors that  effect GPS and map accuracy.  It is essential to read the GPS accuracy document available on the Sendero site or in the other documents folder on your cf card or in the document zip file."

"The better you understand the variables of GPS, the more you will understand the information you receive."

"The purpose of this Sendero GPS tutorial is to familiarize you with your new GPS system, covering many but not all points."

"Each section in this tutorial is separated by the sound you heard at the beginning so you will know when a section ends and when a new one begins."

"Here's a quick overview of what is covered in the GPS tutorial.  This tutorial is structured to mimic a Sendero GPS training sesion.  It's approximately 126 minutes in length."

"Major pauses are eliminated for the sake of brevity."

"Following this introduction, there are 11 more sections in this tutorial and they are:

1.  Installing Sendero GPS software- Learn how to install SENDERO GPS software onto your Braille Note or Voice Note.

2.  Downloading maps, points of interest and software from mysendero.com.

3.  Pairing your Bluetooth GPS receiver

4.  Starting Sendero GPS for the  first time

5.  Sendero GPS help menu system

6.  virtual mode and exploring routes

7.  Points of interest

8.  Using Sendero GPS in a vehicle

9.  Walking around with Sendero GPS

10.  GPS replay files

11.  the Conclusion"

"For items not covered in this tutorial, please refer to the Users Guide, Changes in readme document, or sendero Group support and documentation website."

"Go to senderogroup.com and select the support and ducumentation link and then the Sendero GPS link."

"Now without further ado, here's SENDERO GPS training specialist Chris Grabowski to get you started with your Sendero GPS!”

02 Installing Sendero GPS

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator, Chris Grabowski.

"Throughout the course of this tutorial, I'll be using a Braille Note QT for the demonstrations, but I'll be giving the BT and PK keystrokes when relevant."

"In this section, I'm going to show you how to install the Sendero GPS onto your Braille Note or Voice Note."

"I'm going to turn the Braille Note on.  On the left side, you'll find a rocker switch.  If you flip it towards you, the Braille Note will turn on."

"On the PK, you press and hold c-4.  C-4 is the little round button all the way to the right of the spacebar."

BN- "Main menu."

"Now we're at the main menu.  So the next thing I'm going to do is insert my compact flash card into the compact flash slot.  If you notice, the compact flash card has a lip on it.

"If you have the Braille note in its operating position, that would be the Braille display closest to you, the lip needs to face down and be furthest away from you.  If you were to lay the card flat on the table, the lip would be closest to the table."

"You take the card and you slide the card into the slot on the back of the Braille Note." I'm going to do that now."

BN- "Compact flash card ready."

"The Braille Note announces that the compact flash card is ready."

"If your GPS is on the SD slot, if you feel off to the left of the compact flash slot you'll find the SD card slot and you'd slip the SD card in.  It only goes in one way as does the compact flash card, so don't force it."

"I'm going to press the space bar until I get to utilities, then I'm going to press enter."

BN- "Word processor, planner, address list, email, internet, media center, book reader, scientific calculator, database manager, games, file manager, utilities."

"I'M going to press enter here."

BN- "Utilities menu."

"I'M going to press the space bar until I get to application program and then press enter."

BN- "Connect to active sync, synchronization, backup or restore files, pronunciation dictionaries, date and time set, install KeySoft extension, application program.."

BN- "Run program from which drive? Press enter for flash disk."

"The Braille note prompts us to select a drive.  We need to select the drive that our GPS installation file is on.  So I'm going to press c for compact flash.  If your GPS was on the SD slot, you would select s or you can just press space to get a list of your drives so you don't have to remember the hot keys if you don't want to."

BN- "Compact flash.  Directory name? Press enter for slash"

"I'm going to press enter for the default directory."

BN- "Program name?"

"I'm going to press space bar until I get to the GPS installation program."

BN- "List of programs in compact flash- GPS 5.0 complete install MPower."

"That’s the file I want, the GPS 5.0 complete install MPower.  If you are installing on a PK your filename would be GPS 5.0 complete install PK.  So I'll press enter."

BN- "Command line?"

"Press enter on command line."

BN- "Welcome to GPS 5.0 complete install MPower installation.  To start the installation press enter.  To cancel, press escape."

"On a BT or PK I would press space with e instead of escape to exit, so I'm going to press enter to continue."

BN- "Utilities menu." (Several beeps are heard.)

BN- "Installation complete.  After resetting, go to the KeySoft options connectivity menu and activate the Bluetooth GPS receiver, then press n from the main menu to run Sendero GPS.  To continue, press reset with no keys held down."

"So I'M going to press reset.  But before I do that, I'm going to remove the compact flash card."

BN- "Please wait.  Compact flash card removed."

"Now the reset button is just behind the on-off switch.  On the PK, you would reset by pressing and holding c-3 and c-4 and those keys are to the right of the space bar."

BN- (A tone is heard, followed by default BN start-up tone.)

BN- "KeySoft c e version 7.5.  Build 29.  Main menu."

"Now the Braille Note is reset and the GPS program is installed."

"Note- if you're upgrading from an earlier version of GPS before running the installation program, you must do a 1-2-3 or an s-d-f reset.  To do that, you press and hold dots 1 2 3 on the BT or PK and on the QT you'd press and hold keys s d f.  While doing that, you press the reset button, and keep 1 2 3 or s d f held down until the Braille Note resets and comes up talking.  The equivalent of the reset button on the PK is keys c-3 and c-4.  So you would press and hold down those along with the dots 1 2 and 3 on the PK."

"In the next section, we'll pair and activate our Bluetooth GPS receiver."

03 Downloading maps

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator, Chris Grabowski.  Also for this tutorial, the abbreviation ie will be used to indicate dialogue from Internet Explorer.

"In this section, I'm going to show you how to download maps using the Sendero demo account."

"As of version 5.0 Sendero GPS includes a 15-day serial period.  All users can access this 15-day trial before registering the product and you have access to all maps Sendero offers."

"I'm currently in Internet Explorer, and I'm going to press control o to type in a new web address."

Ie- "o.  Dialogue- open."

"I'm going to type in gosendero.com and the reason I'm using gosendero is because it takes us directly to the login page.  I'm then going to press enter."

Ie- "Mysendero.com window internet explorer.  9 links, 2 frames, 1 table."

"I'm going to press the key that I need to use to move to the edit area so that I can type in the username."

Ie- "Edit box serial number."

"Press enter."

Ie- "Browse off, serial number."

"We're going to use the word demo as the username.  I'm going to press tab."

Ie- "Password, edit box."

"I'm going to type in Sendero as the password and press enter."

Ie- "Mysendero.com windows internet explorer.  Login button, 25 links, 2 frames..."

"So now let's explore this page."

Ie- "SELECT language.  Go button.  Link skip to main content."

"I'm going to press enter to skip to the main content."

Ie- "25 links 2 frames 3 tables.  Hello demo user.  Heading 1- maps- By downloading maps and POI's, you are agreeing that you will not give this data to anyone else and after your trial has expired you will delete all copies made.  Copyright Sendero Group 2006-2008 all rights reserved."

Ie- "Map and POI data copyright by Tele Atlas.  Choose map.  USA combo box."

"So now I'm going to choose a map and I'm going to press enter because I want to switch to a different country."

Ie- "Browse off.  Choose map- USA combo box."

"I'm going to scroll down."

Ie- "Canada, UK, France, Germany,."

"The one I want is Canada, so I'm going to scroll up to Canada."

Ie- "France, UK, Canada."

"I'll now press the tab key."

Ie- "Show downloads button."

"I'll press enter."

IE- "25 links."

"Now the page has changed.  It was displaying the United States maps, now it's displaying the Canada maps."

IE- "Link skip to main content."

"Again, we'll use this link."

IE- 25 links 2 frames.  Hello demo user.  Heading 1- maps.  Choose map- Canada combo box."

"Now that is changed from USA to Canada.  I'll scroll down some more."

IE- "Show downloads, button.  Canada download includes Canada.  Link 2008 POI user Canada Public.zip 1mb.  Link- Alberta 2008.zip $16mb."

"Let's download Alberta.  I'll press enter."

Ie- "Cancel button dialogue file download.  Do you want to save file or find a program online to open? Link- what's the risk? Find, save button."

"Press enter on save."

IE- Dialogue- c users owner downloads file name- Alberta 2008 combo edit box."

"I'm going press enter."

"Our download is almost complete."

IE- "Dialogue- download complete, close this dialogue box when down completes?  Checkbox."

"Now I'm going to tab to open and press enter."

IE- Open.  Alberta 2008.zip."

"Now I'm in a list of files."

IE- "Type, folder Alberta 2008.pro.  Alberta 2008.rmf."

"So what I'm going to do is control a to select all of the files in this zip file and then I'm going to hit control c to copy to the clipboard."

IE- "Copied to clipboard."

"Now I'm going to insert my compact flash card."

IE- "Dialogue autoplay."

"Autoplay comes up."

IE- "Open folder to view files using windows explorer 1 of 8."

"That's what I want to do so I'll press enter."

IE- "Windows “explorer Asap Daisy manual date modified."

"Now I'm going to press m to get to the maps folder."

IE- "M.  Maps.  Date modified 10 05 2008."

"I'll press enter."

IE- "2008 POI tomorrows."

"Then I'm going to press control v to paste the files into the maps folder."

IE- "Pasted from clipboard.  2008 Poia.ms."

"Now if I scroll down the list I'll see."

IE- "Alberta 2008.pro.  Alberta 2008.  Alberta 2008.str."

"So now I can close this window."

"I've logged out of the Sendero demo account and have logged into my own account.  I want to show as how you find your registration key.  This is important because in another section, we will use this key to register the full product."

"I'm going to scroll down."

IE- "Hello Christopher Grabowski.  Link- you may update or view existing user information such as name email, password and SMA status."

"If I follow this link I can update my contact information, my email address, and see how many SMA's I have.  I'll scroll down again."

IE- "Heading 1 license key."

"If we scroll down some more, we'll find our license key.  The license key is only good for my Braille Note with my serial number."

"I'll scroll down again."

IE- "Email license key button."

"If I press enter on this button, it will email me a copy of my license key, providing I have an email address on file."

04 Pairing GPS receiver

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator, Chris Grabowski.

"In this section I'm going to show you how to pair and activate your Bluetooth receiver."

"I have the Braille Note in front of me and I'm currently at the main menu."

BN- "Main menu."

"On the Braille note QT I'm going to press function with o to bring up the options menu.  On the BT, the command is space with o."

BN- "Options men."

"Then I'm going to press space bar until I get to connectivity."

BN- "Date, time, grade for Braille entry, Braille options, keyboard settings, review voice, connectivity."

"I'll press enter."

BN- "Connectivity menu."

"I'm going to press space until I get to Bluetooth."

BN- "Create a new dial-up or LAN connection, active connection, Bluetooth."

"I'll press enter."

BN- "Bluetooth on? Currently no."

"My Bluetooth is off, so I'm going to turn it on by hitting y."

BN- "yes.  Starting Bluetooth, please wait.  Search for devices?"

"Now at this prompt, it says search for devices and I'm going to answer no because I want you to hear what it sounds like when you don't have any devices connected to your Braille Note.  I'm going to hit n."

BN- "No.  Could not find any Bluetooth devices.  Search for devices?"

"IT says could not find any Bluetooth devices and search for devices.  So basically what it it's telling us is that the Braille Note does not have any record of any Bluetooth device being paired to it.  My recommendation is to always answer no and the reason for this is because if you search for devices when your Braille Note is already aware of your GPS receiver, you could in fact confuse the Braille Note making it impossible to connect to your receiver without having to remove the pairing first.  So I'm going to answer yes to search for devices.  First, I'm going to turn my receiver on and hit y for yes."

BN- "Yes.  Searching for Bluetooth devices, please wait."

"The progress beeps are there to let us know that something is going and that the Braille Note is indeed searching for Bluetooth devices.  It may pick up other things such as your cell phone, a Bluetooth printer, things of that nature."

BN- "List of Bluetooth devices."

"Now we are presented with a list of Bluetooth devices and I'm going to hit my space bar."

BN- "Holux m-1000."

"I'm going to press enter."

BN- "List of services for Holux m-1, 000."

"Then I'm going to space bar until I get to active sync and press enter."

BN- "Active sync.  Pair with Holux m-1, 000 and activate Active Sync?"

"So it wants to know if we want to pair with or connect to the GPS receiver and activate active sync.  I'm going to hit y for yes."

BN- "Yes.  Authentication code?"

"Now, we're prompted for an authentication code.  If you are using KeySoft 7.5 or greater, you do not need to enter an authentication code on the PK or the mPower.  If you are using KeySoft 7.2, you do need to enter the authentication code on the PK but not on the mPower.  On the mPower you can just press enter.  Some Bluetooth receivers use 0000 as their authentication code while other Bluetooth receivers would use 1234.  So I'm going to press enter."

BN- "Paired with Holux m-1, 000.  Active Sync, active."

"It said paired with receiver name active Sync is active.  One thing I want to mention too, is if you do a reset, just a plain reset with no keys held down, your receiver remains paired but your active status will disappear.  So I'm going to do that now."

BN- resets and says- "KeySoft CE version 7.55 build 29.  Main menu."

"So now I'm going to press function with o or space with o on the BT in order to bring up the options menu."

BN- "Options menu."

"I'm going to press c for connectivity."

BN- "Connectivity menu."

"I'm going to press b for Bluetooth."

BN- "Bluetooth on? Currently no."

"I answer yes."

BN- "Yes.  Starting Bluetooth, please wait.  Search for devices?"

"I'M going to answer no."

BN- "No.  List of Bluetooth devices."

"This time we got a list of Bluetooth devices, where the other time it said that it couldn't find any Bluetooth devices.  We'll hit our space bar."

BN- "Holux m-1,000 paired."

"If you notice, it says paired, but the active status is gone.  We'll press enter."

BN- "Remove pairing with Holux m- 1,000 or activate active sync?"

"PRESS a to activate active sync."

BN- "Activate list of Bluetooth devices."

"Now we're back to the list of blue tooth devices so I'm going to hit the space bar."

BN- "Holux m-1,000 paired and active."

"So now if you notice, it's paired and active.  That's one good habit to get into when you do a reset, you'll always want to go to the options menu connectivity, Bluetooth, and activate your Bluetooth receiver just like I'm demonstrating here."

"In the next section, we will start the Sendero GPS for the first time."

05 Starting Sendero GPS First Time

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator, Chris Grabowski.

"In this section, I'm going to show you how to start the Sendero GPS for the first time.  If your dealer installed the GPS for you, some of these steps may not apply."

"I have the Braille Note in front of me and I'll start by inserting the compact flash card into the Braille Note."

BN- "Compact flash card ready."

"Then I'll go to the main menu."

BN- "Main menu."

"And I'll press the letter n for navigation."

BN- "Sendero GPS version 5.0 mPower.  Press y to read the license agreement, or press n to return to the main menu."

"The first question is view Sendero license agreement.  This step is required.  If we answer yes, the license agreement launches in the book reader.  If we answer no, we exit the GPS application.  I'll press n for no just to see what happens."

BN- "No.  Sendero license agreement not accepted.  Exiting GPS.  Main menu."

"So now we're back at the main menu.  I'll hit the letter n for navigation."

BN- "Sendero GPS version 5.0 mPower.  Press y to read the license agreement, or press n to return to the main menu."

"This time, I'm going to view the Sendero license agreement."

BN- "Yes.  Launching book reader.  Top of document.  Sendero!"

"We are now in the book reader with the contents of the license agreement displayed.  So we would use standard book-reading commands in order to read the license agreement.  I'll do that."

BN- "By accepting this license agreement, you acknowledge the following warnings..."

"I will not make you listen to the entire license agreement during the audio tutorial but you should read it.  If the license agreement was accepted on your behalf by a dealer, you can still view the license agreement in the information center.  The information center will be discussed in a later section."

"I'm going to press escape on the QT.  and space with e on the BT or PK in order to exit the license agreement."

BN "Exit.  Do you accept the terms of the Sendero license agreement?"

"Now it's asking me "Do I accept the terms" and yes I accept the terms.  If I answered no at this point, I will once again be thrown out of the GPS application.  I must answer yes."

BN-  "Yes.  Which version would you like to use? Trial version or register full version of Sendero GPS?"

"What version would I like to use? That's the next prompt.  Do I want to run the trial version? Did I download this and I want to play with it for a little while before I purchase it or do I already have a license key that I can register right now?"

"I'm going to press t for the trial version."

BN- "Trial version.  15 trial days remaining.  To download your maps and POIS go to www.gosendero.com. For the serial number use "demo" and for the password use "sendero".  Once logged in choose the country and maps you wish to download.  Unzip them on your PC and copy those unzipped files to a storage media card, in a subdirectory called maps.  Exit this program, insert the storage media card into your mPower and start Sendero GPS again by pressing n.  Press space to continue."

"So basically, this is giving you instructions on how to access  the demo account.  The user name is "demo" and the password is "sendero".  The website is www.gosendero.com.  This will allow you to download any map that Sendero has to offer and you can use those maps for the duration of your trial."

"I'm going to press space to continue."

BN- "7 maps loaded.  Enter the name to use when creating new user POI's.  Your initials are a good example."

"When you view the details of any point of interest, there is now an author field.  This tells you who created the point of interest.  For commercial points of interest, the name you'll see is "Tele-Atlas." For user points of interest, you may see somebody's name or somebody's initials.  This will allow people to know who created that user point of interest.  So if you share the user points with your friends, when they view the point of interest they will know that it was created by you."

"I'm going to type in my initials and press enter."

BN- "CG.  POI athuor has been updated.  GPS receiver set to none, in virtual mode.  Select the map to set your virtual position to- New York."

"Now we're presented with a list of maps that are in the maps folder to set our virtual position to because we do not have a virtual position.  This is a quick way to place your position on a map.  These are pre-defined positions which usually would be the capital of the state or province, capital of the country, or it can be a tourist attraction.  It's just a quick way to get your position  onto a mas so that you  can virtually explore some place."

"I'll press enter for New York."

BN- "New York chosen.  Virtual position set to New York Albany city."

"It just said Albany city, which Albany is the capital of New York and its the category is city.  We'll get to categories and points of interest in a different section."

"Now I'm going to exit the application by pressing escape on the QT or space with e on the BT or PK."

BN "Exit GPS? Yes No."

"I'm going to answer yes."

BN "Yes Main menu."

"Through the miracle of audio editing 24 hours has passed.  I'm at the main menu and I'm going to press n for navigation."

BN Sendero GPS version 5.0  mPower.  14 trial days remaining.  7 maps loaded.  GPS receiver set to none in virtual mode."

"If you notice, my 15 day trial is now a 14 time trial."

BN "Albany, city."

"So now that we have evaluated the GPS application it's now time to register the product."

"I'm going to do that now by pressing o for options."

BN "GPS options menu."

"Now I'm going to press a for advanced GPS options."

BN "Advanced GPS options menu."

"I'M going to press r for register product or new map."

BN "Enter license key."

"So now we're prompted to to enter our license key.  The key is not case sensitive and on the BT or PK you would use computer braille."

"So when entering numbers, dot 2 would be 1 dots  would be 2-3 would be 2 etc."

"I obtained my product key from my account at www.gosendero.com.  There you have the ability to view your license key and you can also have your license key emailed to you if you have an email address on file."

"So now I'm going to type in  my product key and press enter."

BN "Successfully registered 12 countries.  Exiting options menu.  7 maps loaded.  13 commercial  poi files loaded.  0 personal user points."

"So I've registered the product it told me how many countries I have access to.  To verify, I'm going to exit the GPS.  I'm going to press escape on the QT or space with e on the BT or PK."

BN- "Exit GPS? Yes no."

"I answer yes."

BN- "Yes.  Main menu."

"Now I'm going to press n for navigation."

BN- "Sendero GPS version 5.0 mPower.  7 maps loaded.  GPS receiver set to none, in virtual mode.  Albany, City."

"If you notice, my product is now converted into a fully registered copy."

"The final topic I would like to cover in this section is setting your GPS receiver.  Remember in the pairing Bluetooth receiver section, the receiver and Braille Note both know about each other.  Now we need to tell the GPS program the name of our receiver."

"If you noticed, when we came in so far, the GPS receiver has been set to none, or no receiver."

"So what I'm going to do is press the letter o for options."

BN- "GPS options menu."

"Then I'm going to press g for set GPS receive."

BN- "GPS receiver currently set to none.  Receiver selection menu."

"So now the GPS receiver receiver is set to none meaning no GPS."

"So now I'm going to press b or I can space-bar down until I get to Bluetooth and press enter.  The navigation of the menu system will be covered in a later section as well." That's why I haven't spent much time in navigating and I've been just giving hot keys."

"I'm going to press b."

BN- "Attempting to connect to Holux m-1000."

"So now it's looking for the receiver.  The receiver is on."

BN- "Successfully connected to receiver.  exiting options menu.  Holux m-1,000 detected.  No fix, acquiring satellites."

"Now the GPS program knows about the GPS receiver and you should really never need to go into the GPS receiver menu unless-

1.  you change to a new receiver, or

2.  you are utilizing the GPS replay functionality."

"I'm going to press escape on the QT or space with e on the BT in order to exit the GPS application."

BN- "Exit GPS? Yes no."

"I'M going to answer yes."

BN- "Yes.  Main menu.”

06 Menu and Help System

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator, Chris Grabowski.

"In this section, I'm going to show you how to navigate the Sendero GPS menu system."

"The GPS program has loads of hotkeys that you can use to ask for information around you such as your nearest intersection, the nearest address, and even heading and distance to your next turn."

"Sometimes, though, it's hard to remember hotkeys.  So what we have done is created a menu system.  You access the menu system by pressing the context sensitive help key."

"On a QT, it's the help key and on a BT or a PK it is space with h when you're in the program."

"Once you're in the help menu you will notice that it is divided into categories."

"You have categories such as where am I commands, GPS commands, virtual commands, etc.  Pressing enter on any of those menu items will bring you into a list of other menu items that you can browse through and press enter on to execute."

"My Braille Note is in front of me and I'm at the main menu."

BN- "Main Menu."

"I'm going to press n to go into navigation."

BN- "Sendero GPS version 5.0 mPower."

"The maps are loading."

BN- "7 maps loaded.  Turn on Holux m-1, 000.  To connect press c, for GPS mode without connecting, press g, or for virtual mode, press v."

"So the receiver is not turned on and the GPS program is letting us know that.  It's giving us options.  If we forgot to turn the receiver on, we could press c to connect after we turn the receiver on.  We can press g to go into GPS and we'll be placed at our last known GPS position.  Pressing v allows us to go into virtual mode."

"I'm going to press v to go into virtual mode."

BN- "In virtual mode."

"It says that we are in virtual mode.  I'm going to press the help key on a QT or space with h on the BT or PK to bring up the help menu."

BN- "GPS help menu."

"Now we're at a list of menu items and you would navigate the menu system the same way you would at the Braille Note's main menu.  You hit your space bar to go forward, and backspace to go backwards.  I'm going to press the space bar."

BN- "WHERE am I commands help w."

"Where am I commands, these will tell you things such as what city you're in, your nearest address, your nearest intersection, things of that nature.  I'll hit the space bar again."

BN- "GPS commands, help g."

"You'll find GPS related commands such as being able to connect to your GPS receiver, getting speed information, getting altitude, things like that.  Space bar."

BN- "Virtual commands, help v."

"ANY command dealing with virtual mode, how to switch to virtual mode, how to navigate in virtual mode, even how to set an address or a POI in virtual mode." Space bar."

BN- "Destination commands, help d."

"Again, anything dealing with your destination: how far your destination is from you, how to set an address as a destination, things of that nature."

BN- "Points of interest commands, help p."

"Anything dealing with points of interest, creating or searching, you'll find in the points of interest menu."

BN- "Route commands, r"

"Now if you notice, it said route commands r, and not route commands help then r or space with h then r.  This means that you can press the letter r from within the GPS application to pull up the route commands.  I'll press the space bar."

BN- "LookAround commands, read with m."

"LookAround commands are the same way.  You can press read with m on a QT or a space with m on the BT and that will bring you directly into the LookAround commands menu.  Space bar."

BENEATH- "Multiple repeat commands m."

"MULTIPLE repeat commands, pressing m will bring you into the multiple repeat commands menu without entering help."

BN- "MISCELANEOUS commands help c."

"Miscellaneous commands help then c, or space with h then c.  The commands found in the miscellaneous items do not belong anywhere else.  Space bar."

BN- "Option commands o."

"Pressing enter on options brings us into the GPS options menu, where you would set your units your map and POI folder, things of that nature."

BN- "Information center, help I."

"The information center is where you would go to read documentation, the manual, the command summary, the changes from the previous version, etc."

BN- "Exit program, escape."

"I can exit the program by pressing escape.  What I'm going to do though, is I'm going to press backspace, because I want to move to the information center."

BN- "Information center, help I."

"I'm going to press enter."

BN- "Information center."

"I'm going to press the space bar."

BN- "About GPS help I a."

"About GPS help I a or space with h then the letter I then the letter a and that will bring you into the about GPS option.  Let's press enter and see what we have in here."

BN- "About GPS menu."

"So again, we're in a menu, so I'm going to press space bar."

BN- "SENDERO GPS version 5.0 mPower."

"This tells us the Sendero GPS version number and the type of device that it's running on.  If it's running on the MPower or if it's running on the PK.  Space bar."

BN- "SENDERO GPS SDK version 3.0."

"Ok.  I silenced that, but that's the software development kit version.  You'd be able to go in and see what version of the SDK you're running.  Space bar."

BN- "7 maps loaded."

"7 maps loaded.  That's how many maps are loaded in my maps folder.  Space bar."

BN- "13 commercial POI files loaded."

"That's how many commercial Points of interest files I have running.  Space bar."

BN- "0 personal poi points."

"I have not created any personal points user points of interest so it says 0."

BN- "Powered by Sendero GPS Copyright 2002-2008 Sendero Group LLC.  Map data copyright by Tele Atlas.  Poi data by Info USA Copyright 2007-2008.  All rights reserved."

"Ok.  That's the copyright information.  So I'm going to press escape to get out of the about GPS menu."

BN- "Exit.  Exiting about menu.  Information center."

"I'm going to press space bar again to go down the information center menu."

BN- "About GPS help I a.  Command summary help I c."

"The command summary, pressing enter on this option will launch the book reader.  I’m going to do that now."

BN- "Launching bookreader.  Top of document.  Braille Note."

"So now we're at the top of the document.  We would use our book reading commands in order to go through the command summary."

"The command summary is modeled after the help menu, so you will find that very familiar.  Let's scroll through the command summary for a minute."

BN- "Description and QT commands- where am I commands, help then w."

"So again, where am I commands you press the help key then w will bring you into the where am I commands.  This is just a text document that you can read."

BN- "Nearest intersection if you are not moving, next intersection if you are moving, x."

"So it told us nearest intersection, next intersection, and the command actually is x.  So if we were in the GPS application, we would just press x.  So this is a great reference tool to let you browse through and read every single command that the GPS application has."

"I'm going to press escape."

BN- "Exit.  Exiting help menu."

"So now we’ve exited the help menu we are back in the GPS application.  I’m going to press the help key or space with h on the BT or PK."

BN- "GPS help menu."

"I'm going to press space."

BN- "Where am I commands, help w."

"I’m going to press enter on where am I commands."

BN- "Where am I commands list."

"I'm going to press the space bar to go down the where am I commands list."

BN- "Nearest intersection when not moving or next intersection ahead when moving x."

"So if you notice, this is exactly what we heard when we were in the commands summary which was just that document.  I'm going to press the space bar."

BN- "Detailed description the nearest intersection when not moving or next intersection ahead when moving read with x."

"I'm going to press the space bar."

BN- "Street address a."

"From within the GPS application you can press a or you can go to the help menu and go to where am I commands find street address and hit enter on it.  Space bar."

BN- "City, state, c."

"So if we press enter on city and state, it will tell us what our current position is.  I'm going to press enter."

BN- "Albany, New York."

"We're actually in Albany New York.  Our virtual position is set to the Capital of New York State which is Albany."

"I'm going to go back into the help menu."

BN- "GPS help menu."

"I'm going to press space."

BN- "Where am I commands help w,."

"I'm going to press space again."

BN- "GPS commands, help g."

"I'm going to press enter to go into GPS commands."

BN- "GPS commands list."

"Press the space bar."

BN- "Speed, s."

"If I press enter here it will give us the speed.  Space again."

BN- "Approximate GPS distance accuracy and number of satellites, g."

"So if you notice they're grouped into logical categories.  I'm going to press escape."

BN- "Exit.  GPS helm menu."

"Now I'm back to the main GPS help menu and I want to press o to go into the GPS options."

BN- "GPS options menu."

"So you could space bar down or you can press o.  It really depends on how you want to get there.  Once you're more comfortable with where things are located in the help menu, then you can just use quick keys in order to move down to the option that you would like to listen to in the   miscellaneous commands."

"Miscellaneous commands you can only get to from the help menu.  I'm going to press space bar to go through the GPS options menu."

BN- "Map and POI folder, compact flash\maps o m."

"Map and POI folder, this is where maps and points of interest are stored.  If you notice it said o m so o would be how you would get there from the GPS application and then m would select the map and POI folder."

BN- "Heading orientation left/right o h."

"Ok.  Let's press enter on heading orientation and see what we have."

BN- "Heading orientation? l left/right c clock, b combo of both."

"So I want to change mine to combo of both so I'm going to press b."

BN- "Heading orientation set to combo."

"Now we have exited the options menu and we set back in the GPS application.  Let's not go through the help menu but let's press o."

BN- "GPS options menu."

"THEN I'm going to space bar down."

BN- "Map and POI folder compact flash\maps o m."

"Space again."

BN- "Heading orientation, combo o h."

"So now instead of saying left right, it says combo.  I'm going to press enter on heading orientation and change it back to left right."

BN- "Heading orientation,? l left/right, c clock, b combo of both."

"I'm going to hit l for left and right."

BN- "EXITING options menu.  Heading orientation set to left/right."

"So now we're back in the GPS application.  Let's press o again to go into the options menu."

BN- "GPS options menu."

"I'm going to press the letter s to go into sounds."

BN- "Sounds submenu."

"I'm going to hit the space bar."

BN- "Turn on or off all sounds, turned on, o ss."

"Pressing enter would turn off all sound notifications.  You wouldn't hear any of the sounds.  Space bar."

BN- "Play nearest intersection sound, o s x."

"Pressing enter would play the sound.  I'm going to do that now."

BN- Plays nearest intersection sound and says- "Sound submenu."

"So that's what you would hear when you're not moving and you ask for your nearest intersection.  Space bar."

BN- "Play background operation sound when the Braille Note is busy, o s b."

"This sound is familiar to you because you've heard it when we started the application and it was loading the maps."

BN- "Plays background sound.  Sounds submenu."

"I just pressed enter.  Space bar."

BN- "Play destination arrival alert sound o s d."

"I'm going to press enter on this one."

BN- plays destination arrival alert sound and says- "Sounds submenu."

"I'M going to press escape to exit the sounds menu.  You can feel free to explore the rest of the sounds."

BN- "Exit.  GPS options menu."

"Press escape again to exit the GPS options menu."

BN- "Exit.  Exiting options menu."

"Now let's go to the help menu."

BN- "GPS help menu."

"I pressed the help key on the QS or the space with h on the BT or PK.  Now I want to press m for multiple repeat."

BN- "Multiple repeat off, select item."

"So if you notice, it says multiple repeat off select an item.  That means nothing will multiply repeat."

BN- "Turn off all multiple repeat announcements, m m."

"If you have multiple repeat announcements going, you would press enter on this or from the main GPS application you can press m once and the letter m again and that will turn off all multiple repeat announcements.  Space bar."

BN- "Multiple repeat delay, every 12 seconds m r."

"So if you don't want your multiple repeat items to trigger every 12 seconds, so every 12 seconds it would announce heading and distance to your intersection if you had that turned on, you could change it to 30 seconds or 45 seconds or a minute.  Every minute it would announce the multiple repeat items."

BN- "Trigger multiple commands now, control with s."

"If you wanted to trigger all of your multiple commands at once, you can press enter on this command or press control with s it would trigger the multiple commands automatically.  Space bar."

BN- "Intersections, turned off m x."

"Now if you notice, it says intersections turned off.  I'm going to press enter to turn intersections on."

BN- "Turning on intersections."

"So we have exited the multiple repeat menu and we are back within the GPS application.  Again we're going to bypass the help menu, we're going to press the letter m and go into the multiple repeat."

BN- "Multiple repeat on, select another item."

"Now if you notice, it says multiple repeat on, select another item.  So we'll space bar down."

BN- "Turn off all multiple repeat announcements, m m."

"Again."

BN- "Multiple repeat delay, every 12 seconds, m r."

"Again."

BN- "Trigger multiple commands now control with s."

"Again."

BN- "Intersections, turned on, m x."

"Ok, snow if you notice, intersections are turned on.  Not only do you have a comprehensive menu, but you have an interactive menu system because it's giving you the status of each item before you change it.  I'm going to press enter to turn intersections off."

BN- "Exiting multiple repeat menu.  Turning off intersections."

"One more menu I want to take you into is the look around menu.  I can press the help key and then space down to look around or I can press help key and press l for look around, or I can press read with m or space with m without going into the help menu.  I'm going to do that now, read with m."

BN- "LookAround commands list."

"I'm going to press space bar."

BN- "LookAround turned on, read with m l."

"Space again."

BN- "POI search direction, ahead, read with m d."

"Let's press enter on POI search direction."

BN- "Search direction? ahead."

"Ok, so search direction ahead, that means it will look for POIS that are ahead of you.  Space bar."

BN- "Right."

"To the right of you."

BN- "Behind."

"POIS behind you."

BN- "Left."

"To the left of you."

BN- "End of list.  left."

"We also have directional search and I'm going to press the backspace key."

BN- "Behind, right, ahead, west, south, east, north."

"If you choose north, south, east or west, it's going to search for points in that direction.  I'm going to go back down to ahead and press enter."

BN- "East, south, west, ahead.  LookAround poi direction search direction set to ahead.  Exiting LookAround menu."

"So now we're back in the GPS application.  I'm going to use the menu system to exit the GPS application.  I'm going to press the help key or space with h."

BN- "GPS help menu."

"Then I'm going to press the space bar until I get to exit program."

BN- "Where am I commands, LookAround commands, multiple repeat, miscellaneous, options, information, exit program escape."

"On the BT or PK, it would say exit program space with e.  So I'm going to press enter here."

BN- "Exit GPS? yes, no."

"Then I'm going to answer yes to exit the GPS application."

BN- "Yes.  Main menu."

07 Virtual Mode and Exploring Routes

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator, Chris Grabowski.

In this section, we're going to discuss virtual mode and creating routes."

"Virtual mode is a mode that you'd go into if you wanted to explore a location without physically being there.  Now it could be 2 or 3 streets away from your home, or hundreds of miles away."

"This gives you the ability to figure out the lay-out of streets or points of interest that may be around the area that you would like to explore."

"You can explore virtually without being in a   route or you can create a route and follow that route virtually so that you can get turn by turn instructions and see what streets the route takes you down or points you pass along the way."

"If you have a route that was created in GPS mode, you can use that route in virtual mode and vice versa.  If you have a route that was created in virtual mode, you can use that route while you're in GPS mode.  So routes are interchangeable between the 2 modes."

"Again, the only difference between GPS and virtual mode is that with GPS you are outside alive, being tracked by the satellites and virtual mode you can be sitting in your house and exploring a position hundreds if not thousands of miles away as long as you have the map."

"So I have the Braille Note in front of me once again and I'm at the main menu."

BN- "Main menu."

"I'M going to press the letter n for navigation."

BN- "SENDERO GPS version 5.0 mPower.  7 maps loaded.  Turn on Holux m-1, 000.  To connect, press c, for GPS mode without connecting, or for virtual mode press v."

"So again our GPS receiver is not on and we are given the options to connect, to go into GPS mode, or to go into virtual mode.  and now I'm going to go into virtual mode."

BN- "Virtual mode.  Albany, City."

"So my virtual position is Albany New York, but I'm going to change that.  I'm going to do an address lookup and that is read with s on the QT or space with s on the BT or PK."

BN- "Address lookup.  Enter state."

"It says enter state so I'm going to type in New for New York and press enter."

BN- "NEW York chosen.  Enter city."

"I'm going to type Niagara and press enter."

BN- "NIAG.  Found 13 possible matching cities.  Cambria area Niagara."

"Now I'm going to press the letter n until I get to Niagara Falls."

BN- "Newfane area, Niagara area, Niagara, Niagara Falls."

"I'll press enter."

BN- "Niagara Falls chosen.  Enter address number."

"I’m going to type in an address number."

BN- "501."

"I'll press enter."

BN- "Enter street name."

"For the street name I'm going to type in Rainbow."

BN- "Rainbow.  Found 4 possible matching streets.  Rainbow Boulevard 269.69 miles left."

"So if you notice, it says Rainbow Boulevard and it's giving us the distance from our current position to that street.  So I'm going to press enter."

BN- "RAINBOW Boulevard chosen.  Address actions menu set or create."

"Now we're brought into the address actions menu. Here is the list of things that you can do to this address.  Now I'm going to hit the space bar."

BN- "Both explore position and destination, b."

"I can set it as both my destination and explore position.  Space bar."

BN- "Create user POI c."

"I can create a user point of interest.  Space bar."

BN- "Destination only, d."

"I can set it as my destination."

BN- "Explore position only, e."

"I can se it as my explore position or virtual position."

BN- "Pedestrian route p."

"I can create a pedestrian route to this position."

BN- "VEHICLE route, v."

"Or I can create a vehicle route to this position."

"I'm going to choose explore virtual position.  I'm going to press the backspace key."

BN- "Pedestrian route, p.  Explore position only, e."

"I'm going to press enter."

BN- "Virtual explore position set to Rainbow Boulevard."

"So our virtual position explore position is now set to the address that I had found.  I'm going to press the letter c to see what city we are in."

BD- "NIAGARA Falls, New York."

"Iamb going to hit a."

BN- "Near 499 Rainbow Boulevard."

"That told us we're near 499 Rainbow Boulevard.  In order to explore virtually, there are some commands that we're going to need to know.  I will give the QT commands first and then the BT commands."

"TO EXPLORE forward or move myself forward adown a street, I would press control with up arrow on the QT and dots 2 3 5 6 on the BT or PK."

"If I want to turn myself to the left, I would press control with left arrow on the QT or dots 2 3 on the BT or PK."

"If I want to turn myself to the right, I press control with right arrow on the QS or dots 5 6 on the BT or PK."

"If I'd like to undo my last virtual move, I would press control with down arrow on the QT or space with dots 2 3 5 6."

"Now I'm going to press control with right arrow because I want to see what happens.  Remember, we just defined this virtual position.  We don't have a heading yet, and I can verify that by pressing h."

BD- "Heading unknown."

"So I'm going to press control with right arrow to try and turn myself."

BN- "Explore forward to move to the nearest intersection."

"So we are told to explore forward to move to the nearest intersection.  I'm going to press control with up arrow on the QT or dots 2 3 5 6 on the BT or PK."

BN- "NOW on Rainbow Boulevard heading west at 5th street, right."

"So now we're at Rainbow Boulevard heading and 5th street is to the right."

"So let's move forward again.  Control with up arrow or 2 3 5 6."

BN- "347 feet west, 4th street left and right.  Falls Side Cafe, Souvenirs Restaurant, left side."

"4 street is left and right; that's the next intersection.  There is also a look around point of interest, and it told us that the point of interest is on the left side of the street.  Because we have a heading now,"

BN- "West 273 degrees."

"I pressed h the GPS program can tell the side of the street that that POI is on.  If I virtually turn myself around so that I'm heading east, that POI will now be on my right side.  I'm going to go forward again."

BN- "394 feet west, 3rd street, left and right."

"So now I'm starting to notice that we are passing the numbered streets, 3rd street, 4th street, and 5th street." Now let's turn to the right so that we can go down 3rd street.  I'm going to press control with right arrow on the QT or dots 5 6 on the BT or PK."

BN- "Now on 3rd street heading north at Rainbow Boulevard left and right."

"Now we did not physically go down that street, we just turned ourselves so we pivoted at the intersection."

BN- "Winter Harden Development, GRP, Tourist Information office."

"When walked down that street.  Again, another look around point of interest had triggered but if you notice, this does not tell us the side of the street.  It could be because the point of interest is not located on our street.  In order to get left right designation from a point of interest, it has to match the heading and it also has to match the current street that you're on.  I can verify that by going into the poi details.  Let's do a space with p on the BT or PK or read with p on the QT."

BN- "DETAILS.  Winter Garden Development Grp Travel/transportation, Tourist information office 56 feet slight right."

"So it's giving us a little bit of information.  What we're interested in is the street that this point of interest is on.  So I'm going to press space bar until I get that information."

BN- "Winter Garden Development Grp Name.  Travel slash transportation, category.  Tourist Information office, subcategory.  716-285-2801 phone.  300 address Rainbow Boulevard street."

"So this point of interest is not located on 3rd street, it's located on Rainbow Boulevard.  I'm going to exit the POI details by pressing escape on the QT or space with e on the BT or PK."

BN- "Details closed."

"Then I can continue to move forward down the street by pressing control with up arrow on the QT or dots 2 3 5 6 on the BT or PK."

BN- "42 feet north, Rainbow Boulevard left and right.  533 feet north, Dougan Drive, right.  Lack David c Pa, Accountants left side."

"So now we could make a right down Dougan Drive if we wanted to.  So let's do that.  Let's do a control with right arrow on the QT or dots 5 6 on the BT or PK."

BN- "NOW on Duggan Drive, heading east at 3rd street left and right.""

"We did not move down the street we just turned at the corner.  So now I'm going to go forward by pressing control with up arrow on the QT or dots 2 3 5 6 on the BT or PK."

BN- "344 feet east, 4th street, left and right."

"Then I'm going to turn left down 4th street.  So I'm going to do control left arrow on the QT and dots 2 3 on the BT or PK."

BN- "Now on 4th street, heading north at Duggan Drive, left and right."

"Then I'm going to go forward."

BN- "701 feet north, Wendel Way, left."

"Then I'm going to go forward again."

BN- "390 feet north Niagara Street, left and right."

"So now we're at Niagara Street.  Now that I've finished exploring around Niagara Falls, I'd like to show you how to create a route and follow that route"

"But there are some keystrokes we need to keep in mind when traveling along a route."

"To move forward waypoint by waypoint, on the QT, you would press read with up arrow, and on the BT, you would press dots 2 3 5 6."

"To move back waypoint by waypoint, on the QT you would press read with down arrow and on the BT or PK, you would press space with dots 2 3 5 6."

"To move forward turn by turn through your route, on the QT you would press read with right arrow and on the BT or PK you would press dots 3 6."

"To move back turn by turn, on the QT you'd press read with left arrow, and on the BT or PK you would press space with dots 3 6."

"Now I'm going to do a point of interest search so I'm going to do a space with f on the BT or read with f on the QT."

BN- "SELECT category all."

"I’m going to hit s until I get to sport."

BN- "School, Shop slash service, sport."

"Press enter."

BN- "Sport chosen.  Select subcategory, all."

"I'M going to press enter on all."

BN- "All chosen.  Enter POI search string or press enter for nearest POI."

"I'm going to press enter."

BN- "BILLIARD'S Empire, Sport, Billiard Parlor, Pool hall, 0.90 miles slight right."

"I'm going to scroll down with down-arrow or space with dot 4."

BN- "Holodome Bowling Lanes Sport, Bowling alley, 1.53 miles right.  Niagara Falls Ice Pavilion, Sport, Skating Rink 1.90 miles right."

"That's the one that I want, so I will pull up the POI Actions menu by doing a backspace with p or control with p."

BN- "Point of interest actions menu set or create."

"I'm going to press v for vehicle.  This will create a vehicle route from my current position to the destination."

BN- "Creating vehicle route.  Route created.  Arrived near beginning of route waypoint 1, 4th, north."

"So now I'm going to travel through my route waypoint by waypoint and to do that I'm going to press read with up arrow on the QT or dots 2 3 5 6 on the BT or PK."

BN- "53 feet, waypoint 2 4th and Niagara.  842 feet, Waypoint 3 4th and Ferry.  Turn right on Us-62 Ferry Avenue heading east."

"Now if you notice, that sound means that this a turning point and it gave us turn instructions.  So again I'm going to move forward along the route."

BN- "169 feet waypoint 4 Ferry.  174 feet waypoint 5 5th and Ferry.  164 feet waypoint 6 Ferry."

"Now I’m going to press the letter t to find out the distance to my next turn."

BN- "1.58 miles, ahead, waypoint 55, 31st and Ferry, turn left on 31st street heading North 3 minutes."

"So it gave us the distance and it gave us the estimated time of arrival.  On a vehicle route, because we're doing it virtually, the miles per hour is assumed to be 25."

"So now I'm going to press read with right arrow or dots 3 6 on the BT or PK to move through the route turn by turn."

BN- "1.58 miles waypoint 55 31st and Ferry.  Turn left on 31st street heading north."

"So if you noticed, we skipped all those point.  Now I'm going to hit t just to see what my distance to the next turn is."

BN- "0.25 miles left, waypoint 57, 31st and Pine.  Turn right on Us-62 bus route, Pine avenue heading east less than one minute."

"Let's just go forward waypoint by waypoint.  Again we're going to do a read with up arrow or dots 2 3 5 6."

BN- "729 feet, waypoint 56 31st and Walnut.  598 feet, waypoint 57 31st and Pine.  Turn right on Us-62 bus route Pine Avenue heading east."

"I'm going to go forward."

BN-"153 feet waypoint 58 Hide and ark and Pine."

"So now I'm going to go forward again turn by turn."

BN- "732 feet waypoint 61, Robins and Pine.  Turn left on Robins drive heading north."

"Let's find out the distance to our destination when I do a d it's going to give to us line of sight.

BN- "278 feet left Niagara Falls Ice Pavilion North, Destination right side."

"I'm going to go forward turn by turn again."

BN- "278 feet arrived near destination waypoint 62, Niagara Falls Ice Pavilion, Destination right side."

"So now that we've made it to our destination.  I'm going to exit the GPS application by pressing escape or space e on the BT or PK."

BN- "Save route before exiting? Yes, no cancel."

"Now it asks us if we want to save this route because the route has not been saved or it has been modified.  I'm going to answer yes."

BN- "Yes.  Save route and exit.  Drive? Press enter for compact flash."

"That's the drive that I want so I'm going to press enter."

BN- "Folder name? Press enter for routes."

"Routes is actually a folder that I created where I would store my routes and that's what I want so I would press enter.  Or I can type in a new folder name or hit the space bar to get a list of folders that I want to place my route into which are standards that KeySoft commands.

BN- "ROUTE to create?"

"I’m going to type a route name.  I called it test to save a route.  I'm going to press enter."

BN- "ROUTE.  File saved.  Main menu."

"Now when you're naming your routes, you'd want to have route names that are more meaningful like home to the bank or something so that you know what's in that route when you open it.

08: Points of interest

note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicate explanations by the narrator, Chris Grabowski.

"In this section I'm going to show you how to work with points of interest."

"Points of interest are items that you may pass along your route that might be interesting like a business."

"Points of interest can be used as clues to help you orient in your area.  So if you're constantly walking the to the same place, and you hear Starbucks then you know you're in the right general area.  If you hear something else, then you might have to reorient yourself depending on the point of interest.  Again, points of interest can be restaurants, they can be tourist attractions, schools, anything that you think might be interesting is a point of interest."

"Now there are 2 types of points of interest.  There are commercial points of interest, which come with the system, and then there are user points of interest."

"Commercial points of interest would be your businesses and all that.  User points of interest would be points that you would put in yourself and that could be anything from the entry to your house to a picnic table in a park that you may need to get back to."

"Points of interest are also different than route waypoints because, one, when you put a point in or you see a point of interest it's global, meaning it's not defined to the route.  So you could see Starbucks from home to work.  You'd also see Starbucks if you had another route loaded from home to the bank or you could see points of interest when you have no route loaded and you are just walking along the map."

"I'm at the main menu"

BN- "Main menu."

"I'm going to hit n for navigation."

BN- "Sendero GPS version 5.0 mPower.  7 maps loaded.  Turn on Holux m-1,000.  To connect, press c, for GPS mode without connecting press g or for virtual mode press v."

"I'm going to press v to go into virtual mode."

BN- "In virtual mode."

"The first thing I want to do is change my map.  So I will do o for the GPS options menu."

BN- "GPS options menu."

"Then I'm going to press the space bar until I get to map and POI folder."

BN- "MAP and POI folder, compact flash\maps m m."

"Press enter."

BN- "MAP and POI folder currently compact\maps.  Choose maps and POI folder? Press enter for flash disk."

"I'm going to hit c v for compact flash."

BN- "Compact flash.  Directory Name? Press enter for /."

"I'm going to hit the space bar until I get to maps Northeast."

BN- "Subdirectories of compact flash."

"Again."

BN- "Maps."

"Space again."

BN- "Maps Northeast."

"I'll hit enter."

BN- "Directory name? Press enter for /maps Northeast."

"I'm going to press enter to confirm that that's the folder that I want to change to."

BN- "Exiting options menu.  13 maps loaded.  20 commercial poi files loaded.  2 personal user poi points.  Map and Poi folder set to compact flash\maps Northeast."

"The reason I switched my map is because there's a certain city that I want to work with.  It's point of interest.  So I'm going to do an address lookup.  I'm going to do a space with s on a BT and PK or read with s on the QT."

BN- "Address lookup Enter state."

I'm going to type Mas."

BN- "Mas.  Massachusetts chosen..  Enter city."

"I'M going to type Boston and press enter."

BN- "Boston.  Boston chosen.  Enter address number."

"I'm going to type 500 and press enter."

BN- "500.  Enter street name."

"I'm going to type Atlantic and press enter."

BN- "Atlantic.  Found 2 possible matching streets.  Atlantic Avenue 409.13 miles, right."

"I'll press enter."

BN- "Atlantic Avenue chosen.  Address actions menu set or create?"

"So now we're back to the address actions menu and I'm going to press the space bar until I get to explore virtual position or I can press the letter e."

BN- "Explore position only, e.  Virtual explore position to Atlantic Avenue."

"So now my virtual position is in Boston and I c..."

BN- "Metropolitan group, Bonus."

"That was a look around event that triggered and it said it was from Bonus.  It's giving us the name of the point of interest and the category that it is in.  So I'm going to press the letter c."

BN- "Boston Massachusetts."

"If I want to pull up a list of points of interest near my virtual or GPS location (this will work either way) I would press f as in find and I will do that now."

BN- "Metropolitan group bonus, 15 feet southwest."

"So it's giving us the name of the point of interest, the category, the subcategory if there is one, the distance from the position to the point of interest and the direction."

"So now to scroll down this list, I'll press the down arrow on the QT or space with dot 4 on the BT or PK."

BN- "Residences at the Inn.  Hotel/accommodation, condominium 15 feet southwest."

"Again."

BN- "BOSTON Atlantic parking LLC Automobile, car parking, 15 feet southwest."

"Again."

BN- "Presentation services Bonus, 47 feet southwest."

"Again."

BN- "Miel, restaurant, French, 47 feet southwest."

"So it's giving us this and the restaurant is the category and French is the subcategory."

BN- "Intercontinental Boston, Hotel/accommodation, hotel, motel, 47 feet southwest."

"The Inter-continental Boston hotel.  Now let's say we want more information on this point of interest.  We can get it's name, it's address and it's phone number and other useful information.  To do that we would press space with p on the BT or PK or read with p on the QT."

BN- "Details.  Inter-continental Boston, Hotel/accommodation hotel motel 47 feet southwest."

"So basically it just told us name of the point of interest and gave us  a little bit of details.  If we wanted more information without stepping through the details, we can use the summary command and that is space with s on the BT or PK or read with s on the QT and I'll do that.

BN- "Inter-continental Boston, Hotel/accommodation, hotel motel, 617-747-1000.  510 Atlantic Avenue, Boston, Ma USA 02210 commercial."

"So it gave us a lot of information and we maybe didn't get that or we wanted to repeat it or it was just too much information at one time for us to digest.  We can press the space bar to go down through the point of interest details.  I'm going to do that."

BN- "INTER-CONTINENTAL Boston.  Name."

"It says Inter-continental and it says name."

BN- "Hotel/accommodation, category."

"Hotel accommodation is the category."

BN- "Hotel motel, subcategory."

"The sub-category is hotel motel."

BN- "617-747-1000, phone."

"There's the phone number."

BN- "5010 address."

"There's the address number."

BN- "ATLANTIC Avenue Street."

"There's the street."

BN- "Boston, city."

"This is the city."

BN- "Ma, state/province."

"Massachusetts."

BN- "USA, country."

"There's the country."

BN- "02210, zip/postal code."

"There's the zip postal code."

BN- "Commercial POI."

"It's a commercial POI.  That means it's not a POI that you created.  It's one that came with the system.  Space again."

BN- "Public, POI."

"This means it's a public point of interest again all commercial POIS are public.  POIS that you create yourself are considered private unless you move them into a different category which then they become public." BN- "TELEATLAS, author."

"There's the author.  Remember in the beginning when we ran GPS for the first time it asked us for the POI author name.  If this was a point of interest that you created, your initials or your name would be in this area.  Space bar."

BN- "LABELS, on."

"Labels are on.  Labels are the things that you hear after the data.  So where it said 510 and then it said address, you could turn the address portion off.  So it would just say 510.  Let's press enter and turn labels off."

BN- "Labels, off."

"Then I'm going to hit the backspace key."

BN- "Tele-atlas."

"See? It says, Tele-atlas, no author, because that field name is turned off."

BN- "Public."

"Public , commercial.  I'm going to go to the labels and turn them back on.  I can press the letter q to do that quickly."

BN- "LABELS, on."

"Now the labels are actually on."

BN- "Public, POI."

"So now it says public POI.  Backspace."

BN- "Commercial, POI."

"Again."

BN- "02210 Zip/postal code."

"Basically, any of these fields you can jump !was using a character.  Now it's not always the first character.  You can press p and it will just  jump to phone."

BN- "617-747-1000, phone."

"You can hit s and it will jump you to,"

BN- "ATLANTIC Avenue, street."

"The street, but if I s again,"

BN- "Atlantic Avenue, street."

"So you only have one character assigned to each field.  Press space with h or the help key and then you can scroll up and down the POI details help which will jump give you a list of each one of the hot keys.  These hotkeys are also listed in the points of interest section of the manual.  I'm going to press escape to exit the POI details."

BN- "Details closed."

"So now we found our hotel and we've explored the address of the hotel and the phone number, and now we want to search for tourist attractions or things to do around our hotel.  I could press f and then scroll then scroll I find a tourist attraction that I might be interested in but that's just a little too cumbersome to do.  So I'm going to use what we call a simple find and that is a space with f on the BT or PK or a read with f on the QT."

BN- "Select category.  All."

"Select category all.  Now we're presented with a list of categories.  I can hit the space bar."

BN- "Alcohol establishments, automobiles."

"I'm going to now press the letter t because I want to move to travel transportation so you could do it either way.  You could space bar down or you can use the first character of the category you're interested in."

BN- "Travel/transportation."

"I'm going to press enter."

BN- "Travel/transportation chosen.  Select subcategory all."

"So now we have all as a sub-category and I'm going to hit space bar and see what we have here."

BN- "Airline access, airport.  Border crossing.  Bus terminal.  Ferry terminal.  Monument or historic site.  Railway station.  Rest area.  Toll plaza.  Tourist attractions."

"I'm going to press enter."

BN- "Tourist attractions chosen.  Enter POI search string or press enter for nearest points."

"But I don't know what exactly I'm looking for.  I know it might be a tourist attraction so I'm going to just press enter.  If I knew what I was looking for I might be able to type a name here."

BN- "Benjamin Franklin Statue travel/transportation tourist attractions 0.43 miles northwest."

"Now what I'm going to do is scroll down the list of POI's and it's only going to give me points in the tourist attraction subcategory.  So to scroll down the list I'm going to hit the down arrow or space with dot 4."

BN- "FAYNWILL Hall Marketplace, travel/transportation, tourist attractions 0.44 miles northwest.  Grannery Burial Ground travel/transportation tourist attractions 0.51 miles northwest."

"So I can do a number of things here.  I can set it as my destination by pressing space with d read with d, I can set it as my virtual position by pressing space with v or read with v and there are a few other things I can do with it.  Or if I don't remember all of those commands, I can bring up the POI actions menu.  I press backspace with p on the BT or PK or control with p on the QT."

BN- "Poi actions menu set or create,"

"So now I'm going to hit the space bar to see what we have."

BN- "Both explore position and destination b."

"I can hit b and it will activate that option.  So what I want to do is set this as my destination and create a vehicle route to it.  So I'm going to select vehicle and I'm going to hit v for vehicle."

BN- "Creating vehicle route.  Route created.  Arrived near beginning of rouse waypoint 1 Pearl, Northwest."

"So now I'm going to go through our route turn by turn and end up at our destination.  So to go turn by turn I'd press dots 3-6 on the BT or PK or I would press read with right arrow on the QT."

BN- "912 feet, waypoint 4 Franklin and Pearl.  Turn left on Franklin Street heading southwest."

"Again."

BN- "0.23 miles waypoint 10 Bromfield and Washington and Franklin.  Turn ahead on Bromfield Street heading Northwest."

"Again."

BN- "737 feet, arrived near destination waypoint 13, arrived near  Franklin Burial ground."

"So now we've made it to our destination and we decided that it's that to look for a place to eat.  So we're going to do another simple find space with f on the BT or PK or read with f on the QT."

BN- "Select a category.  Travel/transportation."

"It remembers the last category that we had accessed.  So I'm going to hit r for restaurant."

BN- "Restaurant.  Restaurant chosen.  Select subcategory all."

"I'm going to press enter for all."

BN- "All, chosen.  Enter POI search string or press enter for nearest points."

"I'm going to press enter for the nearest POIS.

BN- "Beantown Pub, Restaurant, 163 feet behind."

"So I can check that out or I can scroll down the list of POIS.  Again, space with dot 4 or down arrow."

BN- "Silvertone Bar Grill restaurant, American, 276 feet behind right side."

BN- "Mo Prime Restaurant, Steak House 278 feet behind."

"So if you notice it's giving us orientation and the first one it said right side and that's because that point of interest is along our street matching our heading.  Let's edit this restaurant here.  So let's go into the POI details, read with p or space with p."

BN- "Details.  Co Prime Restaurant Steak house, 278 feet behind."

"Now I'm going to space bar down  through the details."

BN- "CO Prime, name.  Restaurant, category.  Steak House subcategory.  617-772-0202, phone."

"Now I heard from the locals that they changed their number, so I need to update their phone number.  I'm going to press enter."

BN- "Phone number? Press enter for 617-772-0202."

"I'm going to type in the phone number."

BN- "617-555-1212."

"I'm making up the phone number.  I'm just demonstrating the point here."

BN- "POI field updated.  90, address."

"There's the address.  Now let's press space with e on the BT or PK or escape on the QT."

BN- "Save POI changes? Yes No Cancel."

"I'm going to answer yes to save POI details."

BN- "Yes.  Save POI and exit.  Poi changes saved.  Details closed."

"Now let's go into the POI details and see if the phone number has indeed changed."

BN- "Details.  Co Prime restaurant, Steak House 278 feet behind."

"Now let's hit the letter p for the phone number."

BN- "617-555-1212, phone."

"Now the only way I can get that data back is if I reinstall the Massachusetts points of interest.  I'm going to press escape."

BN- "Details closed."

"So the next find I want to talk about is the advanced find and this gives you a lot of customization on how you want that search to take place.  So to do an advanced find on the BT or PK we do backspace with f and on the QT we do control f."

BN- "SELECT category.  Travel/transportation."

"I'm going to press enter because I’d want to do another travel transportation search."

BN- "Travel/transportation chosen.  Select subcategory, tourist attractions."

"I'm going to press enter again on tourist attractions."

BN- "Tourist attractions chosen.  Enter starting distance for the POI search."

"So this is prompting us for a starting distance we could type in anywhere from 1 mile to whatever however far out we think the POI might be or we can just press enter and your speed of your search is going to depend on how populated the category is.  So if you did a search that I'm about to do using the category bonus it's definitely going to take longer then demonstrated here because I'm using attractions which is a sub category that has very few points of interest in it compared to a category like shop service or bonus.  So I'm not going to select a starting distance but if I did, we would be prompted for an ending distance."

BN- "Search direction? full circle."

"Search direction is full circle and that means around you.  You're not specifying a direction to search, you're interested in points of interest that are surrounding you.  You can choose from other options and I'll press the space bar."

BN- "North east, south, west, ahead, right, behind, left."

"So if you're going in a specific direction and you want to search for restaurants ahead of you, you can do that here.  If you want to search for them behind you, because you want to turn around, you can do that as well.  I happen to know that the POI that I'm interested in is west of my current location.  So I can press the letter w."

BN- "West."

"And Press enter."

BN- "West.  Chosen.  Search field? Name."

"Now we're presented with search field and we can search on any of those fields that are in the POI details."

BN- "Name, phone number,."

"This is a really cool way to get all points of interest on a specific street in your area.  So if you wanted to find what restaurants were along Main Street, you would select street, and type in Main.  So you can really search on any of these fields that we've seen in the POI details.  I'm going to press n for name."

BN- "Name."

"And press enter."

BN- "Name, chosen.  Enter POI search string or press enter for nearest points."

"Now this time I do know the name so I'm going to type in enough of the name and press enter."

BN- "Liberty.  Statue of Liberty, Travel/transportation tourist attractions, 192.44 miles slight left."

"It found it.  Now let's go and delete a point of interest that does not exist.  Using the address lookup I've virtually placed myself on the street of Kenmore in the city of Buffalo, New York.  I'm currently heading east.  Now I'll press h to verify that."

BN- "East 95 degrees."

"So now I'm going to do an advanced point of interest search and search for all restaurants along Kenmore Avenue.  So I'm going to press control with f or backspace with f."

BN- "Select category, all."

"I'm going to hit r for restaurant and press enter."

BN- "Restaurant, chosen.  Select subcategory.  All."

"Press enter on all."

BN- "All, chosen.  Enter starting distance for the POI search."

"Press enter."

BN- "Search direction? full circle."

"I happen to know the restaurant I'm looking for is east of my current position so I'm going to hit e for east."

BN- "East.  East chosen.  Search field? Name."

"I'm going to hit s for street."

BN- "Street.  Street chosen.  Enter POI search string or press enter for nearest points."

"I'm going to type in Kenmore and press enter."

BN- "Vince's Pizza plus, restaurant, Pizzeria 0.21 miles ahead left side."

"Now if you notice, it said it was ahead of us and it's on the left side of the street.  I'm going to scroll down."

BN- "Franco's Pizza, restaurant pizzeria, 1.43 miles ahead left side."

"Again, that one's on the left side of the street."

BN- "Torches, restaurant, 1.46 miles ahead left side."

"Scroll down."

BN- "Hourglass restaurant, restaurant 1.76 miles ahead left side."

"That's the restaurant that I wish to delete, so I'm going to go into the POI details by pressing space with p or read with p."

BN- "POI details.  Hourglass Restaurant, restaurant 1.76 miles ahead left side."

"Then I'm going to press backspace with dots 2 3 5 6.  or control with 8 on the QT."

BN- "Delete this commercial POI?"

"I'm going to answer yes."

BN- "Yes.  POI erased.  Details closed."

"The details are closed.  Now let's verify that that point of interest is indeed gone.  I'm going rather quickly do another advanced POI find."

BN- "Select category, restaurant."

"Press enter."

BN- "Restaurant, chosen.  Select subcategory, all."

"Press enter for all."

BN- "All chosen.  Enter starting distance for the POI search."

"Enter again."

BN- "Search direction? east."

"Enter."

BN- "East, chosen.  Search field? street."

"The street is what I want, so I'll hit enter."

BN- "Street, chosen.  Enter POI search string or press enter for nearest points."

"I'm going to type in Kenmore and press enter."

BN- "Kenmore.  Vince's Pizza plus, Restaurant Pizzeria 0.21 miles ahead, left side."

"Now let's scroll down."

BN- "Franco's Pizza Restaurant, Pizzeria, 1.43 miles ahead left side.  Torches, Restaurant 1.46 miles ahead left side.  Sinatra's Restaurant, Restaurant 1.83 miles ahead right side.  Curry's Pub Restaurant, Restaurant, 1.95 miles ahead right side."

"So as you notice the Hourglass Restaurant has been deleted from the POI database.  The final point of interest topic I want to talk about is are user points of interest and you create user points of interest at places that you want to be notified of.  They're your specific landmarks.  There might be a bench or it might be a picnic table or a tree that you're tired of walking into or brushing up against.  Anything that you want to put in as a user point of interest you most certainly can.  To create a user POI you would press space with dot 3 on a BT or PK or read with r on the QT.  I'm going to do that."

BN- "Point of interest name?"

"Just for the sake for it we're going to call it a tree.  This is creating it at our virtual position because we're not outside tracking.  I'm going to press enter."

BN- "Tree select category.  User private."

"So now we can select any category that we want, but I'm going to call mine user private and press enter."

BN- "User POI saved as tree.  Tree, user."

"User POI saved as tree.  Then it said tree, user.  the it told it was saved and it told us the name and the category.  Now if I pull up its details with the read with p or space with p, we can see what we have here."

BN- "Tree, user, near."

"I'm going to space bar down."

BN- "Tree, name.  User, category.  Military road, street."

"So what it did is it pulled the nearest street that we were on and it automatically filled that in for us."

BN- "Tanawanda city.  New York, state/province. USA, country.  40217, zip/postal code."

"So it automatically fill the all of that information in for us depending on our GPS position or our virtual position.  Again, it might fill in the wrong street.  So say for example you are adding your house in as a point of interest and it just so happens that you're on a corner, but it grabs the other street.  You could come in edit street and put your own street back in there.  It's just pulling the data from your current position whether it's GPS or virtual mode."

"There's one more search that I  want to briefly touch on and that is the user configurable search.  By default, the user configurable search is set for user as the category and private as the subcategory.  So if I press a hot key, it will do a search out for all user private points and the hot key is backspace with dot 3 on that BT and PK or control with u on the QT.  You can think on the QT control with u for user definable."

"First I'm going to press escape to close the POI details and then I'm going to press control with u on the QT or backspace with dot 3 on the BT or PK."

BN- "Details closed."

"Now I'm going to do control u or backspace with dot 3."

BN- "Tree, user, near."

"So it told us we were near our fictitious tree that we put in there.  I'm going to scroll down the list."

BN- "Outback Steakhouse front door, restaurant, 4.28 miles northeast."

"So if you don't want the user definable hot key to pull up user points of interest, maybe you want restaurants.  Maybe you want shops and services, you can go into the options menu and I'll do that now."

BN- "GPS options menu."

"I pressed o.  Then I'm going to hit a for advance."

BN- "Advanced GPS options menu."

"Then I'm going to space bar until I get to the user definable hotkey."

BN- "GPS, street names, arrival distances, user definable POI search, user, private, o a u."

"Press enter."

BN- "Select category.  User."

"I'm going to hit backspace"

BN- "Travel/transportation."

"Again."

BN- "Talking lights."

"Again."

BN- "Sport, shop service, school,."

"So let's press enter on school."

BN- "School, chosen.  Select subcategory, all."

"I could select subcategory, but I'm going to hit enter on all."

BN- "All, chosen.  User definable search category and subcategory has been changed to school, all.  Exiting options menu."

"Now when I press control u it will not look for user points of interest, it's going to look for schools.  So let's do that."

BN- "LITTLE weeping Willow, School, nursery, 460 feet east."

"So now control u or backspace with dot 3 is defined to look in the category school every time I press it.  So I'm going to press escape on the QT or space with e on the BT or PK to exit the GPS application."

BN- "Exit GPS? yes, no."

"I'm going to enter yes."

BN- "Yes.  Main menu."

09 Vehicle Travel

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicate explanations by the narrator, Chris Grabowski.

"We're in a car and we're going to plot a route to IHOP and I'm going to do a POI search, a space with f or read with f."

BN- "Select category all."

"I'm going to select restaurant."

BN- "Restaurant chosen.  Select subcategory, all."

"Press enter on all."

BN- "All.  Chosen.  Enter POI search string or press enter for nearest points."

"Type IHOP."

BN- "IHOP.  IHOP restaurant American, 2.69 miles slight right."

"Now I'm going to bring up the POI actions menu with backspace with p or control p.  Then I'm going to press v for vehicle route."

BN- "POI actions menu, set or create.  Creating vehicle route.  Route created.  Meadow Lane.  Destination set to waypoint 38 IHOP Restaurant.  Next, waypoint 10, Calvin and..."

BN- "Approaching right turn at waypoint 10 Kolvin and Sharidon 0.22 miles."

"Now we are approaching a right turn at Kolvin and Sharidon in 0.2 miles and we'll; making a right."

BN- "Turn right on Ny-324, Sharidon Drive heading east."

"It told us to turn right.  I apologize for talking over the speech, (underlying speech not transcribed) but it's announcing streets as we are passing them."

BN- "Continue straight.  1.76 miles to waypoint 25 Niagara Falls and Sharidon.  Turn left on Ny-324 Niagara Falls Blvd heading north.  On Sharidon Drive heading east."

"Our next turn is Niagara Blvd in 1.76 miles and it's announcing points of interest as we're passing them on the right side or left side of the street.  I'm going to hit t for my next turn."

BN- "1.52 miles, ahead, waypoint 25 Niagara Falls and Sharidon, Turn left on Ny-324 Niagara Falls Blvd heading north.  3 minutes."

"Now we're continuing on, and there's a street that just announced Logan Avenue."

BN- "Sharidon Animal Hospital, Veterinarian left side."

"It's giving us progress updates as we're going down by telling us the streets that we are coming to as well as points of interest.  This gives you an idea of what streets you're passing along so you get to learn them.  I'm going to hit t for our next turn."

BN- "0.93 miles, ahead, waypoint 25, Niagara Falls Blvd and Sharidon turn left on Ny-324 Niagara Falls  heading north 1 minute.  Willmington Avenue.  Parker Blvd."

"More streets.  We should get our approaching turn sound shortly."

BN- "APPROACHING left turn at waypoint 25 Niagara Falls and Sharidon in 0.31 miles.  Turn left on Ny-324 Niagara Falls Blvd heading west."

"NOW we need to turn left on Niagara Falls Blvd.  I'm going to hit x for my next intersection."

BN- "93 feet ahead to Niagara Falls Blvd left and right."

"Now we're making our left and we should get updated turn instructions."

BN- "Continue straight.  0.66 miles to waypoint 33 Maple and Niagara Falls.  Turn right on Co Highway 192 Maple Road heading east.  Factory Sleep Shop Inn, furniture,/home furnishings left side."

"Next turn is at Maple."

BN- "APPROACHING right turn at waypoint 33 Maple and Niagara Falls in 0.27 miles."

"Our next turn is coming up, Maple and Niagara Falls Blvd, and we'll make a right."

BN- "Turn right on Co Highway 192 Maple Road heading east.  Maple Road."

"We're going to turn right.  This should be the last leg of the journey."

BN- "Continue straight.  0.37 miles to destination waypoint 38 IHOP Restaurant on Maple Road heading east."

"It's announcing streets and POIS as we go along right and left sides.  If it doesn't announce left or right it's possible it's a point that's on one of the cross streets."

BN- "North Bailey Avenue.  Scotch and Sirloin Restaurant Steak House right side."

"Let's find out the distance to the destination."

BN- "288 feet ahead IHOP Restaurant east.  Destination right side.  Arrived near destination 138 feet ahead to destination waypoint 38 IHOP Restaurant route closed."

"We have arrived at our destination."

10 Walk Outside

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicate explanations by the narrator, Chris Grabowski.

"In this section, we're going to use the GPS outside so that you can see how it works and the type of feedback that you get from the system.  In preparation I went to Rite Aid to create a user point of interest at its main entry.  The reason that I did that is because user points that you create myself are more accurate than commercial points because the commercial points are placed at the street and the building may; a hundred feet away or 200 feet away from the commercial point of interest.  So it's always a good idea to create a user point if you're going to go back there multiple times.  I'm going to go into the GPS now."

BN- "Sendero GPS version 5.0 Pk.  12 maps loaded Holux m-1,000 detected.  Acquired position 16 feet 10 satellites."

"Sour position is acquired and we were just told the street we were on and the current cross street which is east of us.  I'm going to do a space with f or read with f to da POI search."

BN- "Select category.  All."

"Then I'm going to hit u for user"

BN- "User."

"Press enter."

BN- "User chosen.  Select subcategory, all."

"I'm going to press enter for all."

BN- "All, chosen.  Enter search string or press enter for nearest points."

"I'm going to type in enough of the point interest name so that it will find it and press enter."

BN- "Rite Aid Front Door bonus, 031 miles southwest."

"Now I'm going to use the POI actions menu backspace with p or control with p."

BN- "POI actions menu set or create."

"Then I'm going to press p to create a pedestrian route to Rite Aid"

BN- "Creating pedestrian route.  Route created.  Approaching left turn at waypoint 2 Hyler and Wilton Parkway in 137 feet."

"So it wants us to go to Wilton Parkway so we will do that."

BN- "Turn left on Hyler Avenue heading south."

"So it wants us to turn left onto Hyler and it gives us the direction, south.  After we make that turn, we'll get updated turn instructions."

BN- "On Wilton Parkway heading west.  On Wilton Parkway, 26 feet slight left to Hyler Avenue."

"Now I'm going to make my left because I'm at the corner.  Once our heading updates,"

BN- "On Hyler Avenue heading west.  Continue straight.  268 feet to waypoint 3.  Approaching right turn at waypoint Hyler and Parkwood in 247 feet."

"So now we were told the distance to our next turn that we're approaching our turn because they are 2 kind of quick turns.  Once we approach the turn we'll get another route instruction to tell us to make a right onto Parkwood Avenue."

BN- "Turn right on Parkwood Avenue heading west.  On Hyler Avenue 122 feet ahead to Parkwood Ave."

"So it gave us our turn instruction and then it told us the distance to our next intersection which was Parkwood, which was 100 and some odd feet away and that's measured to the center of the intersection.  I'm going to make my right and cross Hyler along Parkwood."

BN- "On Parkwood Ave heading southwest."

"We'll get updated turn instructions shortly."

BN- "Continue straight.  0.21 miles to waypoint 6, Parkwood and Kolvin.  Turn left onto Co Highway 118 Kolvin Ave heading South..  On Parkwood Ave 197 feet ahead to Marquette Ave."

"So it gave us our turn instruction, which is Kolvin Ave in 0.2 miles I believe it said and it gave us our next intersection which is Marquette.  As I'm walking along I can press  a for the nearest address."

BN- "Near 168 Parkwood Ave."

"I can press x for the next intersection."

BN- "Marquette Ave left and right."

"It gave us the orientation of the intersection left and right and that tells me that if I'm on the right side of the street I need to be concerned with that intersection.  If it said it was coming at me at the left, then I know if I'm on the right side of the street I wouldn't be concerned with  that street because I wouldn't be physically crossing it.  Although, it's definitely good to know that the street is left and right."

BN- "On Parkwood Ave 198 feet connect to Charleston Ave."

"So Parkwood and Charleston is the next intersection and it made a  little sound to let us know that that's an approaching intersection.  I can press s to find out how fast I'm going."

BN- "2.4 mph."

"I can press t to get the distance to my next turn."

Bn- "717 feet ahead, waypoint 6 Parkwood and Kolvin.  Turn Left onto  Co Highway 118 Kolvin Ave heading south, 8 minutes."

"Now I'm going to cross Charleston.  Now that I've crossed Charleston, I want to get my next intersection, so I'll press x."

BN- "620 feet ahead to Kolvin Ave left and right."

"Ok.  That tells us that our next intersection is our next turn, so we need to be concerned with it  and our approaching turn.  Turn now announcements will occur once we get closer to the intersection.  I'll press x again."

BN- "424 feet ahead to Kolvin Ave left and right."

"I can start to hear the cars along Kolvin."

BN- "Approaching left turn at waypoint 6 Parkwood and Kolvin in 247 feet."

"So there's our approaching turn.  This means we should be on the lookout for our turn."

BN- "Turn left on Co Highway 118 Kolvin Ave heading south.  On Parkwood Ave 124 feet ahead to Kolvin Ave."

"So now we got our turn now message which means be on the lookout for our intersection and then it told us the distance to the intersection.  That was a look around intersection that triggered.  As I can hear, we're getting closer to Kolvin.  I'm going to make my left and continue on."

BN- "Plant Art Inn, antique slash art."

"That was a point of interest that triggered to let us know something of interest."

BN- "On Kolvin Ave heading southwest.  On Kolvin Ave 127 feet slight left to east Hazeltine Ave.  366 feet to waypoint 8 Kolvin.  Turn right onto connecting to Kolvin Ave heading west."

"So we pretty much got our next route instruction."

BN- "Approaching right turn at waypoint 8 Kolvin in 249 feet."

"Once we reach our destination we're going to use the "getting warmer" method to cross the parking lot to actually find the main entry way to Rite Aid."

BN- "Turn right onto Connecting to Kolvin Ave heading west.  On Kolvin Ave 198 feet slight right to Kenmore Ave.  Kenmore Apts.  Apartments, left side."

"It just called out a point of interest that was on our left side.  If I were turn around and my heading changes, that POI would then be on my right side.  Kenmore Ave is the next street that we're going to cross.  Press x."

BN- "Kenmore Ave left and right."

"There it is! When you're using the "getting warmer" method you want to be concerned with the distance decreasing.  The heading you don't really want to be concerned with.  So when it says 75 feet, then 60 feet, you know you're getting closer to that destination.

BN- "Continue straight.  79 feet to destination waypoint 9 Rite Aid Front Door.  On Kenmore Ave.  Arrived near destination 61 feet slight right to destination waypoint 9 Rite Aid front door.  Route closed."

"So now we've reached our destination and our route is closed.  We're still not at the main entry way yet we still have to cross this parking lot.  so I'm going to press the letter d until I get there.  I'm going to continue on my way."

BN- "43 slight right Rite Aid front door north."

"Now I go forward."

BN- "38 feet slight right Rite Aid front door north.  25 feet slight  left Rite Aid front door southeast.  Near Rite Aid front door."

"Here we are.  We made it."

11.  GPS Replay

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicate explanations by the narrator, Chris Grabowski.

"In this section we're going to discuss the GPS replay.  The GPS replay is just as its name suggests.  It is a replay of a trip that you took.  You can have the GPS replay record your movements so that they can be played back at a later time.  Think of the following situation.  You go out with an orientation and mobility specialist and take your GPS with you.  In the background you can set the GPS so that it records your movements, keeping your mind free to pay attention to your lesson.  When you return home, you can play back the replay and go through the lesson just as you did when you were out tracking live.  The GPS replay records the data sent to the GPS receiver.  No keystrokes are recorded.  Your heading, your position, latitude and longitude are all recorded to the file.  In the "walk outside” section, we took a trip to Rite Aid and I used the getting warmer method in order to find the doorway.  I recorded that trip as a GPS replay and we can now view it and see what it does.  I'm at the main menu."

BN- "Main menu."

"I'm going to press N for navigation."

BN- "Sendero GPS version 5.0 mPower.  7 maps loaded.  Turn on Holux m-1, 000.  To connect, press c.  For GPSMODE without connecting, press g, or for virtual mode, press v."

"So I'm going to press the letter g to go into GPS mode."

BN- "In GPS mode."

"In order to use the replay, I need to select it as my GPS receiver.  I'm going to press the letter o for options."

BN- "GPS options menu."

"I’m going to press the letter g for GPS."

BN- "GPS receiver currently set to Holux m-1, 000 paired and active.  Receiver selection menu."

"Then I'm going to press the letter R for GPS replay.  I could space bar up and down the menu."

BN- "Drive? Press enter for compact flash."

"Press enter for compact flash."

BN- "Folder name? Press enter replay files."

"Now here is where I keep my GPS replay files.  Press enter."

BN- "GPS replay to open? Press enter for "replay for tutorial.trc."

"I'm going to press enter on "replay for tutorial" which is the walk to Rite Aid."

BN- "Successfully connected to receiver.  Exiting options menu.  GPS replay detected.  Acquired position 21 feet 9 satellites.  On Wilton Parkway 96 feet east to South Irving Tur.  Pause GPS replay."

"I paused the GPS replay and I did that by pressing control q on the QT or backspace with q on the BT or PK.  The first thing that we heard was GPS replay detected and then we heard the number of satellites that we had when we recorded the replay.  The last item is important and that tells us that we're working with a GPS replay we are not working with live data.  So now let's resume the data and see what happens."

BN- "Resuming GPS replay."

"I'M going to press h for heading."

BN- "Heading unknown."

"So our heading is unknown."

BN- "In Kenmore."

"That was a look around city change."

BN- "On Wilton Parkway 47 feet slight right to Hyler Ave.  On Wilton Parkway heading west.  In New York."

`It just gave us our cross street and it told us we were heading west on Wilton Parkway.  So let's hit h for heading again."

BN- "South 199 feet degrees.  On Hyler Ave heading south."

"So we've already changed directions.  We're already heading down Hyler."

BN- "Hyler Ave 195 feet ahead to Parkwood Ave."

"Press s for speed."

BN- "2.3 mph."

"That's how fast I was walking at the time.  I'll press y to get our latitude and longitude."

BN- "Latitude 42 degrees 57 minutes 37.3560 seconds north.  Longitude 78 degrees, 51 minutes 13.8360 seconds west."

"So this is happening in real time.  This is just as if I was walking along with the GPS receiver connected.  Let's hit the letter x for intersection."

BN- "Parkwood Ave left and right."

"Let's hit a for nearest address."

BN- "Near 52 Hyler Ave.."

"Let's check our direction with h."

BN- "Southwest 204 degrees.  On Parkwood Ave heading southwest."

"There we go.  We're on Parkwood Ave heading southwest.  It made the right turn onto Parkwood."

BN- "On Parkwood Ave 196 feet ahead to Marquette Ave."

"So using this GPS replay I'm able to study my trip.  Ask for speed."

BN- "3.3 mph."

"A for nearest address."

BN- "Near 368 Parkwood Ave."

"Nearest intersection?"

BN- "244 feet ahead to Charleston Ave left and right.  Annotation.

"I just paused the GPS fire hydrant.  Pause GPS replay and I want to explain what an annotation is.  An annotation is a note that I put in the replay as I was recording it.  So you could put things in that you need to be reminded of as you're viewing your replay and I wanted to be reminded of the fire hydrant.  So when I was recording the replay, to add the annotation I would press space with w on the BT and PK or read with w on the QT.  That would pause the replay so I'd type in the text of the annotation and press enter.  The replay starts recording again.  Each annotation is announced at the exact instant that it was recorded in that replay.  If we were following a route, we would get turn-by-turn instructions.  We can take the replay and convert it into a route.  The advantage to converting a replay into a route is that you don't need to have maps under you so it's not tied to streets.  You could be recording a replay as you're hiking along a trail and then convert that replay into a route and then follow that route the next time and you'll get turn by turn instructions for when you made the turns.  Each annotation that you put in the replay then becomes a waypoint that you'll be guided to.  So let's do that now. Let's restart the replay and I'm going to press control with r or backspace with r on the BT or PK."

BN-"Replay restarting."

"So this brings me back to the very beginning of the replay.  Then I'm going to press r for route commands"

BN- "Route commands list."

"Then I'm going to press f to create a route from a replay."

BN- "Drive? Press enter for compact flash."

"Press enter."

BN- "FOLDER name? Press enter for replay files."

"Press enter again."

BN- "GPS replay to open? Press enter for "replay for tutorial.trc."

"That's the one that I want so I'll press enter."

BN- "Creating route from replay file. Route created.  No GPS fix.  Route following will begin once a fix is obtained."

"The reason it says that is because I restarted the replay and the replay is still paused.  So let's resume the replay."

BN-"Resuming GPS replay.  GPS Replay detected.  Acquired position 21 feet 9 satellites.  Destination set.  Approaching left turn at waypoint 2 Wilton Parkway Hyler Ave in 121 feet."

"So now we’re getting our turn by turn instructions."

BN- "Turn left at waypoint 2 Wilton Parkway Hyler Ave.  Hyler Ave heading south."

"Once we make that turn, we'll get our next turn instructions."

BN- "On Wilton Parkway heading west.  Continue straight.  248 feet to waypoint 3 Hyler Ave Parkwood Ave.  Turn right at waypoint 3 Hyler Ave Parkwood Ave, Parkwood Ave heading west.  On Hyler Ave heading south.  On Hyler Ave 195 feet ahead to Parkwood Ave."

"So once we get closer to Parkwood Ave we'll be told our next instruction."

BN- "Approaching right turn at waypoint 3 Hyler Ave Parkwood Ave in 123 feet."

"Let's hit t for our next turn."

BN- "Turn right at waypoint 3 Hyler Ave Parkwood Ave, Parkwood Ave heading west.  38 feet ahead, waypoint 3 Hyler Ave Parkwood Ave, Parkwood Ave turn right heading west, less than one minute."

"That was  the T instruction that I had. I just pressed T.  Once I make the right we'll get our next turn instruction."

BN- "On Parkwood Ave heading southwest.  Continue straight.  252 feet to waypoint 5, fire hydrant.  Proceed ahead at waypoint 5 fire hydrant, heading west.  On Parkwood Ave 196 feet ahead to Marquette Ave."

"Our next important point is the fire hydrant which is the annotation that I had put in.

BN- "Approaching ahead turn at waypoint 5 fire hydrant in 125 feet."

"If the annotation was not there, our turning point would have been Kolvin Ave."

BN-"Proceed ahead at waypoint 5 fire hydrant heading west.  Continue straight.  849 feet to waypoint 9.  Turn left at waypoint 9 Kolvin Ave heading South.  Annotation.  Fire hydrant."

"That's exactly where the annotation" placed in the replay.  That sound effect (boing sound) told us that was an annotation.  Let's hit t for our next turn."

BN- "853655T ahead waypoint 9, Kolvin Ave.  Turn left heading south 4 minutes.  On Parkwood Ave, 199 feet ahead to Charleston Ave.  Paused GPS Replay."

BN- "I paused the GPS replay.  I'm not going to make you go through the rest of the replay with me, but you get the idea.  It's giving us turn by turn instructions and we can play the replay as many times as we want until we feel comfortable and then we can take the route that was created from this replay and follow it live.  So now I'm going to switch back to my GPS receiver.  To do that, I'm going to press o"

BN- "GPS options menu."

"Then I’m going to press c.”

BN- "GPS receiver currently set to GPS replay.  Receiver selection menu."

"It says the GPS receiver is currently set to GPS replay and I'm going to press the letter b for Bluetooth.  Keep in mind my receiver is not on so it's going to try and connect to it but it will fail."

BN- "Attempting to connect to Holux m-1, 000.  Unable to connect to receiver.  Exiting options menu.  Holux m-1, 000 receiver not detected.  To reconnect, turn the receiver on and press control r or press v to enter virtual mode without connecting."

"So now I'm going to press escape on the QT or space with e on the BT or PK."

BN- "Save route before exiting? Yes, no, cancel"

"It asks me do I want to save the route? Yes, no, cancel.  I do not wish to save this route because I could just reconvert it if I wanted to.  I’m going to answer no."

BN-"Route not saved.  Exiting GPS.  Main menu.

"For more information on any of the features discussed in this audio tutorial please see the users guide which is located on your compact flash card.  Or you can find it from within the GPS application by visiting the information center."

12: Wrapping Up

Note- In this, and all subsequent tutorials, prompts by the Braille Note will be indicated by "bn".  All other information in quotation marks indicates explanations by the narrator.

“That concludes our Sendero GPS tutorial. We hope you’ve enjoyed this tutorial and learned a few things in the process.”

“Here’s a review of some of the key points:”

“Sendero GPS is not a mobility aid, it’s an orientation tool. Please pay first and foremost attention to your mobility information and techniques.  GPS is intended to get you in the vicinity of a building or to let you know that a particular building is in the neighborhood. Remember, GPS tracks your movements, so you need to move at least 30-50 feet in a constant direction in order to get accurate heading/directional information.”

“For items not covered in this tutorial, please refer to the GPS manual, the senderogroup.com website, or the readme file found in the docs folder on your storage card. Sendero GPS customers have access to our online training at: senderogrup.com/chat. Inquire for more information.”

“Sendero is available for training throughout the United States and we hope in other countries soon. We’re constantly working to improve the content, the map data, the GPS software and hardware. Sendero Group’s GPS software is the result of input from users and professionals throughout the industry for more than a decade. We look forward to more comments from you so that we can make more improvements. We thank you for your participation and for listening to this tutorial.”

“To contact Sendero, customers call toll-free in the United States or Canada: 888-757-6810.”

“All other customers or international callers dial: 1-530-757-6800. You can also contact us by email at: GPS@senderogroup.com or on the web at: http://www.senderogroup.com. You can contact us as well by fax at: 530-7576830.