# Installing Assisted GPS on the I-Blue 737A+

The iBlue can acquire satellites faster if the AGPS data has been installed. The difference is only a matter of seconds, perhaps acquiring in 5 seconds versus 30 seconds. The improvement in acquisition time varies so this is just an estimation. This is strictly an optional activity and Sendero Tech Support should not be called or Emailed for assistance. It should only be performed by users who are comfortable with Device Manager and COM Serial Port information. If you forget to update the iBlue with AGPS data, it will not adversely affect the receiver. It will just take the usual Cold start time of 35 seconds or so. The AGPS Update should be performed every 10 days or so to obtain maximum benefit.

This document has been marked up with heading navigation. Heading Level 1 is start of document (title). Heading 2 is for each part of the procedure and 3 for the OS and Driver/Application Installation section.

## Connecting Bluetooth GPS Receiver to PC

Plug the receiver into the USB port on the Windows PC. Turn the receiver on and answer any prompts to install the driver for the receiver. This may happen multiple times if you use a different USB port in a subsequent session.

## Installing USB Driver and GPS View Application.

Move back to listing of files.

### Installing the USB Driver

ENTER on USB_Driver and you should see 6 files. Select InstallDriver.exe. If running Windows 64-bit, Select installdrv64.exe. Once complete, select Exit.

### Installing GPS View App

Now, install the Software utility. Copy GpsView v2.0.16 to your Desktop.

## Obtaining COM Serial PORT Number

### XP

Select System in Control panel and choose the Hardware tab and Device Manager. TAB to treeview and scroll down to the item that says Ports (COM and LPT). Press RIGHT ARROW to open that list if it isn’t already and scroll down until you hear GPS or press G to jump through the G items. Make note of the serial com port number. Then exit out of the Device Manager and exit the Control panel.

### Vista and Windows 7

Open Start Menu and type Device Manager. TAB to treeview and scroll down to the item that says Ports (COM and LPT). Press RIGHT ARROW to open that list if it isn’t already and scroll down until you hear GPS or press G to jump through the G items. Make note of the serial com port number. Then exit out of the Device Manager and exit the Control panel.

## Opening GPS View and COM Serial Port Config

Once you click on GPSView, you need to find the field called COM Port. TAB several times until you find the ON button, TAB once more pass this to COM Port selection. Select COM Port number and test by selecting On button. If, an error message is displayed stating the attempt to open the COM Port failed, the wrong COM Port was selected.

## Updating AGPS

TAB several times to the Update button and press ENTER. It comes right after the Reset button. The first time you run the Update you may be prompted with a security alert from Windows or Firewall program. Select Unblock/Allow. Once the update runs, you should hear Update Complete within 10 seconds. Press OK. You should be good for 10 days or so at which time you should run the update again. Obviously, the installation of the software has already been done at that point and you just have to run the update from the same port. It is possible that the com port number could change so if it doesn’t complete, check the com port setting.

## Other features in GPS View:

Speed, type of position fix, performing reset (cold, warm or hot) fix to GPS receiver, latitude and longitude, logging and a few others.