440-1.EXE 	COM1 version

440-2.EXE	COM2 version

440.doc		this file

440-x.EXE is a stand alone compiled basic program designed to control

the KENWOOD 440s or 940s from the keyboard of an IBM-PC computer through

serial ports COM1 or COM2. It is designed to run from a single command

line or through the menus provided in the program itself. By entering all

440-x commands from the command line the program can be used with the

new "!" DOS shell command in version 4.2 of W0RLI CPBBS.

EXAMPLE of a typical command used in the FWD.MB file:

!b			<DOS shell command for port B of the CPBBS

440-2 20 5		<440-2.EXE command string (COM2:)

*** EOF			<END of dos shell

GB0023F W0RLI		<Beginning of next forward list

W0RLI

*** EOF

In the above example the "440-2" runs the 440-2.EXE program then

picks option "20" (SELECT MEMORY CHANNEL) and finally "5" which

selects channel 5. This same kind of command sequence could be used

in the autoexec file to initialize the radio on startup.

WARNING... this program is untested and you should try each command

sequence using the manual menu method before including it in your

FWD.MB or AUTOEXEC.BAT file. The program will ECHO the radio command

to your screen when run through your FWD.MB file, I kept that in

as an indication that the program actually ran. It will be taken out

of the final version when run from a command string if it causes a problem.

This program was written to allow a handicapped fellow ham control his

radio from the keyboard of his PC. My original intent was to write the

thing in C but I could not figure out how to control the PORTS and make

use of the COMBIOS etc.. I knew I could write the program in a few

hours in BASICA but I also knew the result would limit it's operation

to COM1 and COM2 only. To make a long story short I am making the program

available now in compiled basic form while I await additional information

on the control of the communication ports using the COMBIOS programs.

I DO NOT have an interface to control my 440s yet but it is on order so

if any of you have any comments or suggested additions to the program

please leave a message so I can add the features to it.

de KB3MY, CIS 72605,1465 Buster Light, Sliver Spring, Maryland

KB3MY @ KA3OGG

###################