SUPER MORSETM

Ver. 4.06

Copyright 1986-93

M. Lee Murrah, WD5CID

8-1-93

������������������������������������������������ͻ

�     The Original Comprehensive Morse Code      �

� Training Program for the IBM PC and Compatibles�

������������������������������������������������ͼ

������������������������������������������������Ŀ

�                                                �

�                    LICENSE                     �

�                                                �

�  You may use Super Morse for private, non-     �

�  commercial purposes. You may copy and         �

�  distribute Super Morse so long as you charge  �

�  no more than the cost of a diskette not to    �

�  exceed $2 (no profit permitted). Bulletin     �

�  boards may distribute Super Morse so long as  �

�  nothing is charged beyond standard connect    �

�  or membership fees. Super Morse must be       �

�  distributed in its original form and content. �

�  Super Morse IS DISTRIBUTED "AS IS" WITHOUT    �

�  WARRANTIES OF ANY TYPE. THE AUTHOR SHALL NOT  �

�  BE LIABLE FOR ANY DAMAGE OR HARM RESULTING    �

�  FROM USE OF Super Morse.                      �

�                                                �

��������������������������������������������������

��������������������������������������ͻ

�     PLEASE UPLOAD TO YOUR LOCAL      �

�   COMPUTER BULLETIN BOARD SYSTEMS!   �

� ASK SYSOP TO ERASE EARLIER VERSIONS! �

��������������������������������������ͼ

����������������������������������������ͻ

�                                        �

� SUGGESTED SHAREWARE CONTRIBUTION - $20 �

�                                        �

� Payments may be charged to your Compu- �

� serve account in the Shareware Regist- �

� ration Database (GO SWREG, Prog. 649)  �

����������������������������������������ͼ

TABLE OF CONTENTS

FOREWORD . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  1

I. The Morse Code. . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

A. What Is the Morse Code?. . . . . . . . . . . . . . . . . . . . .  2

B. How Morse Code Is Defined. . . . . . . . . . . . . . . . . . . .  3

1. DITS and DAHS . . . . . . . . . . . . . . . . . . . . . . .  3

2. Morse Characters. . . . . . . . . . . . . . . . . . . . . .  3

3. Code Weighting. . . . . . . . . . . . . . . . . . . . . . .  5

4. Code "Speed". . . . . . . . . . . . . . . . . . . . . . . .  5

C. Learning the Morse Code. . . . . . . . . . . . . . . . . . . . .  6

II. Super Morse Basics . . . . . . . . . . . . . . . . . . . . . . . . .  9

A. Conventions Used in Manual . . . . . . . . . . . . . . . . . . .  9

B. Super Morse Program and Data Files . . . . . . . . . . . . . . .  9

1. Distribution File . . . . . . . . . . . . . . . . . . . . .  9

2. Files in Distribution File. . . . . . . . . . . . . . . . . 10

3. Special Super Morse Versions. . . . . . . . . . . . . . . . 10

C. Equipment and System Requirements and Limitations. . . . . . . . 11

1. General Requirements. . . . . . . . . . . . . . . . . . . . 11

2. Code Speed Calibration. . . . . . . . . . . . . . . . . . . 11

3. Video Display Card. . . . . . . . . . . . . . . . . . . . . 12

4. Disk Cache Programs . . . . . . . . . . . . . . . . . . . . 12

5. Floppy Disk Write Protect Tabs. . . . . . . . . . . . . . . 12

6. WINDOWS Capability. . . . . . . . . . . . . . . . . . . . . 12

7. Speaker Volume. . . . . . . . . . . . . . . . . . . . . . . 12

8. Sound Boards. . . . . . . . . . . . . . . . . . . . . . . . 12

D. Installation . . . . . . . . . . . . . . . . . . . . . . . . . . 13

1. In General. . . . . . . . . . . . . . . . . . . . . . . . . 13

2. Upgrading from CW or Earlier Super Morse

Versions. . . . . . . . . . . . . . . . . . . . . . . . . 14

E. Printing the DOC File. . . . . . . . . . . . . . . . . . . . . . 14

F. Morse Code Considerations. . . . . . . . . . . . . . . . . . . . 14

1. The Morse Character Set . . . . . . . . . . . . . . . . . . 14

2. Code Speed. . . . . . . . . . . . . . . . . . . . . . . . . 16

III. Getting Started with Super Morse. . . . . . . . . . . . . . . . . . 17

A. Starting and Quitting Super Morse. . . . . . . . . . . . . . . . 17

1. Starting Super Morse. . . . . . . . . . . . . . . . . . . . 17

2. Title Screen. . . . . . . . . . . . . . . . . . . . . . . . 17

3. User Number . . . . . . . . . . . . . . . . . . . . . . . . 17

4. Quitting Super Morse. . . . . . . . . . . . . . . . . . . . 17

B. Super Morse Organization . . . . . . . . . . . . . . . . . . . . 18

1. Organizational Philosophy . . . . . . . . . . . . . . . . . 18

2. Menus . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

3. The Super Morse Main Display. . . . . . . . . . . . . . . . 19

4. Super Morse Help. . . . . . . . . . . . . . . . . . . . . . 20

C. Super Morse Audiovisual Cues . . . . . . . . . . . . . . . . . . 20

1. Code Output . . . . . . . . . . . . . . . . . . . . . . . . 20

2. Audio Confirmation and Alerts . . . . . . . . . . . . . . . 20

3. Repeating Characters and Words. . . . . . . . . . . . . . . 20

4. Visual Cues . . . . . . . . . . . . . . . . . . . . . . . . 21

D. Program Control. . . . . . . . . . . . . . . . . . . . . . . . . 21

1. The <ESC> Key . . . . . . . . . . . . . . . . . . . . . . . 21

2. Pause . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

3. Screen Control. . . . . . . . . . . . . . . . . . . . . . . 21

4. Timing a Super Morse Session. . . . . . . . . . . . . . . . 22

E. User Input Modes . . . . . . . . . . . . . . . . . . . . . . . . 22

F. The Session Buffer . . . . . . . . . . . . . . . . . . . . . . . 22

G. Reviewing Program Information. . . . . . . . . . . . . . . . . . 22

1. The Review Menu . . . . . . . . . . . . . . . . . . . . . . 22

2. Reviewing the Current Session . . . . . . . . . . . . . . . 23

3. Viewing the Quick Keys. . . . . . . . . . . . . . . . . . . 23

4. Viewing the Morse Alphabet. . . . . . . . . . . . . . . . . 23

5. Viewing the DOC File. . . . . . . . . . . . . . . . . . . . 24

6. Displaying User Statistics. . . . . . . . . . . . . . . . . 24

IV. Settings and Options . . . . . . . . . . . . . . . . . . . . . . . . 25

A. Settings in General. . . . . . . . . . . . . . . . . . . . . . . 25

B. Quick Settings . . . . . . . . . . . . . . . . . . . . . . . . . 25

1. The Quick Keys. . . . . . . . . . . . . . . . . . . . . . . 25

2. Setting Code Speed. . . . . . . . . . . . . . . . . . . . . 26

3. Controlling Audio . . . . . . . . . . . . . . . . . . . . . 28

C. Super Morse Setup. . . . . . . . . . . . . . . . . . . . . . . . 29

1. The Setup Menu. . . . . . . . . . . . . . . . . . . . . . . 29

2. Setting the Code Parameters . . . . . . . . . . . . . . . . 29

3. Configuring the Character Set . . . . . . . . . . . . . . . 33

4. Configuring the COM Ports . . . . . . . . . . . . . . . . . 34

5. Setting Screen Colors . . . . . . . . . . . . . . . . . . . 36

6. Miscellaneous Settings. . . . . . . . . . . . . . . . . . . 36

7. Initializing Super Morse Parameters . . . . . . . . . . . . 38

D. Saving Current Settings. . . . . . . . . . . . . . . . . . . . . 38

1. Save Modes. . . . . . . . . . . . . . . . . . . . . . . . . 38

2. The Settings Files. . . . . . . . . . . . . . . . . . . . . 38

E. Customization of External Files. . . . . . . . . . . . . . . . . 38

1. General . . . . . . . . . . . . . . . . . . . . . . . . . . 38

2. Data Files. . . . . . . . . . . . . . . . . . . . . . . . . 39

3. License Classes File. . . . . . . . . . . . . . . . . . . . 39

4. Text Files. . . . . . . . . . . . . . . . . . . . . . . . . 40

F. Interfacing External Equipment . . . . . . . . . . . . . . . . . 40

V. The Learn Phase . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

A. The Learn Command. . . . . . . . . . . . . . . . . . . . . . . . 41

1. Introduction. . . . . . . . . . . . . . . . . . . . . . . . 41

2. The Super Morse Teaching Method . . . . . . . . . . . . . . 41

3. Repeating Characters. . . . . . . . . . . . . . . . . . . . 42

4. Feedback in the Learn Phase . . . . . . . . . . . . . . . . 42

5. Character Speed in the Learn Phase. . . . . . . . . . . . . 42

B. Lesson <A> - DITS and DAHS, the Basic Code Sounds. . . . . . . . 42

C. Lesson <B> - Sound Combinations. . . . . . . . . . . . . . . . . 42

D. Lesson <C> - Character Steps . . . . . . . . . . . . . . . . . . 43

E. Lesson <D> - Character Runs. . . . . . . . . . . . . . . . . . . 43

F. Lesson <E> - Character Groups. . . . . . . . . . . . . . . . . . 43

G. Lesson <F> - Character Learning Levels . . . . . . . . . . . . . 44

H. Lesson <G> - All Characters. . . . . . . . . . . . . . . . . . . 44

I. Flash Characters . . . . . . . . . . . . . . . . . . . . . . . . 44

J. Oddball Character Drill. . . . . . . . . . . . . . . . . . . . . 44

K. The User Group . . . . . . . . . . . . . . . . . . . . . . . . . 44

L. Viewing the Character Groupings (Steps and Levels) . . . . . . . 44

VI. The Build Phase. . . . . . . . . . . . . . . . . . . . . . . . . . . 45

A. Building Code Speed. . . . . . . . . . . . . . . . . . . . . . . 45

B. Receiving Character Groups . . . . . . . . . . . . . . . . . . . 45

1. Random Groups . . . . . . . . . . . . . . . . . . . . . . . 45

2. Structured Groups . . . . . . . . . . . . . . . . . . . . . 45

3. Variable Groups . . . . . . . . . . . . . . . . . . . . . . 45

4. Group Sending Modes . . . . . . . . . . . . . . . . . . . . 46

C. Receiving Complete Words . . . . . . . . . . . . . . . . . . . . 46

D. Receiving A Message Entered via Keyboard . . . . . . . . . . . . 46

E. Receiving a Simulated Radio Transmission ("QSO") . . . . . . . . 46

F. Receiving Text in a Disk File. . . . . . . . . . . . . . . . . . 47

1. General . . . . . . . . . . . . . . . . . . . . . . . . . . 47

2. Scripted Disk Files . . . . . . . . . . . . . . . . . . . . 48

G  Receiving in the Zoom Mode . . . . . . . . . . . . . . . . . . . 49

VII.  The Enhance Phase. . . . . . . . . . . . . . . . . . . . . . . . . 50

A. Skill Enhancement. . . . . . . . . . . . . . . . . . . . . . . . 50

B. Sending Code . . . . . . . . . . . . . . . . . . . . . . . . . . 50

1.  Sending from the Keyboard. . . . . . . . . . . . . . . . . 50

2.  Sending with a Manual Key. . . . . . . . . . . . . . . . . 50

C. Copying Behind . . . . . . . . . . . . . . . . . . . . . . . . . 51

1.  Head Copy. . . . . . . . . . . . . . . . . . . . . . . . . 51

2.  Lag Copy . . . . . . . . . . . . . . . . . . . . . . . . . 51

3.  Random Lag Copy. . . . . . . . . . . . . . . . . . . . . . 51

4. Word Copy . . . . . . . . . . . . . . . . . . . . . . . . . 52

D. Copying on a Typewriter. . . . . . . . . . . . . . . . . . . . . 52

E. Boost Copy . . . . . . . . . . . . . . . . . . . . . . . . . . . 52

F. Questions and Answers. . . . . . . . . . . . . . . . . . . . . . 52

VIII.  The Measure Phase . . . . . . . . . . . . . . . . . . . . . . . . 54

A. The Measure Command. . . . . . . . . . . . . . . . . . . . . . . 54

B. Exam Sources . . . . . . . . . . . . . . . . . . . . . . . . . . 54

C. Simulated VEC Exam . . . . . . . . . . . . . . . . . . . . . . . 55

1. Auto Exam . . . . . . . . . . . . . . . . . . . . . . . . . 55

2. Exam from File. . . . . . . . . . . . . . . . . . . . . . . 55

3. Checking Exam Contents. . . . . . . . . . . . . . . . . . . 56

4. Displaying and Printing the Exam. . . . . . . . . . . . . . 56

D. Keyboard Receive Test. . . . . . . . . . . . . . . . . . . . . . 56

E. Solid Copy Test. . . . . . . . . . . . . . . . . . . . . . . . . 56

IX. The Operation Phase. . . . . . . . . . . . . . . . . . . . . . . . . 58

A. The Operation Menu . . . . . . . . . . . . . . . . . . . . . . . 58

B. Two Way Practice via Telephone Modem . . . . . . . . . . . . . . 58

1.  The Modem Command. . . . . . . . . . . . . . . . . . . . . 58

2.  General Description of the Modem Function. . . . . . . . . 58

3.  Making a Connection with Another Computer. . . . . . . . . 59

4.  CONTROL Commands . . . . . . . . . . . . . . . . . . . . . 59

5.  Problems with Modem Command. . . . . . . . . . . . . . . . 60

C. Radio Terminal . . . . . . . . . . . . . . . . . . . . . . . . . 60

X. We Need Your Feedback and Help! . . . . . . . . . . . . . . . . . . . 61

Appendix A. - General Information. . . . . . . . . . . . . . . . . . . . 63

1. The Author . . . . . . . . . . . . . . . . . . . . . . . . . . . 63

2. The Software . . . . . . . . . . . . . . . . . . . . . . . . . . 63

3. Other Computers. . . . . . . . . . . . . . . . . . . . . . . . . 63

Appendix B. - Super Morse License. . . . . . . . . . . . . . . . . . . . 64

Appendix C. - Distribution and Support . . . . . . . . . . . . . . . . . 65

1. Distribution Policy. . . . . . . . . . . . . . . . . . . . . . . 65

2. Suggested Contributions. . . . . . . . . . . . . . . . . . . . . 65

3. Courtesy Copies of Super Morse . . . . . . . . . . . . . . . . . 66

4. Support and Inquiries. . . . . . . . . . . . . . . . . . . . . . 66

6. Comments and Suggestions . . . . . . . . . . . . . . . . . . . . 67

Appendix D. - Information about Amateur Radio. . . . . . . . . . . . . . 68

1. Amateur Radio Organizations. . . . . . . . . . . . . . . . . . . 68

2. Amateur Radio Magazines. . . . . . . . . . . . . . . . . . . . . 68

3. On-Line Computer Services. . . . . . . . . . . . . . . . . . . . 69

Appendix E. - Morse Code Table . . . . . . . . . . . . . . . . . . . . . 70

Appendix F. - Troubleshooting. . . . . . . . . . . . . . . . . . . . . . 72

Appendix G. - Interfacing. . . . . . . . . . . . . . . . . . . . . . . . 75

FOREWORD

Super Morse was written primarily to help prospective hams

overcome what many perceive to be the biggest obstacle to

obtaining an amateur radio license--learning the Morse code.

When I began to write Super Morse in 1986, there were only a few

simple Morse code tutorial programs were available for the IBM-PC

and compatibles. However, they were very limited and are often

written in BASIC, which requires a clumsy interpreter.  In

writing Super Morse I sought to create a full-featured program

which provides a variety of modes for learning and practicing the

code and which takes full advantage of the computer as a teaching

tool.

My specific goals in writing Super Morse were:

1.  Professional appearance and operating characteristics.

2.  Comprehensive program covering all aspects of Morse code

learning and practice.

3.  Create new methods of learning and practicing the Morse

code.

4.  Provide ample opportunities for customization by the

user.

5.  Use the unique capabilities of the microcomputer to

teach and learn Morse code.

Super Morse is directed primarily to the United States

amateur radio community, but it can be used be used by commercial

operators.  Amateur radio uses a special subset of the

International Morse code, but Super Morse supports several of the

more common characters used in commercial radiotelegraphy.  In

addition Super Morse supports four Nordic characters used

primarily in Scandinavia.

M. Lee Murrah

Cedar Rapids, IA

April, 1993

I. The Morse Code

A. What Is the Morse Code?

Morse code is a communication code, named for Samuel F. B.

Morse, the inventor of the telegraph, which uses combinations of

timed sound elements to represent the characters of the alphabet.

Morse code was originally used in land telegraphy and comprised a

series of timed clicks.  Today, however, it is usually sent via

radio by turning a radio transmitter on and off to produce the

timed code elements.

Some have stated that Morse code is an obsolete mode of

communications in this age of digital and satellite

communications.  However, modern communications require complex

equipment and could easily be knocked out of service in times of

war or natural disaster.  It is the simplest, most basic form of

radio communications available today, and it has always been

recognized that Morse code can "get through" when voice

communication is impossible.

One of the biggest advantages of Morse code is that it can

be transmitted using very simple, inexpensive equipment.  Modern

voice and digital communications equipment can now be built only

in sophisticated manufacturing facilities, but a Morse code

transmitter can be built at home by even the newest amateur using

readily available parts. Incredibly, some very low power Morse

transmitters have been constructed from a single TTL logic gate

integrated circuit and one or two external components!

Morse code is also a useful skill even outside radio

communications.  Ex-prisoners of war held incommunicado in

solitary confinement in North Vietnam report that they

communicated by tapping messages in Morse code on the walls of

their cells.  Morse code was the only way that the prisoners were

able to communicate with their fellows, and it helped them to

retain their sanity  during several years of imprisonment.

Further, like a foreign language, learning Morse code is a mental

challenge that keeps the mind active and improves thinking

skills.

Today Morse code is most widely known for its use in amateur

radio, but it is also used in military and commercial

communications.

In the United States it is now possible to obtain a limited

amateur radio license without passing a Morse code test.

However, the license is limited to the Technician class which has

limited frequency bands and privileges.  In order to obtain

access to the full range of privileges and frequency bands, you

must pass a Morse code test.

B. How Morse Code Is Defined

1. DITS and DAHS

Morse code is a sound language that uses two tones of

differing lengths in various combinations to represent all the

characters of the Morse alphabet. These two tones are

distinguished by their length, one a "short" tone and the other a

"long" tone.  The short tone is phonetically referred to as

"DIT", and the long tone as "DAH".  You will often see the short

tone represented as a "." (dot) and the long tone as a "-"

(dash), but it is commonly agreed that use of these symbols

encourages a visual approach to the code, which slows code

learning.  You should never think of the Morse code as dots and

dashes or write it in that way!

The original Morse code as used in landline telegraphy was

represented by a combinations of timed clicks.  However, when

radio came into use, the code was revised to use tones which are

more suited for radio use. The original Morse code is used in

land telegraphy is called the American Morse code, but the code

now commonly used in radio is called the International Morse

code.

2. Morse Characters

a.  The Morse Character Set

Amateur radio uses a reduced character set of the 26

letters, the 10 numerals, 4 punctuation marks and 5 special

procedure signs.

b. Characters

The letters are as follows:

A  DIT DAH           N  DAH DIT

B  DAH DIT DIT DIT   O  DAH DAH DAH

C  DAH DIT DAH DIT   P  DIT DAH DAH DIT

D  DAH DIT DIT       Q  DAH DAH DIT DAH

E  DIT               R  DIT DAH DIT

F  DIT DIT DAH DIT   S  DIT DIT DIT

G  DAH DAH DIT       T  DAH

H  DIT DIT DIT DIT   U  DIT DIT DAH

I  DIT DIT           V  DIT DIT DIT DAH

J  DIT DAH DAH DAH   W  DIT DAH DAH

K  DAH DIT DAH       X  DAH DIT DIT DAH

L  DIT DAH DIT DIT   Y  DAH DIT DAH DAH

M  DAH DAH           Z  DAH DAH DIT DIT

The numbers are as follows:

1  DIT DAH DAH DAH DAH     6  DAH DIT DIT DIT DIT

2  DIT DIT DAH DAH DAH     7  DAH DAH DIT DIT DIT

3  DIT DIT DIT DAH DAH     8  DAH DAH DAH DIT DIT

4  DIT DIT DIT DIT DAH     9  DAH DAH DAH DAH DIT

5  DIT DIT DIT DIT DIT     0  DAH DAH DAH DAH DAH

The punctuation is as follows:

.  DIT DAH DIT DAH DIT DAH

,  DAH DAH DIT DIT DAH DAH

?  DIT DIT DAH DAH DIT DIT

/  DAH DIT DIT DAH DIT

-  DAH DIT DIT DIT DIT DAH

b. Procedure Signs

Morse code communications uses several special characters

that convey procedural information.  These are often longer

characters that can be viewed as combinations of alphabetic

characters without and intervening character space.  Following

are the most common procedure signs, or "prosigns", used in

amateur radio:

Prosign          Meaning

������           ������

<AA>           New line

<AR>           End of message

<AS>           Wait, stand-by

<BT>           Double Dash ("=")

<KN>           Over-specified station only

<KA>           Attention

<SK>           End of contact

<SN>           Understood

The latter two prosigns along with <BT> are frequently used

in traffic handling.

c. Non-English Characters

The following European characters are supported by Super

Morse in some modes:

ASCII     Identification

����      �������������

129       {Double dotted "u"}

130       {Forward accented "e"}

143       {Dotted "a"}

146       {Double dotted "a"}

157       {Double dotted "o"}

164       {tilde "n"}

3. Code Weighting

The duration of the elements of Morse code have a standard

relationship among themselves. The DIT is assigned the shortest

duration and is defined as being one time unit in duration, and a

DAH is 3 times as long as a DIT, or 3 time units in duration.

The silence between DITs and DAHs in a character, which we will

call an "element space" is the same length as a DIT, i.e., one

time unit in duration.  The silence between characters, which we

will call a "character space", is 3 time units in duration.

Finally, the space between words, which we will call a "word

space" is 7 time units in duration.

Perfectly sent code uses these standard relationships, but

code is not usually perfectly sent except when a computer is

used.  Each person who sends code manually, develops a weighting

slightly different from the standard, and this unusual weighting

is said to constitute the operator's very recognizable "fist".

Operators who use automatic keyers can usually set the

weighting as a personal preference.  Weighting very far from the

standard become choppy and hard to copy.

4. Code "Speed"

a. How Speed is Determined

The speed of Morse code is measure in words per minute,

which is normally abbreviated as WPM. Code speed is an elusive

measure since words are of various lengths; and when random

characters are being sent, there are no words at all!

Code speed has therefore been arbitrarily defined by

standard words.  The standard word for plain text is "PARIS",

which, if you add up the time units in all the code sound

elements used to send "PARIS" (DITs, DAHs, and spaces between the

DIT's and DAHS, characters and words), you will find that it is

50 units of time long.  One word per minute is defined as sending

"PARIS" using code elements timed to fill one minute, including

the word space following the last word.  Ten words per minute

would be sent at a speed necessary to send "PARIS" 10 times.  The

code elements are calibrated at any particular speed against

"PARIS", and when text is sent at, say, 10 words per minute,

greater or fewer actual words may be sent depending upon their

length.

The word "PARIS" will not work for random characters since

the average Morse code character used to send English text is

shorter than the average character used to send random

characters.  This is because English does not use all the letters

in equal numbers and because Morse code assigns the shortest

codes to the most frequently used letters in English.  The most

frequently used letter in English is "E", and it is assigned the

shorted code, the DIT.  A less frequently used letter such as "Q"

is assigned a longer code, DAH DAH DI DAH.

For random characters, therefore, the standard word "CODEX"

was chosen. If you analyze "CODEX", you will find that it

contains 60 code elements. The effect of this difference is that

the DITS and DAHS in randomly sent characters should sound about

20% shorter than those in plain text at the same nominal speed.

Randomly sent characters therefore should sound 20% faster that

plain words.

b. The "Farnsworth" Method"

Morse code is usually sent with the standard relationship

between character spaces and the elements which make up the

character.  In other words, the DITS, DAHS and element spaces are

sent according to the same speed standard.  However, Farnsworth

discovered that code learning can be speeded up if the DITs, DAHs

and element spaces are compressed as if the code were being sent

at a high speed while the character spaces are expanded increased

to maintain an overall lower send speed.  This is called the

"Farnsworth method", and it is endorsed by the ARRL as the best

way to learn the code.

The Farnsworth method is supported and is used as the

default method by Super Morse.  Two Farnsworth speeds are

provided.  The "Lo" Farnsworth speed, preset at 18 WPM, is used

at send speeds below 20 WPM; and the "Hi" Farnsworth speed,

preset at 26 WPM, is used above 20 WPM. Beware, however, that not

all code tests are given in the Farnsworth format!  Check with

your testing coordinator to determine which method will be used,

and then use that system when studying with Super Morse.

C. Learning the Morse Code

There is no one best way to learn the Morse code.  As with

foreign languages (to which Morse code is similar in some ways),

some people can learn the code faster than others.  The best

attribute that one can possess in learning the code is

persistence.

Super Morse implements one method of learning the Morse

code.  If you follow the lessons provided in the Learn phase, you

should have no trouble in learning the Morse code in a few weeks

or months.

In addition to the method provided by Super Morse several

general tips on learning the Morse code can be provided as

follows:

1.  Learn the characters by sound, not by sight.  Each of

the characters has its own characteristic length and rhythm.  Do

not memorize the characters using the visual dot and dash

symbols.  That will only slow you down and create bad habits that

will be hard to break!

2.  Set the character speed to 13 words per minute or

higher (the ARRL uses 18 WPM) and the send speed at whatever

lower speed you desire. (Super Morse automatically sets the

character speed to 18 WPM). Since your ultimate goal is 13 words

per minute, it is much better to learn the sounds of the

characters at the higher speed even though they are spaced at a

lower speed.  As stated above this is called the Farnsworth

method, and it helps to eliminate the "hump" that most people

experience around 10 words per minute.

3.  Study every day, but if you feel you are stuck and not

making progress, take a few days off.  It's amazing how this will

sometimes clear your head.

4.  Try to copy code in you head as much as possible

.

5.  Obtain access to a ham receiver and try to copy real

transmissions off the air.  Some general coverage short wave

receivers permit reception of Morse code, but in general they are

harder to tune in on a Morse code signal.

6.  As soon as possible learn the sounds of whole words.

You will find that whole words have characteristic lengths and

rhythms that distinguish them from all other words.

7.  Turn on the Super Morse Build/Groups function and let

the computer send in the background while you are doing something

else, such as reading a book or working on a construction

project.  Try to identify characters while doing the other

activity.  This will help your brain shift to "automatic" and

will help the code sounds become a natural "part of the

wallpaper."

8.  Although the code test no longer requires that you

demonstrate an ability to send Morse code,  you should get a code

practice oscillator and practice your sending.  Note that Super

Morse can be used as a code practice oscillator using one of

several keys on the keyboard or an external telegraph key.

9.  As soon as you pass the novice test, get a rig on the

air and operate on the novice bands.  Nothing will increase your

speed and concentration as well as the pressure of an on-the-air

contact.  Don't worry about being perfect--the other guy is

probably in the same boat as you.

II. Super Morse Basics

A. Conventions Used in Manual

This manual uses words or characters in pointed brackets,

such as "<x>" or "<yyyy>" to indicate that the key denoted by the

text or character should be pressed.  Following are several

examples:

<A>      - Press the "A" key

<F1>     - Press the function key "F1"

<CTRL-E> - Press the "Control" and "E" keys simultaneously

<ALT-G>  - Press the "ALT" and "G" keys simultaneously

<ESC>    - Press the "ESCAPE" key

<RETURN> - Press the "RETURN" or "ENTER" key

<SHIFT>  - Press the "SHIFT" key

Brackets are also used in examples to represent textual

information that should be present or entered in a command line.

References to Super Morse menus are often shown in the form

Setup/Code/Speed Factor.  This means that the Speed Factor

selection is listed in the Code menu, which itself is a selection

in the Setup selection from the main menu.

B. Super Morse Program and Data Files

1. Distribution File

Super Morse is distributed as a self-extracting archive file

bearing a filename of the form SMnnn.EXE, where nnn is the

version number.  For example, version 4.01 is distributed as

SM401.EXE.  The archive file contains all the files necessary to

run Super Morse. To extract the files, at the DOS prompt simply

enter:

SMnnn <RETURN>

In some cases Super Morse may be distributed as a plain ZIP

(not self-extracting) file.  In that case you must use the

Shareware de-archiver program PKUNZIP which is available on

computer bulletin boards and from Shareware distributors.

If you share Super Morse with a friend or want to upload it

to a computer bulletin board system, please upload or share the

archive file in its original condition.  Do not add or remove

files, since that can create confusion to other users.  In

addition, do not distribute or share the files in de-archived

form since you may omit an important file!

2. Files in Distribution File

The Super Morse distribution file contains the following

files:

Needed

Name           Description                               to Run?

����������     ����������������������������������        ������

ANTENNAS.SM  - Names of antennas for QSO feature           Yes

SMBLD.HLP    - Help file for Build phase                   Yes

SMBYE.HLP    - Sign off message                            Yes

CITIES.SM    - Names of cities for QSO feature             Yes

CLASSES.SM   - Definition of license classes               Yes

SMENH.HLP    - Help file for Enhance phase                 Yes

GROUPS.SM    - Code groups for Learn phase                 Yes

GROUPS.1     - Alternate code group for Learn phase         No

GROUPS.2     - Alternate code group for Learn phase         No

HAMWORDS.SM  - List of "ham" words for receive feature     Yes

SMHOME.HLP   - Help file for control keys                  Yes

JOBS.SM      - Name of occupations for QSO feature         Yes

SMLRN.HLP    - Help file for Learn phase                   Yes

SMMAIN.HLP   - Main help file                              Yes

SMMEAS.HLP   - Help file for Measure phase                 Yes

SMMORSE.HLP  - Help file for Morse characters              Yes

NAMES.SM     - Names of operators for QSO feature          Yes

SMONLINE.HLP - Help file for the Modem feature             Yes

SMOP.HLP     - Help file for the Operate phase             Yes

Q&A.SM       - Sample question and answer file             Yes

QSO1.SM      - QSO template for Measure phase              Yes

SMRADIO.HLP  - Help file for radio terminal feature        Yes

README.1ST   - Important information about setup            No

REGISTER.FRM - For user registration and suggestions        No

REGWORDS.SM  - List of "regular" words for Receive feature Yes

RIGS.SM      - Names of radio equipment for QSO feature    Yes

RUNS.SM      - Character runs for Learn feature            Yes

SM.EXE       - the main executable file                    Yes

SM.DOC       - This documentation file                      No

SM.PIF       - Program information file for Windows        Yes*

STATES.SM    - States                                      Yes

STEPS.SM     - Character pairings for Learn feature        Yes

WHATSNEW.nnn - New features in version nnn                  No

* Needed for Windows use only

3. Special Super Morse Versions

Versions of Super Morse which have an "N" after the version

number are distributed by the National Amateur Radio Association

and contain additional files relevant to that organization.

Versions which have a "G" after the version number are for use by

the United States Government only.

C. Equipment and System Requirements and Limitations

1. General Requirements

Super Morse can be run on the IBM-PC and compatibles having

384K of memory and one floppy disk drive.  A hard disk is not

required, although you will find that a hard disk will enhance

operation of this and any other program.

Super Morse supports a Microsoft mouse.  In some

configurations, Super Morse uses two serial ports. If you use a

serial mouse, there may be a conflict.  To fully use all Super

Morse features, three serial ports are required.

Your CONFIG.SYS file should authorize at least 20 files.

This can be done with the following statement:

FILES = 20

CONFIG.SYS may be edited using a word processor in the ASCII

output mode or a text editor.

2. Code Speed Calibration

In order for Super Morse to send properly-timed code,

calibration to each computer is required.  Calibration is

affected primarily by the computer clock speed and memory

resident programs.

Super Morse uses two different calibration methods, clock

and loop, in order to handle various types of PC designs and

operating systems. The method is selected from the Setup menu. If

the clock calibration method is used, Super Morse automatically

calibrates each time it loads, and the speed should be

independent of your computer clock speed.  However, if you use

the loop method, you must manually calibrate Super Morse in the

Setup menu.  You must also recalibrate Super Morse each time you

add or remove a memory resident program if you use the loop

method.

The loop method must be used if your computer does not

implement the system clock chip in the same way as the original

PC design.  Super Morse is initially set for the clock method;

but if the code is erratic or choppy on your computer, you should

change to the loop method.

You must also use the loop method if you are running

Microsoft Windows or a PC emualtor on a McIntosh!

3. Video Display Card

Super Morse supports CGA (color), MDA (monochrome) and HGC

(Hercules) video displays.  The proper display adapter can be

selected from the Setup menu.

4. Disk Cache Programs

Super Morse saves changes in the program parameters when you

exit the program.  If you are using a disk cache program, the

parameters may be saved to the cache area in memory and thus may

not permanently saved to disk.  If you are having this problem,

you should try disabling any disk cacheing program which you have

installed on your system.

5. Floppy Disk Write Protect Tabs

If Super Morse is run from a floppy diskette, a write

protect tab should not be used! Super Morse must be able to store

configuration and user information to the diskette.

6. WINDOWS Capability

Super Morse is written as a DOS program, but it will run

under Microsoft Windows under certain conditions.  The author has

confirmed that it will operate under the 386 Extended mode (i.e.,

a 386 or 486 computer is required) using the loop timing method.

You must set up a PIF file, and set the background to foreground

ratio to 10000 to 1 and set Execution to "Exclusive".  These are

available as Advanced options in the PIF editor menu.  To save

you time a PIF file to run Super Morse under the conditions

stated is provided on the distribution diskette.  You should

select either the loop or clock timing method for best operation.

7. Speaker Volume

The volume produced by the PC speaker is very low on some

PC's, but unfortunately there is no way to increase it without an

external amplifier.  Some relief can be obtained by changing the

speaker tone since speakers are frequency dependent.

8. Sound Boards

Super Morse now supports the Sound Blaster card and

compatibles.  Super Morse initializes for use of the PC speaker,

and the Sound Blaster must be selected in the Setup/Code menu.

If you plan to use the Sound Blaster card for sound output,

you must use a text editor, such as EDIT or EDLIN, (or a word

processor in ASCII output mode) to insert a line in the

AUTOEXEC.BAT file specifying an "environment string".  The line

should be in the following format:

SET BLASTER=A<addr> I<int no> D<DMA channel no> T<SB type>

For a Sound Blaster 1.0 using I/O address 220, Interrupt 7 and

DMA channel 1, the string would look as follows:

SET BLASTER=A220 I7 D1 T1

The I/O address, Interrupt number, and DMA channel can be

determined using the TEST-SBC.EXE program provided with your

Sound Blaster.  If you did not change any of the jumpers on the

board when you installed it, you can use the factory default

settings shown in the Sound Blaster manual.

The SB type for Sound Blaster and compatible boards for use

with the "T" parameter is one of the following:

1 - Sound Blaster 1.5 or earlier

Sound Blaster Micro Channel Version

2 - Sound Blaster Pro

3 - Sound Blaster 2.0

Audio Spectrum 16

6 - Sound Blaster 16

Please let the author know if you have a board not in the

above list and its type ("T" parameter) number or other settings

if different from the above.

D. Installation

1. In General

In versions of Super Morse prior to 1.04, it was necessary

to create a special subdirectory and copy all data files into

that directory.  That is no longer required!  The only current

restriction on the location of files is that the files having HLP

and SM extensions must be in the same subdirectory as the SM.EXE

file.

Super Morse may be run directly from the distribution

diskette if diskette is 720K or larger, or it can be copied onto

another diskette or to a hard disk.  If you wish to use another

floppy diskette, simply extract the program files and copy all

the files on the distribution diskette other than the original

archive file.

If you install Super Morse on a hard disk, you may wish to

place the program in a special subdirectory of your choosing.

Simply create the subdirectory as described in your DOS

manual, and copy all the files from the distribution diskette to

the selected subdirectory.  A subdirectory named "\SM" is

suggested.

If you run Super Morse from a floppy disk, do not write

protect the disk!  Super Morse periodically writes to the

diskette, and write protection will cause a disk write error.

2. Upgrading from CW or Earlier Super Morse Versions

The structures of the configuration (SM.CFG) and user

(SMn.USR - SM now supports up to 10 users and their associated

files) files which record the current status of certain program

information have changed several times during the evolution of

Super Morse. If you are upgrading from a version earlier than

4.01, you will need to erase the current version of the

configuration and user files using the following commands:

DEL SM.CFG <RETURN>

DEL SM.USR <RETURN>

and reinitialize Super Morse.  If you fail to do this you, will

either get a "Disk Read" error or the program will operate

improperly.

E. Printing the DOC File

To print SM.DOC (this file) enter the following command:

COPY SM.DOC PRN: <RETURN>

You can also use the PRINT.COM program which comes with DOS or a

word processor program.

F. Morse Code Considerations

1. The Morse Character Set

a. Standard Character Set

Super Morse implements the standard Morse code character set

used in amateur radio.  This includes the 26 letters, 10 numbers,

the comma, period, question mark, the slash mark, and the

prosigns.  In addition, Super Morse supports Nordic characters

and non-amateur punctuation(colon, semicolon, left parenthesis,

and right parenthesis).  The non-amateur punctuation are

available only in functions using the keyboard and disk files and

are not part of the character set for automatic sending by the

computer.

Several Morse code procedure signs ("prosigns") are not

printable as ASCII characters.  These characters are usually

represented as a combination of two printable ASCII characters as

follows:

Prosign      ASCII Representation    Meaning

������       �������������������     ������

<AR>                 +              End of message

<SK>                 $              End of contact

<AS>                 !              Wait, stand-by

<KN>                 #              Over-specified station only

<BT>                 =              Double dash

<AA>                 @              New line

<KA>                 &              Attention

<SN>                 %              Understood

When prosigns are printed on the screen, they are

represented in their double letter form surrounded by pointed

brackets. Prosigns may be entered either as a single ASCII

character or as a character combination. To enter as a  character

combination, depress the <SHIFT> key as both characters of the

combination are entered. Make sure that the <CAPS LOCK> key is

"off" when using the keyboard to enter Morse characters since the

status of the <SHIFT> key is used to distinguish between prosigns

and regular characters. Prosigns will be displayed on the screen

in the mode in which they are entered. In some parts of the

program the ASCII representation are used for display purposes

only.

Except where standard characters have been defined, the ASCII

characters used to represent prosigns are chosen to provide at

least some association with or suggestion of the double-letter

representation of the prosigns. '$' suggests the 'S' in '<SK>',

and '!' the staccato DIT DIT DIT of the 'S' in '<AS>'. '#'

suggests the run-together combination of 'K' and 'N' in '<KN>'.

b. International Characters

In addition to support of the standard U.S. character set

for all operations, Super Morse also supports the following

additional international characters.

ASCII     Identification

����      �������������

129       {Double dotted "u"}

130       {Forward accented "e"}

143       {Dotted "a"}

146       {Double dotted "a"}

157       {Double dotted "o"}

Only the U.S. character set is supported for internally

generated exercises.  It is up to you to set up your computer to

input and display these characters as provided in the DOS manual.

2. Code Speed

a. Speed Standards

The PARIS standard is used for all but the Build/Groups

functions where the CODEX method is used.  As a consequence, that

function will seem to be faster than the other functions, but do

not be alarmed.  That is the way it should be!  You can toggle

the code speed standard at any time by pressing <ALT-S>

b. Character, Send and Word Speeds

Super Morse provides control over three aspect of speeds.

The "character" speed is the speed at which individual elements

comprising the Morse characters (DITS, DAHS, and inter-element

spacing) are sent.  Normally, the spacing between characters is

sent at the same speed as the characters elements.  However, when

the Farnsworth method is used, the spacing between characters is

sent at a slower speed than the characters themselves.

The Farnsworth method is implemented in Super Morse by

permitting you to control the "send" speed, which takes into

consideration the present character speed and increases the

inter-character spacing to provide the indicated overall send

speed.

Super Morse provides a further refinement in permitting you

to also control the "word speed", which controls the spacing

between words or character groups.  This is, of course, available

only when words or character groups are being sent.  It is

especially useful for advanced users who are learning to

recognize the sounds of entire words. Beginner users are not

likely to find any need for this setting.

III. Getting Started with Super Morse

A. Starting and Quitting Super Morse

1. Starting Super Morse

To run Super Morse enter the following command at the DOS

prompt:

SM <ENTER>

2. Title Screen

The first screen to appear after Super Morse is started is

the title screen.  Super Morse will indicate first near the

bottom of the screen that it is loading data files and then it

will ask you to enter a user number.

3. User Number

After all files are loaded Super Morse asks you to enter

your user number at the bottom center of the title screen.  The

user number is a unique number which you should select in the

range 0 to 9. Super Morse supports up to 10 users, and each user

is assigned a separate user file used to record statistical

information.  For example, user 1 is assigned the file SM1.USR,

which is the default.  The contents of the currently used file

can be accessed from the Setup menu with the Stats command.

In order for your statistics to be correct, you should

choose and use a number that is not used by anyone else. If you

are the only user, you are assigned the default user no. 1, and

you can simply press <ENTER> to automatically enter that number.

You will then be taken to the main screen.

4. Quitting Super Morse

Before continuing you should know how to exit Super Morse.

You may do so by selecting the QUIT option from the main menu, by

pressing the ESC key, or by pressing the right mouse button.  If

program parameters have been changed, you are given an

opportunity to save the new ones.

The Morse code exit message can be cut short by pressing ESC

(although you would be deprived of the unsurpassed pleasure of

hearing WD5CID's call sign yet again).

B. Super Morse Organization

1. Organizational Philosophy

Super Morse is organized in a series of menus based upon the

five phases of the Morse code learning process: (1) learning the

characters, (2) building speed, (3) enhancing skills, (4)

measuring progress, and (5) operating.  These five phases are

reflected in the commands available from the main menu.

2. Menus

a. Pull Down Menus

Super Morse uses a pull-down menu system with a bar type

main menu and submenus which "pull down" from the main menu.  In

some cases the submenus have their own submenus that normally

overlay the submenu. The pull-down submenus each comprise a

character followed by a selection name.  The main menu does not

use selection characters.

The current selection in all menus is indicated by a

highlight bar.  The highlight bar can be moved laterally in the

main menu using the left and right arrow keys or by aligning the

mouse pointer with the "<" and ">" symbols in the window upper

side border.  In the submenus the menu bar can be moved up and

down using the vertical arrow keys submenus, or by moving the

mouse pointer to the selection and pressing the left mouse

button.  The highlighted selection can be chosen by pressing

<RETURN> or by pressing the left mouse button.  If the mouse is

used to choose a selection that is already highlighted, the left

mouse button must be clicked twice.

Most menu's have a HELP selection which provides a brief

explanation of the other selections.

The last selection in each menu is normally QUIT, which

exits the menu and returns to a lower level menu.  The same

result can be attained by pressing <ESC> or by pressing the right

mouse button.

b. Line Menus

When Super Morse leaves the pull-down menus and begins to

use the main display area, it sometimes presents choices in the

form of line menus which are integrated into the text display.

Selection may be made using the right and left arrow keys, the

mouse, or the first letter of the selection.

3. The Super Morse Main Display

a. The Message Line (Top)

At all times while Super Morse is running, the top line of

the screen is used to identify the current function and the key

necessary to abort that function.  Most functions can be exited

at any prompt using the <ESC> key.

b. The Main Menu Line

The second line displays the main menu of Super Morse's

pull- down menu system, which provides the following selections:

����������������������������������������������������������Ŀ

�                                                          �

� Learn Build Enhance Measure Operate View Setup Help EXIT �

�                                                          �

������������������������������������������������������������

The current selection is indicated by a highlight bar, which

can be moved using the left and right arrow keys or by pointing

to the menu name with a mouse and double clicking.  The pulldown

menu for the menu name can be displayed by pressing the <RETURN>

key. A selection can also be made by pressing the first letter of

the menu name.

c. The Status Lines (Bottom)

Likewise, the bottom three lines display important program

parameters as follows:

���������������������������������������������������������������Ŀ

� Wt: 3.0  Std: Txt  Zoom: N  Mat: On Int: N  Jit: Y  Sou: Y    �

� Ch: ABCDEFGHIJKLMNOPQRSTUVWXYZ1234567890  Txt: Show ETim: 0:00�

� SSp: 5   Csp: 18   WSp: 5    Class: Novice     Tone: 730      �

�����������������������������������������������������������������

The upper status line displays from left to right the code

weighting ("Wt"), whether text or random code speed is used

("Std"), whether the zoom mode is active ("Zoom"), whether the

match input mode is active ("Mat"), whether interference is

active ("Int"), whether code jitter is active "Jit" and whether

the sound output is active ("Sou").

The middle status line displays the currently active

character set ("Ch"), whether code text will be displayed

("Txt"), and the elapsed time of the current session ("ETim").

The lower status line displays the code send speed ("SS") in

words per minute, the code character speed ("CS") in words per

minute, the code word speed ("WS") in words per minute, the

license class represented by the speed ("Class"), and the

oscillator tone ("Tone").

When the PC speaker is used for sound output, the tone is

displayed as the tone frequency in Hertz; but when the Sound

Blaster card is selected, the tone is shown as a musical note.

The meanings of character speed, send speed and word speed

are defined later in this document.

4. Super Morse Help

a. Help Screens

Help screens for each menu are available by choosing the

HELP selection.  Help for the control keys is available by

pressing HOME.

b. Quick Key Help

The definitions of the function keys and ALT keys can be

displayed at any menu by pressing the HOME key.

c. Displaying the DOC File

This DOC file may be displayed with the Review/DOC command

from the main menu.

C. Super Morse Audiovisual Cues

1. Code Output

Super Morse outputs code sounds either via the PC speaker or

an optional Sound Blaster card.  Super Morse also provides

alternative output with a flashing light square in the upper

right hand corner of the screen.  The flashing light output

permits use of the program in environments where the sound output

would be inappropriate.

2. Audio Confirmation and Alerts

If audio confirmation is enabled (Setup/Misc), Super Morse

makes a "diddle" sound when a selection is made.  When an

erroneous entry is made, Super Morse will make a "buzz" sound.

3. Repeating Characters and Words

In features which send a character or a word and prompt for

a keyboard response, the character or word can usually be

repeated by pressing the <TAB> key.  In some word features, the

word can also be repeated by pressing the <TAB> key.

4. Visual Cues

In some features Super Morse provides visual cues by

printing DITs and DAHs when a character is sounded.  Unless DIT

is the last element in a character, it is usually spelled DI

since this is a more accurate representation of its sound under

those circumstances.  This feature can be disabled in the Setup

menu.

D. Program Control

1. The <ESC> Key

At any time code is being sent, the current operation can be

aborted by pressing the <ESC> key.

2. Pause

During the middle of a code exercise, you may wish to pause

for some reason, e.g., answer the phone.  You may do so by

pressing the <ALT-P> key combination.  Note that you will lose a

character if you are in the midst of it when you invoke the

pause.  Note also that pause will affect timed functions.

Pressing any key continues the previous function.

3. Screen Control

a. Text Display Mode

The ALT-X key combination permits you to control text

display in one of two modes: text and hide.  Pressing ALT-X

toggles between the modes, and the current mode is displayed on

the status line.

The text mode displays the text, while the Hide mode does

the opposite. The hide feature remembers up to 1792 characters

sent when the text is hidden.  When the text is unhidden, these

characters are printed on the screen as they would have appeared

if the had not been hidden.

With some functions the text will be hidden regardless of

the setting of the display mode.

b. Clearing the Screen

The screen may be cleared any time the screen is active

(i.e., no menu is showing) by pressing the <ALT-C> key

combination;

4. Timing a Super Morse Session

Super Morse includes a session timer which is displayed on

the status line as "ETim".  The session timer is automatically

zeroed when Super Morse is invoked, but it may be re-zeroed at

any time by pressing the <ALT-T> key combination.

E. User Input Modes

Super Morse uses several types of receiving/user input

modes.  In some cases, such as in the Learn mode, the user is

prompted to enter an input from the keyboard.

The match mode permits the user to copy the text on the

keyboard, and the accuracy is checked and displayed. Correct

characters are shown in normal print, while incorrect ones are

shown in reverse print.  If there is no input, then nothing is

printed on the screen.  The match mode is automatically selected

in some functions such as the Enhance/Typewriter.  In others,

such as Build/Groups, the user must invoke the match mode using

the ALT-M key until the "Mat" display on the status line shows

"On".  Presently the match mode is available only in the

Build/Groups function.

Some functions, such as the Build functions, can be used

without any user input.  In that case the user can copy either on

paper or in his head.

F. The Session Buffer

Super Morse stores all the characters sounded in a session

and makes them available for later use using the Review selection

from the main menu bar.

The session buffer is stored in a file named SESSION.SM.

This file may also be replayed using the Build/Disk function. The

session buffer is limited to 4000 characters, which is about two

full screens of text on an 80x25 display. Anything in excess of

this is lost.

G. Reviewing Program Information

1. The Review Menu

The Review menu, which may be chosen from the main pull-down

menu, permits you to review and examine various types of program

information as follows:

�������������Ŀ

�             �

� A - Session �

� B - Keys    �

� C - Morse   �

� D - DOC     �

� E - Stats   �

� F - HELP    �

� G - EXIT    �

�             �

���������������

2. Reviewing the Current Session

You can display all the characters sounded in the current

Super Morse session using the Session selection.  The Session

selection displays the following menu:

������������Ŀ

�            �

�  A - View  �

�  B - Play  �

�  C - Erase �

�  D - Print �

�  E - File  �

�  F - HELP  �

�  G - QUIT  �

�            �

��������������

The session buffer may be displayed with the View command

and replayed with the Play command.  It can be printed with the

Print command and filed to diskette with the File command.  When

finished with the session, you can erase it with the Erase

command.

3. Viewing the Quick Keys

Super Morse permits you to use function and ALT key

combinations to set and control various program parameters and

functions.  The functions of these keys can be viewed using the

Keys selection from the Review menu.  The quick keys can also be

viewed by pressing the HOME key at any prompt.

The quick keys are discussed in detail in the Settings and

Options section of this document.

4. Viewing the Morse Alphabet

You may wish review the definitions of the various Morse

characters.  You can do this with the Morse selection in the

Review menu.

5. Viewing the DOC File

You can view this DOC file without leaving Super Morse with

the DOC selection in the Review menu.

6. Displaying User Statistics

Several of the features in the Learn and Measure phases of

Super Morse record information about your progress.

In character based lessons of the Learn section, Super Morse

keeps track of your number of consecutive correct answers.  When

Stats is selected from the Review menu, a color graph showing the

number of correct answers for each character for the last 10

times the character has been sent to you.  Also displayed at the

bottom of the graph is a percentage goal figure.  The goal

referred to is to achieve 10 out of 10 correct for each letter,

for a total of 450 net correct answers.  A perfect score is 100,

but you have to be VERY good to achieve it.  Your score starts at

0 and slowly builds toward 100 as you get better.  If you can

come even close to 100, you KNOW the Morse characters!

In the Enhance section your maximum boost speed and keyboard

copy speed are recorded and displayed in the user statistics.

Also the number of Super Morse sessions and total time spent

using Super Morse are shown.

The user statistics can be cleared with the <E>rase User

command from the Stats display.

IV. Settings and Options

A. Settings in General

Super Morse parameters can be set using the Setup chosen

from the main menu bar, quick keys, or external files.

B. Quick Settings

1. The Quick Keys

Many parameters of Super Morse can be set from the main

screen.

a. The Function Keys

The function keys are active at any input prompt, and while

code is being sent.  They are defined as follows:

F1  - Decrease SEND Speed   F2  - Increase SEND Speed

F3  - Decrease CHAR Speed   F4  - Increase CHAR Speed

F5  - Decrease WORD Speed   F6  - Increase WORD Speed

F7  - Lower Speaker Tone    F8  - Raise Speaker Tone

F9  - Decrease All Speeds   F10 - Increase All Speeds

The operations performed by these keys are discussed in more

detail in several of the following sections.

b. The <ALT> and Other Control Keys

Super Morse parameters may also be set using <ALT> key

combinations and other keys.  Some are active while code is being

sent and others only when an input is required.

The following <ALT> combinations and other keys are

available:

<ALT-C>  - Clear the screen (not active during menu

presentation)

<ALT-E>  - Set to Extra class Farnsworth speed (20,26,20)

<ALT-F>  - Set to User's favorite speed

<ALT-G>  - Set to General class Farnsworth speed (13,18,13)

<ALT-H>  - Context sensitive help

<ALT-I>  - Set Interference type

<ALT-J>  - Toggle code speed jitter

<ALT-K>  - Toggle speaker

<ALT-N>  - Set to Novice class speed (5,18,5)

<ALT-P>  - Pause

<ALT-S>  - Toggle between code speed standards

<ALT-T>  - Reset elapsed time clock

<ALT-V>  - Save new settings

<ALT-W>  - Set code weight

<ALT-X>  - Set text display mode

<ALT-Z>  - Toggle "Zoom" mode

<ALT-1>  - Set class 1 speed

<ALT-2>  - Set class 2 speed

<ALT-3>  - Set class 3 speed

<ALT-=>  - Set cs,ss and ws to the same value

<HOME>   - Quick key help

The 18 WPM character speeds (middle number) indicated in <ALT-G>

and <ALT-N> are the initial value and may be changed in the Setup

menu chosen from the main menu bar.

2. Setting Code Speed

a. Incremental Settings

The code speed can be set at any input prompt, and while

code is being sent, using the function keys as follows:

F1 - Decrease SEND Speed      F2 - Increase SEND Speed

F3 - Decrease CHAR Speed      F4 - Increase CHAR Speed

F5 - Decrease WORD Speed      F6 - Increase WORD Speed

F9 - Decrease SEND, CHAR,     F10- Increase SEND, CHAR,

and WORD Speed                and WORD Speed

You can set the speed at which a character is sent,

separately from the overall send speed (the Farnsworth method).

For example, you can set the send speed at a slow speed but the

character speed  using shorter DITS and DAHS which are equivalent

to a much faster send speed.  This is a highly recommended way to

increase your speed since you will know the sound of the

characters at the higher speed from the beginning.  Super Morse

is initialized for the Farnsworth method to a character speed of

18 WPM for send speeds below 20 WPM and at 25 WPM for speeds

above 20 WPM, but these values can be changed in the Setup menu.

Note that the word speed can be set separately from send and

character speed.  This helps you learn the sounds of whole words

sent at high speeds but spaced at a slower speed.

The minimum code speed is permitted by Super Morse is 3 WPM,

and the maximum is 99 WPM (good luck with that setting!). The

send speed and character speeds are coupled so that the send

speed will never exceed the character speed.  Also in the speed

test mode, the word speed is automatically set to the send speed

so that the test is valid.  In addition, upon entry into the

Learn phase, the send and word speeds may be set to a special

learn speed if enabled in the Setup menu.  If selected, the learn

speeds are set to the values used during the last learn session.

b. Direct Speed Settings

You can quickly and directly set the code speed to the

proper speeds for the various classes of amateur radio licenses

using the <ALT> keys as follows:

<ALT-N> - Novice class (5 WPM)

<ALT-G> - General class (13 WPM)

<ALT-E> - Extra class  (20 WPM)

When these quick keys are used, the "send" and "word" speeds are

set to the license speed standards as indicated above.  However,

the "character" speed is set to the user selected Farnsworth

speed, which is set in the Setup/Code menu.

You can also set your favorite speed with the <ALT-F> key

combination.  The favorite speed is changed in the Setup menu.

For non-U.S. users who wish to redefine the names and speeds

of the license classes as defined earlier in this document, the

following alternative ALT keys are provided:

<ALT-1> - Lowest class license

<ALT-2> - Mid-class license

<ALT-3> - Highest class license

These keys work exactly like <ALT-N>, <ALT-G> and <ALT-E>,

and U.S. users can use them as an alternative if desired.

c. Changing Code Characteristics

i. Code Weighting

The code weighting can be changed using the <ALT-W> command.

Code weighting refers to the ratio between the lengths of the DAH

and the DIT.  Normally a DAH is 3 times as long as a DAH, but in

some instances it is useful to change the weighting to some other

ratio.  Super Morse permits the weighting to be set in the range

2-6 in increments of 0.2. At present, changing the weight from

that used for speed calibration causes a very slight change in

speed.  Thus, you should always calibrate speed at a weighting of

3, and understand that speeds at other weightings may be off by 1

WPM or so.

ii. Adding Speed Jitter

To make code sound more realistic, speed jitter can be added

(or removed) by pressing <ALT-J>.  This slightly varies the

length of the DITS and DAHS in the way that human operators do.

d. Zoom Mode

The File, Groups and Words functions in the Build section may

be exercised using the zoom mode.  In this mode a specified

number of words or groups is sent at a specified starting speed,

and then the send speed is bumped up by the specified WPM

increment.  This process is repeated until a specified end speed

is reached.

The zoom mode is enabled and disabled by pressing the <ALT-

Z> key combination during any menu display.  When the zoom mode

is enabled, the following screen is displayed for parameter

selection:

�����������������������Ŀ

�                       �

� 1 - Start Speed :  3  �

� 2 - End Speed   : 14  �

� 3 - Char Speed  : 18  �

� 4 - Increment   : 1.0 �

� 5 - Num Words   : 10  �

�                       �

�������������������������

Each parameter can be changed by selecting the indicated

parameter number and entering the information when prompted.

The increment parameter specifies the number of WPM the speed is

bumped after the specified number of words or groups is sent.

Any one of four preset increment values may be selected: 0.1,

0.2, 0.5, and 1.0 WPM.

3. Controlling Audio

a. The Oscillator Tone

i. Changing Tone

When the PC speaker is in use, the oscillator tone can be

set to suit your tastes with the function keys as follows:

F7 - Decrease Speaker Tone    F8 - Increase Speaker Tone

ii. PC Speaker

When the PC speaker is used for sound output, the tone is

selected by frequency. The minimum tone frequency is 400 Hz., and

the maximum is 2000 Hz. Unfortunately the volume of the PC

speaker cannot be changed. Some tones are reproduced by the PC

speaker better than others, however, and they seem louder.  The

default tone is 730 Hz. to approximate the 725 Hz. tone believed

to be used for many VEC tests.

iii. Sound Blaster

When the Sound Blaster card is selected as the sound output

device, the tone is selected as notes in the musical scale.

b. Adding Noise

To provide practice in copying code under difficult

conditions, your can add background noise (known as "QRM" to

hams) to the code by pressing the <ALT-I> key combination. Level

0 is no noise, and levels 1, 2, and are noise levels of different

types.  Background noise is available only in the Build and

Enhance phases.

C. Super Morse Setup

1. The Setup Menu

The Setup menu provides the following choices:

�������������Ŀ

�             �

� A - Code    �

� B - Chars   �

� C - Ports   �

� D - Colors  �

� E - Misc    �

� F - Initial �

� G - HELP    �

� H - QUIT    �

�             �

���������������

The "Code" selection sets code speed and other parameters.

"Chars" is used to define the currently active character set.

The COM port parameters for keying I/O interface with external

devices are set with the "Ports" selection.  Screen colors are

set with the "Colors" selection, and other miscellaneous

parameters are set with the "Misc" selection.

2. Setting the Code Parameters

a. The Code Menu

Following is the code menu with example settings shown.

������������������������������������������������������������Ŀ

�                                                            �

� A - Timing Method  : Timer    G - Farnsworth Lo  : 18      �

� B - Timing Factor  : 1083     G - Farnsworth Hi  : 25      �

� C - Calibration    : XXXX     I - Speed Lock     : Off     �

� D - Tone Split     : On       I - Standard Lock  : Diff    �

� E - Set Fav. Speed : 5-14-5   K - QUIT                     �

� F - Learn Speed    : On                                    �

�                                                            �

��������������������������������������������������������������

b. Calibrating the Code Speed

i. Selecting the Calibration Method

Super Morse uses two different calibration methods that may

be used depending upon your equipment requirements.  The first is

the Timer method, which directly uses the PC's clock chip.  This

method provides automatic calibration each time Super Morse is

run and produces more accurate times and thus better sounding

code.  Unfortunately, not all PC's support the clock chip in

exactly the same way, and therefore that method will not work on

all PC's.

In addition, the Timer method seems to be incompatible with

Microsoft Windows.  Finally, the Timer method will not work with

PC emulators for other machines such as the Apple McIntosh.

Due to these incompatibilities, Super Morse versions 4.01

and later reintroduce the software Loop timing method that was

used in versions up to 3.10.  This method should be compatible

with all types of PC equipment, including McIntoshes running the

PC emulator.  It is also compatible with Windows provided that a

special PIF file described elsewhere in this document is used.

The disadvantage of the Loop method is that it requires manual

calibration and must be recalibrated each time a memory resident

program (including Windows) is added or removed.

When switching between the Timer and Loop timing methods,

you will find that the timing factor is radically different.  In

some cases the calibration test may be so slow that you will have

to abort it and guess at a an appropriate timing factor.  As a

rough guide, a 386SX computer running at 16 MHz. requires a Timer

timing factor of about 1100 while the Loop factor is roughly

8000.  The timing factor will not change much with computer

speed, but the Loop factor will increase approximately

proportionately with computer speed.

ii. Manual Calibration

While the Timer timing method automatically calibrates each

time Super Morse is run, you should adjust the timing factor the

first time you run Super Morse using the Timing Factor selection

from the Setup/Code menu.  Once Super Morse is adjusted to your

computer when using the Timer method, you should not need to

change it.

If you use the Timer method, calibration must be performed

manually using the Calibrate selection from the Setup/Code menu.

Once calibrated, however, you will not need to change it unless

you add or delete a memory resident program.

The accuracy of the current calibration may be determined

using the Calibration selection in the Setup/Code menu.  Super

Morse tests calibration by sending the standard word PARIS the

same number of times as the send speed shown on the status line.

The send, character and word speeds can be set to any desired

speed. If Super Morse is correctly calibrated, the test should be

completed in exactly one minute.  Super Morse times the test and

displays the result along with a recommendation for a revised

Timing Factor.  If the recommended Timing Factor is different

from the current setting by more, than say 5,  then the

recommended new timing factor should be entered using the Timing

Factor selection in the Setup/Code menu.  The calibration should

be repeated until the recommended and the current Timing Factor

are very close.

So that you do not spend an inordinate amount of time

seeking the perfect calibration, let's put the problem in

perspective.  For  perfect timing the total time required for the

Calibration test is 60 seconds.  However, you will not likely

attain that number, and you should seek an elapsed time of

slightly under 60 seconds so that any errors will be on the too-

fast side of the indicated speed.  If the speed test requires 58

seconds instead of 60, the code speed will be fast by only about

3 percent.  This is only about 0.6 WPM at 20 WPM, and less at

slower speeds.  If you can attain a total time of 59 to 60

seconds, you will be very close to the actual speed.

iii. Setting Speed Standard

Super Morse supports both the PARIS speed standard for plain

text and the CODEX standard for random characters. The currently

active standard is shown on the command line. The text (PARIS)

standard is indicated by the letter "T" (standing for "text"),

and the random character standard (CODEX) by the letter "R"

(standing for "random"). You can override the automatic standard

selection using the <ALT-S> command at any menu to toggle between

the two standards.

c. Splitting the DIT and DAH Tones

The tones for sending DITs and DAHs are normally the same,

but as an additional aid in distinguishing between the two, Super

Morse permits you to split the two tones such that the DAH is 10

Hz. lower than the DIT.  This is accomplished using the Tone

Split selection in the Setup/Code menu.

The tone split feature is not available when the Sound

Blaster is used for sound output.

d. Code Speed Settings

i. Setting User's Favorite Speed

You can directly set the current speed to a speed that you

commonly use by pressing the <ALT-F> key combination.  The speed

used by the <ALT-F> command is set to the current speed shown on

the status line using the Set Fav. Speed selection in the

Setup/Code menu.  Thus, to set a favorite speed, set the speeds

on the status line using the function keys and then press <ALT-

F>. The speeds shown in the menu are in the same order as on the

status line.

ii. Setting Farnsworth Speed

The character speed for the Farnsworth method can be set for

both low and a high speed ranges.  The Farnsworth speed for send

speeds below 20 WPM is set in the range of 13-30 WPM with the

"Farnsworth Lo" selection; and for speeds above 20 WPM in the

range 20-30 WPM with the "Farnsworth Hi" selection. The

Farnsworth speed is also used in the quick speed settings, <ALT-

N>, <ALT-G>, and <ALT-E> (also in <ALT-1>, <ALT-2> and <ALT-3>).

Test tapes prepared by the ARRL and widely used by Volunteer

Examination Coordinators (VEC's) send at a character speed of 18

WPM in the low range. It is also the authors understanding that

26 WPM is commonly used for the Extra class code speed.

iii. Setting a Separate Learn Speed

Using the Learn Speed selection, you can determine whether

Super Morse switches to special learn speeds when the Learn phase

is entered or whether the speeds remain the same a for other

phases.

iv.  Speed Lock

When using Super Morse for classroom teaching, it may be

helpful to lock the speed settings to prevent the student from

changing them to an easier speed.  This setting permits the speed

change keys to be locked and unlocked selectively.

v.   Standard Lock

Normally Super Morse toggles between the PARIS (text) and

CODEX (random) speed standards as the context dictates.  However,

some users are distracted by the change, and consequently I have

inserted a lock to maintain the PARIS standard regardless of the

type of characters being sent.  In addition the standard lock

also disables the <ALT-S> standard toggle.  I suggest that you

not use this feature since it will help you on the test to have

studied random characters at the seemingly "higher" random

character speed (the CODEX standard).

3. Configuring the Character Set

The set of characters that you desire to receive may be set

by choosing Chars in the Setup menu. This permits you to alter

the currently enabled character set using the following screen:

��������������������������������������������������Ŀ

�                                                  �

�                                                  �

� ABCDEFGHIJKLMNOPQRSTUVWXYZ 0123456789 .,?/- !@#$ �

� xxxxxxxxxxxxxxxxxxxxxxxxxx xxxxxxxxxx ----- ---- �

�                                                  �

�                                                  �

� A - Letters      G - Group 2       M - Default   �

� B - Numbers      H - Group 3       N - Type      �

� C - Punctuation  I - Group 4       O - All       �

� D - Prosigns     J - Group 5       P - QUIT      �

� E - National     K - Group 6                     �

� F - Group 1      L - Group 7                     �

�                                                  �

����������������������������������������������������

The symbol directly below the character indicates whether

the character is enabled, the "x" indicating that it is enabled,

and the "-" that it is disabled. Note that the prosigns are

represented by their single character ASCII equivalent character.

The default character set has the letters and numbers enabled and

the punctuation and prosigns disabled.  The status of characters

can be altered in groups or individually. The letters, numbers,

punctuation, prosign, and national groups can be enabled or

disabled by choosing the correspondingly named selection. The

National command is active only when the special Nordic character

set has been enabled.

The currently active groups 1-7 can be similarly enabled or

disabled.  Note that the alternate character groups file

(GROUPS.2) has only six groups, and the "Group 7" selection will

thus not be active if that file is used. To enable the default

character set, choose the "Default" selection. To change the

status of all characters, choose the "All" selection.

To change the status of individual characters, select "Type"

and then press the desired keys.  When you are finished entering

individual characters, press <ESC> to exit back to the character

menu.

The current status of the character set is saved at your

option when Super Morse is exited or when <ALT-V> is pressed.

4. Configuring the COM Ports

a. The Ports Menu

Super Morse uses two different serial ports, a "keying port"

to interface with a keyer or other device and a "telephone port"

to interface with a telephone modem.  The setting for these ports

are determined using the Ports menu shown as follows with example

settings:

������������������������������������������������������Ŀ

�                                                      �

� A - Phone Port    : COM1    H - RS-232 Input   : DSR �

� B - Baud Rate     : 1200    I - Input Sense    : +   �

� C - No. Bits      : 8       J - Output Keying  : Off �

� D - Parity        : N       K - RS-232 Output  : DTR �

� E - Dial Type     : Tone    L - Output Sense   : +   �

� F - Keying Port   : COM2    M - QUIT                 �

� G - Input Keying  : ALT                              �

�                                                      �

��������������������������������������������������������

b. Configuring the Telephone Port

i. Telephone Port Selection

Super Morse makes use of the PC serial communications ports

for the On Line Practice feature in the Operate phase. You can

set either COM1 or COM2 using the Phone Port selection. COM 3 and

COM 4 are not available for this function.

ii. Setting Communications Parameters

The baud rate for the telephone port can be set at either

300, 1200, or 2400 baud with the Baud Rate selection, the number

of bits can be set either at 7 or 8 using the No. Bits selection,

and the parity can be set at either even, odd, or none using the

Parity selection.

c. Configuring the Keying Port

i. Keying Port Selection

Super Morse provides auxiliary CW signal input and output

using a serial port.  Either COM1, COM2, COM3, or COM4  can be

selected using the Keying Port selection.

ii. Selecting Input Keying Source

The Build phase provides the ability to use a PC key as a

manual Morse code sender comprising either a key on the computer

keyboard or an external telegraph key. The ALT key is the

default, but since memory resident programs often use the ALT key

as a hot key the Super Morse Setup menu permits your to select

either the ALT, CTRL, Left SHIFT, or Right SHIFT for the manual

key.  The EXTERNAL selection permits the telegraph key, an

electronic keyer, or a radio receiver to be used.  The radio

receiver requires a special interface and is normally used with

the <R>adio selection in the <O>perate phase.  In addition, the

left button on a Microsoft compatible mouse can also be used as a

manual key.

iii. RS-232 Input

Either the DSR or CTS pin (6 and 5, respectively) can be

used to receive input keying.  Pin 7 is ground. This can be

selected using the RS-232 Input selection, and the receive sense

can be set at either positive (+) or negative (-) using the input

sense selection.

iv. RS-232 Output

Either the DTR or RTS pin (pins 20 and 4, respectively) can

be used to key an external oscillator or a transmitter.  Pin 7 is

ground. This output capability can be toggled on and off using

the Output Keying selection, and the desired pin can be selected

using the RS-232 Output selection.  The output sense can be set

to either positive or negative using the Output Sense selection.

5. Setting Screen Colors

When the Colors command is selected from the Setup menu, a

color set menu is displayed as follows:

���������������������������������������������������������������Ŀ

�                                                               �

� A - Main Background G - Bar Foreground    M - Info Background �

� B - Main Foreground H - Status Background N - Info Foreground �

� C - Main Highlight  I - Status Foreground O - Blue/Magenta Set�

� D - Menu Background J - Status Highlight  P - Green/Blue Set  �

� E - Menu Foreground K - Line Background   Q - Black/White Set �

� F - Bar Background  L - Line Foreground   R - Quit            �

�                                                               �

�����������������������������������������������������������������

The colors of the screen, the windows, and the highlighting

can be set to suit your taste using the <A>-<O> selections.

Predefined color combinations may be selected with the <O>-<Q>

selections. The current color for each screen area is shown in

the menu (color display only).

6. Miscellaneous Settings

a. The Misc Menu

�����������������������������������������������������������Ŀ

�                                                           �

� A - National Chars : English  F - Show DIT/DAH   : On     �

� B - Video Adapter  : CGA      G - Tie Speeds     : On     �

� C - Boost Factor   : 90%      H - Clear Screen   : On     �

� D - Prompt Sound   : On       I - Sound Device   : Speaker�

� E - Jump Spaces    : Yes      J - QUIT                    �

�                                                           �

�������������������������������������������������������������

b. Controlling Prompt Sound

The prompt sounds ("beep", "diddle", "click")can be toggled

on and off using the Prompt Sound selection from the Setup/Misc

menu.

c. Selecting the Video Type

At program invocation Super Morse automatically determines

the video type used in your computer. However, this can be

changed if necessary (e.g., you have two video cards installed)

with the Video Adapter selection.  Screen colors will be

automatically set for the selected video card, and the new video

setting is saved in the configuration file. The three choices

available are MDA (monochrome), CGA (color graphics adapter), and

HGC (Hercules).

d. Jump Spaces in Disk Files

The Jump Spaces selection permits multiple spaces in a disk

file to be skipped.

e. Setting Boost Goal.

The percentage of correct answers necessary to increase the

speed in the Boost feature can be adjusted using the Boost Factor

selection.

f. DIT and DAH Display

The Learn phase uses a screen display of DIT and DAH to

reinforce the audio.  This feature can be turned on and off using

the Show DIT/DAH selection.

g. Tying Code Speeds

Super Morse permits you to tie the send, character and word

speeds above 20 words per minute for easy one-key settings.

However, this can be disabled using the Tie Speeds selection.

h. Clear Screen

Setting this choice to "On" causes the screen to be cleared

when exiting a function.

i. Selecting the Sound Output Device

Super Morse permits selection of either the PC speaker or

the Sound Blaster card for audio output, or a flashing light for

a silent code output, using the Sound Device selection. in the

Setup/Misc menu.  The Sound Blaster card provides a much higher

output level than the PC speaker and can easily be fed to an

external amplifier or recorder if desired.  In order to use the

Sound Blaster, an environment string must be added to the

AUTOEXEC file as desciribed above.

The flashing light option permits Super Morse to be used

where the audio output is inappropriate.

7. Initializing Super Morse Parameters

Super Morse parameters can be reset to their intial settings

with the Setup/Initial selection.

D. Saving Current Settings

1. Save Modes

The current values of the program settings (parameters) can

be saved with the <ALT-V> key combination.  If the settings have

not been saved during the current session, then your are given

the opportunity to save them when exiting Super Morse. Several

parameters (zoom status, interference status, jitter status and

qrm status) are not saved and must be reset during every session.

2. The Settings Files

Super Morse preserves the most recently used or set

parameters in a configuration file, SM.CFG, and one of several

user files, SMn.USR.

a. Configuration File

The most recently used values for parameters which are least

likely to change during each session are stored in the SM.CFG

file.  If this file is not found when Super Morse is first run,

it is created.

b. User Files

The most recently used values for parameters which are

subject to change during each session are stored in the SMn.USR

file.  Super Morse supports 10 users numbered 0-9 and their

respective files. The user statistics files are given a number

corresponding to the user number.  For example, user 1 is

assigned the file SM1.USR (the default).

E. Customization of External Files

1. General

In addition to internal modifications, Super Morse permits

customization by changing the contents of Super Morse's text

files.  Modifiable files all have the "SM" extension, such as

NAMES.SM. These files can be edited using a text editor or a word

processor. A simple, but very poor, text editor, EDLIN.COM, is

usually provided with your DOS utilities, but other much better

ones are available as Shareware.

If a word processor is used, the output must be ASCII text

only!  Most modern word processors output not only the text that

you enter in the document but also formatting information which

is not displayed on the screen. This invisible formatting

information will cause to Super Morse produce garbage. If you use

a word processor, be sure to check your user manual to determine

how to save text in plain ASCII format without the formatting

information.  If you use WORD PERFECT or DOS, use the "Text Out"

choice from the "Files" pull-down menu, or press <CTRL-F5>.

2. Data Files

Super Morse uses several files containing names, cities, and

other information used in QSO generation (simulated radio

contact), and other files for words used in word-based functions.

If you are not satisfied with the contents of these files, you

may change them to suit your tastes. Non-U.S. users will probably

want to change the contents of the states file to include the

provinces in their own country or other countries.

All data files require that only one item be placed on a

line starting in column 1 (not at the left margin of a word

processor file if the margin is shown!) and that no blank lines

be inserted in the middle of the file.

3. License Classes File

The only data file having a special format is the CLASSES.SM

file.  This file contains three lines in a special order. The

file distributed with Super Morse defines the United States

license classes and speeds as follows:

05 Novice

13 General

20 Advanced

The format is critical!  It must contain three lines and start in

column 1 (not the left margin of a word processor file!). Each

line begins with a speed in words per minute in columns 1 and 2,

followed by a space, followed in column 4 by the name of the

license class beginning at that speed, not exceeding 8 characters

in length.  If a speed is less than 10 WPM, a "0" must be placed

at the beginning.  If only two license classes are used in your

country, repeat the second class definition on the third line as

in the following generic example:

07 Junior

11 Senior

11 Senior

If you have only one class of license, place the same information

on all three lines.  If your country uses more than three classes

of licenses, please let me know, and I will expand this file.

4. Text Files

In some features Super Morse permits you to use a plain

ASCII text file as input.  An example in the Disk File command in

the Build phase.  These files should be created with a text

editor or word processor in the ASCII output mode as described

above.  The lines in these files can be any length and can start

in any column.

F. Interfacing External Equipment

The primary I/O for Super Morse code is via the computer

keyboard and the PC onboard speaker.  Secondary output is

provided by toggling either the DTR or RTS pin at the selected

RS-232 communications port.  Input is provided via either the DSR

or CTS pin at the same port.  You can select whether to expect

either a high (+) or a low (-) voltage in the key-down condition

and the opposite in the key-up condition. Output via DTR/RTS is

available whenever code is being sent, but input via DSR/CTS is

available only in the Manual key function of the Enhance phase

and in the Operate phase.

I/O via either COM1, COM2, COM3, or COM4 can be selected

from the Setup menu selected from the main menu bar. With a

proper interface DTR/RTS can be used to drive an external

oscillator for making high quality code tapes or to key a

transmitter. Likewise, DSR/CTS can be interfaced with a hand key,

electronic keyer, or a receiver.

Code output to the speaker and COM port can be selected from

the Setup menu. Code input can be selected from either one of

several keys on the computer keyboard or the COM port using the

Input Keying selection in the Setup/Ports menu.

A detailed explanation of how to use the DTR/RTS and DSR/CTS

signals for the aforementioned purposes is beyond the scope of

this manual and is left to your ingenuity.  However, limited

information is provided in Section XI. The author would like to

receive diagrams of tested circuits designed by users to take

advantage of COM port I/O.

V. The Learn Phase

A. The Learn Command

1. Introduction

To help you learn the Morse code, Super Morse provides a

Learn feature, which conducts a series of lessons to gradually

and logically introduce the Morse characters.  The Learn menu is

as follows:

�������������Ŀ

�             �

� A - DIT/DAH �

� B - Combos  �

� C - Steps   �

� D - Runs    �

� E - Groups  �

� F - Levels  �

� G - All     �

� H - User    �

� I - Flash   �

� J - Oddball �

� K - View    �

� L - HELP    �

� M - EXIT    �

�             �

���������������

2. The Super Morse Teaching Method

Unlike standard Morse code teaching methods, Super Morse

does not begin by introducing characters. Instead Super Morse

starts with sounds only -- characters are not taught or displayed

in the early lessons.  The idea is to emphasize that Morse code

is a sound language and to teach the fundamentals of code sounds

without the added complication of visual stimuli. Two sound-only

exercises are provided. First, the basic DIT and DAH sounds are

taught. Second, Super Morse puts the basic sounds together into

the four fundamental two-sound combinations.

Only after the basic sounds and sound combinations are

taught does Super Morse start identifying characters.  Before the

standard code groups are introduced, code "steps", which are

groups of two similarly sounding characters, are sent to

gradually introduce the characters in pairs. The idea is to teach

the student to distinguish similar sounding characters from the

beginning, such that those which do not sound so much alike will

fall into place by themselves.  Also the characters are slowly

learned in an indirect fashion in that the student is encouraged

to concentrate on the differences rather than the content of the

characters themselves. Only when the steps are completed are the

standard code groups

introduced.

The Learn selections are <A> through <G> are ordered to

slowly build up your knowledge of the characters in a very

logical way.It is recommended that the Learn subcommands be used

in the same order as the selections are presented.

3. Repeating Characters

In the <L>earn mode (except Flash) characters can be

repeated by pressing the <TAB> key at any prompt.

4. Feedback in the Learn Phase

To enhance the learning experience, Super Morse provides

feedback after each response in the Learn mode in the form of a

report of the number of answers correct, the number of answers

incorrect, a percent score, and the number of consecutive correct

answers for the current session. Also, if an answer is wrong, the

correct answer is displayed and a brief review is provided.  The

consecutive answers report is useful in determining when you

should go on to the next step in the Learn mode.

5. Character Speed in the Learn Phase

Super Morse permits you to use special speeds in the Learn

mode as determined by a selection in the Setup menu. This forces

characters to be learned at speeds which permit you to more

easily reach the General class speed.  Initially, the speeds are

set to (5,18,5), and these speeds are automatically selected if

the learn speed mode is selected. These values are automatically

replaced by the last values used in the learn mode.

B. Lesson <A> - DITS and DAHS, the Basic Code Sounds

Lesson <A> teaches the basic DIT (short) and DAH (long)

sounds.  After sending DITS and DAHS in various formats, you are

asked to identify randomly presented DITS and DAHS using the "1"

key for DITs and the "2" key for DAHs.

C. Lesson <B> - Sound Combinations

Since Morse characters are comprised of combinations of DITS

and DAHS, Lesson <B> is provided to introduce the four simplest

combinations: DIT DIT, DIT DAH, DAH DIT, and DAH DAH.

D. Lesson <C> - Character Steps

Lesson <C> combines the previously learned DITS and DAHS

into complete character sounds.  The characters are paired with

similar (or confusing opposite) sounds in a series of

increasingly harder steps, and you are asked to identify

randomly-selected characters from each step.  You should start

with step 1 and learn to distinguish the characters in each step

before continuing on to the next higher step. After all 69 steps

have been mastered, you should be ready to start with the group

and levels lessons.  Some users have found this tedious, but it

will pay dividends if you

persevere.

If you desire only to practice the characters in a step, you

can skip introduction of the step characters by selecting the

"Drill" option when the line menu appears.

E. Lesson <D> - Character Runs

The character steps idea is extended in Lesson <D>.  This

lesson forms groups of characters, each successive one of

which differs by only a DIT or a DAH from the previous

character.

F. Lesson <E> - Character Groups

Lesson <E> begins the traditional code groups as defined and

used by the ARRL and others.  This lesson lets you study each

character group separately.  The characters from the selected

group are presented randomly, and you are asked to identify them.

The groups used in Super Morse are those used in the ARRL

publication "Tune in the World with Ham Radio". If materials

having a different grouping are used, the groups can be changed

by altering the contents of the file named GROUPS.SM as described

earlier in this document. The initially installed groups are

included in the file GROUPS.1, and a second grouping used in an

older ARRL code booklet is included in GROUPS.2.  To use either

of these groups, just delete the current GROUPS.SM file and copy

either GROUPS.1 or GROUPS.2 to the filename GROUPS.SM.  Note that

only the letters and numbers can be changed, and no more than 8

user groups can be entered. Punctuation and prosigns are

automatically added as the last two groups.

If you desire only to practice the characters in a group,

you can skip introduction of the group characters by selecting

the "Drill" option.

G. Lesson <F> - Character Learning Levels

Lesson <F> permits you to study the various groups in Lesson

<E> in a progressive manner starting with the simple Level 1

sounds and adding characters with each level. Each level is

simply a combination of groups 1 as defined above through the

group whose number is the same as the selected level number.

After the new characters for each level are presented, you are

asked to identify randomly presented characters from that level.

H. Lesson <G> - All Characters

Lesson <G> permits you to sum up all you have learned in the

previous 6 lessons in that all letters, numbers, punctuation, and

prosigns are sent for identification in the same manner as the

preceding lessons.  Lesson <G> is adaptive in that the more

recently missed characters are sent more often than the others.

I. Flash Characters

The Flash selection simulates flash cards by randomly

displaying characters from the currently active character

group in large display format. No user keyboard response is

required in this feature.  To flash characters by groups, use

Setup  to change the currently active character set.

J. Oddball Character Drill

The Oddball character function presents a group of five

characters with one different from the others, and you are asked

to enter the differing character.  The two characters used in

this feature are always very similar or otherwise easily confused

and are randomly selected from the STEPS.SM file.

K. The User Group

The User selection permits you to practice a customized

group of characters. The currently active character set selected

with the Setup command in the main menu is randomly sent for

identification in the same manner as the preceding lessons.

L. Viewing the Character Groupings (Steps and Levels)

The character steps and groups used in the preceding

selections may be displayed using the View selection.

VI. The Build Phase

A. Building Code Speed

Once the character sounds have been mastered, speed can be

increased by receiving code in the Build phase. The Build

menu is as follows:

�������������Ŀ

�             �

� A - Groups  �

� B - Words   �

� C - Message �

� D - QSO     �

� E - File    �

� F - HELP    �

� G - EXIT    �

�             �

���������������

All Build features require that you copy using pencil and paper.

B. Receiving Character Groups

The Groups selection permits you to practice with several

different types of groups.  The choice is made from a line menu

which is presented when the Groups selection is made.

1. Random Groups

Random groups are 5 letter groups of characters randomly

selected from the current character set.  This is a classic way

of practicing code reception and is used in most code tapes.

2. Structured Groups

Structured groups are similar to the Random groups except

that the first and second randomly selected characters of a group

are repeated in reverse order as the fourth and fifth characters.

The third character is always a randomly selected number.  This

permits you to receive random code but have the ability to check

his answer without looking at the screen.  The number is sent

even though you may have deselected the numbers.

3. Variable Groups

Variable groups are also similar to Random groups except

that the length of the groups is variable from 1 to 10.  This

helps you become accustomed to copying normal words, which are of

differing lengths.

4. Group Sending Modes

When the type of group has been selected, you are asked to

select either sending continuously, sending a specified number,

or sending for a specified time.  If number or time are selected,

you are asked to enter the number of groups or time desired in

minutes (or decimal fractions thereof).  In the timed mode,

sending stops only after the last group has been completed, even

if the time expires in the middle of the group.

C. Receiving Complete Words

If you would rather work with real words rather than groups,

the Words function can be used.  A choice of either "regular"

words, "ham" words, call signs, or words from a user file is

given.

The regular and ham words are selected from files of fixed

names REGWORDS.SM and HAMWORDS.SM, but user words are taken from

any file whose name you enter when prompted.  You should prepare

the file as described earlier in this document. Each word (or

short phrase) should be placed on a single line starting in

column 1 and should be no longer than 20 characters. Up to 1000

words may be entered.

Call sign "words" are randomly generated and cannot be

configured by the user. Note that the call signs generated are

not limited to call signs actually issued by the FCC. At 15 WPM

and below each character is displayed as sent. Above 15 WPM the

entire word is sent before it is displayed.

As words are being sent, the current word can be repeated by

pressing the <TAB> key.

D. Receiving A Message Entered via Keyboard

With the message function you can have a helper enter text

messages via the keyboard, and then attempt to copy it.  The

maximum length of the message is 255 characters.  The last

message can be repeated by pressing the <TAB> key. This command

differs from the Enhance/Keyboard Send function in that nothing

is sent until RETURN is pressed and the length of message is

limited.

E. Receiving a Simulated Radio Transmission ("QSO")

You should use the QSO function if you would like to copy a

simulated radio contact between two stations (know to hams as a

"QSO").  Random text is sent containing randomly chosen call

signs, names, etc., providing a variety of QSO combinations. The

call signs are randomly selected from all the possible call

signs, which means that some unissued calls signs will be used.

At the prompt you can choose either a single QSO or a

continuous series of QSO's.  The latter is useful in preparing

practice tapes.

The names, antenna types, city names, and rig types can be

changed to reflect your desires by editing the files ANTENNAS.SM,

NAMES.SM, CITIES.SM, RIGS.SM, and JOBS.SM as described earlier in

this document. The maximum number and length of the entries are

as follows:

File             Max. No. Entries     Max. Length Each Entry

----             ----------------     ----------------------

ANTENNAS.SM            50                      15

CITIES.SM             200                      15

JOBS.SM               100                      20

NAMES.SM              200                      10

RIGS.SM               100                      15

STATES.SM             100                      15

If the QSO is sent in the text Hide mode, you are given the

opportunity to review the QSO before exiting.

F. Receiving Text in a Disk File

1. General

This command sends the contents of a disk text file in Morse

code.  At the prompt press RETURN to select the most recent file

used or enter the name of a new file (including the drive and

subdirectory) to be sent.  If you do not know the name of the

file you wish to receive, you may press <SPACE> to obtain a disk

directory.  At the prompt enter the file specification in the

form D:\SUBDIR\FILENAME.EXT, where "D:" is the drive designator,

"SUBDIR" is the subdirectory name if applicable.  If all files

are desired, press RETURN to select the default "*.*" in place of

FILENAME.EXT.

Disk files can be sent either normally or in reverse by

entering proper character when prompted by the line menu.

You can practice by having someone create a text file of

unknown content.  Another idea is to exchange disks with a friend

who is also studying the code and carry on a "disk QSO".

The disk file required for this command can be any ASCII

text file which you have created. Unprintable characters are

ignored. Multiple blanks may be skipped if desired in the Setup

menu. To familiarize yourself with the use of this command, you

can enter the name of the words file (WORDS.SM) or any of the

other data file having the SM extension, although this will not

provide very useful code practice.

2. Scripted Disk Files

Disk files can be "scripted" by placing a period (".") in

the first column of any line followed by a command character and

a space and text, if applicable. Scripting may be used for

example to prepare code tapes which have voice instructions

intermixed with the code.  With scripting Super Morse follows

exactly the same line spacings as are used in the script file.

Thus, if you want lines skipped before and after the message,

place blank lines in your script file.

Scripting permits the following commands to be inserted in a

file:

a. The Message Command

The "M" command line as follows causes the <text> to be printed

in highlight on the screen:

.M This a message to the user

This will print "* This is a message to the user" on the screen.

b. The Pause Command

The same effect as the message command, except with a pause

for you to press a key to continue, is provided by the "P"

command as follows:

.P Ask the user to do something

This will print "* Ask the user to do something" and instruct you

to press any key to continue.

c. Set Speeds Command

Speeds can be programmed into the script at the indicated

send speed ("<ss>"), character speed ("<cs>") and word speed

("<ws>") with the "S" command as follows:

.S <ss> <cs> <ws>

With all commands the period should appear in the first column,

the command should be a capital letter, and a space must be

placed between the command character and the text or other

information which follow it.

d. Speed Lock Commands

The set speed keys can be disabled with the .L command and

re-enabled with the .U command.  The .L and .U commands are not

followed by text.

G  Receiving in the Zoom Mode

The Groups and Words functions can be sent in the "zoom"

mode, which is enabled and disabled by pressing the <ALT-Z> key

combination during any menu display.  The zoom mode sends a

specified number of words at one speed and then bumps the speed

to a higher speed in a specified increment. The current status of

the zoom mode is shown on the bottom line of the screen.

When the zoom mode is active and either the Groups or Words

functions is invoked, you will be asked to enter a start speed,

an end speed, a speed increment, and the number of characters to

be sent at each speed.  After the specified number of characters

is sent, the code speed is increased by the specified increment

until the end speed is reached. Increments of 0.1, 0.2, 0.5, and

1.0 WPM are available.

VII.  The Enhance Phase

A. Skill Enhancement

The Enhance phase includes modes which are designed to "put

the icing on the cake" by putting your code knowledge to work in

advanced ways.

The Enhance Phase menu is as follows:

����������������Ŀ

�                �

� A - Boost      �

� B - Head       �

� C - Lag        �

� D - Q & A      �

� E - Typewriter �

� F - Word       �

� G - Keyboard   �

� H - Manual     �

� I - HELP       �

� J - EXIT       �

�                �

������������������

B. Sending Code

The send-related commands in the Enhance phase permit you

to practice sending Morse code either from the keyboard or

manually using a designated keyboard key or an external

device as a straight key.

1.  Sending from the Keyboard

This function permits you to enter individual keys to hear

how they sound. You can type ahead of the Super Morse sender by

80 characters maximum.  Unlike the Build/Enter command, you do

not have to press RETURN to start sending characters.

2.  Sending with a Manual Key

You may also wish to practice your "fist", i.e., your manual

sending ability, by using the a designated key or mouse button as

a telegraph key.  The default designated key is the ALT key, but

this can be changed using the <M>anual Key function in the

Setup/Ports menu to one of several keyboard keys, to an external

key, or to the left button of a Microsoft compatible mouse.  In

this function Super Morse decodes the character sent and displays

it on the screen. Super Morse is initialized to the send speed

displayed on the status line. However, Super Morse automatically

adjusts to your sending speed as it changes. Since this

adjustment takes place slowly, you may have to send several

characters before Super Morse determines your speed and displays

the correct character on the screen.  The dot/dash ratio is

constantly updated and displayed on the status line as the "Wt"

factor.

The external key is accessed via the DSR or RTS pin of the

designated port. A hand key can be interfaced by simply

connecting the key to pins 4 and 6 of the selected COM port.

C. Copying Behind

In order to learn to copy Morse code at high speeds, it is

necessary to remember several characters before the one presently

being sent.  Super Morse has several Enhance phase exercises to

help you develop this ability to "copy behind". In all the copy

features, the characters used are selected using the Setup

command in the main menu.

1.  Head Copy

The Head Copy function helps you learn to copy "in your

head" by developing your memory for a string of characters. In

this function a character sequence of increasing length is sent,

and you are asked to enter the sequence exactly after it has been

sounded.  A new sequence is started each time an incorrect answer

is entered.  The current sequence length and the best length for

the session are reported after each correct answer.  The maximum

sequence length is 50 characters (Good luck!).

2.  Lag Copy

The Lag copy function helps you learn to record a previous

character while remembering a later sent character. Instead of

entering the character that was most recently sent as in all

other functions of Super Morse, this function asks you to enter

the character sent a specified number of characters ago.  The

default lag is 1 character, but you may enter a lag of up to 10

characters at the prompt. This exercise is very hard at first,

and you may have to use pencil and paper to get the hang of it.

3.  Random Lag Copy

This function tests your ability to remember a sequence of 3

randomly sent characters.  After the group of 3 characters is

sent, you are asked to enter either the first, second, or third

character.

4. Word Copy

This function sends complete words from the REGWORDS.SM file

and asks you to enter them correctly.  A score is displayed on

the status line, and words may be repeated using the <TAB> key.

D. Copying on a Typewriter

The military services train their Morse code operators to

use a typewriter to record incoming messages since it is much

faster than copying by hand.  The Typewriter function permits you

to use the keyboard for "copying on the mill", as that method is

termed.  In this function a simple text checking algorithm is

used to determine copy accuracy.  Characters which are judged

correct are shown in normal display, and incorrect characters are

shown in reverse video.  A running tally of correct and incorrect

characters is also displayed.

The text checking algorithm uses only a single character to

determine context.  Consequently, it should work fine so long as

you copy each character when sent.  However, if you copy too far

behind and get "out of sync", it may have difficulty in

determining which character you are copying and consequently may

judge one or more characters to be incorrect until it catches up.

E. Boost Copy

Boost copy automatically boosts your speed to your maximum

copy speed.  The send speed starts at 3 WPM and automatically

adjusts upwardly in 0.1 WPM increments as long as you maintain a

copy rate specified in the Setup menu.  If the rate drops below

the specified level, the speed stays constant until the specified

level is again attained.  Each time you start boost copy, the

speed is bumped back by 0.5 WPM to give you a slight edge at the

beginning.

F. Questions and Answers

The Q&A feature sends questions stored in a user-created

file in code without screen presentation and asks you to enter

the answer on the keyboard.  This feature may be used to ask

trivia questions or other easily answered questions.

Files for this feature can have any name, but they must be

constructed in a special way.  The odd numbered lines contain

questions (no question mark required) and the even numbered lines

contain answers.  Since the emphasis is upon correctly copying

the questions and not upon the answers, the questions should be

easily answered and the answers should preferably use only one

word without punctuation. Number answers should be in number form

and not written out in text. A short sample file Q&A.SM is

included with Super Morse.

As each question is sent you should copy it on a piece of

paper and enter the answer when prompted.  The question may be

repeated by pressing the <TAB> key at the prompt. After the

answer is entered, the full text of the last question is shown on

the screen.

VIII.  The Measure Phase

A. The Measure Command

The Measure command provides several ways to test your Morse

code progress.  The Measure menu is as follows:

��������������Ŀ

�              �

� A - Auto     �

� B - File     �

� C - Check    �

� D - Display  �

� E - Print    �

� F - Generate �

� G - Keyboard �

� H - Solid    �

� I - HELP     �

� J - EXIT     �

�              �

����������������

B. Exam Sources

Several of the Measure functions use examinations based upon

simulated radio contacts, or QSO's.  The format and content of

these QSO's are determined either by an internal QSO generation

routine or an external template.  The internal routine is used

exclusively in the Auto exam function and is the default

selection in the Generate exam function.

The external template may be selected in the Generate exam

function.  The external template is contained in a disk file

named QSOn.SM where "n" is a number from 1 to 99.  The file

QSO1.SM is provided as an example, but you may created other

ASCII files yourself.

An exam QSO template file comprises any desired text built

around up to 20 pieces of randomly selected data.  When data is

desired in the QSO file, insert the identifying number of the

data type surrounded by curved brackets ("{" and "}").  The

identifying numbers are as follows:

1 - calling station          11 - countries worked

2 - responding station       12 - band 1

3 - rst report               13 - band 2

4 - name                     14 - job

5 - city                     15 - rig type

6 - state                    16 - antenna type

7 - age                      17 - antenna height

8 - years a ham              18 - power (watts)

9 - license class            19 - weather

10 - states worked            20 - temperature (Degrees F)

The following is an example of a line in a QSO template:

My name is {4}, and I live in {5},{6}.

Super Morse randomly generates a name and substitutes it for

{4} and a city and state and substitutes them for {5} and {6}.

An example of the result is as follows:

My name is Joe, and I live in Northfield, KS.

At least 10 pieces of data should be used in any exam file

to support the 10 questions that are asked in the exam.

C. Simulated VEC Exam

This group of commands permits you to take a simulated code

test much like the one you would take when sitting for the

license exam given by the FCC-approved Volunteer Examination

Coordinator (VEC).  All VEC exam-related functions are based upon

the simulated contact used for the <Q>SO command discussed above.

In the exam functions the student is asked to set the exam speed

either at novice, general class, extra class speed, or other

user-specified speed.  If one of the three license classes is

chosen, you are further given the opportunity to select either

regular or Farnsworth spacing.

1. Auto Exam

This is a fully integrated self-test for the student.  All

exam questions are displayed on the screen, and the exam is

automatically graded.

2. Exam from File

An exam may be prepared in advance and given using the Make

exam file and Run exam file commands. These commands are intended

primarily for use by an instructor to create and give a simulated

test to a student or students. The Make command generates an exam

QSO either internally or from an external template as selected

and stores it in a special file on disk. The Make command also

permits a new set of questions to be created for the existing

exam.

The Run command loads the exam previously created using the

Make command and sounds it at the <N>ovice, <G>eneral, or <E>xtra

exam speed which you have selected. You can also select n<O>ne,

and the speed will default to that shown on the status line.

3. Checking Exam Contents

The Check exam function tallies and reports the characters

used and not used in the current exam file.  This may be used to

determine whether an exam meets the requirements for an official

code test.

4. Displaying and Printing the Exam

You can review the text of the test QSO by displaying it on

the screen or by listing it on a line printer with the Display

and Print commands. Either command permits you to select either

the text of the exam, the related exam questions, or the exam

answers.

In the Print mode you are prompted to turn on the line

printer and press a key to begin.  If the printer is not on, an

error will occur, and the program will terminate.

D. Keyboard Receive Test

This command lets you test your approximate receive speed.

You are prompted to choose either random characters or the

contenst of a disk file. The computer sends a character and waits

for you to type in the correct key.  The accuracy of the copy is

displayed on the status line.  When this function is exited using

the ESC key, the computer prints out the approximate send and

receive speeds.

If the incorrect key is typed, the character is printed in

reverse video (color monitors only) and sent again.  The computer

also remembers wrong characters and sends them more often than

the other characters.

The character set used in the test can be configured using

the Setup command in the main menu as described elsewhere.

E. Solid Copy Test

This function tests your ability to copy random characters

without a mistake.  A sequence of characters of increasing length

is sent, and you are asked to type in the sequence exactly as

sent.  When a mistake is made, the test ends and you are given a

report of the number of consecutive characters copied correctly

and the number of minutes and seconds during which "solid copy"

occurred.

IX. The Operation Phase

A. The Operation Menu

The operation menu provides functions which permit you to

use your code knowledge to communication with another person.

The following choices are provided:

�����������Ŀ

�           �

� A - Modem �

� B - Radio �

� C - HELP  �

� D - EXIT  �

�           �

�������������

B. Two Way Practice via Telephone Modem

1.  The Modem Command

The Modem command provides a simple terminal program which

permits you to use a telephone modem connected to your computer

to call another similarly equipped user and carry on a real time

QSO in Morse code.  A modem using the Hayes AT command set is

assumed.

2.  General Description of the Modem Function

When Modem is selected, the computer's selected

communications port is initialized to the parameters stored in

the configuration file, the default values of which are COM1,

1200 baud, 8 bits, no parity, and 1 stop bit.  The modem is

initialized to automatically answer calls from another computer

on the first ring.  In order for a connection to be successfully

made, both computers must be set to the same baud rate and

communications parameters.

The Modem commands may be displayed using the usual <ALT-H>

specific help key.  All On Line commands are invoked using the

<CTRL> key in combination with another key.  For example, the

Morse mode is initiated by pressing the <CTRL-C> key combination.

Initially, the Modem function is in the "Terminal" mode,

which means that it will act just like a normal modem program and

will not sound Morse code.  The terminal mode should be used to

make a connection with the other computer, and then you should

switch to the "CW" mode by pressing <CTRL-C>.  You can switch

back to the terminal mode by pressing <CTRL-T>.

Initially Super Morse will sound both the received and sent

characters.  This can be selectively changed using the <CTRL-R

and CTRL-S> commands.  Super Morse also adds line feeds to all

carriage return characters.  If received lines are printed

double-spaced, this can be changed with the <CTRL-L> command.  If

you would like to see characters sent to the modem during the

time that Super Morse is not connected to another computer,  use

the <CTRL-E> command.

To dial another computer, use the <CTRL-D> command.  To

disconnect the phone, use the <CTRL-H> command.  In either mode

characters can be sent from the keyboard, or a disk file can be

sent using the <CTRL-F> command.  In addition to these commands,

all the Hayes "AT" commands can be used.

3.  Making a Connection with Another Computer

To connect with another computer you should prearrange the

call with the other operator.  Both copies of Super Morse should

be in the Modem function, and both should set Super Morse On Line

to the terminal mode with <CTRL-T>.  Both must set their

communications parameters, other than the communications port, to

the same values.  Decide who will initiate the call and who will

receive it.  Since both modems are already initialized to answer

a call on the first ring, it does not matter which computer is

chosen.

The call initiator presses <CTRL-D> and enters the telephone

number of the call receiver and presses <RETURN>.  He then

selects either tone (if he has a touch tone phone) or pulse

dialing (if he has a rotary dial phone) and presses <RETURN>. He

will hear the call receiver's telephone ring and the his modem

answer the phone, followed by a brief carrier tone. Both screens

should indicate "Connected to Remote SUPER MORSE."

Both users should then enter the CW mode by pressing <CTRL-

C> and proceed with the landline QSO.

4.  CONTROL Commands

Following is a summary of the CONTROL commands used in the

Modem function:

<CTRL-C>  - Activate CW (Morse) mode

<CTRL-D>  - Dial a number

<CTRL-E>  - Echo/No echo of characters sent to modem

<CTRL-F>  - Send a disk file

<CTRL-H>  - Terminate telephone connection (hang up)

<CTRL-L>  - Add/remove linefeeds from received text

<CTRL-M>  - Hang up modem

<CTRL-P>  - Set baud rate and other communications parameters

<CTRL-R>  - Enable/disable Morse mode for received characters

<CTRL-S>  - Enable/disable Morse mode for transmitted characters

<CTRL-T>  - Activate Terminal (non-Morse) mode

5.  Problems with Modem Command

Experienced modem users will recognize that some of the

above commands can be duplicated using the standard Hayes "AT"

commands.  If you have any difficulty with the Modem function,

just remember that it is nothing more than a simple terminal

program which has the capability of sounding transmitted and

received characters in Morse code.  If you need help, check with

a friend who understands the operation of Hayes and compatible

modems.

If the screen shows "Character Not Sent - Check Modem and

Port Selection", either the power to your modem is not switched

on or the wrong port has been selected.  To correct the latter

use the Setup/Ports selection in the main menu to set parameters.

Correct the problem and try again!

C. Radio Terminal

The Radio command provides a very simple terminal for

sending and receiving Morse code via a radio, if an appropriate

interface circuit is used.  The same COM pins are used in the

other Morse input-output function as discussed elsewhere in this

documentation.

When the Radio function is entered, Super Morse is in the

receive mode.  To change to the send mode, press <CTRL-S>.  To

return to the receive mode, press <CTRL-R>.

X. We Need Your Feedback and Help!

A. Feedback

It is my goal that Super Morse be absolutely the best Morse

code training program available anywhere, commercial or

otherwise.  If it is not the best, I will endeavor to make

it so.  To achieve this I NEED YOUR INPUT!  Please let me

know what you like about Super Morse, what you don't like

about it, any bugs or cosmetic defects you have discovered, what

you would like to see changed or added, etc.  If you

have used another Morse code program that you like better,

tell me the name of the program and why you like it better.

Please take the time to answer and return the questions in

the registration form contained in the file REGISTER.FRM to me.

If the registration form is inadequate, write me a

letter.  If a letter won't do the job, give me a call!

I particularly need feedback from those who use Super Morse

to train novices.  If I know your needs, I will endeavor to add

those features to Super Morse.

Your help would be much appreciated.  Remember Super Morse

is for you!  So tell me what you want it to do for you.

Please give me your constructively frank opinion.

B. Help Others Learn About Super Morse

If Super Morse is helpful to you, you can help others learn

about it in the following ways:

1.  Give a copy to your friends.

2.  Upload Super Morse to your local computer bulletin

board system.

3.  Leave a message on your computer bulletin board

recommending Super Morse.

4.  Tell your ham club about it, especially the person in

charge of license classes.

5.  Write a letter to a ham radio magazine recommending Super

Morse.

6.  Write a review of Super Morse for a ham radio or

computer magazine.

C. Send Me Your Good News!

If Super Morse helps you obtain your license or upgrade to a

higher class of license, I would love to hear about it!  Please

take the time to drop me a note and let me hear your good news!

D. Correspondence with Author

The author is happy to correspond by mail with users, but

please send a self-addressed, stamped envelope for the reply!!!

APPENDICES

Appendix A. - General Information

1. The Author

The author holds an advanced class license to operate

amateur radio station WD5CID.  He has a "5" call because he hails

from Texas, the home of real chili (red hot with no beans) and

real music (with steel guitar and/or fiddle with no

synthesizers). He is a registered patent attorney working with

Rockwell International Corporation in Cedar Rapids, IA (the old

Collins Radio Company).

2. The Software

Super Morse is written in Turbo Pascal version 6.0.  The

development of Super Morse began in early 1986 under the name

"CW" and has gone through a number of versions.  New versions

will be released as features are added and corrections are made

to previously released versions. The latest version of Super

Morse can usually be found on GEnie in the IBM-PC section and

Compuserve in HAMNET.

Credit must also be given to users who have made suggestions

for improving Super Morse.  Many of their suggestions have been

incorporated into Super Morse.

3. Other Computers

Super Morse is available only for the IBM PC and

compatibles.  It is not available for the Apple IIe, McIntosh,

Amiga, or other computers, and there are no plans to port it to

those machines.  Super Morse will operate with some machines with

an IBM emulator program, but the loop timing method must be used.

Appendix B. - Super Morse License

As you may have noticed I have retained the copyright in

Super Morse, primarily in order to control its orderly

development.  However, I am granting the right to anyone to use

the program for private, non-commercial purposes without further

permission from me.  That is, you can copy it, use it, and share

it (including distribution at hamfests and in ham radio equipment

stores) so long as you do not charge any money for it other than

the cost of the disk or other transfer medium, not to exceed $2.

You may not make a profit or recover labor costs for copying or

distributing Super Morse. Super Morse must be distributed in its

original form and content. Please note that you MUST return a

completed registration form as contained in the file REGISTER.FRM

as a condition of using Super Morse.

Shareware distributors may distribute Super Morse so long as

they include on or in close associate with the distribution

diskette a PROMINENT NOTICE stating that the user must also make

a Shareware contribution to the author.  The copy fee may not

exceed $5.

Appendix C. - Distribution and Support

1. Distribution Policy

Super Morse is distributed primarily through Compuserve

"Hamnet" and computer bulletin board systems.  If possible, users

should obtain the program in that manner.  If the user does not

have a modem, the author will make a courtesy copy if the user

will send a formatted diskette (preferably high density--either 5

1/4 or 3 1/2) and a stamped, addressed diskette mailer.  A

contribution to support further development and distribution of

the program as described below is requested but not required.

The contribution is not a purchase price, and the author

therefore will not accept "orders" for Super Morse--a diskette

and mailer must be provided by the user in order to receive a

copy.

The author does not notify the users of updates.  The users

should check Compuserve "Hamnet" and other bulletin boards

periodically for new versions.  If you wish, you can send the

author a diskette and mailer as described above to be held until

a new version is released.

2. Suggested Contributions

a. U. S. Users

In order to cover expenses associated with the development

and distribution of Super Morse, such as equipment, software,

telecommunications charges, postage, supplies, and long distance

calls to users,  I suggest that you make a voluntary contribution

of $20 (or whatever is affordable to you--or more if you think it

is worth it) for each copy of the program. For use by ham clubs

in organized classes, I suggest a contribution of $5 for each

student, unless the students make their own contributions

individually. If you have already made a contribution for a an

earlier release, you do not need to make an additional

contribution for a new release.

If you cannot afford a monetary contribution, at least

write the author a note and give me an "attaboy" or two.  The

author is a always a sucker for an ego massage.

b. International Users

Please send contributions denominated in U. S. Dollars only.

Changing foreign currency in Cedar Rapids, IA requires a trip to

a downtown bank, and I am extremely lazy. If you cannot obtain

U.S. currency, you can send an International Money Order through

your national post office. The order will be forwarded to the

United States Postal Service which will process and mail a U. S.

money order to the author.

c. Credit Cards

Some users have asked about payment by credit cards.  I do

NOT have this service available at this time.

d. Payments via Compuserve

Super Morse contributions may also be made via Compuserve's

Shareware Registration Database.

3. Courtesy Copies of Super Morse

If you cannot obtain a copy of Super Morse elsewhere, the

author will be happy to make you a courtesy copy, if you will do

the following:

a. Send a request to the above address.  Make sure the

request has sufficient postage so that it does not arrive postage

due!  It's aggravating to have to pay postage for the privilege

of distributing a free program.

b. Include a FORMATTED diskette, preferably 1.2 MB or

1.44MB/720K, but 360K is acceptable.

c. Include a STAMPED, SELF-ADDRESSED diskette mailer. Use

a standard diskette mailer if possible.  If you have a U.S.

address, do not substitute cash for stamps.  Enclosing loose

stamps and stick-on mailing label is acceptable.

d. Include a Shareware contribution as described above if

you can afford it.

If you would like to have a copy of the DOC file in Word

Perfect format to permit you to print a manual with numbered

pages, please request it when you send a diskette.  The Word

Perfect format can be read by most popular word processors.

4. Support and Inquiries

The author is happy to help you with problems in using Super

Morse.  However, please keep in mind that there is only one of

the author and thousands of you.  Therefore, before contacting

the author, please read this document and if possible consult

with a friend who is also a Super Morse user. If you are have a

question about DOS or WINDOWS, please check with your computer

vendor or a knowledgeable friend first.

The author can be contacted on Compuserve (ID No.:

71016,1355), GEnie (Address: L.MURRAH), on packet @WA0RJT or by

U.S. mail at the following address:

10 Cottage Grove Woods, S.E.

Cedar Rapids, IA 52403

If you write the author, you must enclose a self-addressed,

stamped envelope.  The author does not normally respond without

one.

The author can also be reached by telephone at 319-365-6530.

Please do not call the author at his workplace!

6. Comments and Suggestions

The author likes to receive comments and suggestion for

improvements to Super Morse.

Appendix D. - Information about Amateur Radio

Information about amateur radio can be obtained from a

number of sources.

1. Amateur Radio Organizations

There are two organizations devoted to the promotions of

radio.

The National Amateur Radio Association (NARA)

is a specialized new organization dedicated to increasing

participation in amateur radio, especially by the youth of

America.  NARA's address is:

National Amateur Radio Association

16541 Redmond Way, Suite 232

Redmond, WA 98052

The American Radio Relay League (ARRL) is  a general

membership amateur radio organization serving the United States

and Canada.  Sister organizations exist in many other countries.

The ARRL publishes a monthly magazine named QST and provides many

useful services and publications. The ARRL's address is:

American Radio Relay League

225 Main Street

Newington, CT 06111

The W5YI Group publishes several amateur radio courses and

manuals, some of which are available through Radio Shack stores.

This company can be contacted directly at the following address:

The W5YI Group

P.O. Box 565101

Dallas, TX 75356

Tel: 817/461-6443

2. Amateur Radio Magazines

Several major magazines devoted to amateur radio are

available at most amateur radio and electronics stores and at

many large newsstands and supermarket and drug store magazine

racks.  The following is a list of several of these magazines:

QST (published by ARRL for members)

73

CQ

World Radio

3. On-Line Computer Services

A vast amount of information about amateur radio can be

obtained from on-line computer services.

One of the primary on-line sources is the COMPUSERVE

information utility, which provides a special interest area

called HAMNET to its subscribers.  Most computer and software

stores can tell you how to subscribe to COMPUSERVE.

The GEnie on-line utility also has a special interest area

for radio and broadcasting interests.  To subscribe to GEnie you

can call 800-638-9636.

Most communities of substantial size have an amateur radio

club or at least one ham.  To find the nearest club or ham

contact the ARRL or call businesses who are likely to have a ham

on the payroll.  Try a local radio dealer (amateur or commercial)

or TV or radio station. The author found a ham to help him out by

looking for suspiciously large antennas in the neighborhood.

Another good tactic is to look for amateur radio call sign

automobile license tags and make contact with the driver (try

leaving a note on the windshield).

Appendix E. - Morse Code Table

Morse Code Table

Letters

A  DIT DAH            N  DAH DIT

B  DAH DIT DIT DIT    O  DAH DAH DAH

C  DAH DIT DAH DIT    P  DIT DAH DAH DIT

D  DAH DIT DIT        Q  DAH DAH DIT DAH

E  DIT                R  DIT DAH DIT

F  DIT DIT DAH DIT    S  DIT DIT DIT

G  DAH DAH DIT        T  DAH

H  DIT DIT DIT DIT    U  DIT DIT DAH

I  DIT DIT            V  DIT DIT DIT DAH

J  DIT DAH DAH DAH    W  DIT DAH DAH

K  DAH DIT DAH        X  DAH DIT DIT DAH

L  DIT DAH DIT DIT    Y  DAH DIT DAH DAH

M  DAH DAH            Z  DAH DAH DIT DIT

Numbers

1  DIT DAH DAH DAH DAH

2  DIT DIT DAH DAH DAH

3  DIT DIT DIT DAH DAH

4  DIT DIT DIT DIT DAH

5  DIT DIT DIT DIT DIT

6  DAH DIT DIT DIT DIT

7  DAH DAH DIT DIT DIT

8  DAH DAH DAH DIT DIT

9  DAH DAH DAH DAH DIT

0  DAH DAH DAH DAH DAH

Punctuation

.  DIT DAH DIT DAH DIT

,  DAH DAH DIT DIT DAH DAH

?  DIT DIT DAH DAH DIT DIT

/  DAH DIT DIT DAH DIT

-  DAH DIT DIT DIT DIT DAH

Procedure Signs

+  DIT DAH DIT DAH DIT           <AR>

!  DIT DAH DIT DIT DIT           <AS>

#  DAH DIT DAH DAH DIT           <KN>

$  DIT DIT DIT DAH DIT DAH       <SK>

=  DAH DIT DIT DIT DAH           <BT>

%  DIT DIT DIT DAH DIT           <SN>

@  DIT DAH DIT DAH               <AA>

&  DIT DAH DIT DIT DAH           <KA>

Note: The phonetic "DIT" is used in every appearance in the above

table for the sake of clarity.  However, "DIT" is normally

rendered as "DI" when it precedes another sound element and

written out fully only when it is the last element in a

character.  The reason is that "DIT" seems to have the "T" ending

only when it is the last element in a character. For example, the

character "C" is normally written phonetically as "DAH DI DAH

DIT".

Appendix F. - Troubleshooting

If you have difficulty in using Super Morse, you should

begin by reading the instructions!  Most problems are due to a

misunderstanding of how Super Morse operates, and the

instructions should clear up most problems.  However, if you

continue to have a problem after reading the instructions, the

following points may be helpful in resolving your problem.

1.  Random Characters Faster than Plain Text

This is NOT a problem.  Due to the way code speed is

calculated, random characters sound faster than plain text. For

an explanation see the discussion of code speed earlier in the

documentation.

2.  'File Not Found' or 'I/O Error 02' Message

a.  Make sure that all files with an HLP and an SM

extension are copied to the same subdirectory or diskette

containing SM.EXE.

b.  Make sure all the required files listed early in this

documentation are on the program disk.

c.  If Super Morse is distributed in ARC format,

de-archive only with ARC 5.12 or later, or PKXARC. For ZIP

format use PKUNZIP 1.02 or later.

d.  Make sure the program disk is in the currently

active drive when run.

e.  Make sure that SM.CFG is present and is not shown

with 0 bytes.  If the latter happens, erase SM.CFG and start the

program again.

f. If you de-archived onto a floppy, copy the distribution

file to a hard disk and try again.  The floppy has

overflowed, and not all necessary files were de-archived.

3.  'I/O Error 100'

Erase the SM.CFG file. If that does not work, erase the

SMn.USR files. The current version of one of these files is

probably longer than the previous version.

4.  'I/O Error 101'

Your disk is full.  Usually happens when trying to run

Super Morse from a floppy.  Run it on a hard disk, or remove all

unnecessary files, such as sm.doc, from the floppy.

5.  'I/O Error 150'

Remove the write protect tab from your diskette!  Super

Morse must be able to write to the diskette to create the

configuration and user statistics files.

6.  Strange Values on the Status Line and Other Weird

Happenings

Erase the SM.CFG and/or SMn.USR file and start over!

7.  'Disk Read Error'

Erase SM.CFG and/or SMn.USR and start over.

8.  No Video on Laptop Computer

It has been reported to me that Super Morse will not run on

some laptop computers with LCD screens.  If someone knows why and

knows how to fix the problem, please let me know.

9. Erratic or Choppy Code Sound

Switch to the software loop code timing method.  Microsoft

Windows or non-standard computer designs are not compatible with

the clock timing method.

10. Code Speed Changes

If the loop timing method is used, the code speed will be

affected each time you add or remove a memory resident program.

You must recalibrate each time.

11. Mouse Does Not Work after Running Super Morse

In some functions, Super Morse can interfere with a serial

mouse. Reboot the computer to correct the problem.

12. External Key Does Not Work

Check the following:

a. The RS-232 connector pins are small and easily confused.

Remember that the connector pins on the computer are a mirror

image of those on the cable connector.

b. Super Morse uses two COM ports, and you should make sure

that different ports are set for the Modem function and the

keying function.

c. Be sure that the "External" keying source is selected.

13. Speaker Not Loud Enough

Aside from adding external amplification, about all you can

do is change the speaker tone since speakers vary in their

loudness at different frequencies.  Super Morse does not support

sound boards such as the Sound Blaster.

Appendix G. - Interfacing

1. Using an External Hand Key

To RS-232 DSR    o ���������Ŀ

(Pin 6) or CTS               �

(Pin 5)                      o

/  Morse Hand Key

(COMPUTER)                    /   ("Straight" Key)

o

To RS-232        o �����������

Pin 7

2. Driving a Transmitter or Other Device

The following circuit designed by John Swancara, WA6LOD, can

be used to drive a keyer, code oscillator, or a transmitter.  It

s described in QST Magazine, February, 1990, page 36.  A

similar�ircuit was provided by John Dilks, K2TQN.

If these connections do not work, first check the Port

settings in the Setup menu.  Then check your computer's RS-232

information in case you computer uses something different from

the standard.

�����������������������Ŀ

�  Radio Shack TIL-113  �

�                       �

���Ŀ                   ���Ŀ

�   �                   �   �

RS-232 Signal   o ���Ĵ 1 ���Ŀ    ����������Ĵ 6 � o NC

ground (Pin 7)        �   �   �    �          �   �

�����   �    �          �����

�     �    �            �

�     �    �            �

�    \ /   �            �

�     v    �            �

�     �    �            �

�     �    �            �

�     �    �            �

D1     �     � \  �            �

���Ŀ   �  \ �          ���Ŀ

RS-232     R1      /   �   �   � \  �          �   �

Pin  o ��� \/\/ Ĵ< ��Ĵ 2 �����  \ �      ���Ĵ 5 � o To

4       �          \   �   �        �          �   �   Positive

�              �����        �     /    �����   Keying

�                �          �   �/       �     Line

���  C1           �          �   �        �

���               �          �   �        �

�                �          ���Ĵ        �

�                �              �        �

�                �              �        �

�                �              �\       �

o Ĵ                �                \      �

�              ���Ŀ               _|  ���Ŀ

�              �   �                   �   �

�         NC o � 3 �               ���Ĵ 4 ��� o To

-              �   �                   �   � �   Trans-

�����                   ����� �   mitter

Interface            �                       �   �   Chassis

Board                �                       �   �

Ground               �������������������������   �

�

�  Interface

-  Board

Ground

D1  = 1N914

R1  = 1K Ohm, 1/4 Watt

C1  = 0.01 Microfarad, 50 Volt

DTR = Data Terminal Ready pin

NC  = no connection

Note: (1) Voltage on pins 4 and 5 of TIL-113 should be kept below

20 volts, and the current draw should not exceed 100

ma.

(2) The Philco ECG 3041 may be substituted for the Radio

Shack TIL-113.

3. Using a Volume-Controlled Headphone

Splice a 50 ohm variable resistor and a headphone jack into

the PC speaker line as shown below.  The headphone jack should

break the speaker circuit and close the headphone circuit when

the headphone plug is inserted.

Headphone Jack

o ������Ŀ           o     o

\          �

PC        50   / < ��������������������Ŀ  /�

Speaker     ohm  \                        � / �   PC

Line           /                �       �/  �   Speaker

�       �\  �

�                 �       � \ �

o ����������������������������������  \�

4. Using an External Oscillator

Following is a simple code oscillator driven from the COM

port of a PC designed by Jeff Furman, KD6MNP.

DB-25

FEMALE

1N4001

�����͸

�     �        \

�     �������Ĵ \�����������Ŀ

�  4  �       � /�       �   �

�     �        /         �   �  555

�     �             ���������������Ŀ

�����͵             �               �

�     �             �    4   8      �

�     �             �               �

�  7  �������������Ĵ 1             �

�     �      �      �               �

�     �      �      �               �

�����;      �      �               �

�      �    6   2   3  �             1/8" stereo

�      �����������������             headphone jack

�           �   �   �

�    ��     �   �   �    ��

����Ĵ�����������   ����Ĵ������������������ V    �

�    ��       �     �    ��            ��� v      �

�             �     �                             �

�              \/\/\                   ������������

� 0.001 uF             0.01 uF         �

�   film        680k   ceramic or      �

�               ohms   film            �

�                                      �

����������������������������������������

This easily fits inside the DB-25 shell.

Adjust the 680k ohm resistor to change the pitch--

higher value for lower pitch, etc.

Decrease the value of the 0.01 uF cap. for lower volume.

Super Morse Setup:

Keying Port  : <your choice from what's available>

Output Keying: ON

RS-232 Output: RTS

Output Sense : -

5. Using an External Sound Output

The following was submitted by Bruce G. Kinney, San Jose, CA

������������������Ŀ

30K    Shield�                  �

Pin 20 >  \/\/\/ ���� O � MOD IN           �      /�

�     �               O  ����� / �

.68uf  ���  � �               U  �   <   � Speaker

���  � �               T  ����� \ �

�   � �                  �      \�

Pin 7  > ��������������Ĵ GND              �

�                  �

��������������������

LEADER 1300S

Function Generator