PC/Computing               June 1991             ATSEND.DOC

Send Modem Commands via Serial Port

We wanted to leave DIAL and DIALV2 as dialer-only programs,

so we created ATSEND.COM, which sends modem commands out of

a serial port.  For example, use ATSEND to silence your

speaker with the following command:

ATSEND ATM0

Or, if you want to dial the phone very rapidly, give this

command:

ATSEND ATS11=50

ATSEND is configured to use COM1.  Choose the serial port

by putting a 0, 1, 2, or 3 (for COM1, COM2, COM3, or COM4)

at offset hex 0102.  Insert the actual hex address of the

port with debug at offsets hex 0104 and hex 0106.

Material in this document may not be reproduced in any form

without permission. Written requests for permission should

be addressed to Chantal Lavelanet, Ziff-Davis Publishing

Company, One Park Avenue, New York, NY  10016.