PC/Computing                June 1991          	 DIALV2.DOC

Dialing with COM3/COM4

The syntax for using DIALV2 is simple.  To dial a phone

number, simply enter:

DIALV2 1-800-555-1212

DIALV2 dials the phone number and then prompts you to press

any key after you've picked up the phone.

By default, DIALV2 is configured to use COM1.  However,

it's possible to choose the COM port that output will be

sent to by entering a 0, 1, 2, 3 (for COM1, COM2, COM3, or

COM4) at offset 0102 in hexadecimal notation in the source.

If you configure it for COM3 or COM4, DIALV2 assumes that

the serial port's base address is hex 3E8 (COM3) or hex 2E8

(COM4).  If your port is configured differently, you'll

need to enter its address at offset hex 010E for COM3 or

hex 0110 for COM4.

For example, to change DIALV2 to work with COM3 and a base

address of hex 338, use the following steps:

DEBUG DIALV2.COM

E 0102 02

E 010E 08 33

W

Q

The first line loads DEBUG and opens DIALV2.COM.  The

second line changes offset 0102 so that it will work with

COM3 (remember that numbering starts with 0 for COM1).

The third line changes the I/O address at offset 010E.  The

last two lines save the file and quit debug.

Material in this document may not be reproduced in any form

without permission. Written requests for permission should

be addressed to Chantal Lavelanet, Ziff-Davis Publishing

Company, One Park Avenue, New York, NY  10016.