Speak-Out and Sounding Board

Voice Synthesizers for your computer

Speak-Out and the Sounding Board family are voice synthesizers

designed specifically to meet the special needs of blind and

visually-impaired computer users.  They all provide responsive,

good quality speech that may be used with a variety of speech

programs from a number of different developers.  They have many

advanced features which provide greater independence and

increased productivity.  Some of these features are

capitalization alert, format alert, fast forward/rewind, pause

and hyperscan.  The Sounding Board was designed for standard

desktop PC compatible computers whereas the Sounding Board Lite,

LT and XE were designed for a variety of laptop computers.

Speak-Out is a battery powered synthesizer which can be connected

to any computer or other device with a serial or parallel port.

Since it is an external synthesizer, Speak-Out is the perfect

solution for someone who needs to take their speech with them -

easily moving from computer to computer.  It's small size and

rechargeable batteries make it ideal for mobile, portable use.

Once installed, Speak-Out or the Sounding Board will speak any

combination of letters and numbers immediately when requested.

They operate much like your printer - they sit quietly until

information is sent their way.  When your printer receives

information to be printed it immediately begins printing this

information on paper.  In the same way, they speak the

information sent to them.

FEATURES

DIMENSIONS

Sounding Board is a full size, 13" board which requires an 8, 16

or 32 bit ISA slot.  The Sounding Board Lite, LT and XE  are

sized for their respective laptop computers.  The Speak-Out is

approximately 5-1/4" square by 1-1/2" tall and weighs

approximately 1 lb.  The Speak-Out comes with a carrying case

which is approximately 6" x 8" x 3".

MEMORY

If the Sounding Board text-to-speech software is loaded into DOS

RAM it uses 28K.  If it is loaded into Sounding Board's 16K of

on-board RAM then it uses 12K of DOS RAM.  The Sounding Board

Lite, LT and XE do not have the on-board RAM.  Speak-Out has a

synthesizer driver which takes less than 3K of DOS RAM. The text-

to-speech software is loaded inside Speak-Out and doesn't consume

any of your computer's memory.

Buffers may be increased or decreased as desired.  With the

Sounding Board family this will increase DOS RAM usage slightly.

The Speak-Out buffers do not affect DOS RAM.

FEATURES

- One year, all parts and labor warranty

- Rate of speech - 1 of 10 levels

- Voice pitch - 1 of 10 levels

- Voice tone - 1 of 26 levels

- Volume - 1 of 10 levels

- All of our synthesizers have an internal speaker.  Except for

the Sounding Board XE, they all have an external volume control.

The Sounding Board XE's volume is controlled by a control in-line

with the headphones.  The Sounding Board also has an external

tone control.  Sounding Board includes our external Super

Speaker.  The Sounding Board Lite, LT and XE includes mono

headphones.

- Indexing support (Parallel not supported)

- Phonetic pronunciation dictionary

- Fast forward/rewind and pause from the keyboard

- Hyperscan

- Acronym, spell and capitalization processor

- Format alert for extraneous spaces

- 10 variable word and sentence boundary delays

- Time and date hotkey

- All 12 hotkeys definable by the user

- Full support with Vocal-Eyes

SOUNDING BOARD SPECIAL FEATURES

The Sounding Board Interrupts and Base Address assignments may be

changed to conform to the specifications of the computer and

other expansion boards.

SPEAK-OUT  SPECIAL FEATURES

26 alarms can be set for a specific time and/or date.  An

optional custom message can be attached with each alarm.

Speak-Out also supports a count-down timer which can be set for

up to 99 minutes.

Speak-Out has a special reload switch used to reload software.

There are no EPROMS in Speak-Out.  You are able to upgrade the

text-to-speech software or add an additional feature by uploading

through the serial port.

Speak-Out has a standard Centronics parallel connector and 9 pin

D serial connector on the back and both ports are active.  It

includes a serial cable - a parallel cable is optional.

It's rapid charge circuit allows Speak-Out to be fully recharged

in 1-1/2 hours.  If you are in a hurry, even a 15 minute charge

provides operation for a long time.

The microprocessor has both a STANDARD mode (4.77 MHz) and a

TURBO mode (9.54 MHz) which is configurable from the buttons on

the front panel.  The standard mode provides the maximum battery

life and the turbo mode provides the maximum responsiveness.

The carrying case includes a lower section for the AC

adapter/charger, a disk pocket, a belt clip, a short carrying

strap and a long carrying strap which can be slung over the

shoulder or worn around the waist "fanny pack" style.

The optional automobile accessory power adapter allows use of

Speak-Out without running down the rechargeable batteries. This

adapter will operate Speak-Out but will not charge its batteries.

EQUIPMENT SUPPORTED

The Sounding Board is compatible with any IBM PC or true

compatible with a full-length internal 8 bit ISA slot.  The

Sounding Board Lite, LT and XE are designed for a number of

laptops.  Please call for help determining which models are

supported.  Speak-Out was designed for PC compatible computers

but it may be connected to any device that has a parallel or

serial port (you must have the necessary software drivers for use

with each device).  Speak-Out includes software for use on PC

compatible computers.