INTRCPT.DOC - Version 3

May 5, 1986

Intercept is a memory resident program which monitors interrupts 40H and

2FH << Function 17, operation 35 >>. Basically, what this program does

is allows AT users with a high capacity drive in position A to format,

read, and write 720K disks using the cheaper low density diskettes. This

way you can double the storage without spending an arm or a leg for the

higher priced ultra density diskettes and achieve compatibility with other

MS DOS systems using quad density drives! Since the high capacity drives

really are 96TPI, FM and MFM, this program along with DRIVER.SYS opens

up a new storage method.

To use this program, you first must be running under DOS 3.2 or greater,

be running with an AT, have DRIVER.SYS installed as a device driver in

your CONFIG.SYS file, and have a HIGH CAPACITY drive in drive position

A. Your config.sys entry must be as follows:

DEVICE=DRIVER.SYS /D:0

When the system boots, it will tell you what the NEW drive letter is,

remember it since you must enter it in the command line when calling

INTRCPT. In my system which I used to write this program, I have two

high capacity drives, 2 hard disks, and a RAM disk; therefore, my new

drive became F. In my AUTOEXEC.BAT file, I have an entry as follows:

INTRCPT F

This entry in the command line is extremely important because it tells

the software which drive specifier to monitor.

Two interrupts are monitored: 40H and 2FH. The 40H interrupt is the BIOS

interrupt which handles the floppy disks and the 2F interrupt is the

MULTIPLEXER interrupt which allows me to monitor the drive specifier.

Since the function that I am intercepting is "INTERNAL TO DOS", a

considerable amount of time was spent trying to verify that I wouldn't

cause any problems with other functions.

The following files are included on this disk:

1) INTRCPT.DOC           This file

2) INTRCPT.COM           The load module

3) INTRCPT.CON           An example of the entry in CONFIG.SYS

4) INTRCPT.BAT           An example of the entry in AUTOEXEC.BAT

[Library note: The assembler source code is available on the appropriate

assembler source disk from the Public (Software) Library.]

THIS SOFTWARE IS PLACED IN THE PUBLIC DOMAIN for your use and UNDER NO

conditions may a fee be charged for it.

This software is given "AS IS" without any guarantees. The user takes

full responsibility. Under NO conditions will I take the liability for

any damages caused by its usage.

Sanford J. Zelkovitz

Alpha Computer Service

5300 Orange Ave.  Suite 108

Cypress, CA   90630

Voice:       714-828-0286  << 82-80286 >>

Data:        714-898-8634

Additions to version 3:

DISKCOPY and DISKCOMP NOW work with the software. Also, INT 21H - function

44H - subfunction D is monitored to allow DISKCOPY & DISKCOMP to operate.

A VERIFY command is performed whenever the IOCTL interrupt in intercepted.

This may cause an unexpected read to be performed; however, this is normal

and is necessay.