LETTERS

Another Sound Alternative

from

KanSys, Inc.

Copyright (c) 1992

by

KanSys, Inc.

1016 Ohio St.

Lawrence, KS 66044

Telephone: 913-843-0351 or 800-279-4880

USER'S GUIDE

The LETTERS program displays a text file in block letters on

a braille embosser with graphics capability. Such block letter

displays are used in creating cover pages for braille manuals,

restaurant menus, conference agendas, and similar documents. They

allow sighted persons to identify such documents at a glance, and

also allow blind persons who are not braille readers to easily

identify such documents.

Most braille embossers operate in a "character mode" and in

a "graphics mode."  In character mode, standard six dot cells are

placed at 0.25 inch intervals across each line, and lines are

placed at 0.4 inch intervals down each page. A typical page of

characters consists of 25 lines of 40 cells each, a 10 inch

square area on an 11.5 by 11 inch page.

The space between two adjacent dots within a braille cell is

3/32 inch, and between two adjacent dots in neighboring cells is

5/32 inch horizontally, and 7/32 inch vertically. These inter-

cell spaces mean that if a page is completely filled with braille

cells in which all six dots are present, one can still identify

individual cells because of the extra space between cells. It

also means that in character mode, smooth unbroken lines cannot

be formed simply by composing them from parts of braille cells.

In graphics mode on the other hand, the extra space between

cells horizontally and vertically is eliminated. A page filled

with six dot cells appears as a uniform field of dots, and

individual cells can no longer be identified. Dot spacing is the

same between adjacent dots, regardless of whether they are in the

same cell or in adjacent cells. Smooth unbroken lines can easily

be formed, and other interesting graphic effects can also be

achieved.

Some braille embossers retain the 3/32 inch dot spacing in

graphics mode, while others reduce this figure to an even smaller

distance to achieve a higher resolution. Graphics mode on the

BookMaker and the Juliet from Enabling Technologies both use

0.080 inch spacing; the Braille Blazer from Blazie Engineering

uses 0.090 inch spacing; the Cranmer Modified Perkins Brailler,

The Romeo from Enabling Technologies, the VersaPoint from

Telesensory Systems, and the Index family of embossers, all use

the standard character mode distance of 3/32 inch (0.094 inch) in

their graphics modes.

Some braille embossers require a mechanical adjustment to be

placed in their graphics modes, while others require that a

software escape sequence accompany the data to be embossed. A few

embossers require both a mechanical adjustment and a software

escape sequence. Among those embossers mentioned earlier, the

Braille Blazer is mechanical only, the Romeo is both mechanical

and software, and the others are software only. Mechanical-only

embossers generally cannot intermix normal character mode braille

and graphics mode braille on the same page. Software-only or

mechanical-and-software embossers generally do support mixed

formats on the same page.

Most embossers that use software escape sequences have

adopted a standard method of entering and exiting graphics mode.

The two characters "Escape 1" enter graphics mode, and the two

characters "Escape ]" exit graphics and return to text mode. The

Index braille printers use "Escape ^A" and "Escape ^B" instead,

where the caret (^) indicates that the "A" or "B" is "Control A"

or "Control B". Check your printer manual to determine what

software escape sequence, if any, is required for your printer.

The LETTERS program assumes you are using a high-resolution

graphics embosser, that you are printing on wide paper (11.5 by

11 inch), that your printer uses standard escape sequences to

enter and exit graphics mode, and that you want only one copy of

the output. These defaults can be changed by including an

appropriate switch on the command line, or by running the program

in its interactive mode.

The simplest way to run LETTERS is to type its name at the

DOS prompt. This begins an interrogation process in which you may

select the resolution, the paper width, the escape sequence to

use, and the number of copies you wish. Except for the number of

copies, these choices automatically become the program defaults

when you next run the program. You are then asked for an input

file containing the text to be displayed, and an output file

where the data will be written. Pressing "Enter" in response to

the last question causes output to go directly to your printer

port, LPT1.

You may avoid this question-and-answer sequence by including

the name of the input file on the command line. In that case, the

default settings will be used, and the output will go to LPT1. If

you include both the input-file and an output filename on the

command line, the data will be written to that file. Finally, you

may substitute another choice for a default setting by including

a "switch" on the command line. The switch must be included

between the program name and the name of the input file. The

following switches are recognized:

/H -- High Resolution. Make 13 block letters per line on

narrow paper, 18 on wide paper, with 14 lines per page.

/L -- Low Resolution. Makes 11 block letters per line on

narrow paper, 16 on wide, with 12 lines per page.

/N -- Narrow Paper. Notebook size, 8.5 by 11 inches.

/W -- Wide Paper. Standard size, 11.5 by 11 inches.

/X -- Do not use escape sequences.

/S -- Use standard escape sequences.

/D -- Use escape sequences defined in an earlier interactive

session and remembered by the program.

/C## -- Create ## copies of the output, where ## is one or

two digits, and is greater than 0 and less than 100.

If you wish, you may use a hyphen (-) instead of the slash

(/) in the switches. Be sure that no blank spaces occur within a

switch, but be equally sure that a space occurs to the left of

each switch. Switch letters may be either upper or lower case.

Using a switch to change a setting will not permanently affect

the default for that setting.

You MAY NOT use wildcard characters in an input filename. If

you enter "CON" for the input file, you may enter text directly

from the keyboard. Type ^Z (Control Z) on a new line and press

Enter to finish your entry.

The LETTERS program contains a seven pixel by nine pixel map

for the ASCII characters with decimal values from 32 to 126. Thus

it can display both upper case and lower case letters, the ten

digits, and the standard punctuation symbols normally seen on a

computer screen. It cannot display the extended ASCII set

sometimes seen on IBM compatible PC screens. No effort is made to

avoid splitting words, to center words on the page, or to perform

any other kind of formatting; you must do that manually when

preparing the input file.

Be sure to use an ASCII editor such as QEDIT or WordStar

non-document. If you MUST use WordPerfect, be sure to save your

file as a DOS TEXT file on the Control-F5 menu.

The LETTERS program cannot by itself produce a page

containing both standard character mode braille and graphics mode

block letters, even if your embosser has that capability.

However, the LETTERS program logic is now an integrated feature

of Turbo Braille Version 3.2 and later, in which mixed format

pages can be produced. Call or write us for more information

about Turbo Braille and other KanSys, Inc., products.

KEEP IN TOUCH!

##################