*********************************************************************

*  NOTE:  The following ASCII text file (without graphics)          *

*         is contained in a printed technical paper available       *

*         from Broadcast Electronics Inc.  Unfortunately, it        *

*         was not possible to reproduce the graphics portions       *

*         of this paper within this text file.  If you find the     *

*         information in this file of interest, you may request     *

*         a complimentary, printed, copy including figures and      *

*         graphics from: BROADCAST ELECTRONICS INC.                 *

*                        P.O. BOX 3606                              *

*                        4100 N. 24TH  STREET                       *

*                        QUINCY, IL. 62305-3606                     *

*                        ATTN: SALES DEPARTMENT                     *

*                        PH  217-224-9600                           *

*                        FAX 217-224-9607                           *

*                                                                   *

*         The contents of this technical paper are                  *

*         Copyrighted (c) 1984, by Broadcast Electronics Inc.       *

*         All rights reserved.                                      *

*********************************************************************

SCA BASICS

BY:

Geoffrey N. Mendenhall, P.E.

Vice President - Engineering

Broadcast Electronics Inc.

Quincy, Illinois

SCA OVERVIEW

The recent de-regulation of broadcast SCA usage has spurred a new interest

in using the SCA for data transmission and paging.  SCA stands for "Subsidiary

Communications Authorization", which allows FM broadcast stations to "piggy-

back" up to two additional audio or data channels on the main carrier of the

station.

This article will explain the basic theory behind SCA transmission and will

also describe the equipment required to add an SCA to the FM broadcast signal

HOW SCA's ARE ADDED TO THE BROADCAST SIGNAL

SCA's are added to the broadcast FM carrier by a technique called frequency

domain multiplexing which allows the additional subchannels to be separated

from each other and from the main channel by use of specific frequency bands

for each subchannel.  In the case of an FM station broadcasting in stereo with

two SCA's, the frequencies used are 67KHz and 92KHz for the two subcarriers.

The main channel and stereophonic information are transmitted in the frequency

band extending from 30Hz to 53KHz while the SCA information is transmitted

above this frequency range in a band extending from 54KHz to 99KHz. The SCA

subcarriers each modulate the main FM carrier by a maximum of 10% of the total

modulation.  This means that the effective coverage will not be as great as

the main channel since the SCA does not have full use of the transmitted power.

The sum of all the different components being transmitted is called the

composite baseband. It includes the following individual components as shown

in figure 1A.

(FIGURE 1A)

Figure 1B shows what the composite baseband looks like if viewed on an

oscilloscope with the peak-to-peak amplitude shown as a function of time.  It

is difficult to identify the various components as a function of time.

Figure 1C shows the composite baseband as viewed on a low frequency

spectrum analyzer.  This is a representation of amplitude as a function of

frequency.  It is now easy to identify the various frequency components within

the composite baseband.

(FIGURE 1B  AND  FIGURE 1C)

The method of modulation used is frequency modulation of the SCA subcarrier

itself which in turn frequency modulates the main transmitter carrier.  This

type of "FM on FM" system is difficult for many people to fully understand.

Think of it this way, the main carrier is deviated a constant plus and minus

7.5KHz (10% of the total) by the 67kHz subcarrier while the 67KHz subcarrier is

itself deviated up to plus and minus 6KHz by the audio or data being fed into

the SCA modulator.  The key points to remember are that the deviation of the

main carrier is dependent only on the level of the subcarrier and is not

effected by the modulation applied to the subcarrier while the deviation of the

subcarrier is dependent only on the modulation applied to the subcarrier and is

not related to main channel deviation.

RECEIVING THE SCA

The SCA receiver must first FM demodulate the entire baseband, filter out

the desired subcarrier from the rest of the baseband components, and then FM

demodulate the SCA information.  This requires two IF strips and two FM

demodulators.  The first IF strip is usually operated at 10.7 MHz with a

bandwidth of about 250 KHz while the second IF strip is located at the SCA

frequency (67 KHz or 92 KHz) with a narrow bandwidth (typically 25KHz or less).

The first FM demodulator is usually a quadrature or pulse counting type while

the second FM demodulator is usually a Phase-Locked-Loop to minimize

interference from the main and stereo channels.

This dual demodulation process places some limitations on the performance

of the SCA receiver.  The amplitude and phase response of the IF filters will

have a significant effect on the crosstalk into the SCA from the main and

stereo channels.  This type of receiving system is also more susceptible to

multipath than a simple FM system.

The SCA subcarrier being received only has access to a maximum of 10% of

the transmitter power.  The additional noise factors introduced by the "FM on

FM" nature of the system cause further degradation of the signal to noise ratio

which may yield a total penalty in coverage of more than 20db when compared to

main channel.

TRANSMITTING THE SCA

The SCA is added to the broadcast signal by connecting an SCA generator to

the FM exciter located in the FM broadcast transmitter.

The SCA generator is usually a self contained device that accepts audio

and/or DC coupled data inputs and provides a frequency modulated subcarrier (67

KHz or 92 KHz) at its output.  The SCA generator may also have additional

features which allow automatic muting of the SCA transmission under certain

conditions, multiple inputs, active filtering of the inputs, and modulation

indication.  Since the SCA generator is located at the broadcast transmitter,

it is usually supplied by the broadcaster.

The FM exciter must have a wideband SCA input to accept the subcarrier from

the SCA generator and add this signal to the rest of the baseband for FM

modulation onto the main radio frequency carrier.

In order to prevent interference among the main channel, the stereo

channel, and the SCA the exciter should be a state-of-the-art unit employing an

ultra-linear modulator.  This type of modulator will minimize the generation of

intermodulation and harmonic products within the baseband.  This is important

because non-linearity within the modulation process will alter the composition

of the baseband, which results in distortion of the demodulated signal at the

receiver.

Modulator linearization has reduced harmonic and intermodulation distortion

to less than 0.05% in newly-developed equipment.

Any distortion of the baseband signal caused by the modulated oscillator

will have secondary effects on stereo and SCA crosstalk, which are quite

noticeable at the receiver in spite of the rather small amounts of distortion

to the baseband.  For example, if the harmonic distortion to the baseband is

increased from 0.05% to 1.0%, as much as 26dB additional crosstalk into the SCA

can be expected. This amount of crosstalk would seriously degrade the

performance and utility of the SCA.

Assuring that the composite baseband signal undergoes minimal distortion in

the modulation process will suppress undesired harmonic and intermodulation

products in the baseband, making the FM exciter transparent to the SCA signal

coupled into it.

The stereo generator's characteristics play an important role in preventing

interference to the SCA.  The second harmonic components of the stereophonic

subcarrier fall directly on top of the SCA so it is important to use an stereo

generator that suppresses these spurious components.

The bandwidth of the FM transmitter and antenna system will also affect the

performance of the SCA.  The distortion in any practical FM system will depend

on the amount of bandwidth available versus the modulation index being

transmitted.  Relating the specific quantitative effect of bandwidth

limitations imposed by a particular transmitter to the actual interference to

the SCA is a complicated problem indeed.  Some factors which will improve the

situation are:

1. Maximize bandwidth by using a broadband exciter and a

broadband IPA stage.

2. Use a transmitter of single tube design or broadband

solid state design where feasible.

3. Optimize both the grid circuit and plate circuit tuning

for best possible bandwidth.

4. Use a broadband antenna system with low standing wave

ratio on the transmission line.

ADJUSTING THE TRANSMITTER FOR BEST SCA PERFORMANCE

Start out by remembering that all optimization should be done with the

transmitter connected to the normal antenna system rather than to a dummy load.

The transmitter is first tuned for normal output power and proper

efficiency.  A simple method for centering the transmitter passband on the

carrier frequency involves adjustment for minimum synchronous AM.  Synchronous

AM is AM modulation of the carrier caused by frequency modulation of the

carrier frequency.  If the bandwidth is narrow or skewed, increasing amplitude

modulation of the carrier will result.

A typical adjustment procedure is to FM modulate 100% at 400 Hz in the

monaural mode and fine tune the transmitter for minimum 400 Hz amplitude

modulation as detected by a wideband envelope detector.  At the optimum point,

the residual demodulated AM will double in frequency to 800 Hz.  It should be

possible to minimize synchronous AM while maintaining output power and

efficiency.

Transmitter tuning becomes very critical when minimizing crosstalk into the

SCA.  A more sensitive adjustment procedure involves modulating one channel

only on the stereo generator to 100% with a 4.5 KHz tone.  This will place the

lower second order (L-R) stereo sideband on top of the 67 KHz SCA.  Activate

the SCA at normal injection level without modulation on the SCA.  Tune the

transmitter for minimum output from the SCA demodulator.  A similar adjustment

can also be made by listening to the residual crosstalk into an unmodulated

SCA while normal stereo programming is being broadcast.  In any of these tests,

the grid tuning is frequently more critical than the plate tuning.  This is

because the impedance match into the input capacitance of the grid becomes the

bandwidth limiting factor.

BRINGING IT ALL TOGETHER

SCA's do offer an important new opportunity to transmit information over

existing FM broadcast facilities.  Whether SCA's are used primarily for paging,

data transmission to fixed points, or other uses remains to be seen.  In any

case, it is important that each subsystem be individually optimized before the

complete transmission and reception system can perform properly.  The

complexity and technical limitations of the SCA system makes attention to

details critical to the success of the operation.#########################################################################################################################