FSEND VERSION 11.2

A program to help you learn or generate practice in the

international morse code.

By Rick Hayner.

Copyright (C) 1990, 1995 By Rick Hayner.

All rights reserved.

Fsend was written to help people who are interested in

obtaining an Amateur Radio license, or who are in the process of

upgrading to a higher class of Amateur Radio license. I am a morse

buff, and I originally wrote the program to help me

increase my receiving speed.

FEATURES

Fsend has many features.  I wrote it to operate in four modes.

First of all, Fsend can read a text file and send its' contents in

morse code, or CW.  Cw is what Hams or Radio Amateurs call Morse

Code.  In this mode, Fsend ignores extra spaces and carriage

returns to prevent long pauses in the sending.

Secondly Fsend can generate random code practice by using its'

internal random code generator.

Fsend also has a keyboard mode.  This mode can help you learn

the code, because it sends any character you type on the keyboard

while it is in this mode.

Fsend also has a message mode.  In this mode, you can type a

message up to 80 characters in length, and fsend will send the

message for you.  Fsend also has an environment variable called

fcaps.  if this variable is set to yes, fsend will change pitch

when a capital letter is sent, and will return to the pitch you

specify when a lower case letter, number, or punctuation mark is

encountered.  to set this variable to yes, type the following from

the dos prompt:

set fcaps=yes

To turn this feature off, type:

set fcaps=

COMMAND-LINE PARAMETERS

Fsend has several command-line parameters which must be typed

in a specific order.  you can type zeros in place of any parameter

after the second one.  All parameters must be separated from the

last one or following one by a space.

First of all, you must type fsend.

The second parameter can contain one of several things.

If you type the name of a standard text file, fsend will send

the contents of it for you.

If you type the word keyboard in either case, fsend will enter

keyboard mode.

If you type the word message, in either case, fsend will enter

message mode.

If you type the word random in either case, fsend will enter

random code generation mode.

If you type the word time in either case, fsend will send the

current time as displayed by dos, and then exit to dos.

The third parameter is the speed in words per minute.    The

value for speed can range from 5 to 95 words per minute, and it

controls the initial speed of the code practice session.

The fourth parameter is the pitch in hertz.  This can range

from 100 to 10000 hertz.

The fifth parameter is the weighting.  This is the dit to

space and daah to space ratio.  It can range from 1 to 45.  1 is

very light weighting, and 45 is very heavy weighting. 23 is normal

weighting.

The sixth parameter is  word spacing.  It can range from 1 to

100.  this value determines word spacing in increments of 4 times

dit length.  Normal spacing is 4 dit lengths, and if you enter a

value of 3 for word spacing, you will then have word spacing of 12

dit lengths.

The seventh parameter is character spacing.  It can range from

1 to 100, and changes character spacing in increments of twice dit

length.  if you enter a value of 3 for character spacing, you will

have character spacing equal to 6 dit lengths.  If you enter a

value for character spacing, and a 0 for word spacing, you will

have twice the word spacing as you have character spacing.

The eighth parameter is called Limit.

Limit determines whether there will be a

change in the code speed during

a practice session.  It works in the

following way:  If 0 is entered

for limit, there will be no speed change during

the following practice session.  If a value is entered

that is less than the initial speed that

was entered for the speed parameter,

the program will send for a default time of

10 minutes, and then decrease speed by 5 words per minute

until the speed becomes less than limit.  When this occurs, the

program will send "End of code practice session=

Thanks Friend for using Fsend=  73 and good luck*

xx8xxx de k9rh cl" and exit to dos.

This is referred to as the sign off message.

If you enter a value for limit that is greater than the

initial speed, the program will

send at the initial speed for 10 minutes, and then will increase

the sending speed by 5 words per minute and continue this sequence

until the speed becomes greater than limit.  When this occurs, the

sign off message will be sent,

and the program will exit to dos.

The ninth parameter is called speedchange.  It can range from 1 to

20 words per minute.  It controls the amount of change that will

occur in the speed if limit does not equal 0.  It defaults to 5

words per minute.

The tenth parameter determines the length of time

that the program will send at a given speed if limit does not equal

0. It is referred to as length, and can range from 1 to 30 minutes.

It defaults to 10 minutes.

The eleventh and last parameter controls the number of

minutes the program will send before it pauses and asks you to turn

over a tape.  it's default is 480 minutes, which is 8 hours. In

order to make code tapes with fsend, you must put a jack in your

machine.  I used one of the metal plates that go where the cards

would go.  This jack is connected across the speaker in the

computer.  caution!!! make sure the jack is insulated from ground.

Here is an example of  using fsend to read a text file called

fsend.doc.

fsend fsend.doc 25 500 2 15 0 90 0 0 0

This causes the following to happen.

Fsend will send the file fsend.doc at 25 words per minute. The

pitch will be at 500 hertz.  Since weighting is 2, weighting will

be very light.

Since word spacing is set at 15, word spacing will be 60 dit

lengths.   Since character spacing is 0, the

character spacing will be normal.

limit is set to 90, so the speed will continue to increase until

it reaches 90 words per minute.   Speedchange is set to 0, so the

speed will increase in steps of 5 words per minute.  if 0 is

entered for any parameter, that parameter uses its' default value.

Since length is set to 0,  the length of time that the program will

send at  any

given speed will be 10 minutes.  Likewise since number of minutes

is 0, it defaults to 480 minutes, or 8 hours.  the file will be

sent long before that amount of time elapses.  By the way, pressing

the space bar causes fsend to pause the sending until any key

except escape, control-q, or the space bar is pressed.  Escape and

control-q causes the program to display the message "Thanks friend

for using fsend.  73 and good luck.  xx8xxx this is k9rh." and exit

to dos.  any of the other keys on the keyboard will cause fsend to

resume sending.

FSEND'S KEYBOARD MODE.

If you follow fsend by keyboard on the command-line, you will

put fsend into keyboard mode. You will be told that fsend is in

keyboard mode, and you should press any key to continue, and then

press f1 for help.  The following commands are available from

keyboard mode.

Control-a puts fsend into message mode.  This is as if you

typed  message after fsend on the command-line.  you can type in

any message of up to 80 characters, and after pressing return,

fsend will send the message.  Pressing return will repeat the

message.  To exit this mode, type the word end in either case.

Control-b causes fsend to send the current time.  It is like

following fsend by the word time on the command-line.  After fsend

finishes sending the time, it will return to keyboard mode.

Control-c clears the screen.

Control-d displays the current parameters that can be

changed from keyboard mode.

Control-i or the tab key causes fsend to send a 3 by 3 cq 3

times.  the call that is now programmed into fsend is "xx8xxx".

After fsend finishes this, it returns to keyboard mode.

Control-p allows you to change the pitch.  It defaults to 880

hertz.  If you press this key combination by mistake,

pressing return will exit back to keyboard mode without changing

the pitch.  Again, the pitch can range from 100 to 10000 hertz.

Control-q or escape  causes fsend to display the message "thanks

friend for using fsend.  73 and good luck.  xx8xxx this is k9rh."

and exit to dos.

Control-r causes fsend to reset all parameters that can be

changed from keyboard mode to their defaults.  The following are

fsend's defaults.  wpm 20, pitch 880, limit 0, length 10,

speedchange 10, weighting 23, word spacing 1, normal spacing of 4

dit lengths, character spacing 2, normal character spacing of 2 dit

lengths, number of minutes, nm, 480 or 8 hours. Control-r resets

pitch, speed, and weighting.

Control-s allows you to change the speed.  it can range from

5 to 95 words per minute.  If you accidentally enter this mode,

just press return to return to keyboard mode.

Control-t Checks speed calibration.  It does this by sending

dits for 1 minute using the computers clock to time it.  While it

sends dits, it keeps track of the number it has sent, and divides

this value by 25 to get the speed.  After it finishes, it

displays the speed in words per minute, and returns to keyboard

mode.  I used a different time function to time this procedure, and

found the speed was not accurate  above 95 words per minute, so

that is why I increased the upper limit for speed.  Please notify

me if the speed is off more than 2 words per minute at 95 words per

minute or below.

Control-u enters random code generation mode.  Pressing any key

except escape, control-q or the space bar  should return you to

keyboard mode. If fsend is sending a text file, or random code, the

space bar will pause the sending, and any other key except Control-

q or Escape will resume the sending.

Control-v shells to dos.  To return to fsend, type exit at the

dos prompt.

Control-w allows the weighting to be changed.  it can range

from 1, very light weighting, to 45, very heavy weighting.  If you

enter this mode by accident, just press return to exit to keyboard

mode without changing anything.

Control-y sends the quick brown fox jumps over the lazy dog's

back, and exits to keyboard mode.

RANDOM CODE GENERATOR

Fsend's random code generator can be entered in two ways. You

can enter it either through keyboard mode, or from the

command-line.  Entering from the command-line allows you to

generate random code practice at multiple speeds just like you can

send a text file at multiple speeds.

FOLLOWING IS A LIST OF CHARACTERS YOU CAN PUT INTO TEXT FILES, AND

WHAT THEY SEND.

All letters and numbers send their correct characters.

! sends dit dit dit daah dit.

$ sends dit dit dit daah dit dit daah.

% sends daah dit daah dit daah.

^ sends dit daah dit dit daah.

& sends dit daah dit dit dit.

* sends dit dit dit daah dit daah.

( sends daah dit daah daah dit.

) sends daah dit daah daah dit daah.

- sends daah dit dit dit dit daah.

= sends daah dit dit dit daah.

+ sends dit daah dit daah dit.

> sends dit daah dit daah.

" sends dit daah dit dit daah dit.

' sends dit daah daah daah daah dit.

REGISTRATION

Fsend is not free, but it is shareware. Registration is

$10.00.  You will never have to pay for an upgrade.  If you have a

modem, I will put your call and name in any upgrades and upload

them to you.  If you want the upgrade on a disk, you must furnish

the diskette and mailer.  Your upgrade will be returned within 24

hours of my receipt of the diskette.  I can supply fsend in all ms-

dos formats.

Registering fsend gets your name and call programmed into the

program.  It also makes you feel good that you have helped this

programmer who has spent about 3 years getting this program where

it is today.  You will also receive unlimited telephone support.

Thanks for registering in advance.

Send checks drawn on an US bank or a money order made out to

Rick Hayner to the following address.

Rick Hayner

4410-A Lilac Lane Apt. 144

Kalamazoo, Mi  49006

You may also phone me between the hours of 10 a.m. to 10 p.m.

eastern time at

(616) 372-1847.  I'm sorry, but I can't accept credit card

orders.

CHANGES  IN THIS VERSION.

The calibration routine was modified to send a string of dits

for 1 minute.

As I said earlier in the manual, I increased the upper limit

for speed and limit to 95 words per minute, because the accuracy

was not good above 95 words per minute. The % now sends daah dit

daah dit daah.  I changed this to make it like the PK232.  The ^

now sends dit daah dit dit daah.  This character is used between

the whole number and fraction of a mixed number.

FINAL COMMENTS

This program was written in Borland's Turbo C.  It should run

on any ibm compatible machine.  The program is only about 40k in

size. I am also an Amateur Radio operator, and have been for 31

years. My call is K9RH.

I will now wish you the best of 73 and good luck learning the

code or getting your speed up.  I wish you years and years of fun

and enjoyment in the Amateur radio hobby.#