___________________________________________________________________________

STACKER NOTE                             Stac Electronics Technical Note

Subject: Stacker and Anti-Virus Programs.

Tec037 - 6/22/92

____________________________________________________________________________

Stacker is compatible with anti-virus programs such as Norton Anti-Virus (NAV)

and Central Point Anti-Virus (CPAV).  However, there a few guidelines to

follow for them to peacefully coexist.

I. Installing Stacker.

Before installing Stacker, disable any TSRs and Anti-Virus programs that are

memory resident.  Device drivers such as NAV&.SYS, NAV_.SYS, and VWATCH.SYS

should be disabled by REMarking the drivers in Config.sys and rebooting.

Failure to disable these could result in an incomplete installation.

II. Configuring Device Drivers.

It is generally recommended that any anti-virus devices be loaded after the

Stacker device in Config.sys.  Loading devices such as NAV_.SYS before Stacker

may cause the system to hang while loading the driver.

III.  Immunization.

CPAV allows you to "immunize" executables by adding signature information to

the file. Do not immunize the Stacker driver files.  These files use their

own signature information for a checksum self check at boot time. Immunizing

the files disrupts this checksum information and prevents them from loading.

Exempt the files from immunization by adding them to the immunization

exception list.  Add the following files: STACKER.COM, SSWAP.COM, STACKER.XT,

STACKER.AT, STACKER.MC.  If these files have been immunized accidently,

restore them from the Stacker diskette or remove the immunization.

IV. BOOTSAFE from Central Point Anti-Virus:

Bootsafe is Central Point's partition table and boot sector security

system. Usually loaded in Autoexec.bat, it examines the boot sector and

partition information of the default drive (usually C:).  If Stacker is

swapping drive letters, Bootsafe it will attempt to read this information

on the Stacker logical drive and report: "WARNING partition/boot sector has

been modified!"   This solution is simply to make Bootsafe look at the

original boot partition.  The syntax is:     BOOTSAFE d:

(where d: is the letter of the SSwapped boot drive.)

-----------------------------------------------------------------------------

1992 STAC ELECTRONICS