������������������������������������������������������������ͻ

�                 M I C R O S C A N   2 . 0                  �

�                                                            �

�      The shareware SSTV & FAX program for the IBM-PC       �

�                                                            �

�                (c) Copyright 1995 CombiTech                �

�     Morelstraat 60, 3235 EL Rockanje, The Netherlands      �

�  This program may be copied in its original integral form  �

������������������������������������������������������������ͼ

To enable printing to any kind of printer, all special printer

codes except formfeed (12), and all graphic symbols and

pictures have been removed from this document. Some codes have

been replaced with the following text:

{up}   arrow up symbol

{down} arrow down symbol

This document is formatted to be printed double-sided, with 58

lines per page. Every new page is preceded by a formfeed.

Type the following command to print this manual:

PRINT MSCAN-EN.DOC

TABLE OF CONTENTS

=================

1 INSTALLATION  . . . . . . . . . . . . . . . . . . . . .     1

1.1 Installing the MSCAN software . . . . . . . . . . . .     1

1.2 Installing the Multiscan interface  . . . . . . . . .     4

1.3 Installing the Microscan interface  . . . . . . . . .     4

1.4 Adjusting the hardware (Multiscan only) . . . . . . .     6

1.5 Computing the calibration factor  . . . . . . . . . .     8

2 OPERATION . . . . . . . . . . . . . . . . . . . . . . .     9

2.1 Screen layout . . . . . . . . . . . . . . . . . . . .     9

2.2 Selecting the program mode  . . . . . . . . . . . . .    10

2.3 Loading and saving pictures . . . . . . . . . . . . .    10

2.3.1 Setting the path  . . . . . . . . . . . . . . . . .    10

2.3.2 Loading a picture from disk . . . . . . . . . . . .    11

2.3.3 Loading a picture from the auto directory . . . . .    12

2.3.4 Loading a picture from memory . . . . . . . . . . .    12

2.3.5 Loading a standard image  . . . . . . . . . . . . .    12

2.3.6 Loading a picture from the Iris videodigitizer  . .    13

2.3.7 Saving a picture to disk  . . . . . . . . . . . . .    13

2.3.8 Saving a picture to memory  . . . . . . . . . . . .    14

2.4 Text editor . . . . . . . . . . . . . . . . . . . . .    14

2.5 Receiving and transmitting pictures . . . . . . . . .    15

2.5.1 Receiving pictures in SSTV modes  . . . . . . . . .    15

2.5.2 Transmitting pictures in SSTV modes . . . . . . . .    17

2.5.3 Receiving pictures in FAX modes . . . . . . . . . .    18

2.5.4 Transmitting pictures in FAX modes  . . . . . . . .    21

2.6 Processing pictures with the toolkit  . . . . . . . .    22

2.7 Managing the spectrum . . . . . . . . . . . . . . . .    22

APPENDIX I - TECHNICAL INFORMATION  . . . . . . . . . . .    25

III

A Start- and stoptones  . . . . . . . . . . . . . . . . .    25

B Troubleshooting . . . . . . . . . . . . . . . . . . . .    25

APPENDIX II - MODE DESCRIPTIONS . . . . . . . . . . . . .    27

A SSTV  . . . . . . . . . . . . . . . . . . . . . . . . .    27

B FAX . . . . . . . . . . . . . . . . . . . . . . . . . .    28

MSCAN 2.0   Copyright 1995 CombiTech

CombiTech,

Morelstraat 60,

3235 EL  Rockanje,

The Netherlands,

Tel/Fax: +311814-4252.

Agents:

Australia           Kevin Cavanagh VK4SP, 222 Brisbane Valley

& New Zealand       Highway, Wanora, Queensland 4306, Tel/Fax:

+61-74-643954

Belgium             Spitz telecom, P. Verhaegheplein 15, 8310

Bruges, Tel: 050-353639, Fax: 050-353938

Denmark             Norad Tele-center a/s, Frederikshavnsvej

74, DK-9800 Hj rring, Tel: 98909999, Fax:

98909988

Germany             Garant-Funk, Kommerner Str. 119, 53879

Euskirchen, Tel: 02251-55757, Fax: 02251-

59214

Switserland         ALINAG, Dammweg 53, 5000 Aarau,

Tel: 064-227066, Fax: 064-241263

United Kingdom      AMDAT, 4 Northville Road, Northville,

Bristol, BS7 ORG, Tel: 01179-699352, Fax:

01179-872228

The Graphics Interchange Format(c) is the Copyright property

of Compuserve Incorporated. GIF(sm) is a Service Mark Property

of Compuserve Incorporated.

IV

1 INSTALLATION

MSCAN 2.0 is a program for receiving and transmitting SSTV and

FAX images. For convenient operation, MSCAN 2.0 requires an

IBM-compatible computer with at least a 80386 processor, SVGA

video capable of displaying in truecolour and a Microsoft

compatible mouse.

MSCAN is available in two versions: one for the Multiscan

modem, and one for the Microscan modem. When a description in

this manual is only intended for one of these versions, it is

marked with '[Multi]' or '[Micro]'.

Multiscan [Multi]

The Multiscan modem consists of two boards (one for receiving

and one for transmitting) which can be connected to each

other. The receive board can also be used stand-alone. The

Multiscan uses a high-quality PLL circuit for decoding and a

sinewave generator for generating signals. This ensures the

best pictures even under worse conditions.

Microscan [Micro]

This is a very simple interface circuit which consists of a

single OPAMP for decoding signals. Signals are generated using

the PC speaker. This interface is also known as HAMCOMM or JV-

FAX interface. The Microscan version is also distributed as

shareware. When a description in this manual only refers to

the shareware version, it is marked with '[share]'.

Some parts of this manual only apply to the receive & transmit

(TRX) version of MSCAN, these parts have been marked with

'[TX]'.

MSCAN is a unique program. No other program offers the multi-

tude of multitasking possibilities you will find in MSCAN.

While you are transmitting or receiving a picture, MSCAN

enables you to load another picture, or digitize it, and

overlay it with text. To make all of this possible, MSCAN uses

many of the advanced (often not used) features your computer,

graphics adapter and mouse have to offer. It is therefore

absolutely necessary that these parts have a high degree of

compatiblity and that MSCAN will not be influenced by any

other program which would be present in memory (so called

resident programs).

MSCAN consists of two files:

MSCAN.EXE - the main program

MSCAN.CNF - the configuration and registration file.

1.1 Installing the MSCAN software

P.S.:

If a memory manager like QEMM, EMM386 or 386MAX has been

installed in your computer, then disable it first. On some PCs

memory managers can greatly influence timing. This can cause

1

MSCAN to function incorrect or even not at all! You can dis-

able the memory manager by placing the text 'REM ' in front of

where the command is placed in your AUTOEXEC.BAT or CONFIG.SYS

file. Then restart your computer and run MSCAN and check its

proper functioning. If all is ok, then enable the memory

manager, reboot your system and run MSCAN again. If you detect

any problems, or a degraded picture quality, then you cannot

use this memory manager while running MSCAN.

The MSCAN software is supplied on floppy disk which should be

copied to the computer hard disk before use. At the DOS prompt

type :

MD \MSCAN

CD \MSCAN

Now copy the software from the distribution disk to this new

directory, e.g.:

COPY A:*.*

Before the software can be used it must first be configured.

To enter the calibration menu type:

MSCAN /C      followed by a <Enter>.

A number of questions will be asked in sequence, with each

entry the current status is reported (...). If you do not want

to change a parameter a simple <Enter> is sufficient.

SVGA chipset 1=Trident 8900D  4=ET4000(VESA)  5=PVGA1/WD90Cxx

7=Cirrus  8=VESA

(...) :

Select the chipset which is present on your graphics adapter.

If MSCAN does not support your chipset select VESA. The

following question will then be asked:

0=standard VESA  1=hybrid#1  2=hybrid#2  (...) :

Select standard. If you do not get the correct image on your

display, then try starting MSCAN again with the hybrid#1 and

hybrid#2 settings. It is possible that your graphics adapter

requires a VESA driver to be loaded in order to be VESA

compatible.

[Multi]

For the Multiscan the next question will appear:

Spectrum calibration factor low  (...) :

Spectrum calibration factor high  (...) :

If you have a previous version of MSCAN, enter the same values

here which are displayed in the configuration menu of your

previous version. The calibration factors are used for the

calibration of the spectrum, and interrelated to this the

2

calibration of the demodulator. If a ready built unit has been

bought the figures will already have been entered for your

interface. They should not be changed unless the unit is to be

recalibrated. Unless you need to change these values simply

press <Enter>.

[Micro]

For the Microscan version you can now select which type of

interface you want to use.

RX interface 1=DTR+/RTS-/PTT  2=RTS+/DTR-  (...) :

For the Microscan and HAMCOMM interface select type 1. For the

(older) JV-FAX interface, which has no PTT connection, select

type 2.

COM port 1/2  (...) :

Enter the serial port number on the PC to which you have

connected your hardware. Only port 1 and 2 are supported.

Initial program mode 1=SSTV B&W  2=SSTV Wraase  3=New SSTV

4=B&W-FAX  5=Colour FAX  (...) :

Select the initial program mode. MSCAN will auto-select this

mode after startup.

RX display sequence 1=1  2=2  3=1&2  (...) :

TX display sequence 1=1  2=2  3=1&2  (...) :               [TX]

These questions allow you to determine the default receive and

transmit screen settings in the SSTV modes. You can always

change these settings afterwards in the program itself.

Default path to picture directory : ...

The default directory in which you want to store your SSTV and

FAX pictures. You may choose for example the current directory

(.\) or a directory named 'pictures'.

Crystal calibration factor  (...) :

Both new SSTV and FAX modes rely heavily on synchronism

between transmitter and receiver, for this the PC has to be

able to precisely set timing. Therefore you will have to

calibrate the PC. The computed correction factor (see

'Computing the calibration factor') can be entered here. If

you have already computed this factor for a previous version,

simply enter the old factor here.

Call to be transmitted  (...) :                         [share]

For the shareware version of Microscan you can now enter your

callsign which is transmitted with every picture.

3

1.2 Installing the Multiscan interface                  [Multi]

First install the Multiscan interface by connecting it to a

suitable radio and to an unused serial port on the PC.

Radio connections

Pin 1     -    transmit audio (connect to MIC

input of transmitter)

Pin 2     -    ground connection

Pin 3     -    receive audio (connect to audio

output of receiver)

Pin 5     -    PTT (connect to PTT of

transmitter)

Computer connections

Pin 2     -    Receive Data from

Multiscan

Pin 3     -    Transmit Data to Multiscan

Pin 5     -    ground

1.3 Installing the Microscan interface                  [Micro]

If you do not have a Microscan-like interface, you will have

to build one yourself. Please refer to the Microscan schema-

tic. Connect CN1 to the serial port of your PC (COM 1 or COM

2). Connect your receiver to 'audio in', and your transmitter

(mic or line input) to 'mic input'. Connecting the PTT

connection to the PTT input of your transmitter will enable

MSCAN to remote control it. Now connect the wire labelled

'speaker' to one of the speaker terminals of your PC. Connect

the wire labelled 'ground' to the chassis of the PC. Check the

audiosignal which is applied to your transmitter and adjust

its volume with R3. If the sound is weak and distorted,

connect the 'speaker' wire to the other speaker terminal.

4

DEMODULATOR

===========                                       RS-232 PC

(25p) (9p)

� /�

����Ĵ< ����� DTR (20)  (4)

�    � \�

�\�

��100n         3� \7   1N4148

audio �����Ĵ��������������Ĵ  \ 6

in    �  ��   �          �741>���������� DSR (6)   (6)

�      ��    ����Ĵ  /

�      � �    �   2� /4   1N4148

�����    � �    �    �/�

�����    � �    �      �    �\ �

� 10n  � �    �      ����Ĵ >����� RTS (4)   (7)

�      ���    �           �/ �

�       �100k �

ground ������������������������������������� GND (7)   (5)

PTT-CIRCUIT

===========

PTT ����������Ŀ

�       1k          1N4148

\�   �����Ŀ   � /�

���Ĵ     ���Ĵ< ������� RTS (4)   (7)

/�   �������   � \�

�

� BC547B

ground ������������

MODULATOR

=========

100n   2k2         2k2

��  �����Ŀ     �����Ŀ

speaker ���Ĵ��Ĵ     �����Ĵ     ����������Ŀ

pc (1)      ��  �������  �  �������   �      � 1k

�            �     ��

�����        �����   � �

�����        �����   � �<��� mic input

�            �     � �

� 100n       � 100n���

�            �      �

ground  ������������������������������������������� ground

pc (2)

(1) connect to speaker terminal with strongest signal

(2) connect to PC chassis

All C's ceramic

This interface is designed to be used with the Microscan

version of MSCAN, and is equipped with a PTT connection.

5

1.4 Adjusting the hardware (Multiscan only)             [Multi]

RECALIBRATION OF THE (DE)MODULATOR IS NOT REQUIRED IF A BUILT

AND TESTED UNIT HAS BEEN BOUGHT!

Before using the program and matching hardware you may have to

adjust a few things. For adjusting the demodulator a software-

approach is used. The demodulator can be found on the Multi-

scan receive board. The modulator uses a hardware adjustment

by means of adjustable resistors. The modulator is placed on

the transmit board of the Multiscan. The adjustments are

necessary in connection with tolerances in the used parts.

Calibrating the demodulator

Calibration of the demodulator should only be required if you

have just completed building the interface unit or you have

changed any parts. If you have bought a built unit the

software should already be calibrated for the interface.

When calibrating the demodulator you are in fact adjusting the

software to match the characteristics of the hardware (soft-

ware calibration). Calibration should be done using a tone

generator which has an adjustable frequency. It is however

also possible with the help of another amateur station.

Use the following steps for calibration:

1)   Switch the program to a SSTV mode and press the spectrum

menu button.

2)   Apply a 1200 Hz tone to the audio

input of the Multiscan. A bar will

appear in the spectrum. In the top-

right corner the frequency is

displayed, this will have to be

1200. You do this by clicking on the

lower and  higher menu buttons. With these you can shift

the entire spectrum downwards or upwards. The bar will

now be positioned exactly under the syncindicator.

3)   Apply a 2300 Hz tone. To get the

frequency '2300' displayed in the

spectrum use the  expand and  shrink

buttons to expand or shrink the

entire spectrum. The bar will now be

positioned above the right marker

(dot on the right).

4)   When ready repeat the entire procedure again. Now you can

write the calibration factors to the configuration file

using the save button.

If you do not have access to a tone generator you can also ask

another amateur to transmit the tones for you, if he also uses

6

MSCAN this will be no problem since the program has a special

feature for this purpose. In this case simply connect the

Multiscan to the audio output of your receiver.

It is also possible to calibrate the demodulator using a high-

quality SSTV signal. This will however give a less exact

result. The procedure for calibrating using an SSTV signal is

not much different:

1)   Apply the SSTV signal to the Multiscan. Use the  lower

and  higher buttons to get a good synchronization (1200

Hz adjustment). You will now see a bar under the

syncindicator.

2)   Use the  expand and  shrink buttons to get the right

greyshades in the picture. These values have to be spread

all over the 1500-2300 Hz part of the spectrum (between

the two markers).

3)   You can now store the acquired calibration factors in the

configuration file using the save button.

Calibrating the modulator [TX]

Calibration of the modulator should only be required if you

have just completed building the interface unit or you have

changed any parts. If you have bought a built unit the

modulator should already be calibrated.

The modulator is calibrated as follows:

1)   Switch the program into the transmit mode by clicking on

the RX indicator. The indicator will now show TX.

2)   Enable the spectrum by clicking the spectrum menu button.

The buttons 1200, 1500 and 2300 allow you to generate the

tones 1200, 1500 and 2300 Hz.

3)   Select the tone 1200 Hz. Measure the frequency of the

signal that the modulator generates. You can do this at

the output of the board. The outputlevel can be adjusted

with R4 on the Multiscan transmit board. If your counter

is not sensitive enough you can also measure the signal

at R3. Adjust the frequency with R7 to exactly 1200 Hz.

4)   Now select a tone of 2300 Hz. Adjust the measured

frequency with R6 to exactly 2300 Hz.

5)   Repeat the previous procedure. The modulator is now

precisely calibrated. When you now select a tone of

1500 Hz you should measure this frequency at the output.

A deviation of a few Hz is allowed.

7

6)   Press your right mousebutton to leave the spectrum mode,

and click the TX indicator twice to switch back to the

receive mode.

If you do not possess a frequency counter you can also simply

transmit the signal and ask another amateur to measure it for

you. If this amateur also uses MSCAN then this is easy: he

only has to press the spectrum button and read the displayed

frequency.

1.5 Computing the calibration factor

Precise timing is vital for especially the new SSTV and FAX

modes. A slight deviation in the crystal frequency of the PC

will result in bad synchronization (picture skew) when

receiving pictures. If you detect this picture skew when

receiving 'commercial' FAX stations (i.e. calibrated stations)

then the crystal in your PC will have a slight deviation. This

deviation can be compensated with the calibration factor,

which is entered in the configuration menu. This is done using

the following method:

1.   Switch the program to FAX mode by clicking on the program

mode indicator.

2.   Select the param menu and use the +lpm and -lpm buttons

to receive a picture without skew.

3.   Click on the calib button and confirm by clicking calib

again.

4.   Repeat steps 2 and 3 until you cannot detect any skew

anymore.

Beware that commercial stations can transmit skewed pictures,

this is not caused by deviation in linetime but simply because

someone placed to chart askew in the FAX machine! A good

alternative is to use a timepulse station (look around 10 MHz)

which will transmit a bleep e.g. every second. These bleeps

appear as small lines on your screen. When your calibration is

ok, these lines must appear directly under each other.

[picture]

8

2 OPERATION

To start the main program type

MSCAN     followed by <Enter>.

The program will enter the default mode which was defined

during configuration.

2.1 Screen layout

In the SSTV modes most of the screen is filled with two

screens, numbered 1 (left) and 2 (right). These screens are

used to display incoming and/or outgoing images. Below this

the 14 memories are displayed. Since FAX modes require display

of high-resolution pictures, here only one big screen is

visible.

[picture]

At the bottom of the screen you can see the console, which is

made up of the spectrum, the indicators and the menu buttons.

The spectrum shows the in- or outgoing audiosignal. The

spectrum contains a sync-indicator. When a correct synchroni-

zation is established this indicator lights up. In SSTV modes

you can see three markers at the bottom of the spectrum, these

9

mark the frequencies (from left-to-right) 1200, 1500 and 2300

Hz. In FAX modes the 1200 Hz marker is omitted since it has no

function. The other two markers either mark 1500 and 2300 Hz

for 800 Hz shift or 1750 and 2050 Hz for a 300 Hz shift.

In the console of MSCAN you can find both (menu)

buttons and indicators. Menu buttons are visible in

the lower part of the console and make up the menu.

Indicators can have a display function only, or can

have both a display and button function. In the

latter case 'pressing' an indicator will activate a

corresponding function, or will select a new setting. You can

'press' a button or indicator by positioning your mousepointer

above it and click your left mousebutton. Some indicators have

more than one function, by repeatedly clicking on the indica-

tor other functions or settings are shown. A function or

setting will become active when the indicator is not changed

for one second, or when another indicator is clicked. The

right (and middle) mousebutton have an escape function and are

used to leave a (sub)menu.

2.2 Selecting the program mode

B&W SSTV ->  Wraase  -> New SSTV -> B&W FAX  -> Col FAX

To the right of the spectrum you can find the program mode

indicator, which shows the current program mode (like SSTV or

FAX). Clicking on it selects another program mode.

2.3 Loading and saving pictures

You can load a picture from disk, from the auto directory or

from one of 14 memories (which are in fact stored on disk).

MSCAN will load pictures in GIF and JPEG format. Because of

the high datacompression pictures will always be saved in JPEG

format. Pictures can be loaded and saved at all times, even

when receiving or transmitting another picture. A picture can

be loaded into the entire screen or just to a part of it

(called a box). In the same way the entire screen or just a

part of it can be saved. MSCAN will automatically scale

pictures to fit in a screen or box. In b&w modes picture

colours will be translated to greyvalues.

2.3.1 Setting the path

picture   ->    Path

Here you can temporary change the path to the picture

directory which you entered in the configuration menu. This

10

path is used when loading or saving a picture from/to disk.

After selecting Path a cursor will be visible, you can now

enter the new path, e.g. 'a:' to read a picture from disk a:.

2.3.2 Loading a picture from disk

picture   ->  diskLoad  ->   screen

box

multi

delete

Depending on the mode you are in, you will be able to select

screen 1 or screen 2, or only one screen. If you do not want

the picture to fill the entire screen but just a portion of

it, select box. Press the left mouse button to select the top

left corner of the box, then drag the box to size and press

again the left mouse button. The picture will be loaded and

fits within the selected box size.

After your selection a cursor will appear: you can now enter

the name of the picture to be loaded, or press <Enter> or left

mousebutton and pick a file from the directory. Use the {up} and

{down} buttons to show more entries.

If you want to load multiple pictures, select multi. In SSTV

modes MSCAN will now load pictures to screens 1 and 2 alterna-

tely. When transmitting, picture load is synchronized with

picture transmission. When both screens are selected for

transmission, this feature enables you to transmit several

picture in a row. Pictures are loaded alphabetically, starting

with the picture you selected.

In FAX modes a collection of small pictures will be showed

across the screen.

Press the delete button if you want to delete a picture. If

you have loaded a picture previously and not leaved the

picture menu, the name of this picture will be showed now. You

can use this name, or enter a new one, or press <Enter> or

left mousebutton to pick a picture from the directory. Use the

{up} and {dpwn} buttons to show more entries.

11

2.3.3 Loading a picture from the auto directory

picture   ->  Autoload  ->   screen

box

multi

delete

This works exactly the same as loading a picture from disk,

only the picture is not loaded from the picture directory (see

configuration menu) but from the auto directory. This is the

directory where pictures are stored that are received in the

SSTV or FAX auto mode.

2.3.4 Loading a picture from memory

picture   ->  memRead   ->   screen

box

Pictures which are stored in memory are in fact stored on disk

in a special format. This format enables the pictures to be

loaded and saved very fast and without loss of information due

to compression.

The procedure to load a picture from memory is almost the same

as loading a picture from disk. The only difference is that

you do not have to enter a name, but immediately can pick a

memory from the directory.

showmems

In SSTV modes you can see 14 little boxes in the lower part of

the screen. These boxes display the contents of all 14

memories. To (re)load these boxes with the contents of the

memories press the menu button showmems.

2.3.5 Loading a standard image

picture   ->   Image    ->   screen

12

A number of images are available in MSCAN. You can use these

images for testing or just for a background.

Depending on the mode you are in, select screen 1 or screen 2,

or only screen and then pick the desired image from the

directory. There is one special image called spectrum.

Choosing this image will copy the current spectrum to the

selected screen. You can use this to show your opposite

station his spectrum.

2.3.6 Loading a picture from the Iris videodigitizer

digitize  ->   screen

box

multi

input

One of the unique features of MSCAN is that you are able to

digitize video-images just as easy as loading pictures from

disk, even when receiving or transmitting a picture! To be

able to do this the optional Iris videodigitizer has to be

installed in your PC.

The procedure to digitize a picture is almost the same as

loading a picture from disk. Just select the desired screen or

box and the picture will be digitized and loaded. Use input to

select either the composite or the S-VHS/Hi-8 input. The multi

command works the same as that when loading a picture from

disk. The FAX modes have no multi command available for

digitizing.

2.3.7 Saving a picture to disk

picture   ->  diskSave  ->   screen

box

Depending on the mode you are in, you will be able to select

screen 1 or screen 2, or only one screen. If you do not want

to save the entire screen but just a part of it, select box.

Press the left mouse button to select the top left corner of

the box, then drag the box to size and press again the left

mouse button. After your selection a cursor will appear: you

can now enter the name of the picture to be saved. If a file

with this name already exists, you will be asked if it is ok

13

to overwrite this file. Select overwr if you want to, or press

again if you want to enter a different name. The picture will

now be saved to disk.

2.3.8 Saving a picture to memory

picture   ->  memWrite  ->   screen

box

The procedure to save a picture to memory is almost the same

as saving a picture to disk. The only difference is that you

do not have to enter a name, but immediately can pick a memory

from the directory.

2.4 Text editor

MSCAN has a built-in screeneditor which can be used even while

receiving or transmitting a picture. Simply type a character

on your keyboard and it will be placed at the position which

is marked by the textcursor. You can move the cursor around

with the cursor keys on the keyboard. The cursor will

disappear if during 5 seconds no character has been typed.

Pressing a key will make the cursor visible again. The next

keys also control the position of the cursor:

<Home>         moves the cursor to the top-left corner of the

screen

<End>          moves the cursor to the bottom-right corner of

the screen

<Ctrl>+< >     moves cursor to the right of the screen

<Ctrl>+< >     moves cursor to the left of the screen

<Backspace>    moves the cursor one position to the left and

erases that position

<Del>          clears screen and moves the cursor to the top-

left corner of the screen

<Tab>          only for SSTV modes: moves the cursor to the

other screen

You can control the character's size and colour by clicking on

the fontsize and -colour indicators.

t   s   m   l   t   s   m   l

The fontsize indicator shows which size is selected: tiny,

small, medium or large. If the indicator is blue the

characters will show transparent on the screen, if the

indicator is black the characters will erase the background.

14

c   c   c   c   c

|   |   |   |   |

|   |   |   |   |

|   |   |   |   +-  blue

|   |   |   +-----  green

|   |   +---------  red

|   +-------------  black

+-----------------  white

The fontcolour indicator shows the colour of the characters.

[TX] In B&W modes coloured characters will be transmitted in

their corresponding greyvalue.

2.5 Receiving and transmitting pictures

MSCAN will be in RX mode after startup. Click on the program

mode indicator to select the mode which you want to use to

receive or transmit pictures.

2.5.1 Receiving pictures in SSTV modes

Click on the mode indicator, which is to the right of the

program indicator, to select the desired SSTV mode.

B&W SSTV:

7.2 ->  8  -> 16  -> 32

Wraase SSTV:

24 -> 48 -> 96

New SSTV:

M1 -> M2 -> S1 -> S2

Select the screen that you wish to use to display the received

pictures, by clicking on the display sequence indicator. You

can find this indicator to the right of the large message

indicator. It has four settings:

1X -> X2 -> 12 -> XX

15

1X   display pictures on screen 1 (left)

X2   display pictures on screen 2 (right)

12   display pictures alternating on both screens

XX   pictures are not displayed, reception is off

Incoming SSTV signals will now be displayed fully automatic on

the selected screen(s), leaving MSCAN available for other

tasks. Should manual control of reception be required, you can

use the indicator to the left of the message indicator and the

menu button param. The indicator has multiple functions,

depending on the SSTV mode. Each function can be selected by

clicking one or more times on the indicator.

In B&W and New SSTV modes:

top -> cls

In Wraase SSTV mode:

rgb -> top -> cls

rgb  only available in Wraase mode: kicks r-g-b sequence one

colour back (causes the colour of the receiving picture

to change), pressing the <Ins> key has the same effect

top  start reception at the top of the screen, pressing both

the <Ctrl> and <Home> keys has the same effect

cls  does the same and also clears the screen

The New SSTV mode has an extra indicator, which is positioned

to the right of the mode indicator.

syn -> lin

Default setting is syn(chrone), which meanse the reception of

a picture is started after receiving the frame startpulse (or

VIS code). After that reception is fully synchronous and will

continue until the next startpulse is received. In lin(e) mode

every line is written after receiving a linesync pulse, just

like in B&W SSTV modes. This mode is more sensitive to noise

but can be useful in the next cases:

1.   If the receiving signal is not crystal-controlled (like

from a taperecorder) synchrone mode will not work.

2.   If a startpulse is not received ok the receiving picture

can be shifted to the left or the right. Selecting line

mode for a second to achieve line synchronisation and

then switching back to synchrone mode will position the

rest of the picture ok.

16

param

The param menu button's functions also depend on the selected

SSTV mode. In both the B&W and Wraase SSTV modes clicking the

param button toggles between manual and autosync mode. The

manual mode is default. In this mode every line is written

after reception of a linesync pulse, and every image is

started after reception of the framesync pulse. If signals are

very noisy and thus no synchronisation is achieved, selecting

the autosync mode will make reception less sensitive to noise.

By looking 'intelligent' to the synchronization of the SSTV

signal, display of received images will improve, which is

especially helpful in the Wraase colour SSTV mode to prevent

coloursync hickups. This mode however only works when the

transmitting stations have correct timing.

In the New SSTV mode clicking the param menu button will

select another menu.

param    ->   + lpm

- lpm

left

right

RGB

auto

You can temporary change the linetime by clicking on the +lpm

and -lpm menu buttons. Move reception to the left or to the

right by clicking on the corresponding buttons. Click RGB to

change the colour.

The 'auto' command activates the auto mode. In this mode MSCAN

automatically starts reception in both screens and saves the

pictures in the auto directory afterwards, using the filename

mmddhhmm (month|day|hour|minute).

2.5.2 Transmitting pictures in SSTV modes                  [TX]

Click on the mode indicator, which is to the right of the

program indicator, to select the desired SSTV mode.

17

Select the screen that you want to transmit by clicking on the

tx display sequence indicator. You can find this indicator to

the right of the rx display sequence indicator. It has three

settings:

X2 -> 12 -> 1X

X2   use screen 2 (right) for transmisson

12   use both screens alternating for transmission

1X   use screen 1 (left) for transmission

Now click on the RX/TX indicator to start the transmission.

The transmission will stop automatically at the end of the

picture (indicated by the message 'auto stop'). By clicking

twice on the RX/TX indicator transmission will be continuous.

The picture(s) will be transmitted fully automatic, leaving

MSCAN available for other tasks such as preparing the next

picture or adding text to the picture.

Click on the RX/TX indicator until it shows RX if you want to

abort the transmission.

2.5.3 Receiving pictures in FAX modes

To fully understand the operation of some functions in the FAX

modes, first the terms 'scrolling' and 'endmarker' will be

explained.

Scrolling

To be able to display long FAX pictures which can not be

displayed on the standard full screen size, MSCAN uses

scrolling (vertical movement) of a picture, which enables the

rest of the picture to be shown. This scrolling is done mostly

automatic. Only when the cursor is visible, no scrolling will

take place (unless the cursor itself is moved).

The endmarker

The endmarker is used to mark the lower border of the current

picture. The endmarker works mostly automatic. When a FAX

image is received and reception is stopped (in auto mode this

is done automatically) the endmarker will be placed at the

last received line, the same is done after for example a

picture is loaded from disk. Processes like transmitting or

saving use the endmarker to determine the end of a picture.

The endmarker is displayed as a black & white chequered line.

Click on the mode indicator, which is to the right of the

program indicator, to select the desired FAX mode for

reception.

18

In B&W FAX:

120     180     240     60      90

In colour FAX:

240     360     120     180

The indicator shows the number of lines per minute (LPM).

Select the desired IOC by clicking on the IOC indicator, which

is to the right of the mode indicator.

In B&W FAX mode:

288   576   864

In colour FAX mode:

288   204

The Index Of Cooperation indicates the aspect ratio of the

picture. When you select 360 LPM automatically an IOC of 204

will be set.

To the right of the IOC indicator you will find the shift

indicator.

800   300

This is the difference between the frequency for black and the

frequency for white video.

[TX]

When transmitting a shift of 800 Hz is automatically set.

In b&w FAX mode you will find the next indicator to the right

of the shift indicator.

top -> cls

19

In colour FAX mode it has one extra setting:

rgb -> top -> cls

rgb  only available in colour mode: kicks r-g-b sequence one

colour back (causes the colour of the receiving picture

to change), pressing the <Ins> key has the same effect

top  start reception at the top of the screen, pressing both

the <Ctrl> and <Home> keys has the same effect

cls  does the same and also clears the screen

Use the rx display sequence indicator to switch on and off

reception.

1    X

1 = screen 1 (= reception on)

X = no screen (= reception off)

Various parameters in the FAX mode can be changed by clicking

the param menu button.

param    ->   + lpm

- lpm

left

right

Video

Calib

Auto

You can temporary change the linetime by clicking on the +lpm

and -lpm buttons. Move reception to the left or to the right

by clicking on the corresponding buttons.

Click the video menu button to do some realtime image

processing.

20

Video    ->   Invert

Mirror

invert    invert the incoming videosignal (black becomes white

and white becomes black)

mirror    the incoming videosignal will be written to screen

in the opposite direction

You probably will only use these commands for shortwave

reception of commercial (weather)stations in B&W FAX modes.

Auto

For automatic reception of FAX pictures click the auto menu

button. This will activate the FAX auto mode. MSCAN will now

wait for a starttone. Once detected it will determine the

transmissions IOC and mode (b&w or colour). After that it will

wait for the synchronization signal and use it to determine

the LPM and shift and synchronize reception. It will then wait

for the start of the picture and begin reception. When the

stopttone is detected the picture will be saved to the auto

directory and MSCAN will wait for a next transmission.

Pictures will be saved with the following name: mmddhhmm

(month/day/hour/minute).

The auto mode can be aborted by pressing <Esc> or right mouse

button.

2.5.4 Transmitting pictures in FAX modes

After loading or digitizing a picture notice that the endmar-

ker will be automatically placed at the lower border of the

picture. If you want to transmit more or less of this picture,

you can change the position of the endmarker by clicking the

endmarker indicator:

--

Now position your mouse at the desired position and click

again, the endmarker will move to this new position. Optional-

ly select the desired LPM and IOC and press the RX/TX indica-

tor to start the transmission. To abort the transmission click

the RX/TX indicator until it shows 'RX'.

21

2.6 Processing pictures with the toolkit

Press the toolkit menu button to activate all image processing

functions.

toolkit   ->   mirror

colour

aspect

move

draw

debug

Click mirror to swap left and right sides of a screen.

Click colour (only in colour modes) to change the colours of a

picture on screen (colours will be 'rotated').

Click aspect to correct a picture which is received in the

wrong IOC.

Click move to redefine the top-left corner of a picture. This

will be useful in FAX modes if a picture has been received

with bad synchronization.

Click draw to draw white pixels on the screen using your left

mousebutton.

The debug menu button will only be shown in B&W SSTV mode. It

is meant for technical support only.

2.7 Managing the spectrum                               [Multi]

The spectrum occupies the left part of the console and shows

the frequency spectrum of the received or transmitted signal.

In SSTV modes it also features a sync-indicator which is the

little square box above the 1200 Hz marker. Whenever MSCAN is

correctly synchronized to an SSTV station a white dot will

appear in this box.

By pressing the spectrum menu button the following submenu

will be presented.

22

spectrum  ->   shrink

expand

lower

higher

default

load

save

This menu is used when calibrating the Multiscan interface. It

can also be used to (temporary) change the spectrum calibra-

tion, e.g. to compensate for a station which has a frequency

deviation in sync- and or videotones.

The reception spectrum can be shifted using the  lower and

higher menu buttons. Pressing the  lower button decreases all

frequencies (darker video) and pressing the  higher button

increases all frequencies (brighter video). You can also

shrink (less contrast) or  expand (more contrast) the spec-

trum, this will leave the 1200 Hz frequency unchanged. Other

commands are:

d)efault: loads the default calibration

l)load:   loads the last stored calibration

s)ave:    saves current calibration to disk.

[TX]

In transmit mode the spectrum menu button shows the following

submenu.

spectrum  ->   1200Hz

1500Hz

2300Hz

APT

23

Pressing a button causes MSCAN to transmit the corresponding

tone for 5 seconds. Pressing APT shows:

APT     ->   200Hz

300Hz

450Hz

675Hz

Pressing a button causes MSCAN to transmit the corresponding

starttone, which are used in FAX modes.

24

APPENDIX I - TECHNICAL INFORMATION

Here is some technical info about MSCAN, and some very useful

tips to solve problems.

A Start- and stoptones

In accordance with international standards, MSCAN uses the

following start- and stoptones in b&w FAX modes:

IOC  start     stop

288  675 Hz    450 Hz

576  300 Hz    450 Hz

The colour FAX mode has been developed by radio amateurs, and

unfortunately no international standard has been set. However

the following start- and stoptones are semi-standard:

IOC  start     stop

204  200 Hz    450 Hz

288  200 Hz    450 Hz

As you can see both IOCs have the same starttone of 200 Hz.

Because the auto mode of MSCAN uses the starttone to determine

the IOC, it will always select an IOC of 288, except for the

360 LPM mode which is mostly used with an IOC of 204.

B Troubleshooting

Because MSCAN is a complex multitasking program, it will take

up all capacity of the PC. Therefore no extra tasks can be

performed by the PC. Some problems which may occur because of

this are discussed here.

Use of resident programs

Resident programs (permanently present in memory) may cause

extra system interrupts. Some caching programs for harddisk

and/or floppydrives temporary disable the system interrupts.

This is not allowed because this will desturb timing in MSCAN.

Some memory managers (such as EMM386, QEMM or 386MAX) do an

even better job in disturbing timing and may crash the system.

If you experience problems, try booting from a 'clean' DOS

diskette and then start MSCAN. If there are still problems,

try removing hardware (such as scanner cards) from your PC

which could be generating interrupts or DMA requests.

No image on the monitor

Check if you have selected the correct chipset. If you choose

VESA, check whether your graphics adapter requires a VESA

driver to be loaded. Are you sure no memory manager or

25

incompatible mouse is causing the problem? Is your adapter

capable of displaying 16.8 million colours (truecolour) with a

resolution of 640*480?

Interference in reception

If loading a picture in one SSTV screen interferes with

reception (or transmission) in another screen, chances are

some caching program is the cause. If not, probably your

computersystem is too slow. Remove all caches, memory managers

etc. Use of the Multiscan modem instead of Microscan relieves

the computer from one task and probably solves the problem.

Else use a faster CPU or graphics adapter.

If after trying the above you are still unable to get MSCAN to

perform satisfactory then contact your supplier who will try

to help solve your problem.

26

APPENDIX II - MODE DESCRIPTIONS

MSCAN 2.0 supports a number of modes. The outline

specification for these modes is defined below.

A SSTV

SSTV pictures are transmitted only by radio amateurs. They can

be received on HF (e.g. 3730 kHz LSB and 14.230 MHz USB) and

on VHF (e.g. 144.500 MHz FM) and UHF frequencies.

B&W SSTV

SSTV is used by radio-amateurs to transmit images via a narrow

(3kc) bandwidth radiochannel. The image is transmitted in

lines, each line is preceded by a line syncpulse of 1200 Hz.

Each image is preceded by a longer frame syncpulse of 1200 Hz.

The video is modulated on a 1900 Hz carrier with an 800 Hz

shift (1500 Hz is black, 2300 Hz is white).

[picture]

Wraase colour SSTV

Derived from B&W SSTV. To be able to transmit a colour image,

each line is sent three times: one for the red colours, one

for the green colours, and one for the blue colours. The

receiver combines the three lines to one colour line. When

transmitter and receiver are not synchronized, false colours

will appear at the receiver.

New colour SSTV modes (martin and scotty)

Instead of sending three lines, every line is divided into

three parts, one for every colour. After receiving a line

27

syncpulse, the receiver cannot be mistaking about the correct

colour sequence.

[picture]

B FAX

Modulation

FAX pictures on short-wave and long wave are normally

transmitted in FM. That means that the transmitter is keyed

between two frequencies of which the one corresponds to white,

the other to black. When transmitting half tone pictures, the

transmitter frequency is continuously shifted between the

frequency for black and the frequency for white.

Half the difference between the frequency for white and the

frequency for black is called signal deviation. This deviation

is, by standard, 400 Hz for short-wave and 150 Hz for long

wave transmissions. Furthermore, the higher transmitter

frequency should correspond to white. The shift is twice the

deviation.

The FAX signals should be received with an SSB receiver so

that it outputs an FM-modulated audio sub carrier.

This sub carrier is then fed to the interface that converts

the different audio frequencies into digital intensity

information that can be processed by the computer.

Two terms you will have to know are IOC and LPM. IOC stands

for Index Of Cooperation, which is a measure for the vertical

resolution of a picture; an incorrect IOC will result in a

picture which is too short or too long. The speed in which a

picture is transmitted is indicated in LPM, which stands for

Lines Per Minute.

28

Automatic Picture Transmission

Automatic Picture Transmission (APT) is used by commercial FAX

stations to enable a fully automated unattended reception of

the transmitted pictures. At the start of the transmission a

start tone is transmitted for some seconds, which is recogni-

sed by the receiving unit. At the end of the transmission a

stop tone is sent that switches the receiving unit back to

standby mode. The tone which is sent can be used to identify

the IOC of the transmitting station. Most amateurs now also

use APT in their transmissions.

Colour FAX

By looking at colour SSTV modes, radio amateurs have developed

a method of transmitting colour FAX images. Every line is sent

for this purpose three times: one time for the red colours,

one time for the green, and one time for the blue colours.

Commercial FAX stations can be received all over the long- and

shortwave bands, e.g. on 134 kHz (USB), 4778 kHz (USB) and

8145 kHz (USB). Radio amateur FAX transmissions can be

received on 14.230 MHz (USB) and 144.700 MHz (FM).

Enjoy MSCAN!

Mike Versteeg PA3GPY

CombiTech

29