SYNTHIA ver. 0.1(alpha)

Written by

Michal Wiernowolski

e-mail:

mike@math.amu.edu.pl

mike@plpuam11.bitnet

mail:

M.Wiernowolski

Os.Rusa 115/5

61-245 Poznan

POLAND

*******************************************************************************

Basics

*******************************************************************************

Synthia is a program that tries to get more 'synthy' sounds out of GUS.

It allows you to load and re-programm normal GUS patches (it's sort

of a patch player). It has the following features:

1) multiple layers (although there is a place for multi-layered

samples in GUS patches, drivers do not accept them. This is

simply not required for General MIDI sounds)

2) multi-point volume envelope with loop

3) multi-point pitch envelope with loop

4) sample tuning

5) delayed attack

6) overall volume, panning

7) 32 voice polyphony

*******************************************************************************

Hardware requirements

*******************************************************************************

1) PC-compatible with at least 80286 processor

2) VGA graphics

3) mouse

4) Gravis Ultrasound

*******************************************************************************

Getting started

*******************************************************************************

I think that the intefrace is self-explanatory, so I do not intend

to write a whole story.

Use mouse buttons to select options (right one gives accelerated action).

Use computer keyboard to play sounds.

You can change octave by pressing left/right shift keys.

Things to try:

1) Select Demo (you need the file METALPAD.PAT in %ULTRADIR%\MIDI).

This will automatically load and setup a sound.

2) Load PAT, copy it to another layer, select opposite pannings,

delay first layer or finetune it.

This will produce interesting 'thick' sound.

3) Load PAT, change pitch envelope and activate it.

This will produce automated pitch shifting for each sound.

4) Load PAT, copy it to all layers and tune them accordingly.

This may produce cluster sounds or cords.

*******************************************************************************

Bugs

*******************************************************************************

This program is NOT intended as a professional tool and has many

limitations. It's been written to play a little bit with patches

and to learn programming the GUS. Due to time restrictions I wasn't

able to test it thoroughly. Please report any problems via e-mail.

*******************************************************************************

Copyright information

*******************************************************************************

This software is provided 'as is', without any warranty

expressed or implied.

You are free to use it for any commercial or non-commercial

purposes.

You may also distribute it as long as you follow the rules:

(1) files SYNTHIA.EXE and SYNTHIA.DOC are included

without any changes

(2) no fee is charged for this program (excluding typical

media charge)

(3) if you put it on a CD-ROM send a free copy to me :-)

This software is CARDWARE. If you like the program

send a postcard to me (if you don't - simply delete it).

******************************************************************************