TRUST COMMUNICATOR FIRMWARE UPGRADE

(Int.Art.No.: 10035, 10038, 10039, 10040, 10041, 10042, 10798)

1.	Contents

The firmware upgrade consists of three parts:

- Modem driver v2.00 for Windows 95/98/NT

- V.90 firmware upgrade

- K56Flex firmware upgrade

2.	Before you upgrade

- Please make sure that you have the correct driver for your modem. Trying to upgrade your modem with the wrong firmware (wrong type or country-model) may lead to problems during or after upgrading.

Here are some tips to determine what type and model modem you have in your computer. The modem information can be found as follows:
- Click on ‘Start’, ‘Settings’, ‘Control Panel’, ‘Modems’.
- Select the tab 'Diagnostics'.
- Select the COM-port of the modem.
- Click on the button 'More Info...'.

ATI5 reports country code parameter
		002 = Belgium
		006 = Germany
		008 = Italy
		010 = Netherlands
		016 = United Kingdom

- Make sure that the service you are calling (for instance your Internet Provider) supports the V.90 protocol. If your provider does not support the V.90 protocol but the K56Flex protocol, the firmware upgrade is not needed. If you have upgraded to V.90 but want to go back to K56Flex, a ‘downgrade’ is also available (see the last chapter).

- Close all programs and resident applications (like virus scanners).

NOTE: 	The latest V.90 firmware does not support ASVD (Analog Simultaneous Voice and Data) function.

3.	Upgrade to V.90

- In the subdirectory ‘V90’ of the firmware upgrade start the program ‘FLASHCOM.EXE’

- The program starts to detect COM-ports and search for connected modems. (see figure 1). When a modem is detected, figure 2 appears.


Figure 1:	Modem detection


Figure 2:	Modem Flash Memory Loader

- Select the COM port on which the modem in installed and click the button 'Load Now' to flash the modem to V.90.

- A confirmation window appears. Click 'Ok' to continue. Now the modem will be upgraded and figure 3 appears.


Figure 3: Loading Flash Memory

NOTE: 	Do not turn off your computer or modem or otherwise stop this process until all modules are loaded.

- When finished, figure 4 appears. Click 'Close' to continue. You return to the start window (figure 2)


Figure 4: Flash Load Successful

- Click Close button again to finish the program.

4.	Update Modem driver

- Start the program 'MDMDEL.EXE' from the ‘driver’ subdirectory to remove the old modem driver.

- Start the program 'REGDEL.EXE' from the ‘driver’ subdirectory to remove the modem settings of your registry. See figure 5.


Figure 5: registry checking

- Click the Ok-button to continue.

- Restart the computer when finished and install the new driver that is located in the ‘driver’ subdirectory.

5.	Downgrade modem to K56Flex

NOTE: 	only do this if you need to go back from V.90 to K56Flex, for instance when your provider only supports K56Flex.

NOTE: 	If you downgrade your modem to the K56Flex protocol, you still need the version 2.00 modem driver that is included in this package.

Procedure

- In the ‘K56’ subdirectory of the firmware upgrade, start the program 'FLASHCOM'

- Follow the instructions given by the program (see chapter 3, the V.90 upgrade) to complete the operation.