NEC Technologies, Inc.

FDCD.SYS

CD-ROM Device Driver Version 2.31NEC

I. GENERAL PRODUCT OVERVIEW:

1)  Product description:

The FDCD.SYS CD-ROM device driver program is to be used with

Future Domain SCSI Host adapters to support one or more CD-ROM

drives on a DOS-based host system.  The device driver can support

from 1 to 24 CD-ROM drives (via up to 4 SCSI adapters) on the

PC, PC/XT, PC/AT and Micro Channel Architecture (PS/2) environments.

Photo-CD is supported on drives as noted in section 2.

2)  Table of supported NEC CD-ROM drives on 8-bit and 16-bit adapters:

SUPPORTED NEC CD-ROM DRIVES

---------------------------------------------------------

CDR-25, CDR-38, CDR-55, CDR-72, CDR-73, CDR-74, CDR-74-1,

CDR-83, CDR-84, CDR-84-1

(M-series are also supported)

3)  Table of NEC Drives supporting Photo-CD:

MODEL NUMBERS      SINGLE-/MULTI-SESSION

-------------	   ---------------------

CDR-25	   Single-session

CDR-38	   Single-/Multi-session

CDR-74	   Single-session

CDR-74-1	   Single-/Multi-session

CDR-84	   Single-session

CDR-84-1         Single-/Multi-session

II. SYSTEM REQUIREMENTS:

- Minimum 512KB of System Memory needed by INSTALL program.

(This amount of memory is required by the automatic installation

program - INSTALL.EXE.  Refer to "Manual Installation" in the

CD-ROM device driver Installation Manual Guide, if you cannot

run the automatic installation program.)

- MS or PC DOS Version 3.x - 6.0.

- MS DOS Extensions Version 2.20-2.22 (Version 2.22 included)

- A Hard Disk Drive. (Optional)

(Not necessary for device driver or host adapter but

may be necessary for some CD-ROM application programs.)

III. INSTALL/TECH TIPS (READ BEFORE INSTALLATION):

1)  Command line parameter '/I' (controls Disk Change status)

If this switch is not specified, the driver will not return an

"Invalid Disc Change" error if the SAME disc is reinserted into

the drive.  This may cause a problem if two different disc's

having the same volume size are being used.

If you are experiencing this problem, specifying the /I switch

will force the driver to always return an "Invalid Disc Change"

error anytime a disc is changed. (SAME or DIFFERENT disc)

Usage:  DEVICE=C:\DEV\FDCD.SYS /D:MSCD000 /I

2)  LASTDRIVE = Z in the CONFIG.SYS file.

The install program will default to using drive letter 'Z'.

This drive letter should be changed so that DOS does not reserve

more drive letters than are necessary.

Example:  If you have one fixed drive with two partitions using

drive letter C & D and one CD-ROM drive using drive

letter E, set LASTDRIVE to E (LASTDRIVE = E). This is

very important for Network users so that drive letters

will be available to access Network drives as in a

Novell environment.

3)  For compatibility with the Future Domain's FDDEVICE.SYS device

driver, load the FDDEVICE.SYS driver before loading FDCD.SYS

in the CONFIG.SYS file.

IV. TESTED SOFTWARE SUPPORT:

Microsoft:

- TESTDRV.EXE (CD-ROM device driver test program)

- CDSPEED.EXE (CD-ROM Data Streaming Transfer Rate Test)

- Windows 3.0 with Multimedia extensions V1.0

- Windows 3.1

- BookShelf

- Programmer's Library

- Musicbox with a variety of audio discs

- Multimedia Beethoven

National Geographic Society:

- Mammals: A Multimedia Encyclopedia

- The Presidents

Sierra Online:

- Mother Goose

Software Toolworks:

- CD Game Pack

- Illustrated Encyclopedia

- US Atlas

- World Atlas

Animotion:

- MCS CDMaster

NOTE:  The CD-ROM device driver will work under Windows 3.0/3.1 in REAL,

STANDARD and ENHANCED mode.  Depending on the application being

used, you may be limited to using REAL mode.  Contact the

manufacturer of the application for compatibility under Windows.

V. TECHNICAL SUPPORT:

NEC's Technical Support Department can be reached at:

NEC Technologies, Inc.

1414 Massachusetts Avenue

Boxborough, MA  01719

Telephone #:  (800) 388-8888

Fax #:        (508) 635-4666

Monday - Friday, 8:30 A.M. - 8:00 P.M. EST.