Autodesk - ADI 4.2

==================

The  ADI  4.2 driver provided with your Cirrus Logic VGA  is  the

TurboDLDClassic display list driver from Panacea. It has only two

purposes:

To speed up AutoCAD REDRAWs, PANs, and ZOOMs

To provide a more productive, user-friendly, interface to AutoCAD

(via the features described later in this manual).

The driver is memory-resident and inserts itself between  AutoCAD

and  the  graphics  board. It has no  other  effect  on  AutoCADs

operation besides speeding the program up; it runs with AutoShade

2  with  RenderMan and 3D Studio to  provide  enhanced  rendering

support,  but does not affect the speed of these programs,  since

they do not support Display Lists.

Installing  TurboDLDClassic  does not change any of  the  AutoCAD

program  files or alter any of the drawing files stored on  disk.

TurboDLDClassic was designed to be an easy-to-use, feature-filled

product that makes using AutoCAD faster and more productive,  not

one that makes AutoCAD more complicated and difficult to use.

How does TurboDLDClassic make AutoCAD run faster? There are three

things the driver does to speed operation:

AutoCAD stores drawings in a hierarchical structure, with  simple

elements  intermixed with complex ones. Every time the screen  is

updated,  AutoCAD  must decode  this  structure.  TurboDLDClassic

works  differently.  While  you are working,  it  translates  the

normal  hierarchical structure into a Display List, a  series  of

vectors  or polygon fills. When you pan or zoom,  TurboDLDClassic

uses  the Display List, then writes the resulting vectors to  the

video  board hardware. Since the hierarchical structure does  not

have to be decoded, drawing proceeds very quickly.

TurboDLDClassic also maintains a Drawing Cache. The Drawing Cache

is  a  compressed list which contains the current contents  of  a

viewport. This pre-scaled portion of the Display List allows  for

even faster pans and zooms and redraws.

TurboDLDClassic  gives you numerous new user-interface  features,

such as the bird's eye view. (via the features described later in

this manual).

By  how much does TurboDLDClassic increase the speed of  AutoCAD?

PANs  and ZOOMs, aided by only the Display List, run from two  to

twelve  times faster than a non-display list driver. The  Drawing

Cache  further speeds things up to the point that REDRAWs can  be

up to twenty-five times faster with TurboDLDClassic, compared  to

the graphics drivers shipped with AutoCAD.

Features:

---------

The features provided by TurboDLDClassic include:

Bird's eye view.

Accelerated redraws, pans, and zooms.

Easy to use - no new commands or special menus to learn.

Protected-mode  ADI  4.2  driver  -  completely  compatible  with

AutoCAD  Release  12, Release 11/386, 3D Studio and  AutoShade  2

with RenderMan.

No memory conflicts.

Works with AutoCAD's built-in Virtual Memory Manager

Includes  CustomColors,  which lets you  interactively  customize

your logical and physical colors from within AutoCAD.

Completely compatible with all Autodesk ADI 4.2 compatible  third

party software.

Supports all AutoCAD Release 12 features, including rendering  to

viewports and 31-Bit regen space.

TurboDLDClassic  requires a '386, '486 or Pentium based PC  which

supports AutoCAD Release 12, 11/386, AutoShade 2 with  RenderMan,

or 3D Studio V1.x/2.x. Additional extended memory is  recommended

for optimal performance.

Quick start guide

-----------------

This  section  summarizes  the basics  of  installing  and  using

TurboDLDClassic. Please read the rest of the manual for details.

What does it do?

----------------

TurboDLDClassic  's  main function is to speed up  AutoCAD  pans,

zooms  and redraws. The driver accomplishes this by creating  and

maintaining a Display List - a fast-displaying object list of the

current drawing - dramatically increasing performance on pans and

zooms.

In   addition  to  offering  display  list   speed   enhancement,

TurboDLDClassic  provides a bird's-eye view to allow you to  move

around large drawings.

Whats the catch?

----------------

The  only resource used by TurboDLDClassic is memory. The  driver

actively uses about 300Kbytes of RAM from extended memory for its

operation.  This memory is drawn from AutoCAD's memory  pool  and

therefore  does  not affect normal DOS operation. The  driver  is

loaded  by  AutoCAD  at AutoCAD load time and  is  unloaded  when

AutoCAD is exited.

Additionally,  the Display List size can vary from one  tenth  to

three  times the size of the current drawing  file,  particularly

when using AutoCAD 12's 31-bit regen space.

You  may  wish  to purchase and  install  additional  RAM  before

installing TurboDLDClassic, since it shares memory with  AutoCAD.

If AutoCAD is using a lot of memory, TurboDLDClassic may not have

enough. If there is significant hard disk activity while you  are

using  TurboDLDClassic, it may be an indication that  you  should

add more memory to your system.

Installation

------------

Insert  the Cirrus Logic drivers diskette into Drive A: (or  B:).

Run  the  Cirrus Logic installation  program  (INSTALL.EXE),  and

choose  the option for the Autodesk/AutoCAD drivers  and  specify

the  drive and directory where you want them copied to  (such  as

C:\ACAD\DRV).

Since the start-up is a little bit different for AutoCAD  Release

12 than it is for older versions, please follow the  instructions

for  the  version of AutoCAD you will  be  using  TurboDLDClassic

with.

To configure AutoCAD 12

-----------------------

Begin AutoCAD with the reconfigure switch by typing

ACAD -R[Enter]

Choose  option  3,  Configure  Video  Display  from  the  AutoCAD

configuration menu.

Type  Y  at the Do you want to select... message to  display  the

available video options for AutoCAD.

Select TurboDLD Classic by Panacea Inc. from the list of  display

options.

If  you  chose to install TurboDLDClassic  into  a  sub-directory

other  than ACAD\DRV, be sure to modify the  ACADDRV  environment

variable   to   include  that   sub-directory.   Otherwise,   the

TurboDLDClassic  selection  will  not  appear  in  the  list   of

available drivers.

To configure AutoCAD 11/386

---------------------------

Run the FASTACAD.BAT that was copied to the TurboDLDClassic  sub-

directory by typing:

C:\TURBODLD\FASTACAD[Enter]

The  above  example shows that TurboDLD Classic was installed  on

drive C: in the TURBODLD sub-directory.

Start  AutoCAD  and  reconfigure it  to  use  TurboDLDClassic  by

selecting option 5, Configure AutoCAD from the AutoCAD main menu.

From  the  next menu, select option 3, Configure  Video  Display.

Choose  Item  1, P386 ADI 4.0/4.1 (R11) as your  display  device.

(For  more  information,  see  your  AutoCAD  'Installation   and

Performance Guide.')

TurboDLDClassic configuration

-----------------------------

After  selecting the proper display device,  the  TurboDLDClassic

driver configuration menu will appear.

Set-up  of TurboDLDClassic requires configuration of its  various

operating  parameters.  These  parameters  have  been   logically

grouped  into  menus based on their interaction with  AutoCAD.  A

quick  trip  through each menu will  complete  the  configuration

process. Context sensitive help can be obtained by typing [?]  at

any  menu option. Please note that the DLDSETUP.HLP file must  be

present in order for help to appear. If you get an error message,

be  sure  that  the file DLDSETUP.HLP is  in  the  ACAD\DRV  sub-

directory (R12), or in the TURBODLD sub-directory (R11).

If  at  any point during the configuration process  you  want  to

change an item on a previous screen, just press the [ESC] key  to

back  up. Selecting NO SAVE, EXIT at the main configuration  menu

will return you to the AutoCAD configuration menu without  making

any changes.

At  a  minimum, a graphics board and screen  resolution  must  be

chosen  from  the Select Graphics Board/Resolution  menu.  If  no

display options are configured, the driver will be  automatically

configured for Generic VGA, 640 x 480 resolution at 16 colors.

The  first  menu, Select  Graphics  Board/Resolution,  configures

TurboDLDClassic for the graphics board, and display and rendering

resolutions to be used. Prior to configuring TurboDLDClassic, run

the Clmode utility and check the first screen to determine  which

Cirrus Logic chip and how much memory is on your VGA adapter.

Select Graphics Board - Selects the graphics chip being used

Select  Display  Resolution  - Selects  AutoCAD,  3D  Studio  and

AutoShade main display screen resolutions

Select  Rendering Resolution - Selects AutoCAD 12, 3D Studio  and

AutoShade rendering resolutions

When configuring TurboDLDClassic's display options, the Rendering

Board/Resolution  options  described  below  will   automatically

appear ONLY when configuring AVE Render, 3D Studio or  AutoShade.

Otherwise, the Display Board/Resolution options will be the  only

menu choices available.

Basic  Configuration options set AutoCAD screen  characteristics,

number of text lines in the command-line area, font size and dual

screen mode.

AutoCAD  Text Lines - Selects number of lines in AutoCAD  command

prompt area. The default is 3 lines, but values from 1 to 10  can

be  specified. A number larger than 3 might be useful if you  are

using  this  driver in a high resolution mode with  small  fonts,

eliminating  the need to frequently swap back and  forth  between

the text and graphics screens.

If  you  do  not  want any lines of text at  the  bottom  of  the

display, use AutoCAD to disable the command area (see the AutoCAD

'Interface,   Installation  and  Performance  Guide'   for   more

information). Press [Return] to continue with font selection.

Font  Size - Selects AutoCAD display screen font or font file  to

be  used.  Choose the Font Size you would like to  use  for  your

AutoCAD     menus,     pull-downs,     and     dialog      boxes:

8x8/8x14/8x16/12x20/12x24.   For  resolutions  over  800x600   we

recommend the 12x20 or 12x24 fonts. The default is the 8x14 font.

Dual  Screen Mode - Enables or disables dual screen operation  of

AutoCAD.

User  Interface  options include the parameters  for  maneuvering

within  AutoCAD. Using these options, the Big Picture,  Panacea's

bird's-eye  view, can be configured to suit your needs  or  allow

for better differentiation within a complex drawing.

Double  Click  - The Double Click option allows you  to  set  the

delay  for  TurboDLDClassic's  Big  Picture  pop-up.  This   time

represents the largest amount of time that can elapse between the

release  of the digitizer button and the subsequent press of  the

same button in order to detect a double click and display the Big

Picture. Setting this number too low will result in the  computer

seeing  two separate clicks, while setting the time too high  may

slow down response time for single button presses. Values between

10 and 30 are recommended.

The  Double Click time is measured in hundredths of  seconds.  PC

systems  have  a  timing  resolution of  5/100ths  of  a  second,

therefore,  the  entered time is rounded by the  driver  to  most

closely approximate the nearest multiple of 5. This means that  a

time setting of 23 is the same as a setting of 27, both of  which

get  rounded  to  25 (or one-quarter of a  second).  In  simplest

terms,  when  adjusting  the  timing value, it  is  best  to  use

multiples of 5.

BP  Button - Sets mouse/digitizer button to use for Big  Picture.

With  the  BP  Button  option,  TurboDLDClassic  gives  you   the

flexibility to use a double click on an available digitizer/mouse

button  as  a Big Picture pop-up button. On the  BP  Button  menu

line,  enter the number of any available digitizer  button  other

than  button 1 - button 1 is reserved as the pick button.  Within

AutoCAD, simply double click the chosen button to invoke the  Big

Picture.

BP  Highlight Mode - Controls how the Big Picture will appear  on

the  screen. Patt Line will display the Big Picture using  dotted

lines to form the outer bounding box and the zoom crosshairs. XOR

Rect will use a contrasting rectangle to display the Big Picture.

The Both option will use a combination of Patt Line and XOR  Rect

to distinguish the Big Picture.

BP  Refresh  -  In it's fastest mode,  TurboDLDClassic  does  not

refresh  the  Big Picture until a DLDREFRESH command  is  issued.

Depending  on  a  drawing's complexity, in  this  mode,  the  Big

Picture  could become confusing, displaying vectors that are  now

actually erased, and no longer part of the drawing. To  configure

the  Big  Picture to be updated as objects are drawn  or  erased,

enable the BP Refresh. If speed is a major concern, disable it.

BP Cache - Enables or disables TurboDLDClassic's internal  memory

cache  for  Big Picture functions. This cache will speed  up  Big

Picture operations on some graphics platforms, primarily on TIGA-

based graphics boards. For this reason, the BP Cache is  normally

disabled. As a reference, on VGAs, the performance benefit of the

BP  Cache is as little as 1%. With a TIGA-based  graphics  board,

however,  the performance benefit of the BP Cache is as  high  as

400%.

The   Expert   Configuration   menu   sets   the   function    of

TurboDLDClassic itself. This menu allows for customization of the

driver for speed or to adjust for memory constraints.

Display  List - Enables or disables the display list  feature  of

TurboDLDClassic.  This  option should always be  set  to  enable,

disabling  the Display List will cause TurboDLDClassic to run  as

an ordinary non-display list driver.

Drawing  Cache - Enables or disables  TurboDLDClassic's  internal

drawing  cache, a compressed list of the current viewport,  which

speeds up pans zooms and redraws. As in the Display List  option,

Drawing   Cache  should  normally  be  enabled.  In  low   memory

situations,  it  may be desirable to disable the  Drawing  Cache.

Disabling  the drawing cache will free-up memory for AutoCAD  but

may  or  may  not  have a visible effect on  your  ZOOM  and  PAN

performance. For example, on VGAs, the performance benefit of the

drawing  cache  is as little as 5%. With  a  TIGA-based  graphics

board,  however, the performance benefit of the drawing cache  is

as high as 400%.

AutoCAD  Logical  Drawing  Space - Using  AutoCAD  31-Bit  Space,

configures  TurboDLDClassic  for use with  AutoCAD  R12's  31-Bit

logical  drawing  space.  When set to Yes, the  driver  will  use

AutoCAD's extended 31-Bit drawing space. Selecting No, will use a

15-Bit drawing space, similar to that of AutoCAD R11.

The  31-Bit  logical space allows you to extend  your  Regen-less

zooming  ability by a factor of several million, at the  cost  of

more memory. Additional memory is used by AutoCAD for the drawing

space,  and  by  TurboDLDClassic for  the  Display  List.  31-Bit

zooming  and  panning  is about 10%-20% slower  than  for  15-Bit

logical  space. The Use AutoCAD 31-Bit Space setting  is  ignored

for AutoCAD R11 installations.

Internal  Command  Echo - The echo  of  TurboDLDClassic  internal

commands  can  be enabled or disabled with the  Internal  Command

Echo option. If you would like to see TurboDLDClassic's  internal

commands  display  at  the  AutoCAD  command  line  as  they  are

executed, enable this option.

Big Picture Zoom Mode - Sets the Big Picture (BP) zoom definition

area  display options. In a zoomed view of the static BP, as  the

current viewport is zoomed or panned, Float mode causes the image

in  the BP to move around within the bird's-eye  window,  keeping

the  zoomed viewport area fixed in the center of the  bird's-eye.

Freeze  mode  will  lock the current BP contents  into  place  to

provide a better overall frame of reference.

Regen  Mode - As a convenience feature, a Regen Mode  option  has

been  added  to  TurboDLDClassic. A Fast  Regen  will  store  the

AutoCAD drawing until the Display List has been created, and then

display  it,  all  at once. The  Incremental  mode  displays  the

drawing in 'chunks' as the display list is created. The Fast mode

causes  Regens  to process approximately 5%-10% faster  than  the

incremental. Neither mode changes memory requirements.

After  all  options have been set, use the arrow keys  to  scroll

down  to the Save and Exit option and then press the [Enter]  key

to continue.

Next,  configure the AutoCAD screen display  characteristics  and

then type [Y] to accept the changes.

Exit   to   the   AutoCAD   drawing   editor   to   begin   using

TurboDLDClassic.

The  last step in setting-up TurboDLDClassic is to configure  the

colors   for  AutoCAD.  From  the  AutoCAD  Command  Line,   type

DLDCOLOR[Enter],  to start CustomColors, TurboDLDClassic's  color

configuration  utility. Make any desired color changes  and  then

Save the new color palette and then Exit to return to the drawing

editor.

Verifying your TurboDLDClassic installation

-------------------------------------------

To   verify  that  TurboDLDClassic  is  running   and   installed

correctly, follow one of these two simple tests.

If  you have the AutoCAD side menu enabled, look for the  Panacea

Logo in the lower right-hand corner.

If   you   are  running  AutoCAD  without  a  side   menu,   type

DLDVER[Enter]  at the AutoCAD command prompt. If  TurboDLDClassic

is  loaded and running, this command should return  your  current

version, and serial number.

Reconfiguring TurboDLDClassic

-----------------------------

To reconfigure TurboDLDClassic follow the instructions below  for

your version of AutoCAD, Release 12 or Release 11.

AutoCAD Release 12

------------------

If  you need to make changes to your Release  12  TurboDLDClassic

configuration,  type CONFIG[Enter] at the AutoCAD command  prompt

or use ACAD -R[Enter] when starting the program. Select option 3,

Configure  Video  Display. Answer No[Enter] to the Select  a  new

videodriver ... prompt to start the TurboDLDClassic configuration

program. Make the desired changes to the driver and then Save and

Exit to continue to the AutoCAD drawing editor.

AutoCAD release 11

------------------

To change a Release 11 configuration, select option 5,  Configure

AutoCAD   from  the  AutoCAD  main  menu,  and  then   from   the

configuration  menu,  select option 3, Configure  Video  Display.

Answer  no to the Select a new video driver ... prompt  to  start

the  TurboDLDClassic  configuration program. Change  the  desired

driver   options.  Save  and  Exit  to  return  to  the   AutoCAD

configuration menu. Open or begin a new drawing.

To   completely  reconfigure  TurboDLDClassic   using   Panacea's

defaults,  delete  DLDSETUP.DAT from either  the  \ACAD\DRV  sub-

directory  (R12)  or the TURBODLD sub-directory  (R11)  and  then

follow  the  Driver Configuration instructions  earlier  in  this

chapter.

Configuring an AutoCAD 11 environment

-------------------------------------

During  the TurboDLDClassic installation process, a  FASTACAD.BAT

file  is  created  and placed into  the  TURBODLD  sub-directory.

FASTACAD.BAT   contains  four  lines  that  set   four   separate

environment  variables: DLDCFG - used by TurboDLDClassic to  find

all of its configuration files; and DSPADI, RCPADI, RDPADI - used

by AutoCAD, 3D Studio and AutoShade 2 w/RenderMan,  respectively,

to  find  the  driver file. FASTACAD.BAT must  be  run  prior  to

starting  AutoCAD  R11 and only needs to be run once  per  system

boot.  For  automatic  loading  of  the  environment   variables,

FASTACAD.BAT may be added to the AUTOEXEC.BAT file or an  AutoCAD

start-up batch file.

To add FASTACAD.BAT to your AUTOEXEC.BAT file, insert the line

CALL D:\TURBODLD\FASTACAD

anywhere in the file. The above example assumes that FASTACAD.BAT

resides in a sub-directory on drive D: called \TURBODLD.

If  you dont wish to put FASTACAD in your AUTOEXEC.BAT file,  you

may  put it in a batch file which also starts AutoCAD, or  simply

remember to run the file before starting AutoCAD.

If  you  get the message Out of Environment Space  when  you  run

FASTACAD.BAT, you will need to enlarge your system's environment.

This is accomplished by adding the following line,

SHELL=C:\COMMAND.COM /P /E:768

to your CONFIG.SYS file. The /E:768 specifies an environment size

of 768 bytes. Change this number as appropriate. You will need to

reboot  after  modifying your CONFIG.SYS file, in order  for  the

changes to take effect.

Configuring AutoShade, 3D Studio and AVE Render

-----------------------------------------------

AutoShade

---------

To configure AutoShade v2.0 to use TurboDLDClassic, first run the

FASTACAD.BAT file from your TurboDLDClassic sub-directory to  set

the  AutoShade environment variables. Next, start AutoShade  with

SHADE  /R[Enter], which will allow you to reconfigure  AutoShade.

For  the  display device, select P386 Autodesk  Device  Interface

display  driver,  and for the rendering display select  the  P386

Autodesk  Device Interface rendering driver. If you  are  running

the display and rendering screen on the same monitor (i.e. single

monitor),  make  sure to tell AutoShade this.  A  single  monitor

approach  will  require a redraw of the display  screen  after  a

rendering screen.

Next, follow the same installation steps that were used to select

the  graphics  board and display and  rendering  resolutions  for

AutoCAD.

3D Studio

---------

Configuring  3D Studio requires three steps. First, you  need  to

set  the  environment  variables for 3D  Studio  by  running  the

PANA3DS.BAT  file that was copied into your TurboDLDClassic  sub-

directory.  Next,  edit  the 3DS.SET file, located  in  your  3DS

directory.  The following three lines may be changed to  use  the

RCPADI rendering driver. Locate the lines that begin with

RENDER-DISPLAY

MAIN-DISPLAY

MATERIAL-DISPLAY

and change them to read

RENDER-DISPLAY=RCPADI

MAIN-DISPLAY=RCPADI

MATERIAL-DISPLAY=RCPADI

Make sure to remove the ';' or any spaces that may be present  at

the beginning of the line.

It  is  only necessary for the RENDER-DISPLAY line to be  set  to

RCPADI in order to render at high resolution. If you do not  need

a high resolution main display screen or if you will not be using

the  materials  editor, you may keep MAIN-DISPLAY  and  MATERIAL-

DISPLAY set to their defaults.

For  3D  Studio  1.x,  there  is  no  RENDER-DISPLAY  line.   The

corresponding  line is DEFAULT-DISPLAY and should be  changed  to

read

DEFAULT-DISPLAY="RCPADI"

The  quotes  around RCPADI must be used for this version  of  3D

Studio.

The  MATERIAL-DISPLAY should only be configured for  RCPADI  when

BOTH the Display AND Rendering Resolutions of TurboDLDClassic are

configured for 256 color resolutions. An unpredictable  Materials

Editor  screen will appear if anything other than 256  colors  is

selected.  If you are unsure about the function of the  Materials

Editor  screen, use the Materials Editor as VGA, you  should  not

have  any problems at all. Please note too, that the use  of  the

Materials  Editor  will not give you any more  colors  to  choose

from.

Save the above changes and start 3D Studio to begin the last part

of the configuration procedure.

From  your  3D  Studio directory, delete the  file  3DADI.CFG  by

typing

DEL 3DADI.CFG[Enter]

This  will cause 3D Studio to start in its reconfiguration  mode.

Start 3D Studio by typing 3DS[Enter].

During  the 3D Studio reconfiguration start-up, you are  prompted

with  a series of questions. After the first 3D  Studio  question

appears  and  is  answered,  the  TurboDLDClassic   configuration

program  will  appear. Select a graphics board  and  display  and

rendering resolutions as you would for AutoCAD use.

Because  RCPADI  device  drivers,  by  definition,  are  combined

display and rendering devices, during 3D Studio  reconfiguration,

you  will  be brought to the TurboDLDClassic  configuration  menu

more  than one time, once each for Display and Rendering.  It  is

not  necessary to select display and/or rendering  resolutions  a

second  time.  Simply press the [Enter] key to  remove  the  help

screen  and then highlight Save and Exit to continue to the  next

question. Also if FASTACAD.BAT is used instead of PANA3DS.BAT  to

define  the  AutoCAD operating environment,  the  TurboDLDClassic

configuration menu will also appear for RDPADI and DSPADI devices

if they are present.

TurboDLDClassic  only  provides still rendering  support  for  3D

Studio. TurboDLDClassic will not play back rendered .FLI or  .FLC

files. This is a limitation of 3D Studio.The Mapping Icon colors,

which  are  usually yellow and green, will appear as  black  when

using any external ADI driver. Also, 3D Studio has problems  with

large  fonts.  It is recommended that you use  the  default  font

settings.

AVE Render

----------

AutoCAD   12's  AVE  Render  uses   TurboDLDClassic's   rendering

capabilities  to render objects and drawings. If AVE  Render  has

never  been configured, when you first select the AutoCAD  render

command,  you  will be forced to run  through  the  configuration

process. You will be prompted as follows.

Select  a  Rendering Display Device. Since TurboDLDClassic  is  a

combined  display/rendering  device,  choose  item  1,  P386  ADI

Combined Display/Rendering Driver from the available choices.

Configure  the Rendering Graphics Board and Resolution. Here  the

TurboDLDClassic configuration program will appear on the  screen.

Press  the  [Enter]  key to continue past  the  help  screen  and

display the Rendering Configuration menu. Choose Select  Graphics

Board/Resolution  to display graphics board and  resolution  menu

selections. First, choose Select Render Graphics Board to  select

the graphics board to be used for renderings. Next, choose Select

Render Resolution to select the desired rendering resolution from

the list of available choices. Select Return to Previous Menu and

then select Save and Exit.

Select   Render  Mode.  Then  select  the  rendering   mode   for

TurboDLDClassic.  Select either Render to Viewport or  Render  to

Screen  depending on how you wish to view your  renderings.  Note

that in order to render to a viewport you must be using a display

resolution of at least 256 colors. Otherwise, AVE Render will not

allow a render to viewport selection.

Select a Render Hard Copy Device. If you are using a render  hard

copy device select your device type from the list of choices.  If

you are not using a hard copy device, accept the default of NULL.

After  configuring AVE Render, you will return to the drawing  to

render the current drawing.

If you have previously configured AVE Render, type RCONFIG[Enter]

at  the  AutoCAD command prompt to manually  display  the  Render

Configuration  menu.  Follow  these  steps  to  re-configure  AVE

Render.

Select  option  2,  Configure Rendering Device to  choose  a  new

rendering driver. Answer [Yes] to the Select Different  Rendering

Device question.

Select option 1, P386 Combined Display/Rendering Driver.

Configure  the rendering graphics board and resolution. Here  the

TurboDLDClassic configuration program will appear on the  screen.

Press  the  [Enter]  key to continue past  the  help  screen  and

display the Rendering Configuration menu. Choose Select  Graphics

Board/Resolution  to display graphics board and  resolution  menu

selections. First, choose Select Render Graphics Board to  select

the graphics board to be used for rendering. Next, choose  Select

Render Resolution to select the desired rendering resolution from

the list of available choices. Select Return to Previous Menu and

then Save and Exit.

Select  the  rendering mode for  TurboDLDClassic.  Select  either

Render to Viewport or Render to Screen depending on how you  wish

to  view  your  renderings. Note that in order  to  render  to  a

viewport you must be using a Display resolution with at least 256

colors. Otherwise, AVE Render will not allow a render to viewport

selection.

Select  Exit to the Drawing Editor from the Render  Configuration

menu  and  then type [Y] to keep the changes  you've  just  made.

Press the [F1] key to change to the graphics screen if necessary.

Changing colors

---------------

Included   with   TurboDLDClassic  is  the   CustomColors   color

configuration  utility. What is so unique about  CustomColors  is

that  it gives you the ability to modify all of  your  changeable

AutoCAD  colors, including menu colors, text colors,  dialog  box

colors, and even drawing colors, while running AutoCAD.

CustomColors  simulates  an  AutoCAD screen,  complete  with  all

possible  objects.  At the AutoCAD drawing editor  command  line,

type  DLDCOLOR[Enter] to edit the color configuration.  Once  you

enter   the  utility,  you  are  provided  with   the   following

configuration menu at the bottom of the screen:

(O)bject,

(D)rawing,

(P)hysical,

(A)DI reset,

(V)GA reset,

(L)oad,

(S)ave,

(E)xit

The menu items perform the following functions.

Object - Selecting this option will allow you to change the color

of  any  AutoCAD  screen  object,  such  as  the  graphics   area

background  color, the menu area text color, or the  border  line

color.

Select  the object whose color you want to change by  moving  the

highlight box around with the left and right cursor control  keys

and  pressing  [Enter]  when you  have  highlighted  the  desired

object.  Note that a one line description of the object  type  is

displayed at the top of the screen as you move the box around.

Once  you  have selected the object, another highlight  box  will

appear  around  the  color boxes nearest to  the  bottom  of  the

screen.  These  are  the physical colors  that  the  video  board

supports.  Use  the  left and right cursor  keys  to  select  the

physical  color to be used for the object you have  selected  and

press [Enter]. The screen will quickly redraw with the new  color

selection for the object you have chosen, and you will be back at

the  start  of object selection in case you want  to  change  the

color of another object.

Note  that at any time during color configuration, you can  press

the [Esc] key once or twice and return to AutoCAD. Also, pressing

the question mark [?] key will provide you with context sensitive

help.

Drawing - This option allows you to modify AutoCAD drawing colors

0,  8  and 9, in a fashion similar to the way you  change  object

colors.

When  this option is selected, a highlight square appears in  the

drawing  color area of the simulated AutoCAD display. The  square

can  be  manipulated using the four cursor keys. You  may  notice

that  the drawing color portion of the display is laid  out  just

like  the  CHROMA drawing supplied with AutoCAD. When  you  press

[Enter], the highlight moves down to the 16 physical colors, just

as  it  does during the object color selection. Using  the  arrow

keys, pick the physical color you want to represent the  selected

drawing color. Press [Enter] to have your selection take effect.

Physical - choosing the Physical option from the menu allows  you

to  alter  the red, green, and blue components  of  the  physical

colors.  A  highlight box will appear in the row of  16  physical

colors at the bottom of the screen.

Use the arrow keys to select the color you wish to edit and press

[Enter].  Three  sliders  will  appear near  the  bottom  of  the

display,  with the horizontal position of the slider for each  of

the  color components (red, green, or blue - RGB) indicating  the

relative  intensity  of the component. You can use the  left  and

right  arrow keys to move the slider for the selected  component,

or type in a number from 0 to 255.

Many graphics boards do not support 255 different intensities for

each  color  component. Therefore, when you enter  an  intensity,

Custom  Colors will round it to the nearest intensity which  your

graphics board supports. For example, VGAs support 64 intensities

per  RGB  color  component. This means  intensities  increase  in

multiples of 4 (256/64), therefore an intensity of 0 is the  same

as an intensity of 3.

The  up and down arrows allow you to select which  component  you

want to alter. As you manipulate the sliders, all objects on  the

display that are of the same color as the selected physical color

will change in hue. This way you can visually determine the  most

appropriate  setting for your display. Pressing [Enter] will  set

the RGB values you have selected for the physical color you  were

modifying.

ADIreset - constructs a default ADI color palette, as defined  in

the Autodesk Device Interface Driver Development Kit.

VGAreset - constructs a default VGA color palette in the first 16

color  entries. The remaining palette colors remain identical  to

those used for ADI devices.

Load - reloads the color information from the file  DLDCOLOR.DAT.

It  will  be  loaded  from the directory  pointed  to  by  DLDCFG

(AutoCAD  R11),  or  from  the AutoCAD  R12  \DRV  directory.  If

DLDCOLOR.DAT cannot be found, an error message will be displayed.

This  command is useful because it allows you to go back to  your

previously edited color configuration in case you have made  some

mistakes in configuring your colors that you want to undo.

Dont  Accidently  Wipe  Out  Your  Changes.  Selecting  ADIreset,

VGAreset,  or  Load will irrevocably wipe out any  current  color

changes  you may have made, unless you have just saved  your  new

setting,  so use them only when you really need to, such as  when

you  have  made so many color changes that you cant seem  to  get

back to a reasonable place and just want to start over.

Save  - Saves the current color palette to DLDCOLOR.DAT. It  will

be  saved to the directory pointed to by the  DLDCFG  environment

variable  in  FASTACAD, or to the \ACAD\DRV  (R12)  directory  if

FASTACAD is not used.

Exit - Exits CustomColors. If you have made palette changes,  but

not  saved  them,  you will be asked if you  want  to  save  your

changes before exiting.

When running at more than 8 bits-per-pixel (256 colors),  setting

physical colors via DLDCOLOR will have no visible effect until  a

SAVE and EXIT occurs.

TurboDLDClassic commands

------------------------

TurboDLDClassic   offers   AutoCAD  users   many   features   and

productivity options. This section alphabetically lists the basic

commands found in TurboDLDClassic and provides the correct syntax

for their usage. For a brief summary of TurboDLDClassic  commands

type DLDHELP at the AutoCAD command prompt.

The most important feature of TurboDLDClassic is the addition  of

a  world view called The Big PictureTM or BP for short.  Bringing

up the BP allows you to view where in the current drawing you are

currently located in your active viewport, and it also gives  you

the  ability to dynamically move to another part of  the  drawing

without having to exit the function youre currently executing.

DLDBIGPIC

---------

To  call  up the Big Picture, at the AutoCAD command  line,  type

DLDBIGPIC[Enter],  or  double  click the right  mouse  button  or

button 2 on your digitizer puck. (if you havent redefined your BP

Button  double click). This will bring up the BP on the  display.

You  will  see cross-hairs and a highlighted  section  indicating

which  part  of the whole drawing youre currently viewing  -  the

size  of  the  image will be determined by  the  AutoCAD  logical

drawing space being used, 15-Bit or 31-Bit. The selection or pick

box will have an 'X' through it. Move the mouse/digitizer  around

to  locate  your pick box. If you cant see a  small  box  moving,

click  the left mouse button (the pick button) to size  down  the

pick area. In size mode, the pick box will have a right  pointing

arrow  in it ('-->'). If the contents of the BP are too small  to

work  with, you may increase the size of the image in the  window

by  pressing the [+] key on your numeric keypad this performs  an

incremental  zoom within the BP. Likewise, pressing the  [-]  key

will  reduce  the size of the contents of the  BP.  Pressing  the

[Home]  and  [End]  keys will put the BP into  its  smallest  and

largest  size, respectively. The smallest BP size is  defined  as

being exactly the same view as that in the viewport referenced by

the  BP.  The  largest BP size is defined as  the  largest  image

possible without causing a Regen.

The BP pick box is always proportioned to the proper aspect ratio

for the current viewport. Clicking the pick button again will put

you  back  into the pick box move mode ('X'). This  operation  is

very  similar to using the Zoom Dynamic feature of AutoCAD.  Once

you  have  positioned  the pick box on the area  you  would  like

displayed in the active viewport, click any button other than the

pick  button to initiate the zoom, or press the [Enter]  key.  If

you want to abort the operation, just hold any button down, other

than  the pick button, until the BP disappears. You can also  hit

any  key on the keyboard, other than [Enter], to cancel  the  BP.

Note  that the BP is updated with new drawing commands, but  that

moves  or  erases will not update the image unless  BPREFRESH  is

enabled. A DLDREFRESH or a REGEN will manually update the BP with

the current drawing changes.

DLDBPCACHE*

-----------

Enables  or  disables  TurboDLDClassic's internal  cache  for  BP

features.  This  cache speeds up  TurboDLDClassic's  Big  Picture

functions. DLDBPCACHE is normally disabled. Performance  benefits

of the BP Cache with will vary from one graphics platform to  the

next.  For  example, on VGAs, the performance benefit of  the  BP

Cache is as little as 1%.

DLDBPDIM

--------

Resizing  and repositioning the BP can be accomplished by  typing

this  command at the AutoCAD prompt. DLDBPDIM allows you to  tell

the driver where you want the BP placed and how large you want it

to be. The size is limited to being between one-quarter and  one-

half  the width of the display. This command functions  like  the

pan/zoom  selection  box  in the BP, as far  as  positioning  and

dimensioning goes. To abort, hold the right mouse button, or  any

other digitizer button, down until the moving box disappears.

DLDBPFREEZE

-----------

Sets  the Big Picture (BP) zoom definition area display  options.

DLDBPFREEZE  toggles  the  Big Picture  Zoom  Mode  parameter  in

TurboDLDClassic's Expert Configuration Menu.

In  a  zoomed view of the static BP, as the current  viewport  is

zoomed  or panned, Float mode causes the image in the BP to  move

around within the bird's-eye window, keeping the zoomed  viewport

area fixed in the center of the bird's-eye. Freeze mode will lock

the  current BP contents into place to provide a  better  overall

frame of reference.

DLDBPHILIGHT*

-------------

Changes the highlight used to display the Big Picture among three

modes, Patt Line, XOR Rect, and Both. Patt Lines will use  dotted

lines  to outline the Big Picture pick box. XOR Rect will  use  a

contrasting  rectangle to highlight the pick box, and  Both  will

use a combination of Patt Lines and XOR Rect.

DLDBPREFRSH*

------------

This  convenience  feature of TurboDLDClassic,  toggles  the  Big

Picture  update mode. If disabled, the BP is updated manually  by

issuing  a  DLDREFRESH command. When  enabled,  DLDBPREFRSH  will

cause the BP to refresh automatically when an object is drawn  or

erased.   The  BP  operations  therefore  will  be  slower   with

DLDBPREFRSH enabled.

DLDBPSTATIC

-----------

Toggles  the  BP  to stay on the screen at all  times.  Use  this

command if you want the BP on the screen in order to conveniently

move into it whenever you want to zoom or pan. Typing DLDBPSTATIC

will  put up the BP at the position you set with  DLDBPDIM.  Note

that the static BP will disappear when other menus pop-up on  the

screen, and it will also disappear if you draw any objects  which

may  overwrite the BP. With these exceptions, the BP will  remain

on-screen  until you either exit the drawing screen or  you  type

DLDBPSTATIC again.

DLDCOLOR

--------

Invokes CustomColorsTM, Panacea's color configuration program.

DLDCOMPACT

----------

Forces  a manual garbage collect of Display List  memory  thereby

returning  unused display list memory back to the AutoCAD  memory

pool.

DLDDCACHE*

----------

This  command  toggles the TurboDLDClassic drawing cache  on  and

off.  The  Drawing  Cache is a compressed  form  of  the  current

viewport which speeds pans, zooms and redraws

DLDDLIST*

---------

This command toggles the display list function of TurboDLDClassic

on and off.

Please  note that if the display list is turned off, you will  be

running  AutoCAD as though you were using a standard non  display

list  driver - pans, zooms and redraws will be MUCH  slower  with

DLDDLIST disabled.

DLDECHO*

--------

Toggles  internal TurboDLDClassic command echoing at the  AutoCAD

command line. When TurboDLDClassic commands are executed via  the

digitizer or pop-up menus, they generate internal commands  which

will  be  displayed  at the AutoCAD command line  if  DLDECHO  is

enabled. Disable DLDECHO to simplify the command line.

DLDHELP

-------

Provides  a  list  of  TurboDLDClassic  commands  with   one-line

description  of each, within AutoCAD. It's recommended  that  you

flip  to  the text screen by pressing the <F1> key  to  view  the

output.

DLDREFRESH

----------

Refreshes the Big Picture to reflect the most current changes  to

a  drawing. If the static BP is up, it will be updated.  If  not,

the  next  time the BP is brought up, it will contain  a  current

representation  of  your drawing, provided no changes  were  made

between  the  DLDREFRESH command and the DLDBIGPIC  command.  For

Automatic update of the Big Picture, see the DLDBPREFRSH  command

above.

DLDSTAT

-------

Displays  the  current TurboDLDClassic status. A listing  of  the

current  TurboDLDClassic  parameters  will be  displayed  at  the

AutoCAD   command  line.  A  flip  to  the  graphics  screen   is

recommended for this command.

DLDUSAGE

--------

Use DLDUSAGE if you want a to-the-byte breakdown of how memory is

being  used, specifically for display list  processing.  DLDUSAGE

returns  information  regarding  the  memory  each  viewport   is

occupying.  Since  AutoCAD  supports multiple  viewports,  it  is

possible to have multiple display lists.

DLDVER

------

Displays   the   TurboDLDClassic  version,  serial   number   and

registered user's name at the AutoCAD command line.

DLDVISREGEN*

------------

Toggles   between   the   Fast  and  Visible   Regen   modes   of

TurboDLDClassic.  A Fast Regen will create the display  list  and

then  display  the  drawing all at once.  A  Visible  Regen  will

display  the  drawing in chunks as the display list  is  created.

This command is a dynamic form of the Regen Mode parameter in the

Expert  Configuration  Menu. Since TurboDLDClassic's  Fast  Regen

mode is faster than AutoCAD's, we highly recommend its use.

Please  note that the commands above with a '*'  following  them,

when  issued  within AutoCAD, will override the  selections  made

during  TurboDLDClassic  configuration, for the  current  drawing

session  only. Exiting AutoCAD and subsequently  restarting  will

cause  all feature settings to revert back to those  selected  in

the  TurboDLDClassic configuration menu. If you wish to make  the

current changes permanent, reconfigure TurboDLDClassic.

Memory usage and lists

----------------------

TurboDLDClassic  shares  extended  memory with  AutoCAD  via  the

PharLap  Virtual Memory Manager. This means that  TurboDLDClassic

will  automatically page to disk if it uses up all the  RAM  that

AutoCAD  has left for its use. See the AutoCAD 'Installation  and

Performance  Guide'  for  more  information  on  Virtual   Memory

Management.

Please note that if you start seeing excessive hard disk accesses

during  PANs,  REDRAWs,  and  ZOOMs  while  using  AutoCAD   with

TurboDLDClassic, try using the DLDCOMPACT command. If this doesnt

affect  the amount of disk access its probably time to  add  more

memory to your system. Contact your AutoCAD dealer for assistance

in upgrading your memory.

Regarding  display  list memory, its important  to  realize  that

TurboDLDClassic  speeds  up  AutoCAD  operations  by  creating  a

Display  List  in memory, and sending that list to  the  graphics

board  for pans, zooms and redraws. But a display list  takes  up

memory.

How much memory?

----------------

For  production  use, we recommend that at least  1  megabyte  is

available  for  TurboDLDClassic.  To determine  how  much  memory

AutoCAD is using, use the status command while in AutoCAD  (refer

to  the  AutoCAD 'Installation and Performance  Guide'  for  more

information).  The  Display List for a simple  drawing  like  the

shuttle  Columbia  might only require 20 Kbytes for  the  Display

List.  Complex  drawings may require several megabytes.  We  have

seen Display Lists for a drawing range from one-tenth the size of

the drawing file to three times the size; in general, the Display

List  averages  about the same as the DWG file size.  This  means

that you should count on the Display List generally taking up  as

much  as twice the DWG file size. Also, complex objects  such  as

circles and text expand when translated into display list format,

so  a drawing with a lot of complex objects and text will have  a

larger Display List than a simpler drawing.

Keeping the Display List small

------------------------------

Text  takes up a disproportionate amount of space in the  Display

List.  To keep the list small, put text in its own layer  of  the

drawing.  Then dont display the text layer when editing the  rest

of  the figure. This will keep memory consumption down and  speed

up PANs, REDRAWs, and ZOOMs.

Read the AutoCAD manuals

------------------------

The  AutoCAD  manuals have an excellent section  on  performance,

concentrating on memory usage. To get the most performance out of

the  program, read the appropriate sections of those  manuals  as

well as this one.

Tips and tricks

---------------

Upgrading from older Panacea DLD drivers

----------------------------------------

Be  sure to remove any reference to previous DLD driver  commands

(FASTACAD calls or SET parameters) or sub-directories that may be

in  your  AUTOEXEC.BAT file or in AutoCAD start-up  batch  files.

Such  references could cause AutoCAD and TurboDLDClassic to  look

in the wrong place for setup information.

If have been using a previous Panacea DLD driver and wish to  use

the color palette you customized for it with TurboDLDClassic, you

may  copy  the  DLDCOLOR.DAT  file from  your  older  DLD's  sub-

directory   to  the  sub-directory  you  specified   during   the

TurboDLDClassic installation process.

Maneuvering Through AutoCAD

---------------------------

Zooming

-------

When  zooming into an image, be aware of your AutoCAD  grid  snap

setting.  If you are zoomed extremely far into a drawing and  you

are  having  trouble  moving your digitizer cursor,  you  may  be

snapping to a point which is not part of the zoomed viewport.  If

the  cursor only moves to a single point, or is not on screen  at

all, turn the grid snap off.

TurboDLDClassic and Paper Space

-------------------------------

TurboDLDClassic  features do not work in AutoCAD's  Paper  Space.

The execution of any TurboDLDClassic command in Paper Space  will

result in an error message at the command prompt.

Zoom Dynamic

------------

We do not recommended using ANY TurboDLDClassic features while in

the Zoom Dynamic mode of AutoCAD. Because TurboDLDClassic  cannot

determine  when  Zoom Dynamic has been  initiated,  unpredictable

results   will   occur.   Use  the   Big   Picture   feature   of

TurboDLDClassic to accomplish the same task as Zoom Dynamic, in a

much more efficient fashion.

Switching Color Modes

---------------------

When  switching color modes, (i.e. from 16 to 256 or vice  versa)

the  DLDCOLOR command should be run in order to reconfigure  your

color  palette for the number of colors selected. A black  cursor

and disappearing crosshairs are both symptoms of a color  palette

problem.

AutoCAD R12's CONFIG command

----------------------------

Configuring  TurboDLDClassic in the middle of a  drawing  session

using  the CONFIG command is a bit like exiting  and  re-starting

AutoCAD.  If are using the static Big Picture, you will  have  to

re-initialize it after returning from the Configuration menu.

Sticky Cursors

--------------

Sometimes, when using the Big Picture, the digitizer cursor  will

appear to stick to the edge of the bird's-eye. This is normal and

is  the  result  of the digitizer puck responding  to  the  whole

screen  area while the Big Picture only occupies a small  portion

of  the screen. Because of the difference in resolutions  between

the  screen and the digitizer, when the screen cursor enters  the

Big  Picture  area, the digitizer puck has more drawing  area  to

cover in order to get to the same location as the screen cursor.

Using AutoCAD Commands

----------------------

Since  TurboDLDClassic  is  totally  transparent  to  users  with

respect  to  using normal AutoCAD commands to  REDRAW,  PAN,  and

ZOOM, you still have to suffer from some of AutoCADs nuances. One

of  these nuances is that ZOOM ALL and ZOOM EXTENTS both force  a

REGEN, because AutoCAD does not keep track of various  boundaries

necessary  to  avoid  the REGENs. And  REGENs  are  rather  time-

consuming  and  dont  use any display list  processing  to  speed

themselves  up. One way around this problem is to use another  of

AutoCADs built-in features, namely the VIEW command.

When  you first load your image and see the whole drawing on  the

screen  at once, just type VIEW Save ALL[Enter], which will  save

the  display  position you see under a view  named  'All'.  Then,

after youve done some detailed editing and want to return back to

the  big picture, type VIEW Restore ALL[Enter], instead  of  ZOOM

All[Enter]  or ZOOM Extents[Enter], and the full drawing will  be

restored to the display at display list speeds, without a REGEN.

Another nuance of AutoCADs is that if you zoom in too far or  pan

over too far, you may inadvertently cause a REGEN. AutoCAD  again

provides a very simple solution: the REGENAUTO command. Just type

REGENAUTO Off[Enter] at the AutoCAD command prompt, and automatic

REGENs  will be disabled. The REGENAUTO setting is also saved  as

part  of your drawing file, so you only need to execute  it  once

per  drawing.  You  may even want to set REGENAUTO  off  in  your

ACAD.DWG  drawing template so that all your drawings are  created

with REGENAUTO set off.

Third-Party Software

--------------------

If  you are having trouble with TurboDLDClassic  and  third-party

AutoCAD  applications, be sure that the  third-party  application

supports  ADI  4.2. In order to use the  ADI  4.2  specification,

third-party applications require new T-Drivers and therefore must

be  revised. If an application does not specifically say that  it

is  ADI  4.2  compatible,  it probably is  not.  Check  with  the

manufacturer to be sure.

If your third-party application is ADI 4.2 compatible and you are

having trouble using TurboDLDClassic, try running AutoCAD without

the third-party application to try to isolate the problem.  Also,

try  the third-party application with the VESA compatible  driver

shipped  with AutoCAD, this will also help to isolate the  source

of the problem.

Also  note  that  any third-party TSR that needs  to  access  the

display  may  not  work  properly when  using  AutoCAD  with  any

advanced  ADI  display  driver, especially if  the  TSR  switches

graphics  modes.  Most TSRs will not support  the  same  graphics

platforms  as TurboDLDClassic and therefore, will not be able  to

accommodate mode switching back and forth.

Command summary

---------------

This  section  alphabetically lists the basic commands  found  in

TurboDLDClassic. Commands have been listed alphabetically.

DLDBIGPIC        calls up the Big Picture - use digitizer  clicks

to define an area and zoom into it.

DLDBPCACHE   enables   or   disables  the   Drawing   Cache   for

TurboDLDClassic advanced features.

DLDBPDIM         allows  resizing and repositioning  of  the  Big

Picture.

DLDBPFREEZE  toggles the Big Picture Zoom Mode between Float  and

Fixed modes.

DLDBPHILIGHT  selects the highlight mode used to display the  Big

Picture.

DLDBPREFRSH enables and Disables the Big Picture automatic update

mode.

DLDBPSTATIC toggles the Big Picture to stay on the screen at  all

times.

DLDCOLOR invokes  CustomColorsTM,   Panacea's color configuration

program.

DLDCOMPACT  forces  a  manual clean-up  of  Display  List  memory

returning any unused memory back to the AutoCAD memory pool.

DLDDCACHE toggles the TurboDLDClassic drawing cache on and off.

DLDDLIST toggles the display list function of TurboDLDClassic  on

and off.

DLDECHO  toggles internal TurboDLDClassic command echoing at  the

AutoCAD command line.

DLDHELP provides a list of TurboDLDClassic commands with one-line

description of each, within AutoCAD. It's recommended  that you

flip to the text screen to view the output.

DLDREFRESH refreshes the Big Picture to reflect the most  current

changes to a drawing.

DLDSTAT  displays  a  listing  of  the  current   TurboDLDClassic

parameters.  A flip to the graphics screen is  recommended  for

this command.

DLDUSAGE         gives a to-the-byte breakdown of how  memory  is

being used for display list processing.

DLDVER  displays the TurboDLDClassic version, serial  number  and

registered user's name at the AutoCAD command line.

DLDVISREGEN  toggles between the Fast and Visible Regen modes  of

TurboDLDClassic.