Clmode

------

The Clmode utility allows the user to define the type of  monitor

attached  and set the video modes supported by the  Cirrus  Logic

VGA.

Using Clmode's menu-driven interface

------------------------------------

At the DOS prompt type:

CLMODE[Enter]

The  main  popup  window consists of a number  of  buttons.  Each

button  represents  a different option or  menu.  The  underlined

letter  of  a button name specifies the hot key  combination  for

that item. For example, press the [Alt] and the underlined letter

keys  simultaneously or just the underlined letter key to  select

an option. Note if you intend to use a mouse with Clmode, then  a

mouse  driver  should  first be installed prior  to  running  the

Clmode utility.

Choosing the attached monitor type

Selecting the proper monitor type will allow the Cirrus Logic VGA

to display the highest quality output that it is capable of  with

the  attached  monitor. The monitor type  determines  what  video

modes  will be available to your system. It will  also  determine

the  vertical refresh rates available. Generally, the higher  the

refresh,  the better the screen will look. A description  of  the

available  monitor types is given in table 2 at the end  of  this

section.

Select  the  Monitor Type button. The Monitor Type  setup  window

will  be displayed. The current monitor type will be  highlighted

initially. Use the cursor keys or the mouse to choose the type of

monitor attached. Selecting the Advanced monitor type will enable

you  to  customize the monitor timings for the  monitor  you  are

using.

To customize the monitor timings, use the mouse or the tab key to

select the gray Set Advanced button. It will only be available if

the  Advanced monitor type is selected. This will bring up a  new

dialog  box with drop down menus for each  available  resolution.

Select the desired refresh rate for each resolution. Consult  the

manual for your monitor to discover the correct rate. When all of

the settings are correct, select the OK button. If there are  any

problems or you change your mind, select the Cancel button.

Before  saving the new monitor type, you may want to  select  the

Verify button to view each of the new video timings that you have

selected  to verify that they are compatible with  your  monitor.

After  each test screen is displayed, press [Enter] or  the  left

mouse  button to see the next video mode. If there is a  problem,

press  [Esc]  or the right mouse button will return  you  to  the

Monitor  Type  Setup  dialog box. Try a slower  setting  for  the

resolution  that  did  not work and try  verifying  your  choices

again.

Once  you  are sure that everything is correct,  press  the  Save

button  to set the new monitor type. Press the Cancel  button  to

discard  any  changes. Press the Help button to  get  a  detailed

explanation on the monitor types and the VGA refresh rates.

On  some  systems the monitor type will be  remembered  from  one

session to the next. To test this, select a monitor type and exit

the Clmode program. Turn the computer off for ten seconds.  After

rebooting the computer, run Clmode and verify whether or not  the

monitor  that you selected is still enabled. If it is, it  should

not ever need to be set again in normal use. If the monitor  type

was  not  kept then you should select the option to  have  Clmode

save the monitor type in you AUTOEXEC.BAT file. You will be given

this choice after selecting the Save button.

Available video modes

---------------------

Select  the  Video  Mode button. The Video  Mode  Preview  window

displays  all the modes supported according to the  monitor  type

attached  and  the amount of video memory present. This  list  of

video  modes  will tell you which are available in  your  current

configuration  for use with extended resolution drivers.  To  see

what different video modes look like on your monitor, select  the

Preview  button.  After  each test  screen  is  displayed,  press

[Enter]  or the left mouse button to see the next video mode.  If

you  don't  want to see any more video modes press [Esc]  or  the

right  mouse  button will return you to the Video  Modes  Preview

window.

Retrieving the current VGA controller status

--------------------------------------------

The  information in the main window displays the  VGA  controller

type,  the  BIOS version number and the amount  of  video  memory

present.

Getting help

------------

Clmode provides Help for the following items: Monitor Type, Video

Mode Preview, Mouse, Keyboard and About Clmode. The Monitor  Type

help  window explains the different capabilities of each  monitor

type. The Video Mode help window defines the information given in

the  Video  Mode  window. The Keyboard  and  Mouse  help  windows

explain how to use the keyboard and the mouse to make selections.

The About window displays the Cirrus Logic copyright message  and

the Clmode version number.

Exiting the Clmode

------------------

To   exit  Clmode  at  any  time,  press  [Alt]  and  [F4]   keys

simultaneously,  or  click the left mouse button  on  the  system

button of the main window (i.e. The top left corner button of the

window which is shown as a dot), or select the Exit button.  When

the  Clmode utility exits, the current video mode, monitor  type,

and VGA refresh rate will be displayed.

Using Clmode's command line options

-----------------------------------

When  command  line options for CLMODE.EXE are given at  the  DOS

prompt,  the menu-driven windows will not be displayed.  Instead,

monitor type, video mode and refresh rate will be set at the  DOS

prompt. The command line options for CLMODE.EXE are:

CLMODE [[modenum][+*-]] [m[montype] | t6=x t8=x t1=x t2=x]

[modenum]  mode number

[+*-]    + selects 400 lines (default)

* selects 350 lines

- selects 200 lines

[montype]    monitor type

t6=x(Hz)     640x480   @ (0 = 60, 1 = 72)

t8=x(Hz)     800x600   @ (0 = 56, 1 = 60, 2 = 72)

t1=x(Hz)     1024x768  @ (0 = 87i, 1 = 60, 2 = 70, 3 = 72)

t2=x(Hz)     1280x1024 @ (0 = 87i, 1 = Not available)

For  example, to select mode 3 for a Super VGA  monitor  (montype

2), type the following command at the DOS prompt:

CLMODE 3+ m2[Enter]

To select custom monitor timings with 640x480 at 60Hz and 800x600

at 72Hz refresh type:

CLMODE t6=0 t8=2[Enter]

Typing an invalid option will display the command line help text.

Typing  [S]  as a command line option will  display  the  current

Clmode settings.

On the next page is a table of common monitor types.

Monitor  Examples           Horizontal      Vertical      Display

Type                        Frequency       Frequency     Resolutions

(kHz)           (Hz)

0     IBM 8512,8513,8503  31.5            60             640x 480

1     IBM 8514,  8515     31.5            60             640x 480

35.5            43.5 (I)      1024x 768

2     NEC 2A              31.5            60             640x 480

35.2            56             800x 600

3     NEC II              31.5            60             640x 480

35.2            56             800x 600

35.5            43.5 (I)      1024x 768

4     NEC 3D              31.5            60             640x 480

37.8            60             800x 600

37.8            43.5(I)       1024x 768

5     Sony CPD-1304,      31.5            60             640x 480

NEC 3FGx,           48.0            72             800x 600

Nanao 9065S, 9070U  48.0            60            1024x 768

48.0            43.5(I)       1280x1024

6     NEC 4D, 4FG,        31.5            60             640x 480

Nanao T240i         48.0            72             800x 600

56.0            70            1024x 768

48.0            43.5 (I)      1280x1024

7     NEC 5D, 5FG, 6FG,   31.5            60             640x 480

Nanao T550i, T560i, 48.0            72             800x 600

T660i, F550i, F750i 58.3            72            1024x 768

48.0            43.5 (I)      1280x1024

����������������������������������������������������������������Ŀ

�Valid �Monitor Type �Horizontal � Vertical       �  Display     �

�Value �             �Frequency  � Frequency      � Resolutions  �

�      �             � (Khz)     �   (Hz)         �              �

�      �             �           �                �              �

����������������������������������������������������������������Ĵ

�  0   �IBM 8512     �   31.5    � 60 or 70       �  640x480     �

�      �IBM 8513     �           �                �              �

�      �IBM 8503     �           �                �              �

����������������������������������������������������������������Ĵ

�  1   �IBM 8514     �   31.5    � 60 or 70       �  640x480     �

�      �IBM 8515     �   35.5    �87- interlaced  � 1024x768     �

����������������������������������������������������������������Ĵ

�  2   �NEC 2A       �   31.5    � 60 or 70       �  640x480     �

�      �             �   35.1    �    56          �  800x600     �

�      �             �           �                �              �

����������������������������������������������������������������Ĵ

�  3   �NEC II       �   31.5    � 60 or 70       �  640x480     �

�      �             �   35.1    �    56          �  800x600     �

�      �             �   35.5    �87- interlaced  �  1024x768    �

����������������������������������������������������������������Ĵ

�  4   �NEC 3D       �   31.5    � 60 or 70       �  640x480     �

�      �             �   35.5    �87- interlaced  �  1024x768    �

�      �             �   37.8    �                �  800x600     �

����������������������������������������������������������������Ĵ

�  5   �SONY CPD-1304�   31.5    � 60 or 70       �  640x480     �

�      � NEC 3PGx    �   48.0    �    72          �  800x600     �

�      �Nanao 9065S  �   48.0    �    60          � 1024x768     �

�      �Nanao 9070u  �   48.0    �87- interlaced  � 1280x1024    �

����������������������������������������������������������������Ĵ

�  6   �NEC 4D       �   31.5    � 60 or 70       �  640x480     �

�      �NEC 4FG      �   48.0    �    72          �  800x600     �

�      �Nanao T240i  �   56.5    �    70          � 1024x768     �

�      �             �   48.0    �87- interlaced  � 1280x1024    �

����������������������������������������������������������������Ĵ

�  7   �NEC 5D       �   31.5    � 60 or 70       �  640x480     �

�      �NEC 5PG/6FG  �   48.0    �    72          �  800x600     �

�      �Nanao T560i  �   56.5    �    70          � 1024x768     �

�      �Nanao T660i  �   48.0    �87- interlaced  � 1280x1024    �

�      �Nanao F550i  �           �                �              �

�      �Nanao F750i  �           �                �              �

�      �Nanao 9400i  �           �                �              �

�      �Nanao 9080i  �           �                �              �

������������������������������������������������������������������

����������������������������Ŀ

�Min.  �  Display    �       �

�Video � Resolutions � Color �

�Memory�             �       �

�      �             �       �

����������������������������Ĵ

�  256K�  640x480    �    16 �

�  256K�             �   256 �

�    1M�             �   32K �

�    1M�             �   64K �

�    1M�             � 16.7M �

����������������������������Ĵ

�  256K�  800x600    �    16 �

�  512K�             �   256 �

�    1M�             �   32K �

�    1M�             �   64K �

����������������������������Ĵ

�  512K� 1024x768    �    16 �

�    1M�             �   256 �

�    2M�             �   32K �

�    2M�             �   64K �

����������������������������Ĵ

�    1M�1280x1024    �    16 �

�    2M�             �   256 �

������������������������������