CL-GD542X Display Drivers and Utilities

User Guide

Software revision: 1.30

Notice:

Cirrus  Logic,  Inc. believes the information contained  in  this

document  is accurate and reliable. However, Cirrus  Logic,  Inc.

reserves the right to make corrections, improvements, or  changes

to this manual at any time and without notice. No  responsibility

is   assumed  by  Cirrus  Logic,  Inc.  for  its  use;  nor   for

infringements  of patents or other rights of third parties.  This

document  implies no license under patents or  copyrights,  other

than those explicitly stated.

This  manual is copyrighted. All rights reserved.  This  document

may  not, in whole or part, be copied;  photocopied,  reproduced,

translated,  or  reduced  to any  electronic  medium  or  machine

readable  form  without prior consent, in  writing,  from  Cirrus

Logic, Inc.

Copyright 1992 - 1993, Cirrus Logic, Inc.

All rights reserved.

Trademarks

Windows,  MS-DOS,  and  MS  Word  are  trademarks  of   Microsoft

Corporation. AutoCAD is a registered trademark, and AutoShade and

AutoSketch are trademarks of Autodesk, Inc. Display List  Driver-

VGA,  TurboDLDClassic,  CustomColors, are trademarks  of  Panacea

Inc.  GEM is a trademark of Digital Research, Inc. Lotus,  1-2-3,

and Symphony are trademarks of Lotus Development Corporation. PC-

DOS,  OS/2  and  Presentation  Manager  are  trademarks  of   IBM

Corporation.   Ventura   Publisher  is  a  trademark   of   Xerox

Corporation.   WordPerfect   is  a   trademark   of   WordPerfect

Corporation.  WordStar  is a trademark of  WordStar  Corporation.

RenderMan is a registered trademark of Pixar. All other brand and

product  names are trademarks or registered trademarks  of  their

respective companies.

==============

Introduction

==============

This  manual provides operating instructions for  user  utilities

and  installation instructions for the display  drivers  supplied

with your Cirrus Logic VGA.

The Cirrus Logic VGA graphics system is 100% VGA compatible.  The

utilities and drivers supplied with your Cirrus Logic VGA provide

support for additional features of your Cirrus Logic VGA, such as

greater speed, higher resolution, and more available colors.

Before you begin

----------------

The following instructions assume that the user is familiar  with

DOS  and certain DOS commands. Please review the  associated  DOS

commands before performing the installation.

Notational conventions

----------------------

Throughout this manual, the term 'DOS' refers to both MS-DOS  and

PC-DOS, except when noting features that are unique to one or the

other.

Easy installation

-----------------

The  installation  utility is provided to facilitate  the  smooth

installation  of  the display drivers and utility  software.  The

installation program is menu-driven and allows you to select  and

install only those display drivers for software and  applications

currently in use.

*** Important ***

-----------------

It  is important to note that some display drivers need  to  have

the associated vendor's application program already installed  on

the system prior to loading the Cirrus Logic VGA display drivers.

In  other  cases,  the loading of the display driver  may  be  an

integral  part  of  the vendor's  product  installation  process.

Please  review  the  driver product section  below  for  specific

instructions prior to running the installation program.

The installation utility is located on the diskette by UNPACK. To

install  the  desired display drivers and utilities,  insert  the

diskette into the A: drive, type

A:[Enter]

UNPACK[Enter]

Follow  the  instructions  on the screen to  install  the  listed

display  drivers.  Selected  drivers are  simply  copied  to  the

specified disk and directory. Applications may require additional

installation as described in the Display drivers section of  this

manual.

Cirrus Logic BBS

----------------

If  there  are any newer versions of the software  provided  with

your VGA adapter, they will be available on the Cirrus Logic BBS.

It is a WILDCAT! BBS. The phone number is (510) 226-2365. It  can

handle modems running up to 9600 baud. The modem should be set to

NO  parity,  8  data bits, 1 stop bit. Before  you  are  able  to

download  software,  you  must join  the  conference(s)  for  the

product(s) that you are interested in.

Installation

------------

Installing the Cirrus Logic VGA adapter

To  install  Cirrus Logic VGA, you may need some or  all  of  the

following tools:

medium size flat blade screwdriver,

medium sized phillips head screwdriver,

a 3/16 inch nutdriver or wrench.

Preparation

-----------

The  Cirrus Logic VGA adapter may or may not be shipped with  the

zero-wait-state  jumper  installed. If  so,  the  zero-wait-state

jumper  will be set to not assert the zero-wait-state  line.  You

may achieve better performance in some systems by installing this

jumper.  If  any problems are  encountered  with  zero-wait-state

enabled, remove the jumper to disable this feature.

PC, XT and compatible users

---------------------------

The Cirrus Logic VGA card requires a 16-bit expansion slot.  This

means  that  it  is not compatible with PC, XT and  other  8  bit

compatible systems.

IBM AT and AT compatible system users

-------------------------------------

The  IBM  AT, IBM XT-286 and compatible computers use  a  program

typically  called  Setup to tell the computer what  equipment  is

installed  in your system. The Setup program is usually  part  of

the  package of programs provided with your AT or  AT  compatible

computer.  The 'Guide To Operations' (or equivalent manual)  will

explain  the use of the Setup program. After the VGA  adapter  is

installed,  Setup will need to be run to configure  the  computer

for proper operation with the Cirrus Logic VGA card.

Open the computer system

------------------------

Before opening the system, be sure that the power to the computer

system  unit and all devices connected to it is  off.  Disconnect

the cables from the back of the system in order to give more room

to   work.   Note  how  all  cables  are   connected   prior   to

disconnection.

In a typical IBM AT compatible computer all cover mounting screws

are located on the back of the computer. Remove these screws.

Carefully  remove  the cover. On most computers  the  cover  will

slide forward or backward, or tilt off.

Choose an expansion slot for the Cirrus Logic VGA adapter card

Your VGA adapter must use a 16-bit expansion slot. Do not  insert

it into an 8-bit slot.

Installing the VGA adapter in your computer

Check that the power is turned off.

If  necessary, remove a system expansion slot cover  by  removing

its retaining screw and lifting it out. Save the screw.

Hold the Cirrus Logic VGA adapter by its top corners and slide it

into  the system unit. Make sure that it is correctly  seated  in

the expansion slot.

Secure  the VGA card with the screw that you removed in the  step

above.

Are  the  computer system's switches and jumpers  correctly  set?

Check  the  user's manual for your computer  for  any  additional

configuration instructions.

Replace and secure the system cover.

Plug A VGA compatible or multi-frequency display into the 15  pin

'D'  shaped video connector at the back of the Cirrus  Logic  VGA

adapter.

*** Important ***

-----------------

Important  note: If you are using a multi-frequency monitor,  you

may need a special 15 pin PS/2 compatible adapter cable, and  you

will  need  to  make sure that your  multi-frequency  monitor  is

configured properly for a PS/2 compatible analog VGA signal. Some

multi-frequency  displays  have  a  switch  to  select  'TTL'  or

'analog'  operation;  set  this switch for  'analog'.  Check  the

manual that came with your monitor for additional information  on

configuring it.

IBM AT and AT compatible users

------------------------------

After the VGA card is installed in the AT compatible system,  use

the  Setup program supplied with the system to tell the  computer

about the equipment installed. When using AT and compatible Setup

programs  with the Cirrus Logic VGA adapter, please follow  these

steps.

Run Setup to configure the computer. Sometimes this utility comes

on  a disk, sometimes it is built in to your system's  ROM  BIOS.

When you arrive at the part of the program where video support is

specified, the program may ask you if the monitor you are looking

at will be the primary monitor, answer yes. If you are  presented

with a list of video options, select the option for VGA .

If  no  option  for VGA is listed, select support  for  EGA.  The

Cirrus  Logic VGA adapter will look to many systems like  an  EGA

card.  If you don't see an option for EGA, try the option for  NO

MONITOR,  RESERVED or SPECIAL. These will usually work.  If  not,

see  your  equipment  dealer.  At the  conclusion  of  the  Setup

program,  the computer will restart as if you had just turned  on

the power.

Operation

---------

If  you  have  followed the  installation  instructions  in  this

manual,  you are now ready to use your Cirrus Logic VGA  equipped

computer  system.  Most software that is  compatible  with  IBM's

Personal  System/2,  VGA or EGA will run  automatically  on  your

system using the VGA Card. Just turn on your computer system  and

install  your  application package for PS/2 model 50,  60  or  80

video, VGA, or EGA as instructed by the software manufacturer.

=================

FCC information

=================

Note: This equipment has been tested and found to comply with the

limits  for a Class B digital device, pursuant to Part 15 of  the

FCC  Rules.  These  limits are  designed  to  provide  reasonable

protection   against  harmful  interference  in   a   residential

installation. This equipment generates uses and can radiate radio

frequency  energy  and, if not installed and used  in  accordance

with  the instructions, may cause harmful interference  to  radio

communications. However, there is no guarantee that  interference

will  not occur in a particular installation. If  this  equipment

does cause harmful interference to radio or television reception,

which can be determined by turning the equipment off and on,  the

user  is encouraged to try to correct the interference by one  or

more of the following measures:

Reorient the receiving antenna.

Increase the separation between the equipment and receiver.

Connect the equipment into an outlet on a circuit different  from

that to which the receiver is connected.

Consult  the  dealer or an experienced  radio/TV  technician  for

help.

This  device  complies with Part 15 of FCC  Rules.  Operation  is

subject to the following two conditions: (1) This device may  not

cause  harmful interference and, (2) this device must accept  any

interference  received,  including interference  that  may  cause

undesired operation.

Notice  to  user: Changes or modifications to  this  product  not

approved  by the party responsible for FCC compliance could  void

your authority to operate this equipment.

In  order  for  an  installation  of  this  product  to  maintain

compliance with the limits for a Class B device, shielded  cables

must  be used for the connection of any devices external to  this

product.

-----------

DOC Files

-----------

<file_name>.DOC under <DOC> subdirectory.

MANUAL ---> CL-GD542x user's manual.

CLMODE ---> CL-GD542x utility user's manual.

SETRES

ACAD -----> Panacea Autodesk Device driver Note.

ACAD386 --> AutoDesk - ADI 4.2 & TurboDLDClassic user's manual.

OS2 ------> OS/2 2.0 PM display driver user's manual.

FW23 -----> Framework II/III display driver user's manual.

GEM,VP ---> GEM/3 & Ventura Publisher display driver user's manual.

ORCAD ----> OrCAD display driver user's manual.

PCAD -----> Personal CAD (PCAD) display driver user's manual.

VERSACAD--> VERSACAD 386,VERSACAD 2D display driver user's manual.

WORD -----> Word display driver user's manual.

LOTUS ----> Lotus 123,Symphony,Report writer dispaly driver user's manual.

WP5X -----> WordPerfect 5.x driver user's manual.

WS -------> WordStar driver user's manual.