OrCAD

-----------------------------------------------------------------

The  OrCAD version 4.x driver supports a resolution  of  1024x768

with 16 colors. Beginning with version 4.04 of OrCAD,  800x600-16

is  directly  supported using the VESA standard driver  which  is

included  with the program. An 800x600 driver for OrCAD can  also

be built using a tool supplied with OrCAD called GENDRIVE.EXE, if

the  version you are running does not include the  VESA  standard

driver.   Cirrus  Logic VGA systems with one  megabyte  of  video

memory can use the 1024x768-16 driver.

Installing OrCAD drivers

-----------------------------------------------------------------

The 1024x768 OrCAD display driver called WD1K.DRV is installed by

the  ORCAD installation program.  It can be selected  by  marking

the  label 1024x768 16 Western Digital VGA from the menu of  dis-

play drivers presented when installing OrCAD.

The VESA standard 800x600 driver included with OrCAD is copied by

the installation program. Simply mark the driver labelled 800x600

16 VESA Super VGA in the menu of display drivers when  installing

the OrCAD program.

Using the OrCAD GENDRIVE utility

--------------------------------

GENDRIVE.EXE is a utility supplied by OrCAD that has the  ability

to create an 800x600 driver. The utility is located on the  OrCAD

driver diskette that is shipped with OrCAD. GENDRIVE is automati-

cally copied to the OrCAD directory which contains display  driv-

ers, typically ORCADESP\DRV.

The driver may be created as follows:

1. Make the OrCAD driver directory you current working  directory

by typing:

CD \ORCADESP\DRV[Enter]

2. Run the GENDRIVE utility by typing

GENDRIVE[Enter].

3. Press S in response to the following message:

Enter a Display Card Number or S(Special), M(More) or Q(Quit):

4. Type 800[Enter] to create a 800x600 driver in response to  the

following message:

Number of Screen Columns (640, 800):

5. Type 600[Enter] to create a 800x600 driver in response to  the

following message:

Number of Screen Rows (350, 480, 600):

6.  Type 6A[Enter] to create a 800x600 driver in response to  the

following message:

AX Value for Initializing INT 10H (hex):

7. Type 0[Enter] in response to the following message:

BX Value for Initializing INT 10H (hex):

8. Type OrCAD 800x600 generic driver[Enter] to create an  800x600

driver in response to the following message:

Identification displayed during driver initialization:

Driver Name:

9. Press Q to abandon redefinition of EGA/VGA Palette Definition.

10.  Type ORCAD800.DRV[Enter] in response to the  following  mes-

sage:

Please Enter Filename for updated driver:

The OrCAD display driver has now been created.

Configuring OrCAD

-----------------

The  first time OrCAD is used with a display driver, it  must  be

configured  for the computer's hardware. OrCAD may be  configured

for a display driver as follows:

1. Run OrCAD by typing ORCAD[Enter].

2. Select CONFIGURE ESP from the main menu.

3. Select the driver from the menu of display drivers  presented,

if  it is present. Otherwise place the cursor in the  field  con-

taining  the  name of the current display driver and  delete  the

name displayed there. Type:

VESA800.DRV[Enter] for the VESA standard 800x600 driver, or

WD1K.DRV[Enter] for the Western Digital VGA 1024x768 driver,

or ORCAD800.DRV[Enter] to select the driver created with GENDRIVE

as described above.

4. Return to the Main Menu and exit OrCAD.

5. Restart OrCAD to use the driver selected.