Personal CAD (PCAD)

Installing PCAD drivers

-----------------------------------------------------------------

Before beginning a Personal CAD session, the Personal CAD  driver

must be copied to the Personal CAD directory.

Run  the  Cirrus Logic installation  program  (INSTALL.EXE),  and

choose  the option for the Personal CAD drivers and  specify  the

drive  and  directory  where you want them  copied  to  (such  as

C:\PCAD).

The  first  time Personal CAD is used with a display  driver,  it

must be configured for the computer's hardware. The Personal  CAD

display drivers are loadable device drivers which must be identi-

fied  to  the Personal CAD loader. The Personal CAD  system  file

PCADDRV.SYS  is used for this purpose. This file must  reside  in

the root directory of the disk drive which PCAD runs on. The file

contains  information used by Personal CAD when it begins  opera-

tion. An example of the file follows:

SYSTEM     \PCAD\DRV\SIBMPC.DRV

INPUT      \PCAD\DRV\IHPMOUSE.DRV

DISPLAY    \PCAD\DRV\DIBMVGA.DRV

The PCADDRV.SYS file must be modified in order for the loader  to

use  a new display driver. The file can be modified using a  text

editor  or  word  processor. Only the line DISPLAY  needs  to  be

changed. Using a text editor or a word processor in  non-document

mode, make the modification. An example of the modified file  for

a 800x600 16 color mode appears below:

SYSTEM     \PCAD\DRV\SIBMPC.DRV

INPUT      \PCAD\DRV\IHPMOUSE.DRV

DISPLAY    \PCAD\DRV\D800.DRV

For 1024x768 16 color support modify the line DISPLAY to read:

DISPLAY    \PCAD\DRV\D1024.DRV

When the PCADDRV.SYS file has been changed, Personal CAD will use

the new driver the next time the program is run.