SetRES

------

SetRES  is a utility that runs under Windows 3.x. It  allows  the

user  to change the screen resolution, number of  screen  colors,

and select either large or normal fonts and system resources.

After  new  options  have  been selected,  the  user  can  either

immediately  restart  Windows, in which case the  new  resolution

will take effect immediately, or continue working in the  current

resolution, in which case the new resolution will take effect the

next  time Windows is started. Note that under Windows 3.0  large

and normal font selection is not allowed.

This  application  assumes  that the Windows  drivers  have  been

correctly  configured  using Windows Setup.  For  information  on

installing  and configuring Windows display drivers using  setup,

please  refer  to the Microsoft Windows section in  this  manual.

From  Windows Setup select the Multi-Resolution entry. This  will

correctly  configure  the driver for Windows  386  enhanced  mode

operation, and copy the necessary font files to your hard disk.

Installation

Start  Windows. In the Program Manager, choose File Run.  Specify

the  diskette  drive  where the  Windows  Drivers  and  Utilities

Diskette  is inserted, and run the install program. For  instance

if the floppy is in drive A:, type A:\INSTALL[Enter]

When prompted, specify to copy the files to the Windows directory.

SetRES may now be run by selecting the SetRES icon.

Use   the   SetRES  utility  to  select  supported   colors   and

resolutions.

If  your are unsure what resolutions and colors your  video  card

can  support, refer to the Section of this manual on  Clmode  for

information on how to determine supported resolutions.

For  help  on the SetRES utility, select Help  from  the  program

menu.

Display drivers

---------------

The  Cirrus  Logic video controller is 100% VGA  compatible.  The

display drivers described in this manual are supplied to  improve

the resolution for each supported software application package.

It  can  support  improved  text  resolution,  providing  greater

readability  when using the supplied drivers in  text-based  word

processing   programs.   It   also   supports   higher   graphics

resolutions,  providing  greater detail when using  the  supplied

drivers in graphics-based programs.

In  each application section, there will be a brief  introduction

describing  the  application  and  the  revision  level  of   the

application supported by the supplied display drivers.

The installation instructions for each display driver will follow

the introduction section. Follow the instructions carefully to be

sure that each display driver is correctly installed. All of  the

installation  instructions assume that the Cirrus  Logic  Drivers

Diskette  is  located  in  drive A:. If drive  B:  is  used,  the

instructions should changed appropriately.

The  installation  utility (INSTALL.EXE) should be used  to  copy

display   drivers   directly  to  the   appropriate   application

directories  where  they  may be configured  by  the  application

software.

Before you begin

----------------

It is assumed that the user is familiar with DOS and certain  DOS

commands.  Please  review the installation instructions  and  the

associated   DOS   commands   before   attempting   the    actual

installation.

Not  all  video  modes will be available on all  systems.  If  an

extended  mode driver is installed for a video mode that  is  not

available,  the application program will not  function  properly.

There are a number of things that determine the list of available

video modes. Some of these include the current monitor type,  the

amount  of  installed  memory,  and  the  revision  of  the   VGA

controller.  To  determine  which  modes  are  available   before

beginning  the  driver installation, it is recommended  that  the

user  run  the Clmode program and examine the list  of  available

video modes.