VERSACAD

Installing VERSACAD 386 or VERSACAD 2D display drivers

-----------------------------------------------------------------

1.  If you have not already done so, install VERSACAD  386   onto

your  computer  system.  Follow the  instructions  supplied  with

VERSACAD.

2.  Run the installation program (UNPACK.EXE), and choose the op-

tion for the VERSACAD 386 drivers and specify the drive and dire-

ctory  where you want them copied to (such as C:\VCAD).

3.  Run the AVGAP.EXE file from the directory where  the  drivers

were  installed.   This  loads a protected-mode  TSR  driver  for

VERSACAD 386.  This should come up with the driver message.

4. Modify the ENVIRO.CFG VERSACAD 386 configuration file using  a

text  editor (as in EDIT.EXE for MS DOS 5.0 or EDLIN for  MS  DOS

4.0 and below) adding exactly the following statements at the end

of the screen parameters section:

SCREEN: avga800 avgap.exe n y n n n n 15 - - - - -

.TITLE: Cirrus Logic GD540x, GD542x VGA, 800x600, single

.NOTE: 15 colors

.PARAMS: 6B 1 799 599 88 0

.MONITOR: ms

..TITLE: Multisync

..PARAMS: 0.93

--- For VERSACAD 2D as following ---

SCREEN: avga800 avga2d.com n y n n n n 15 - - - - -

.TITLE: Cirrus Logic GD540x, GD542x VGA, 800x600, single

.NOTE: 15 colors

.PARAMS: 7B 1 799 599 88 0

.MONITOR: ms

..TITLE: Multisync

..PARAMS: 0.93

---

The  Install  program will place a file named  CIRRUS.CFG

in the VERSACAD directory that contains these statements.

5. Run the ENVIRO.EXE setup program from your VERSACAD directory.-

6. Once in the Menu, select the [S] Screen Configuration and  the

space  bar [space] or back space [bs] until the  message  "Cirrus

Logic GD540x, GD542x VGA, 800x600" is displayed.

7. Select [Enter] to accept this option and again select  [Enter]

several more times until the following is displayed at the bottom

of the screen:

[N] Next Menu [X] Exit [esc] Aborts all change.

8. Select [X] Exit.  This now saves the screen display configura-

tion.

9. Run VERSACAD and it will be using the 800x600 16 color mode.