CL-GD5424/6/8

VESA Local Bus

Display Card

User's Manual

1. Introduction

Thank you for your purchase of our CL5426/8 VESA standard VGA card. Before

you proceed with installing the card into your computer, make sure your

package has the following:

1. One user's manual.

2. 2 driver diskettes.

3. One CL5426/8 VESA CGA card.

1.1 Features on the CL5426 VGA card:

* Fully IBM VGA compatiable

* Support VESA standard software by BIOS built in functions

* Support VESA local bus system

* 64X64 hardware cursor

* Support 72Hz nertical scan refresh rate

* VESA standard feature connection

* Clock chip and a true color (24 bit, 16 million colors) RAMDAC are built

in the chip

* Support 256KX4, 512KX8, 256KX16 DRAM in 1MB or 2MB memory

* Support VGA, Super VGA, and Enhanced VGA resolution in 16, 256, 64K and

16.8 Million Simultaneously colors which is shown as follows:

Resolution Colors

1. 1MB DRAM on the card : 640X480#16,256,64K,16.8M colors

800X600#16,256,64K colors

1024X768#16,256 colors

1280X1024 16 colors

2. 2MB DRAM on the card : 640X480 16,256,64K, 16.8M colors

800X600 16,256, 64K colors

1024X768 16, 256, 64K colors

1280X1024 16,256 colors

Support Software drivers

Display Drivers disk #1

* ACAD drivers                  * GEM/3

* Microsoft Word                * Word Perfect 5.X

* Word Star                     * Lotus 123 V3.X

* Lotus 123 V2.X, Symphony, and Report Writer

Display Driver disk #2

* Windows 3.1 drviers

1.2 The CL5426/8 Adapter Card

#EMBED MSDraw   \* mergeformat###

DRAM Configuration

1MB DRAM on board: U7-14(256X4) or U24,25(256X16) or U51,52(512X8)

2MB DRAM on board: U7-14 nad U24,25

2. Installation

2.1 Easy Installation

The installation utility is located on the diskette labeled Display

Drivers disk #1.  To install the desired display drivers and utilities,

insert the diskette into floppy driver (for example A driver), type

A: (Enter)

INSTALL (Enter)

Follow the instructions on the screen to install the listed display

drivers at, any time you may press (Esc) to abort the installation

process and go back to DOS.  Selected drivers are simply copied to

the specified disk and directory.  Applications may require additional

installation as described in the READ.ME of the display driver diskettes.

2.2 SetRES

It is located in the Display Drivers disk #2.  SetRES is an utility that

runs under Windows 3.1.  It allows the user to change the screen resolution,

number of screen colors, nad select either large or normal fonts and system

resources.

P/N: TMNCL 542XVL-000

Notice

The information conttained in this document is accurate and reliable.

However, Cirrus Logic, Inc. reserves the right to make amendment without

further notice.

All rights reserved.  This document may not, in whole or part, be copied;

photocopied. reproduced, translated, or reduced to any electronic medium

or machine readable form without prior consent, in writing, form Cirrus

Logic, Inc.

Copyright 1992, Cirrus Logic, Inc.  All rights reserved.

Trademarks

Windows, MS-DOS, and MS Word are trademarks of Microsoft Corporation.

PS/2, VGA, EGA are trademarks of IBM Corporation.  All other brand and

product names are trademarks of their respective companies.