ExpertColor Multimedia - Best S3 VGA From ASIA

END USER INFORMATION

- Drivers & Utilities

- Hardware

- Manufacturer

Web-Site : www.dataexpert.com.tw

- Frequently Asked Questions

- Where do I find updated drivers?

- I have some questions about my S3 video card...

- Where can I find a VESA driver?

- I changed to high color or true color and now the computer reports that I am low on memory...

- How do I install the Windows 95 driver?

- How do I install Windows 3.1x drivers?

- I installed Win32s and now 32-bit applications such as Freecell do not work with the S3 driver but work with the Microsoft  supplied VGA driver.

- Where can I get an S3 graphics server for XFree86?

- How do I set my refresh rate for OS/2 Warp 3.0?

- Why do the S3 Windows drivers support  more color depths than the S3 OS/2 drivers on the same hardware?

- Windows 3.x is reporting General Protection Faults while running Netscape. Help!

- I need drivers that support DirectX or DirectDraw.

- Where can I find Windows NT 4.0 display drivers for my ViRGE based system or graphics card?

- Where can I find OS/2 display drivers for my ViRGE based system or graphics card?

Where do I find updated drivers?

Your best option is to request driver updates directly from  ExpertColor Multimedia manufacturer. By calling their BBS number  (886-2-6981743) you will be able to download driver updates that are compatible with your graphics controller.  You can also download Drivers from the Web-site :

www.dataexpert.com.tw

S3 also maintains an end-user BBS and FTP site. These are not intended to replace the support provided by your board or system manufacturer. Please note that these drivers may not be compatible with your particular board because of BIOS and /or board customizations your manufacturer may have implemented which are not supported by our generic drivers. Secondly, be advised that these generic drivers are not supported by any warranty and require a license to use the software. You may also want to refer to the HW Manufacturers Documentation or Computer Vendor Listing found on  S3's BBS and FTP site for additional referrals and support from ExpertColor Multimedia  manufacturers.

S3 Incorporated End-User BBS: (408) 654-5676 Santa Clara, California

Web-site : www.s3.com

- I have some questions about my S3 video card...

S3 does not manufacture graphics or video cards. S3 Incorporated

supplies GUI accelerator chips, a starting point BIOS and drivers to

ExpertColor Multimedia manufacturer. We then modify the BIOS and

drivers to match the feature set we desire.

As a result of this customization, S3 can provide only limited end user

support. S3 cannot answer questions about memory upgrades,

daughter card options, hardware settings, or any custom software that

ExpertColor Multimedia manufacturer  may have provided.

If you have questions about your graphics adapter, the best option is

to contact the supplier. If the supplier is unable to answer

your questions, their tech support engineer should contact us directly.

If you are unable to reach the manufacturer, you can obtain generic

S3 drivers from their FTP site www.S3.com

- Where can I find a VESA driver ?

S3 does not provide a "VESA Driver", or more precisely a VESA

BIOS extensions TSR*. All S3-generated BIOS's already contain

VESA 1.20 functions, so there should be no need for additional

extensions. For VESA 2.0 functionality, there is a shareware

Universal VESA BIOS Extension package (\others\misc) which is

located on the CD-ROM "Others\Misc" directory. Also, Dietmar Meschede                                         distributes a freeware VESA 2.0 package for S3 chips.

However, S3 does not guarantee that any of these packages will work

with S3 chips.

* Terminate and Stay Resident - A program that remains in memory

at all times, so that it can be instantly activated.

- I changed to high color or true color and now the computer reports

that I am low on memory...

In Microsoft Windows operating systems versions 3.1 and 3.11, after

you change to a high-color display (32K or 64K colors) or true color

display (16.7 million colors), the following error message may be

displayed the following error message may be displayed:

"Extremely Low on Memory: Not enough memory to convert all the

program icons"

The error happens because Program Manager does not have enough

memory while trying to convert the icons to the new color depth.

When Program Manager runs out of memory, it stops converting

icons. The icons not yet converted appear as black boxes.

You can convert the black icons by doing the following:

1. Select the program item with a black icon.

2. From the File menu, select Properties, or press .

3. Click on the OK button.

If you have unusually large groups, you may have to split the group

into two or more separate groups to adhere to the 64K limit for a

Program Manager group.

- How do I install the Windows 95 driver?

The Windows setup program installs and modifies all of the necessary

files. Follow these steps to install the Windows 95 driver.

1. If you do not have factory supplied diskettes and have downloaded a

compressed version of the driver, decompress the file into a temporary

or permanent directory. Note the directory where you decompressed the drivers.

2. Start Windows 95.

3. Click the Start button.

4. Double-click on the Display icon in the Setting->Control Panel

folder. You can also click the right mouse button anywhere on

the desktop and select the Properties option from the pop-out menu.

5. Click the Settings tab.

6. Click the Change Display Type... button.

7. Under Adapter Type click the Change button.

8. Click the Have Disk button.

9. Insert the display driver diskette or set the path to the directory

where you decompressed the files (Step 1) and click the OK button.

10. Select S3.INF and click the OK button.

11. Select the appropriate S3 driver from the list and click the OK button.

If a message stating that one or more driver files is older than the files on the

system, respond YES to overwrite the files.

12. Click the Close button.

13. Select the appropriate resolution using the Desktop Area and Color Palette

options.

14. Click the Apply button. A System Settings Change box appears.

15. Click Yes to restart your PC.

The Windows 95 S3 driver installation is now complete.

- How do I install the Windows 3.1x drivers.

The DOS version of the Windows Setup program installs and

modifies all of the necessary files. Follow these steps to install the S3

Windows 3.1, Windows 3.11, and Windows for Workgroups driver:

1. If you do not have factory supplied diskettes and have

downloaded a compressed version of the driver, decompress

the file into a temporary or permanent directory. Note the

directory where you decompressed the drivers.

2. If you are working in Windows, exit to DOS.

3. From the DOS prompt, change to your Windows directory;

for example, C:\WINDOWS

4. From the Windows directory, type Setup and press <Enter>.

This will start the DOS based Windows Setup program.

5. Use the arrow keys to highlight "Other (Requires Disk

Provided by Hardware Manufacturer)" and press <Enter>.

6. Insert the display driver installation diskette or direct the path

to the directory where the decompressed the display drivers

(Step 1).

7. Select the desired screen resolution and color depth and press

<Enter>.

8. When the choice appears to either press <Enter> to update or

<Esc> to replace, BE SURE TO PRESS "<ESC> TO REPLACE" !!!

9. After pressing <Esc> to replace, Setup may ask for the path or

diskette again. When the file copying is complete, the Setup

program will close.

10. Restart windows. Your driver installation is now complete.

- I installed Win32s and now 32 bit applications such as Freecell do

not work with the S3 driver but work with the Microsoft supplied

VGA driver...

Older S3 drivers had a compatibility problem with Win32s. All the

new drivers are compatible with this package. Make sure you install

the newest driver for your video card or S3 chip.

- Where can I get an S3 graphics server for XFree86?

New versions of XFree86 come with an S3 server which is

compatible with most S3 chips.

- How do I set my refresh rate for  OS/2 Warp 3.0?

The following steps apply to using S3's generic drivers and may differ

depending on the board manufacturer:

1.Make sure OS/2 is currently configured to run properly with

the default VGA driver.

2.Install the S3 OS/2 driver according to the instructions given by

your OEM.

3.Boot up the system into DOS, either through Boot Manger or

Dual Boot.

4.Enable a mouse driver in DOS.

5.To set refresh rate for the Presentation Manager:

For S3 86C801, 86C805 and 86C928, run the refresh rate utility:

REFRESH.EXE.

For S3 Vision and Trio products, run the refresh rate utility:

S3REFRESH.EXE.

Caution! Be sure the refresh rate set for a particular mode does

not exceed what your monitor can achieve. If you are not sure,

stop now before proceeding. Setting the refresh rate too high

may damage your monitor permanently.

6. To set refresh rate for Win-OS/2 session:

* Put either REFRESH.EXE or S3REFRSH.EXE statement in the autoexec.bat

for OS/2.

* For instance, if your monitor is capable of running

640x480@72hz, 800x600@60hz and 1024x768@44hz, the command would be

as follows. Note the x is the letter of the drive.

x:\refresh.exe 640 4

x:\refresh.exe 800 3

x:\refresh.exe 1024 1

7.For dual boot system, simply change the directory to /OS/2.

For systems with boot manager installed, log on to the drive

where OS/2 resides and change directory to /OS/2.

8.Type SVGA ON DOS. The screen will flash a few times.

9.If there is a svgadata.pmi file, rename it to svgadata.old.

Reboot the system into OS/2. The refresh rate should than be set up

accordingly.

If you are having any problems please contact your supplier.

- Why do the S3 Windows drivers support more color depths than the

S3 OS/2 drivers on the same hardware?

There are two reasons why a color depth/resolution combination

might be supported in Windows and other operating systems, but

unsupported in OS/2 with the S3 driver.

1.The Windows and OS/2 drivers both need some off-screen

memory for caching fonts, etc. However, the OS/2 driver

needs some off-screen memory for both the Presentation

Manager and Win-OS/2. This additional display memory

overhead prevents some color/resolution combinations from

being displayed unless you have more video memory.

2.The S3 OS/2 driver does not currently support 24-bits per

pixel for 16.7 million color (true-color) modes. For 16.7M

colors, the OS/2 drivers use 32-bits per pixel only. In

true-color mode, each pixel only requires 24 bits, so 8 bits are

unused for each pixel when using 32-bit color modes. This

prevents 640x480 at 16.7M colors from being displayed on

1MB video hardware and 1280x1024 at 16.7M colors from

being displayed with 4MB. In contrast, the S3 Windows

drivers support both 32-bit and 24-bit color modes.

- Windows 3.x is reporting General Protection Faults while running

Netscape. Help!

If you are using Netscape 2.0 or higher, this problem can be resolved by installing the

latest version of our Windows 3.1 drivers. For the best results, we recommend that

you first contact your supplier for an updated display driver; however, in the event

that you cannot obtain an updated driver from your hardware manufacturer, you can

try our generic drivers. S3 generic or reference display drivers can be downloaded

from the Drivers and Utilities section of S3  web site. For more information regarding

the installation of our generic display drivers, please see the "How do I

install Windows 3.1 drivers" section of our FAQ.

- I need drivers that support DirectX or DirectDraw.

All of our current Windows 95 drivers support the use of DirectX or

DirectDraw. Most programs that require DirectX include a DirectX

setup with the application. In addition, some video card and system

manufactures distribute DirectX with their products.

In order to successfully run an application that requires DirectX, make

sure that you are using a current version of our Windows 95 drivers.

For the best results, we recommend that you first check our Web-site

for an updated display driver; however, in the event that you cannot

obtain an updated driver from your hardware manufacture, you can try

S3 generic drivers. S3 generic or reference display drivers can be

downloaded from the Drivers and Utilities section of our web site.

After you have successfully installed a current Windows 95 driver,

simply follow the installation instructions that are provided with your

DirectX based software.

Run "DirectX.exe" from the CD-ROM. This is release from Microsoft to

support latest DirectSound, DirectPlay, Direct3D, DirectInput and AutoPlay.

Most Microsoft Windows 95 Games will run with the DirectX v3.0 on this release.

- Where can I find Windows NT 4.0 display drivers for my ViRGE

based system or graphics card?

We are working closely with Microsoft in regard to Windows NT

4.0, and we plan to have ViRGE drivers included with Windows NT

4.0 when it's released. In addition, ExpertColor Multimedia manufactures may

release their own versions of these drivers after the official release of Windows

NT 4.0 or check at our WEB-site www.dataexpert.com.tw for any new updates .

- Where can I find OS/2 display drivers for my ViRGE based system

or graphics  card ?

For the best compatibility, we recommend that you contact your supplier and ask for

their latest OS/2 driver release. In the event that you cannot obtain OS/2 drivers from

your supplier, generic OS/2 drivers for ViRGE are available in the Drivers and

Utilities area of  S3 Web site or from S3 BBS (1-408-654-5676).