Protected mode MicroStstion PC driver

-------------------------------------

for

S3 86C911 based cards

INSTALLATION:

-------------

1. Copy the following files to the "\USTATION\DRIVERS" directory:

"MGL911.EXP"	The executable driver file.

"MGL911.MA"	The MDL application that loads the driver.

"MGL911.MGL"	Used by the configuration program to locate

the driver files.

2. Go to the "\USTATION" directory.

3. Run the "USCONFIG" program to select the new device driver.

3a. Select "Display Adapters" entry.

3b. Select "VENDOR SUPPLIED DRIVER" option.

3c. Select "PMGL 86C911 DRIVER" option.

3d. Select second adapter options as needed.

3e. Select "Exit and Save".

3f. Reboot system for installation to take affect.

CONFIGURATION:

--------------

The first time this driver is used, it will ask the user which video

mode to use for MicroStation.  It will present the user with a menu of

all available resolution modes.  One must be selected, and then a

configuration filed will be created that the driver will use for

subsequent MicroStation sessions.

To permanently reconfigure the driver, erase the file

"\USTATION\DRIVERS\MGL911.CFG" and run MicroStation again.

To temporarily change the resolution mode used, you may set the

environment variable "MGL911" to one of the following numbers:

0	640x480x256

1	800x600x16

2	800x600x256

3	1024x768x16

4	1024x768x256

5	1280x960x16

6	640x480x256

7	1280x1024x16

If the driver finds a legal value in the environment, it will use it

to override any variable found in the configuration file.

FEATURES:

---------

This driver will work with a Hercules-compatible MDA card as the

second graphics device.

This can be configured as either the "Primary" (right screen) or

"Secondary" (left side) graphics device.

SWAP screen capabilities are supported if a 256 color mode is

configured.

If the initialization code is unable to determine which mode to use, it

will always default to the 640 x 480 x 256 color mode.

The driver will exit if it is not able to find an S3-based graphics

card.

The graphics screen contents are now saved when shelling out to DOS.