MICROSTATION PC DRIVER DISK

---------------------------

for the

S3 86C911

---------

Features:

* Very Fast-

Written in 100% assembly language

* Very Small-

Uses only 10K, and may be loadhi'd

* Does not require HDILOAD-

Leaves more memory available for MGDS

Works with all versions, including 4.0

* Convenient-

Will remove itself from memory when needed

Hardware Requirements:

* 286 or better CPU (8088 based machines will not work)

* S3 86C911-based board

Resolution:

* 1024 x 768

* 800 x 600

* 640 x 480

* 16 colors

* 256 colors or swap screen (16 colors) with sufficient video memory

I N S T A L L A T I O N

-----------------------

Manual installation:

1. Go to disk drive that has MicroStation.

2. Place the driver install disk in drive A:.

3. Change to the "\USTATION\DRIVERS" directory.

4. Copy "A:\MGLS3.EXE" to the current directory.

5. Copy "A:\MGLS3.MGL" to the current directory.

6. Change to the "\USTATION" directory.

7. Copy "A:\REMOVE.EXE" to the current directory.

8. Run "USCONFIG", selecting the "ATC Graphics S3 86C911" driver.

D R I V E R   U S E

-------------------

The driver is a TSR (Terminate and Stay Resident) driver that must be

invoked before MicroStation PC will run. The driver will accept

certain command line options that will change the behaviour of the

driver.

If your 86C911 has at least 1 megabyte of VRAM, this driver

will support either 2 pages ("swap screen" capability) or will support

256 colors. To select the 256 color option, use the "/noswap" option

When installing the driver in memory. This is usually done by the

"USTATION.BAT" or "MCE.BAT" files, and they should be modified with

an editor.

To remove a previous copy of the driver, use the 'remove.exe' program.

Examples:

'mgls3'	Will install the driver in 16 color mode. If enough

VRAM is installed, swap screen will be enabled.

'mgls3 /noswap'	Will install the driver, telling it to display

the maximum number of colors possible.

The driver will default to the highest resolution possible. If this is

not what you desire, the resolution may be overridden by setting an

environment variable as follows:

"set S31_911=<mode>"

Where <mode> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 960, 16 colors

6	--> reserved <--

7	1280 x 1024, 16 colors

8	--> reserved <--

Please note that not all resolution modes may be available.

T R O U B L E S H O O T I N G

-----------------------------

Driver won't install in memory.

Couldn't find a 86C911

Accept no substitutes- install a 86C911 board.

P E R F O R M A N C E   T I P S

-------------------------------

High loading

To conserve DOS memory, this driver may be loaded into high memory.

This can be accomplished by using a 386 memory manager such as QEMM or

386max.

QEMM Example:

"c:\qemm\loadhi c:\ustation\drivers\mgls3"

386MAX Example:

"c:\386max\386load envfree prog=c:\ustation\drivers\mgls3"

Unloading high loaded drivers

QEMM Example:

"c:\qemm\loadhi /nope c:\ustation\remove"

386MAX Example:

"c:\386max\386load quiet prog=c:\ustation\remove"

MicroStation 4.0 will run faster (redraws 50% faster) if you have a

VCPI memory manager (QEMM or 386MAX) present and correctly configured.

To optimize your VCPI memory for use with 4.0, all memory outside the

DOS 640K range should be configured as EMS memory.