VERSACAD DRIVER DISK

--------------------

Features:

* Very Fast-

Written in 100% assembly language

* Very Small-

Uses only 11K

* Versatile-

Works with all monitor types.

* High performance features-

Full internal display list using EMS

* Does not require HDILOAD-

Leaves more memory available for macros

Works with all applications including VCAD/386

Hardware Requirements:

* 286 or better CPU (8088 based machines will not work)

* 80287 or 80387

* EMS 4.0 memory (512K minimum)

* S3 96C911 (911) based graphics card

Resolution:

* 1024 x 768 or 800 x 600 on MultiSync displays

* 640 x 480 on all others

* 16 or 256 colors

I N S T A L L A T I O N

-----------------------

Automatic installation:

1. Go to disk drive that has your VersaCAD product(s) installed.

Don't worry, it will figure out which ones are present.

2. Place the driver install disk in drive A:.

3. Type "A:\VERSACAD\INSTALL" <cr>.

4. Run ENVIRO program when prompted, selecting this driver.

Manual installation (VersaCAD 5.3):

1. Go to disk drive and directory that has VersaCAD (usually \VCAD53)

2. Place the driver install disk in drive A:.

3. Copy "A:\VERSACAD\VC2_911.COM" to the current directory.

4. Copy "A:\VERSACAD\VCAD53.CFG" to "ENVIRO.CFG".

5. Run ENVIRO program, selecting the "S3 96C911" driver.

Manual installation (VersaCAD Design (5.4, 6.0 & 7.0):

1. Go to disk drive and directory that has VersaCAD (usually \VCAD54)

2. Place the driver install disk in drive A:.

3. Copy "A:\VERSACAD\VC2_911.COM" to the current directory.

4. Copy "A:\VERSACAD\VCAD54.CFG" to the current directory.

5. Run ENVIRO program, selecting the "S3 96C911"

screen driver. You will be able to choose between "Single

Screen" and "Dual Screen" configurations, as well as various

resolutions.

Manual installation (VersaCAD 386):

1. Go to disk drive and directory that has VersaCAD (usually \VCAD386)

2. Place the driver install disk in drive A:.

3. Copy "A:\VERSACAD\VC3_911.COM" to the current directory.

4. Copy "A:\VERSACAD\VCAD386.CFG" to the current directory.

5. Run ENVIRO program, selecting the "S3 96C911"

screen driver. You will be able to choose between "Single

Screen" and "Dual Screen" configurations, along with the various

resolutions available.

D R I V E R   U S E

-------------------

LOADING THE DRIVER

This driver is a TSR type (Terminate and Stay Resident) program. You

only need to install it once in memory each time your machine is

powered up. For this reason, it is recommended that a call to this

driver be placed in your AUTOEXEC.BAT file. If you are using VersaCAD

5.3 or 5.4 (VersaCAD Design), the driver name will be "VC2_911.COM".

If you are using VersaCAD 386, the driver name will be "VC3_911.COM".

HIGH LOADING

For information on loading this driver into the high memory provided

by certain 386 memory managers, please see the PEFORMANCE TIPS section

of this manual.

EMS MEMORY USAGE

As a default, this driver will reserve 512K of EMS memory when

invoked. This will be used to store the display lists needed for fast

redraws. While this should be enough memory to handle most drawings,

you can tell the driver to reserve more EMS space when the driver is

loaded. This is done by setting the environment variable "VCAD_EMS="

to the number of EMS pages you wist to have reserved. Each EMS page is

16K in size.

Example:

"SET VCAD_EMS=64"      will cause the driver to reserve 1024K (64 pages *

16K) of memory when the driver is loaded.

"SET VCAD_EMS=0"       will tell the driver to reserve no pages

of EMS for use. This special case tells the

driver not to reserve any EMS. In this case,

display list processing will be disabled.

SCREEN RESOLUTION

This driver will use the resolution selected by the ENVIRO program. If

that mode is unavailable, VersaCAD will not run.

T R O U B L E S H O O T I N G

-----------------------------

The driver won't start up, and gives an message about driver failure.

Couldn't find a 96C911 board, or BIOS failure detected.

Either a 96C911 board is not installed or the BIOS

needs to be upgraded.   Please see your dealer for a

compatible card or replacement BIOS.

Jagged vectors drawn all over screen.

Early 386 chip.

Early versions of the 386 chip had a problem with 32

bit math operations. Check the 386 for a sigma

(sideways 'M'), and if one does not exist, this is

your source of grief. Possible work-arounds include

installing a later revision 386 chip, or turning off

the display list option in the ENVIRO program.

EMS not fully compatible.

This driver tests the EMS handler upon loading,

checking for correct EMS version (4.0) and looks for

the most common bugs in the EMS handler. Some bugs are

not detectable at this time and slip through. Two

work-arounds exist: update your EMS handler, or turn

off the "built in" display list option

P E R F O R M A N C E   T I P S

-------------------------------

Display List

The main feature of this driver is the built-in display list

capability. In order to take advantage of this, VersaCAD must be told

that the driver can support display listing, and that you wish to use

it. To do this, go into the screen options in ENVIRO, and select the

Built-In option under Display List.

To make the most of the display list, use the display list options

menu inside VersaCAD by pressing Ctrl-F5.

To minimize the number of "regens", (where VersaCAD sends down each

invidual line) you should zoom the display all the way in, then back

out to where you were in the drawing. After doing this, VCAD will

redraw everything from the local display list, (sKetches) and zooming

and panning (with the arrow kayes) will be much faster. This is done

by using the "Full" function from the displist list (LIST) menu, then

using the "Out" function from the same menu.

To speed up the panning operations, (scrolling throught the drawing can

be done with the arrow keys) you should use a keyboard utility to

increase the "Typematic" rate of our keyboard. This will increase the

rate at which the scrolling occurs.

High loading

To conserve DOS memory, this driver may be loaded into high memory.

This can be accomplished by using a 386 memory manager such as QEMM or

386max. Using high memory for the driver works very well with VersaCAD

286 versions. Since the VCAD 386 program uses a minimum of DOS memory,

this will not give any additional performance to VCAD386.

QEMM Example:

"c:\qemm\loadhi c:\vcad54\vc2_911"

386MAX Example:

"c:\386max\386load envfree prog=c:\vcad54\vc2_911.com"

Unloading high loaded drivers

QEMM Example:

"c:\qemm\loadhi /nope c:\vcad54\remove"

386MAX Example:

"c:\386max\386load quiet prog=c:\vcad54\remove"

VersaCAD/386 will run faster (redraws 50% faster) if you have a VCPI

memory manager (QEMM or 386MAX) present and correctly configured.  To

optimize your VCPI memory for use with VCAD/386, all memory outside

the DOS 640K range should be configured as EMS memory. This means that

all the memory that would normally be allocated to "DOS high memory"

should be reserved for use as EMS.