~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~              README OF SCANKIT UTILITY                           ~

~                                                                  ~

~              AUG 24, 1994                                        ~

~                                                                  ~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This file contains the following topics:

*  Running Scankit Utility

*  Editing the scanner device line

*  Special Notice

Before proceeding, it is important that you have completed the

following steps:

Step 1:  Install the scanner interface card in your computer.

Step 2:  Connect the scanner to the interface card.

Step 3:  Run the Install program of Scankit Utility.

If you have not yet done any of the three steps, please refer to the

scanner Installation Guide for more details.

RUNNING SCANKIT UTILITY

~~~~~~~~~~~~~~~~~~~~~~~

1.  Start Windows by typing "win."

2.  Double-click the ScanKit Utility Icon(which appears after installation) to

run the Scankit Utility or,

2.1 Use the Program Manager's File RUN command. Type:

"scanw"

Also include the DOS pathname.

(Example: C:\SCANW\SCANW  )

2.2 Click OK.

EDITING THE SCANNER DEVICE LINE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1.INTRODUCTION

~~~~~~~~~~~~~~

For the computer to recognize that you have installed a scanner,

the CONFIG.SYS file should contain a device line that specifies

the scanner driver and the configuration (i.e. - DMA, IRQ, and

I/O address settings) used by the scanner interface card.

To edit the CONFIG.SYS file, you will need to use a text editor.

If you have DOS 5.0, you can use the EDIT.EXE command.  If your

system is running a DOS version lower than 5.0, you will use the

EDLIN.COM command.  If you are in Windows, you may also use the

Windows Notepad.  Please refer to your DOS (or Windows) manual

for steps on how to use these text editors.

Read carefully the following sections to obtain more information

on how to set the parameters in the scanner device line.

2.  THE SCANNER DEVICE LINE

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The scanner device line should include five (5) items. These are:

*  Device driver

*  DMA setting

*  IRQ setting

*  Scanner type setting

*  I/O address setting

In the CONFIG.SYS file, the correct syntax for the scanner

device line is as follows:

DEVICE=[drive:\][path-directory\]driver-name Dn In An Sn

where:

[drive:\][path-directory\]    is the drive and subdirectory

where the scanner driver is located

driver-name                   is the name of the scanner driver

(e.g., MSCANC.SYS)

Dn                            is the DMA setting of the interface card

(e.g., D3 for DMA3)

In                            is the IRQ setting of the interface card

(e.g., I5 for IRQ5)

An                            is the I/O address of the interface card

(e.g., A2AB for I/O address 2AB)

Sn                            is the scanner type

(e.g., S1 for AmiScan Color scanner)

3.  SETTING THE SCANNER DEVICE LINE PARAMETERS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

When you edit the scanner device line, there are certain

precautions that you should make note of, especially when

choosing the DMA, IRQ, and I/O address settings for the

scanner interface card.

Avoid hardware and software conflicts that can adversely

affect the performance of the scanner by selecting only

available and unused DMA, IRQ, and I/O address settings.

See the Troubleshooting Tips in your scanner user's guide

for more information.

USE THE FOLLOWING OPTIONS FOR SETTING THE SCANNER DEVICE

DRIVER LINE PARAMETERS:

3.1  The scanner device driver  [driver-name]

The scanner device driver is found in the installed

subdirectory. For the scanner to work, it is important that

the scanner driver is loaded properly when you boot your

computer. In the CONFIG.SYS file, make sure the driver in

the scanner device line has the correct path.

Driver-Name:    Hand-held scanner:

------------    ------------------

MSCANC.SYS        AmiScan Color

3.2  The DMA channel  [Dn]

When you assign the DMA setting for the scanner interface card,

make sure it is not used by any other device connected to

your computer. The available DMA channels for the

interface card are:

D1        for  DMA 1

D3        for  DMA 3 (recommended setting)

The devices that might have DMA settings are sound cards,

SCSI devices, anti-virus cards, CD-ROM drives, multimedia

cards, etc. If you have any of these devices in the computer,

please select the DMA channel not being used.

3.3  The IRQ level  [In]

When you assign the IRQ setting for the scanner interface card,

make sure it is not used by any other device connected to

your computer. The available IRQ levels for the interface

card are:

I3        for  IRQ 3

I5        for  IRQ 5  (recommended setting for XT bus)

I10       for  IRQ 10 (recommended setting for AT bus)

The devices that might use IRQ settings are networking cards,

SCSI devices, bus mice, sound cards, etc.  If you have any of

these devices in the computer, please select the IRQ settings

not being used.

3.4  The Scanner type  [Sn]

S1        for  AmiScan Color

3.5  The I/O Address  [An]

The scanner interface card has eight (8) available I/O

addresses and is equipped with a switch for selecting the

address settings.

Inspect the interface card closely. Printed on the circuit

board are the 8 available addresses.  As with the DMA and

IRQ settings, select an I/O address that is not used by any

other device connected to your computer.

It is very important that the I/O address selection in the

scanner device  line should  match  the scanner  interface

card's I/O address setting.

The 8 available I/O addresses are:

*  22B                          *  32B

*  26B                          *  36B

*  2AB (recommended setting)    *  3AB

*  2EB                          *  3EB

The I/O address  3EB will conflict with  COM3  port  and

Western Digital's IDE hard disk controller card. If you are

using the I/O address 3EB, the I/O address on the scanner

interface card (DIP switch) and in the device driver should

be changed to another address (e.g. - 26B)

3.6  Examples of Scanner Device Lines

DEVICE=C:\AMISCAN\MSCANC.SYS D3 I5 S1 A2AB

In the example above, the scanner driver M1904C.SYS is located

in subdirectory AMISCAN of drive C. The parameters used are

DMA 3, IRQ 5, and I/O address 2AB. The scanner model is the

AmiScan Color scanner.

AFTER YOU HAVE EDITED THE SCANNER DEVICE LINE IN THE

CONFIG.SYS FILE, REBOOT YOUR COMPUTER TO MAKE SURE THE

CONFIG.SYS FILE IS READ.

FOR PROBLEMS DUE TO HARDWARE AND/OR SOFTWARE CONFLICTS,

PLEASE SEE APPENDIX C OF THE SCANNER INSTALLATION GUIDE.

SPECIAL NOTICE

~~~~~~~~~~~~~~

1. ABOUT OmniPage Direct

~~~~~~~~~~~~~~~~~~~~~~~~

Please modify SCANMGR.INI in the DEXXA directory as below:

1.1 Set

...

TWAINDS=AmiScan Color

...

1.2 Add

...

[AmiScan Color]

SHOW_UI=1    --------------> Show User Interface

INVERT=1     --------------> Invert image

AUTO_BRI=0   --------------> Text(LineArt) Scan Mode

DPI=1        --------------> Resolution = 300 dpi

DOCTYPE=0    --------------> Document Type = Black & White

ORIENT=4     --------------> Orientation = Landscape(L to R)

1.3 Switch scanner to TEXT scan mode, 300 DPI.

2. ABOUT GO-CR V1.3 & V2.1

~~~~~~~~~~~~~~~~~~~~~~~~~~

Merge function is not supported for this application.