Tablet Initial Utility

WTINIT.EXE allows you to initial tablet by file mode or command

mode as following example.

>WTINIT <filename>

>WTINIT /<command>/<command>/...

<filename>: The file created by utility WTSETUP.EXE

<command> : Refer to commands description

Commands description:

/*: reset tablet to default settings

Sampling mode           : switch stream

Resolution              : 500 lpi

Report Rate             : maximum

Increment value         : 0

Axis update value       : 0

Tablet origin           : lower left

Tablet identifier       : 0

Report format           : 8-bit packed binary, odd parity

Baud rate               : 9600 bps

/t: self test

After the self test, there is a bell sound as pass.

/@: Stream mode

The tablet continuously reports coordinate data in Stream

mode as long as the tablet state is valid.  It is not

necessary to press a cursor or stylus button.  Should the

cursor be out of proximity, the last valid report will be

issued three times.  Using Increment mode in combination with

stream mode prevents redundant report when the cursor or

stylus is stationary.

/A: switch stream mode

The tablet continuously reports coordinate data in Switch

Stream mode as long as the tablet state is valid and a cursor

or stylus is pressed.  If the cursor or stylus is out of

proximity and a button is pressed, the last valid report is

issued continuously.

/B: point mode

the tablet reports one coordinate data point mode as long as

a cursor or stylus button is pressed.  If the cursor or

stylus is out of proximity and a button is pressed, the last

valid report is issued once.

/D: remote request mode

The tablet report one coordinate data each time the host

issues a poll command.  The tablet issues the mode command

'D' once, and then sends a command 'P' for each report.  If

the cursor or stylus is out of proximity and a button is

pressed, the last valid report is issued each time a poll

command is sent.

/P: poll command

See Remote Request command /D

/E: delta mode

In delta mode ,instead of issuing reports of absolute

coordinate, the tablet issues reports as relative coordinates

which are measured relative to the last issued report.

Report can be positive or negative values If the cursor or

stylus is out of proximity, report issue is zero.

/F: absolute mode

In absolude mode, the tablet issues reports as absolute

coordinstes.  It is opposite to delta mode.

/Q: maximum report rate

/R: max/2 report rate

/S: max/8 report rate

/T: max/32 report rate

/d: 100 lpi resolution

/e: 200 lpi resolution

/f: 254 lpi resolution(10 lpm)

/g: 400 lpi resolution

/h: 500 lpi resolution

/i: 508 lpi resolution(20 lpm)

/j: 1000 lpi resolution

/q: 1016 lpi resolution(40 lpm)

/l: 1 lpi resolution

/n: 2 lpi resolution

/p: 4 lpi resolution

/b: upper left Tablet origin

Use the Tablet origin command to define the location of the

tablet's origin (0,0) on upper left.

/c: lower left Tablet origin

Use the Tablet origin command to define the location of the

tablet's origin (0,0) on lower left.

/1: COM1

The command "/1" lets you send the commands that you wish to

send to tablet via COM1.  But you may not specify it, WTINIT

can auto detecte where the tablet is connected.  If you use

the command "/1", put it on the first location in the commands.

/2: COM2

The command "/2" lets you send the commands that you wish to

send to tablet via COM2.  But you may not specify it, WTINIT

can auto detecte where the tablet is connected.  If you use

the command "/2", put it on the first location in the commands.

/M: Mouse Systems Mouse mode

In Mouse Systems Mouse mode, the tablet emulates Mouse

Systems Mouse operations.  After the command is issued,

remote commands are ignored.  Powering down the tablet will

abort the mouse function.  You can use MS-DOS commands to initial

the tablet as mouse.  Example:  Suppose the tablet default

characteristics:

9600 bps, odd parity, 8 data bits, 1 stop bit.

>MODE com1: 96,0,8,1

>ECHO M>com1

/N: Microsoft Mouse mode

In Microsoft Mouse mode, the tablet emulates Microsoft Mouse

operations.  After the command is issued, remote commands

are ignored.  Powering down the tablet aborts the mouse

function.  You can use MS-DOS commands to initial the tablet

as mouse. Example:  Suppose the tablet default

characteristics:

9600 bps, odd parity, 8 data bits, 1 stop bit

>MODE com1: 96,0,8,1

>ECHO N>com1

End of file.