Tablet Setup Utility

WTSETUP.EXE allows you to set tablet operation in desirous mode

after the power on (you may also use WTINIT to set tablet, see

WTINIT.DOC).  You can run WTSETUP by two modes as follows:

>WTSETUP                immediate mode

>WTSETUP <filename>     file creation mode

Immediate Mode:

Run WTSETUP you will find following screen:

������������������������������������������������������������������������������ͻ

�                            Tablet Setup Utility                              �

������������������������������������������������������������������������������ͼ

��� DIAGNOSTIC ����ͻ���������� Current Settings ���������Ŀ��� RESOLUTION ���Ŀ

�y :reset           ��Sampling mode    :                   ��d :100lpi         �

�t :self test       ��Resolution       :                   ��e :200lpi         �

�w :test result     ��Report Rate      :                   ��f :254lpi(10lpmm) �

�x :version check   ��Increment Value  :                   ��g :400lpi         �

�k :echo            ��Axis Update value:                   ��h :500lpi         �

�������������������ͼ�tablet origin    :                   ��i :508lpi         �

��� REPORT MODE ���Ŀ�tablet identifier:                   ��j :1000lpi        �

�@  :stream mode    ��Report Format    :                   ��q :1016lpi        �

�A  :switch stream  ��and Baud Rate    :                   ��l :1lpi           �

�B  :point mode     ������������ Current Status ����������Ĵ�n :2lpi           �

�D  :remote request ��Count :                              ��p :4lpi           �

�E  :delta mode     ��X:        (      ) Y:        (      )���� Miscellaneous Ĵ

�F  :absolute mode  ��switch:( )  ( )  ( )  ( )            ��r :set X,Y scale  �

�I  :increment mode ���������������������������������������ٳu :transmission   �

�G  :axis update    ��������������� Message ��������������Ŀ�b :tablet origin  �

�������������������Ĵ�<ESC> exit the setup program         ��0 :identifier     �

�Q  :maximum        ��Confirm tablet connect with  com1 or ��a :configuration  �

�R  :max/2          ��com2?                                ����� MOUSE MODE ��Ĵ

�S  :max/8          ��Hit '1' or '2'!                      ��M :Mouse System   �

�T  :max/32         ��                                     ��N :Microsoft      �

��������������������������������������������������������������������������������

You can select each command by press the left character on this

row.  When you set completely, press <ESC> to exit.  Following is an

example the tablet set in stream mode, 1/32 report rate, 1000 lpi.

1. >WTSETUP <enter>             run WTSETUP in immediate mode

2. type '1' or '2'              1: COM1         2: COM2

3. type 'y'                     reset tablet

4. type '@'                     set stream mode

5. type 'T'                     set 1/32 report rate

6. type 'j'                     set 1000 lpi resolution

7. type <ESC>                   set complete, exit to DOS

File Creation Mode:

Run WTSETUP <filename> you will find following screen:

������������������������������������������������������������������������������ͻ

�                         (R) Tablet Setup Utility                             �

������������������������������������������������������������������������������ͼ

��� DIAGNOSTIC ����ͻ���������� User's  Settings ���������Ŀ��� RESOLUTION ���Ŀ

�y :reset           ��                                     ��d :100lpi         �

�t :self test       ��                                     ��e :200lpi         �

�w :test result     ��                                     ��f :254lpi(10lpmm) �

�x :version check   ��                                     ��g :400lpi         �

�k :echo            ��                                     ��h :500lpi         �

�������������������ͼ�                                     ��i :508lpi         �

��� REPORT MODE ���Ŀ�                                     ��j :1000lpi        �

�@  :stream mode    ��                                     ��q :1016lpi        �

�A  :switch stream  ��                                     ��l :1lpi           �

�B  :point mode     ��                                     ��n :2lpi           �

�D  :remote request ��                                     ��p :4lpi           �

�E  :delta mode     ��                                     ���� Miscellanious Ĵ

�F  :absolute mode  ��                                     ��r :set X,Y scale  �

�I  :increment mode ���������������������������������������ٳu :transmission   �

�G  :axis update    ��������������� Message ��������������Ŀ�b :tablet origin  �

�������������������Ĵ�<ESC> exit the setup program         ��0 :identifier     �

�Q  :maximum        ��Confirm tablet connect with  com1 or ��a :configuration  �

�R  :max/2          ��com2?                                ����� MOUSE MODE ��Ĵ

�S  :max/8          ��Hit '1' or '2'!                      ��M :Mouse System   �

�T  :max/32         ��                                     ��N :Microsoft      �

��������������������������������������������������������������������������������

The screen is same as immediate mode except "User's Settings"

window.  You can select each command by pressing the left charecter

letter.  When you set completely, press <ESC> to exit.

Following example is to create the file contains COM1, reset, stream

mode, 1/32 report rate, and 1000 lpi five commands.  The steps are

as follows:

1. >WTSETUP MYFILE <enter>      run WTSETUP in file creation mode,

the creation file name is MYFILE

2. type '1' or '2'              1: COM1         2: COM2

3. type 'y'                     reset tablet

4. type '@'                     set stream mode

5. type 'T'                     set 1/32 report rate

6. type 'j'                     set 1000 lpi resolution

7. type <ESC>                   set complete, exit to DOS

Now, you have MYFILE file.  You can send the commands which

contain in MYFILE file by WTINIT. About the details, please refer

to WTINIT.DOC.

Command Description:

y: reset tablet to default settings as follows:

Sampling mode           : switch stream

Resolution              : 500 lpi

Report Rate             : maximum

Increment value         : 0

Axis Update value       : 0

Tablet origin           : lower left

Tablet identifier       : 0

Report Format           : 8-bit packed binary, odd parity

Baud Rate               : 9600 bps

t: self test

After the self test, there is a bell sound as pass.

w: test result

If test is OK test result equals 8Fh.  Execute this command,

the cursor must be on the active area.

x: version check

The checksum is the following six bytes format:  .$HHHH

k: set tablet to echo mode

This mode's character will be transmitted to the tablet and

it will echo back the character to the host. While in this

mode, all remote commands are ignored.  However, you can abort

this command by issuing a reset command "y".

@: stream mode

The tablet continuously reports coordinate data in Stream

mode as long as the tablet state is valid.  It is not

necessary to press a cursor or stylus button.  Should the

cursor be out of proximity, the last valid report will

issue three times.  Using Increment mode in combination with

Stream mode is to prevent redundant report when the cursor or

stylus is stationary.

A: switch stream mode

The tablet continuously reports coordinate data in Switch

Stream mode as long as the tablet state is valid and a cursor

or stylus is pressed.  If the cursor or stylus is out of

proximity and a button is pressed, the last valid report is

issued continuously.

B: point mode

the tablet reports one coordinate data point mode as long as

a cursor or stylus button is pressed.  If the cursor or

stylus is out of proximity and a button is pressed, the last

valid report is issued once.

D: remote request mode

The tablet reports one coordinate data each time the host

issues a poll command.  The tablet issues the mode command

'D' once, and then send a command 'P' for each report.  If

the cursor or stylus is out of proximity and a button is

pressed, the last valid report is issued each time a poll

command is sent.

P: poll command

See remote request command /D

E: delta mode

In delta mode ,instead of issuing reports of absolute coordinate,

the tablet issues reports as relative coordinates which are measured

relative to the last issued report. Report can be positive or negat-

-ive values.  If the cursor or stylus is out of proximity,

report issue is zero.

F: absolute mode

In absolute mode, the tablet issues reports as absolute

coordinates.  It is opposite to delta mode.

I: Increment mode

The tablet reports one coordinate only when the cursor or

stylus has moved a specified distance in X or Y direction.

The distance is defined by the increment value.  After the

Increment mode command and the increment value are sent in

the tablet,the increment value will be the new standard of

measurement.  If the displacement is equal to or greater

then the specified increment value, the tablet reports the

coordinate data of point.  The increment value is in counts

of resolution, and can be any number in the range from 0 to

95(the default is 0).  Add 32, a required offset to the

desired increment, then convert the sum into your prefered

format.  Example as following:

desired  + 32 = increment value = Increment Value

increment       in decimal        in ASCII

20       + 32 = 52              = 4

G: Axis Update mode

Similar to Increment mode, the tablet issues a report only

when a set distance is satisfied in either the X or Y

direction.  Unlik increment mode, the tablet issues the

report only at the grid intersection point.  After axis

update mode command and update value are sent to the tabl-

-et, the tablet uses the first report as a reference point.

From that reference point, when axis has a displacement equal

to the specified update value, a new intersection point will

be obtained.  In the other axis, if the displacement in the

axis is not greater then the update value, the new report of

the axis will remain unchanged.  The update value is in

counts of resolution, and can be any number in the range from

0 to 95(the default is 0).  Add 32, a required offset to the

desired update value, then convert the sum into your prefered

format.  Example as following:

desired  + 32 = update value = update Value

value        in decimal        in ASCII

20       + 32 = 52              = 4

Q: maximum report rate

R: max/2 report rate

S: max/8 report rate

T: max/32 report rate

d: 100 lpi resolution

e: 200 lpi resolution

f: 254 lpi resolution(10 lpm)

g: 400 lpi resolution

h: 500 lpi resolution

i: 508 lpi resolution(20 lpm)

j: 1000 lpi resolution

q: 1016 lpi resolution(40 lpm)

l: 1 lpi resolution

n: 2 lpi resolution

p: 4 lpi resolution

r: Set X, Y scale

Use set X, Y scale to match the tablet resolution to the

resolution of another two-dimensional object.  Set X, Y scale

allows you to define the resolution of each axis

independently.  Resolutions can be from 1 to 1016 lines

per inch.  The command format as following:

1st byte        mode command               : r

2nd byte        X axis resolution low byte : 00 to FFh

3rd byte        X axis resolution high byte: 00 to FFh

4th byte        Y axis resolution low byte : 00 to FFh

5th byte        Y axis resolution high byte: 00 to FFh

The following example describe how to use set X, Y scale:

1.  Determine the resolution for the entire length of the

axis.  If the number is fractional, round it to the next

higher whole number:  X scale:  1200 1200/11.7 = 102.6

---> 103 per inch Y scale:  1500 1500/11.7 = 128.2 --->

129 per inch

2.  Multiply these numbers by the axis lengths,and round the

results to next higher whole number:  103 * 11.7 =

1205.1 ---> 1206 129 * 11.7 = 1509.3 ---> 1510

3.  Convert the axis' resolution value to hexadecimal number:

1206(decimal) = 04B6(hexadecimal) 1510(decimal) =

05E6(hexadecimal)

4.  seperate the hexadecimal number into two-digital parts,

the least significant byte(low byte) and most significant

byte (high byte).  04B6 = 04(high byte) B6(low byte) 05E6

= 05(high byte) E6(low byte)

u: Transmission control

The tablet accepts start transmission and stop.  Transmission

commands to determine whether or not the reports are to be

sent to the host.  On transmission protocols, stop

transmission is equal to XOFF and start transmission

corresponds to XON.

b: Tablet origin

Use the Tablet origin command to define the location of the

tablet's origin (0,0)

0: Tablet identifier

Use tablet identifier command to set a bit in the packed

binary report format to a one or a zero.  This can be helpful

in dual-tablet systems to distinguish between the reports

coming from one tablet versus the other.

a: Report configuration

Use Report configuration command to issue a report to the

host that identifies the Tablet Identifier setting and the

resolution setting of each axis.

M: Mouse Systems Mouse mode

In Mouse Systems Mouse mode, the tablet emulates Mouse

Systems Mouse operations.  After the command is issued,

remote commands are ignored.  Powering down the tablet aborts

the mouse function.  You can use MS-DOS commands to initial

the tablet as mouse.  Example:  Suppose the tablet default

characteristics:

9600 bps, odd parity, 8 data bits, 1 stop bit.

>MODE com1: 96,0,8,1

>ECHO M>com1

N: Microsoft mouse mode

In Microsoft mouse mode, the tablet emulates Microsoft Mouse

operations.  After the command is issued, remote commands

are ignored.  Powering down the tablet aborts the mouse

function.  You can use MS-DOS commands to initial the tablet

as mouse.  Example:  suppose the tablet default

characteristics:

9600 bps, odd parity, 8 data bits, 1 stop bit

>MODE com1: 96,0,8,1

>ECHO N>com1

End of file.