WTTAPI.DOC

This WTTAPI.DOC is a document file about the WTTAPI.  It contains

TAPI introduction; TAPI installation; TAPI command line arguments;

The initial file TAPI.INI and WTTAPI messages.

TAPI introduction

=================

TAPI (Tablet Application Programming Interface) is a TSR (Terminate

and Stay Resident) routine providing intergration interface for

ADI, Mouse, Tablet and Keyboard emulation. WTTAPI V2.0 enables you

to automatically toggle tablet between tablet mode and mouse emulation

mode without reconfiguring the program or re-load the mouse driver.

You can take AutoCAd and Pcshell for example.  You will find TAPIV2.0

enable you to switch ADI driver (AutoCAD) and mouse driver (Pcshell)

more easily and quickly.

TAPI installation

=================

1.  System requirement:

Environments - AT/286/386/486 IBM PC and compatible

Tablet - all KD, WD, LD series tablet

Memory size - 11.8k bytes

2. the steps

(1) Check your computer and tablet well connected.

(2) Run WTTAPI.COM under DOS prompt.  You will see the message:

" TAPI version 2.1

Read initial file C:\EXE\TAPI.INT

XX-XXXX connected on COMX

Driver install successfully.                         "

TAPI return to the ERRORLEVERL CODE 0

If the messages are:

" TAPI version 2.1

Read initial file C:\EXE\TAPI.INT

Digitizer not respond, or unknown model

Abort installation !                                  "

Please recheck your computer and digitizer is well connected.

If the connection is OK and the screen always appears the same

messages:

" Digitizer not respond, or unknown model

Abort installation !                     "

Please contact us or your nearest dealers.

* Note: WTTAPI version 2.1 support serial port from COM1 to COM4.

WTTAPI.COM will automatically search the COM PORT to which

the tablet connect, you needn't to assign COM PORT (only

type "WTTAPI <return>").  However, if you want to use

argument to assign COM PORT, please see the next description

about "Command line arguments".

TAPI Command line argument

==========================

WTTAPI provides some command line arguments.  There are no case

sensitivities in all  arguments, and you can use them by syntex.

WTTAPI [/arg1/arg/2..]

Arguments:

/1    install driver on COM1

This arguments tells your tablet is connected at com2

/2     install driver on COM2

Same as argument/1, but it is at com2

/3     install driver on COM3

Same as argument/1, but it is at com3

/4     install driver on COM4

Same as argument/1, but it is at com4

/?     help

This argument will help you use the the arguments

collectly

/A     absolute mapping mode

This argument tells WTTAPI to work under absolute mode,

and at this moment your tablet cursor will be an absolute

mapping mouse.  Absolute mapping means tablet cursor's

action is fully mapping your screen monitor.  You can use

this argument character in most mouse application like

PCTOOLS or Norton....

/D     enable direction-key simulation

This argument lets you move tablet's cursor like you enter

direction-key from PC's keyboard to help you move screen

cursor or cell speedily on screen.  This argument provides

you two status for your choice:

<Left Ctl>+<Right Ctl> => act direction-key emulation

<Ctrl>+<Shift>         => unact direction-key emulation

/H    help (same as /?)

/I    not to read initial file

This argument tells WTTAPI not to read TAPI.INT (about the

TAPI.INI, please see the description about TAPI.INI)

/R    relative mouse mode

this is a default setting.  If you use the variable

MODE=1 in TAPI.INI, you can use this command line

argument/R to replace it.

/U    uninstall

Release WTTAPI.COM allocated memory

The initial file TAPI.INI

=========================

WTTAPI provides a DOS enviroment variable TAPI=[driver:][path].

According to this variable, the WTTAPI will find out the TAPI.INI,

and read your TAPI.INI predefinition, then prset itself.  If you

don't want WTTAPI to read TAPI.INI, it is not necessary to set the

variable TAPI= in DOS enviroment; however if you want WTTAPI to

read TAPI.INI,  we recommand you to write TAPI=[driver:][path]

statement in AUTOEXEC.BAT.  Now TAPI has six variables which allow

you to use in TAPI.INI.  The six variables are as follows:

COM=  This variable tells WTTAPI on which com port Wintime tablet

is connected.

COM=0  Autodetect the com port which the tablet is connected

COM=1  Means tablet is connected on com1

COM=2  Means tablet is connected on com2

COM=3  Means tablet is connected on com3

COM=4  Means tablet is connected on com4

* Note:  1. the variable setting will be replaced by DOS

command line arguments /1,/2,/3,/4

2. this variable default setting is COM=0

MODE=  This variable makes WTTAPI to work by absoulte or relative mode;

the value 0 means relative mode, 1 means absolute mode.

* Note:  1. the variable setting  will be replaced by DOS command

line arguments /A,/R

2. this variable default setting is MODE=0

Xmin=  This variable will be effective when MODE=1 (absolute mode).

It sets the X coordinate of lower left of tablet's mapping area.

Xmax=  This variable will be effective when MODE=1 (absolute mode).

It sets the X coordinate of upper right of tablet's mapping area.

Ymin=  This variable will be effective when MODE=1 (absolute mode).

It sets the Y coordinate of lower left of tablet's mapping area.

Ymax=  This variable will be effective when MODE=1 (absolute mode).

It sets the Y coordinate of upper right of tablet's mapping area.

WTTAPI messages

===============

When you run WTTAPI, you may meet some messages.  Following are the

messages meanings.

Message:  XX-XXX connected on COMn

It means TAPI detects the tablet model number and the com port

it connected with.

XX-XXX means the model number of tablet

COMn, "n" means com port number

For example:  KD-4000 connected on COM1

Message:  Driver already installed

It means WTTAPI is already loaded and installed into memory .

Message:  Driver install successfully

It means WTTAPI is loaded and installed successfully into memory.

Message:  Driver uninstall successfully

It means WTTAPI is loaded and installed unsuccessfully into

memory.

Message:  Open file error !

It means the WTTAPI can't find the TAPI.INI from the variable

TAPI=driver:\path which you specify in DOS enviroment.

Message:  Read initial file driver:\path\TAPI.INI

It means WTTAPI reads the initial file TAPI.INI.

For example:  Read initial file C:\EXE\TAPI.INI

Message:  Syntex error !

It means WTTAPI checks out the syntex error in command line.

Message:  Unknown command !

It means you use an unknown command in WTTAPI command line.

We welcome your comments for WTTAPI, and thank you for your use !

The END !