Canon BJ-200ex Lotus 123 Release 2.x Printer Driver User notes

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

WYSIWYG and PrintGraph

~~~~~~~~~~~~~~~~~~~~~~

Please note that 123 2.x refers to 123 2.3 and 2.4 only, as WYSIWYG does

not exist in 123 2.2.

This driver uses the Canon Raster Image Command mode for fast printing

of colour raster graphics. The driver automatically switches the printer

from the current emulation mode, to the Canon Raster Image Command mode,

and then back to the original mode once printing has finished.

The same drivers provide support for BOTH PrintGraph AND WYSIWYG, and

configure the printer for different print media.

The print quality/media options are:-

Plain Paper (Special)

Plain Paper (High Quality)

Plain Paper (High Speed)

Transparency (Special)

Transparency (High Quality)

Transparency (High Speed)

NOTE

~~~~

In WYSIWYG if the bottom of your image, or footer, is to close to the

bottom of the page it may fall into a unprintable area and not be printed.

If this happens then please re-print the page using an increased bottom,

and or top margin (the left and right margin in landscape).

123 Text

~~~~~~~~

The 123 text driver CANBJPRO.DRV uses the default BJ-10 (Proprinter) emulation

mode of the BJ-200ex printer. The emulation mode is selected by setting the

printer DIP switches. Details of how to do this can be found in the

BJ-200ex User's Manual.

The text driver does not configure the printer for a particular print media

and print mode. This allows you to use the the print mode switch on the

printer control panel to set the appropriate mode you wish to use for

printing. See your BJ-200ex User's Manual for details.

Your Lotus 123 User's Guide provides advice on when to print using the

123 Print command (/Print), and when to use the WYSIWYG Print command

(:Print).

When printing on A4 paper do not forget to set the printer's page length

DIP switch to 12 inch paper.

Installation

~~~~~~~~~~~~

1.  Place the Canon BJ-200ex Driver disk in drive A:

2.  Change to your Lotus 123 2.x directory using

CD "path" ENTER

Substitute your 123 directory for "path".

For example, if you have 123 installed in the directory C:\123R24, you

would type:

CD \123R24  ENTER

3.  Then run the Lotus program INSTALL.EXE by typing:

INSTALL ENTER

4.  The Introductory Lotus Install screen should now

appear. Press ENTER

5.  Select  "Change selected equipment" then press ENTER

6.  Select  "Add New Drivers to the Library" then press ENTER

7.  You will now be asked to specify the floppy disk drive which contains

the drivers disk. Enter the letter of the drive and press ENTER.

8.  Select  "Modify the Current Driver Set" then press ENTER

9.  Install the graphics driver by selecting "Graphics printer" and pressing

ENTER

10. Select  "Canon" then press ENTER

11. Select  "BJ-200ex" then press ENTER

12. Now install the text driver by selecting "Text printer" and pressing

ENTER.

13. Select  "Canon" then press ENTER

14. Select  "BJ-Series Propr" then press ENTER

15. Select  "Return to Previous Menu" then press ENTER

16. Select  "End Install" then press ENTER

17. You will now be asked if you want to save your changes. Select "Yes"

then press ENTER

18. Now press ENTER to save the changes into the current driver set, or

type the name of a different driver set and press ENTER.

19. Press ENTER to choose a WYSIWYG bitmap font set.

20. Select the font set you want e.g Medium, and press ENTER

21. The font set you selected will now be generated. Once this has been done

select ENTER to exit install. All of the BJ-200ex drivers will now be

installed.

#