Canon BJC-600 Lotus 123 Release 2.x Printer Driver User notes

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

WYSIWYG and PrintGraph

~~~~~~~~~~~~~~~~~~~~~~

Please note that 123 2.x refers to 123 2.3 and 2.4 only, as WYSIWYG does

not exist in 123 2.2.

This driver uses the Canon Raster Image Command mode for fast printing

of colour raster graphics. The driver automatically switches the printer

from the current emulation mode, to the Canon Raster Image Command mode,

and then back to the original mode once printing has finished.

The same drivers provide support for BOTH PrintGraph AND WYSIWYG, and

configure the printer for different print media.

CN2BJC6P.DRV - Plain Paper (High Quality)

CN2BJC6H.DRV - Plain Paper (High Speed)

CN2BJC6T.DRV - Transparency Film

CN2BJC6C.DRV - Coated Paper

CN2BJC6B.DRV - Coated Paper (Enhanced Black)

CN2BJCCM.DRV - BJC-600 driver support file.

All of the drivers configure the printer for printing at 360 dpi, using

the high quality (HQ) print mode, normal print media thickness and normal

ink density, except for the Plain Paper (High Speed) and Coated Paper

(Enhanced Black) drivers.

The Plain Paper driver prints at 180 dpi, using the high speed (HS) print

mode. This is useful for draft printing of WYSIWYG worksheets. The Coated

Paper (Enhanced Black) driver uses a higher black ink density for

emphasized black printing.

NOTE

~~~~

In WYSIWYG if the bottom of your image, or footer, is to close to the

bottom of the page it will fall into a unprintable area and not be printed.

If this happens then please re-print the page using an increased bottom,

and or top margin (the left and right margin in landscape).

123 Text

~~~~~~~~

The 123 text driver CANBJC6.DRV uses the default Epson emulation mode

of the BJC-600 printer. The emulation mode is selected by setting the

printer DIP switches. Details of how to do this can be found in the

BJC-600 User's Manual.

The text driver does not configure the printer for a paticular print media

and print mode. This allows you to use the the print mode switch on the

printer control panel to set the appropriate mode you wish to use for

printing. See your BJC-600 User's Manual for details.

Your Lotus 123 User's Guide provides advice on when to print using the

123 Print command (/Print), and when to use the WYSIWYG Print command

(:Print).

When printing on A4 paper do not forget to set the printer's page length

DIP switch to 12 inch paper.

Installation

~~~~~~~~~~~~

1.  Place the Canon BJC-600 Driver disk in drive A:

2.  Change to your Lotus 123 2.x directory using

CD "path" ENTER

Substitute your 123 directory for "path".

For example, if you have 123 installed in the directory C:\123R24, you

would type:

CD \123R24  ENTER

3.  Then run the Lotus program INSTALL.EXE by typing:

INSTALL ENTER

4.  The Introductory Lotus Install screen should now

appear. Press ENTER

5.  Select  "Change selected equipment" then press ENTER

6.  Select  "Add New Drivers to the Library" then press ENTER

7.  You will now be asked to specify the floppy disk drive which contains

the drivers disk. Enter the letter of the drive and press ENTER.

8.  Select  "Modify the Current Driver Set" then press ENTER

9.  Install the graphics driver by selecting "Graphics printer" and pressing

ENTER

10. Select  "Canon" then press ENTER

11. Select  "BJC-600" then press ENTER

12. Now install the text driver by selecting "Text printer", pressing

ENTER and then repeat steps 10 and 11.

13. Select  "Return to Previous Menu" then press ENTER

14. Select  "End Install" then press ENTER

15. You will now be asked if you want to save your changes. Select "Yes"

then press ENTER

16. Now press ENTER to save the changes into the current driver set, or

type the name of a different driver set and press ENTER.

17. Press ENTER to choose a WYSIWYG bitmap font set.

18. Select the font set you want e.g Medium, and press ENTER

19. The font set you selected will now be generated. Once this has been done

select ENTER to exit install. All of the BJC-600 drivers will now be

installed.